Draft update for NorDig IRD spec v3.0 2017, draft 017

inclusion of HEVC IRD profile

Video/HEVC part

Chapters not changed or relevant to

video has been removed in this draft

NorDig Unified Requirements
for

Integrated Receiver Decoders

for use in

cable, satellite, terrestrial and IP-based networks

Date: dd Month 2017

DRAFTING GUIDELINES / Explanation from the editors related to DRAFT versions:

This NorDig IRD SSU subdraft for v3.0 document is based on the official NorDig Unified IRD v2.6.

- Yellow highlight marking marks changes in text compared to NorDig Unified IRD v2.6

  - New modified text: without strikethrough marks new additional text,

  - Removed text: with strikethrough marks old text proposed to be removed

- Green marked text: highlighting text that under extra scrutiny during this update (not yet agreed).

- Blue marked text: comments or other raw text that will be removed before final version.

- Grey marked text: refers to text that not are relevant to this review/update (SSU).

Guide: To improve version handling and readability, old text from NorDig Unified IRD v2.6 that is proposed to be deleted in future “v3.0” should not be removed from draft version. Use instead strikethrough and yellow highlighted marking. Microsoft Word function “Track Changes”, will be used in addition to highlight changes, BUT from one draft version to another draft sometimes all “Track Changes” are Accepted to easier read changes in updates of proposals during our work.

When drafting a proposal, cross-references should be manually set and same for proposing correction, i.e. yellow mark and manual reference value. NorDig editor will update cross-references when preparing final draft.

Following text is only during drafting and will be removed before inclusion into NorDig IRD specification:

Bugzilla and proposals for future NorDig Unified IRD spec

| Chapter | Status | Incl in this draft | date, description | owner |
| --- | --- | --- | --- | --- |
| Open points from earlier drafting (v2.6 etc) |  |  |  |  |
| 3.4.4.4 Installation mode: Automatic Search, best service and SI chapter |  |  | Nov/Dec 2011, Target Region descriptor Proposed by Petri Hyvärinen, Teracom, during v2.3 update (draft 06), but was missed when sent to Excom (draft 04), with following comment; This section is more logical to this text. Changed to shall requirement because of request from operator. Agreed 2011-09-21 to change should req. Inclusion of following text after 3rd paragraph (but depends on if target region descriptor is included): “If two or several services (same original_network_id, transport_stream_id, but different service_id) request the same service list position signaled in LCN (logical channel number) in LCD, IRD shall use in addition to LCN the TRD, target_region_descriptor, if signaled.” Proposal of also including Target Region descriptor into SI chapter. 2013-09-09, input from Sony; Requirement should be same as on D-Book. 2014-05-13 (during NorDig T meeting): No input proposal yet, if no proposal soon, this point will be postpone to future releases. | NorDig T Petri H Des f RoO |
| 3.4.3 DVB-T2 |  |  | 2014-10-15 (e-mail from Sony, Kari Eriksson), update ref to DVB-T2 spec from v1.2.1 to v1.3.1 and scrambling of L1 post signalling. Kari: In NorDig Unified we are referring to ETSI EN 302 755 v1.2.1 (references point 21). But in section 3.4.3 Modes there’s note 3 “Note 3: DVB-T2 transmissions may simultaneously carry both DVB-T2 Base signal and DVB-T2 Lite signal. DVB-T2 Lite signal contained in FEF part of the DVB-T2 Base signal is according to requirements in EN 302 755 version 1.3.1 or later.” In our opinion is that Note 3 is actually broadcaster issue and therefore should be in rules of operations? Any plan to have EN 302 755 version 1.3.1 mandated for example in NU 2.6? Actually we are getting question about supporting L1 post scrambling. |  |
| ? |  |  | 2015-02-11 Questions from Test subgroup up to NorDig T: Below some unclarified things in nordig u v2.5.1 we identified during the today’s nordig ver test meeting for discussion within nordig/t. Question 1: How to specify OTA for ONid which not follow country code like deployed some cable and satellite networks ? ONId may be allocated per operator or broadcasted as 0 or something else. need for more detailed specification? Answer 1: TBD (Per T, Teracom, investigate) | NorDig T (Per T) |
| 16.2 User preference settings |  |  | 2015-02-11 Questions from Test subgroup up to NorDig T: Below some unclarified things in nordig u v2.5.1 we identified during the today’s nordig ver test meeting for discussion within nordig/t. Question 4: Ch 16.2 User setting country setting states “pre-select” language. what does “pre-select” mean ? does it mean that ird proposes in menu by default majority language of the country ? ”Country setting based on country code [49] for pre-selection of the primary menu, primary audio language, primary subtitle language settings and channel list selection as defined in 12.2.9.3.” Answer 4: (Per T) Same view, default proposes languages. Wording probably needs to be reformulated (same wording since IRD spec v1.0.3 from 2007). Our understanding this intend to refer to this Country value may typically be used by an IRD to also pre-select audio and subtitling languages and maybe also other country related settings. Example if user set “Sweden” as country menu, the IRD pre-select “Swedish” for audio&subt languages to speed up first-time installation process so most user can just confirm language menus during first time installation (similar as many mobile telephones does). | NorDig T (Per T) |
| Chapter | Status | Incl in this draft | date, description | owner |
| New proposals for common templete |  |  |  |  |
| 3.1.1 | new | incl | Per T 2017-02-07, typo: “an IP-based front-end supporting IPTV over managed networks is from here referred to as a terrestrial IPTV NorDig IRD.” | Editor |
| Doc history | New | incl | Des Mac Giolla an Chloig 2017-02-14, typo: “…20th January 2017 on the front page and DD.01.2017 page 11”, proposal to correct DD.01.2017 to 20.01.2017. | Editor |
| Wordings: Hybrid to HbbTV, Basic to Basic IRD | New | incl | Per T/HEVC group 2017-02-14: Propose to change wording when changing from profiles & levels to capabilities (HbbTV, HEVC, PVR etc). - From Hybrid IRD to HbbTV IRD, - From Basic to Basic IRD | Editor |
| Chapter | Status | Incl in this draft | date, description | owner |
| 5.9 Overscan | open | incl | Lars Haglund 2017-03-06: Proposal, edit the following text on page 69 (chapter 5.9): “For services carrying H.264/AVC video, the broadcaster may use the overscan_info_present and overscan_appropriate flags to indicate whether the receiver should apply this typical overscan or should display the complete broadcast video image”. To: “For services carrying H.264/AVC video, the broadcaster may use the overscan_info_present and overscan_appropriate flags to indicate whether the receiver should apply overscan (e.g. by masking with black pixels or by additional cropping plus scaling) or should display the complete broadcast video image after appropriate Frame Cropping (see Chapter 5.8 Frame Cropping).” Proposal, replace the following sentence at two (2) places on page 70 (chapter 5.9.1): “EBU – Recommendation R95, Safe areas for 16:9 television production” [73] With: “EBU R 095 - Safe areas for 16:9 television production” [73] Proposal, replace the following sentence on page 16 (chapter 1.4): [73] EBU, R-95 EBU Recommendation R 95, Recommendation for Safe areas for 16:9 television production With: [73] EBU R 095 EBU R 095 - Safe areas for 16:9 television production, July 2016 (or later). | Lars H (SVT) and David W |
| 5 5.1-5.8 Video chapter | open | Started (not stable) | NorDig video group meeting 2017-03-07, during the webinar meeting a first common walk-through of the text of the video chapter 5.1 to 5.8 and common proposal of what should be updated. - 5.1 adding IRD type NorDig HEVC IRD and basic additional video decoding requirements compared to basic NorDig IRD, reference to DVB. Use same wordings as DVB 154. - 5.2 starting to add HEVC video resolutions - 5.3 adding UHD (3840x2160) full video resolution for HEVC IRDs - 5.4 adding HEVC IRD requirements, later update wording to DVB and MPEG wordings. Table 5.2 added UHD res, later update to DVB wordings (important to refer to DVB’s subset of ITU BT.2100 for HLG and PQ). - 5.5 highlighted second senstense, it needs to be reformulated - 5.6 AVC still pictures, proposal is to remove this subchapter but so far keep it as an empty frame to avoid renumbering the other subchapters. - 5.7 added a first proposal for minimum HEVC bitrate, bitrate needs to be checked. - 5.8 highlighted sentences that need to be updated (AP for Lars Haglund to proposal wordings). | Video group |
| 12.1.4 | open | Started, not stable | Per T 2017-03-13, add HEVC service types. Proposal for prio between services with same LCN value (in basic highest service type value highest prio and inside HEVC highest component type value highest prio) meaning: - first between codecs: MPEG-H/HEVC highest prio, then MPEG-4/AVC and lowest MPEG-2 - then between HEVC: first HFR HDR, then HFR+SDR, HDR+SFR, amd last SDR+SFR. - Proposal: no prio among resolutions (UHD, FHD, HD, SD+). If a service varies over time on some schedule (e.g. over the day or over the week), then service_type (incl comp_type) should be according to maximum case for the service. | Per T |
| 12.3.4 Simulcast Linkage | open | Not started | Per T 2017-03-13, add HEVC service types, NorDig Simulcast replacement service |  |
| 14.2.7 PVR downscale for recording to removable media | open | First proposal | Per T 2017-03-13, propose to update current requirement for downscale from only refer to downscale of HD to all content with higher resolution than SD. Should we allow 960x540p50, in proposal I have incl this. From: “for recording of protected content shall downscale any HD content to SD resolution (maximum 720x576) before storing…” To: “for recording of protected content shall downscale any HD content with higher resolution than SD (i.e. higher than 720x576i25 or 960x540p50) to SD resolution (maximum 720x576 or 960x540) before storing…” |  |
| 14.3.1 PVR Recording gereral | open | First proposal | Per T 2017-03-13, propose to add similar requirements for HEVC content, for example as: “The NorDig HEVC PVR shall in addition as a minimum support recording up to YY Mbps per (Full HD) service and shall as a minimum support recording up to XX Mbps per (UHD) service.” |  |
| 14.3.19 PVR recording simulcast | open | First proposal | Per T 2017-03-13, propose to make text more generic to resolution instead of as current text that has only SD and HD, for example as: “The NorDig PVR should, by factory default, prioritise recording of the event from HD service over the SD service the service with higher service priority according to chapter 12.1.4 (service type) during SD and HD simulcast broadcast with different resolutions of the same programme event. (See SI chapter 12 for more details of the service types for SD and HD services). The NorDig PVR may allow an end user to change the user preference setting regarding or during the time of programming offer the end user to choose which service type has a recording priority during SD and HD simulcast broadcast with different resolutions.“ |  |

| Status |  |
| --- | --- |
| New | New proposal/input/action listed, but not yet presented inside NorDig T |
| Waiting | Proposal been presented inside NorDig T, but no decision whether continue with proposal or close it. Work/activity not yet started. |
| Drafting/open | Proposal been presented inside NorDig T. NorDig T agreed upon continue work and start drafting text proposal (i.e. normally refers to agreed inside NorDig T upon content of the proposal but detailed/final text is still under preparation/drafting). |
| Accept/final | Final text proposal is ready and agreed within NorDig T and to be presented up to NorDig Excom for final approval before publication. |
| Info | Informative point |
| Closed | Proposed point/action closed, no (more) action will be made. No text inserted in any specification/document. |
| Deleted | NorDig T has agreed upon deleting proposed point/action/comment |

END: Above text is only during drafting and will be removed before inclusion into NorDig IRD specification:

1	Introduction	11

1.1	Scope	11

Document History	14

1.2	Terminology	18

1.3	Definitions	18

1.4	References	19

1.5	List of Abbreviations	25

2	General Features of the NorDig IRD	30

2.1	General	30

2.2	Introduction	30

2.3	IRD Hardware and Firmware	31

2.4	System Software and API	34

2.5	General Product Requirement	36

PART A: Hardware and Firmware	38

3	The Frontend of the NorDig IRD	39

3.1	Common Features	Error! Bookmark not defined.

3.2	Satellite Tuner and Demodulator	Error! Bookmark not defined.

3.3	Cable Tuner and Demodulator	Error! Bookmark not defined.

3.4	Terrestrial Tuner and Demodulator	Error! Bookmark not defined.

3.5	IP Based Front-End	Error! Bookmark not defined.

4	MPEG-2 Demultiplexer	40

4.1	General	40

4.2	DVB Descrambler Performance (for IRD with embedded descrambling)	40

4.3	System Clock Recovery	40

5	Video	42

5.1	General requirements	42

5.2	Supported resolutions and frame rates	43

5.3	Up-sampling/Up-converting	45

5.4	Colorimetry	45

5.5	Dynamic changes in the video stream	47

5.6	AVC still picture	Error! Bookmark not defined.

5.7	Minimum video bandwidth	48

5.8	Frame Cropping	48

5.9	Overscan	48

5.10	High Definition Video Output and Display	49

5.11	Down-conversion of High Definition Video for Standard Definition output	50

5.12	Display Format for other Aspect Ratios	50

5.13	Rescaling for HbbTV application	51

6	Audio	52

6.1	General	52

6.2	Audio Decoding	53

6.3	Multichannel Audio	56

6.4	Stereo Audio	56

6.5	Audio Prioritising	56

6.6	Audio Output Formats	59

6.7	Audio Video Synchronisation	62

6.8	Audio Output Signals	63

6.9	Dynamic Changes	64

6.10	Clean Audio	65

6.11	Supplementary Audio	65

6.12	IRD Internal Reference Level	68

6.13	Loudness Levels, – Dynamic Range Control and Downmixing	68

7	Teletext and Subtitling	71

7.1	General	71

7.2	EBU Teletext	72

7.3	DVB Subtitling	72

8	Interfaces and Signal Levels	74

8.1	Introduction	74

8.2	RF- bypass (option)	74

8.3	Two-way Interface	74

8.4	SCART Interfaces (Option)	74

8.5	Audio Output Interfaces (Option)	75

8.6	HDMI Interface	75

8.7	User Control functions (Remote Control)	80

9	Interfaces for Conditional Access	85

9.1	General	85

9.2	Use of the Common Interface	85

9.3	Use of Smart Card Reader	86

10	The System Software Update	88

10.1	General	Error! Bookmark not defined.

10.2	SSU functionality	Error! Bookmark not defined.

10.3	System Software Update via local interface or IP-based interface (Internet)	Error! Bookmark not defined.

10.4	Network Management and Provisioning for IP based IRDs (IPTV)	Error! Bookmark not defined.

10.5	System Software Update via broadcast channels	Error! Bookmark not defined.

10.6	The descriptors for the SSU Notification in the UNT are specified in Section 12.7.CIP-CAM software updates	Error! Bookmark not defined.

11	Performance	89

11.1	Introduction	89

11.2	Video Performance of RGB and PAL Signals	89

11.3	Audio Performance of the Decoded Digital Signal	89

11.4	Zapping Time for TV Services	89

Part B: 	The system software with application	91

12	Service Information	92

12.1	General	92

12.2	Network Information Table (NIT)	98

12.3	Service Description Table (SDT)	110

12.4	Event Information Table	111

12.5	Time and Date Table and Time Offset Table	114

12.6	Conditional Access and Program Map Tables	115

12.7	SSU UNT Descriptors	121

12.8	Related Content Table (NorDig PVR only)	121

12.9	NorDig Broadcast Record List syntax (NorDig PVR only)	121

13	Navigator	122

13.1	General	122

13.2	Service List	122

13.3	Event Schedule Guide (ESG)	124

13.4	Service Discovery and Selection for IRDs with IP-based front-end	127

13.5	User Information (UI) about service components (audio and subtitling)	127

13.6	Accessibility menus and settings	128

14	NorDig PVR feature requirements (NorDig PVR only)	129

14.1	Introduction - PVR	129

14.2	General - PVR	129

14.3	PVR Recording	133

14.4	Playback	144

15	IRD System Software and API	147

15.1	NorDig Basic	147

15.2	NorDig Hybrid	147

16	User Preferences	148

16.1	General	148

16.2	User Preference Settings	148

16.3	Deletion of service lists	149

16.4	Reset to factory mode	149

Annex A : NorDig Members and Acknowledgement	151

1	NorDig Full Members	Error! Bookmark not defined.

2	NorDig Associated Members	Error! Bookmark not defined.

3	Acknowledgement and support	Error! Bookmark not defined.

Annex B : Background and options for IRDs with a terrestrial front-end	152

1	Terminology and Definitions for Single Frequency Networks Performance Parameters	Error! Bookmark not defined.

2	List of DVB-T/T2 centre frequencies	Error! Bookmark not defined.

3	Hierarchical mode reception	Error! Bookmark not defined.

Annex C : Placeholder for changes in 13.4 for IP-based IRDs.	153

Annex D : Implementations Guidelines for best service selection in automatic channel search in terrestrial networks	154

Annex E : Raw carrier to noise values, (C/N)RAW	155

Annex F : Recommendation for menu wordings and translations	156

Annex G : Guidelines for NorDig IRD audio selection	160

Annex H : Loudness levels, – Typical IDTV Audio Block diagram	161

Annex I : Examples of Signalling to be used for audio property	161

Annex J : Comparison of NorDig profiles	162

1	Introduction	Error! Bookmark not defined.

2	Legend	Error! Bookmark not defined.

# Introduction

## Scope

This document specifies a set of minimum equipment requirements for reception of DVB-based and related services from cable, satellite and/or terrestrial broadcast networks; in addition, it includes requirements for reception via IP-based networks. This NorDig specified receiver is hereafter denoted as an IRD (Integrated Receiver Decoder) and target consumer (viewer) usage.

The specifications cover IRDs, both as separate units (set-top-boxes) and as relevant parts of integrated digital TV-sets.

The NorDig IRD technical specifications are established with the aim to ensure that IRDs in the Nordic and Irish market satisfy a common set of minimum requirements, independent of operator/service provider and transmission media.

The specifications cover all kinds of IRD types, like as separate units (set-top-boxes) and as relevant parts of integrated digital TV-sets.

A NorDig IRD may be implemented in different IRD variants:

| FrontEnd variants | satellite, cable, terrestrial or IPTV IRD, see section 3 |
| --- | --- |
| Implementation type variants | STB or iDTV (IRD refers both STB and iDTV) |

The NorDig IRD may be implemented with minimum of capability, NorDig Basic IRD, or implemented with one or several optional capabilies in addition to the minimum.

NorDig has following optional additional IRD capabilities:

| HEVC | a NorDig IRD with UHDTV HEVC HDR&WCG SFR capability |
| --- | --- |
| HEVC HFR | a NorDig IRD with UHDTV HEVC HDR&WCG HFR capability (planned for the future, this profile is not yet specified) |
| HbbTV | a NorDig IRD with HbbTV capability according to section 15. An HbbTV IRD has (Internet access) connectable capability. |
| PVR | a NorDig IRD with capability for recording services and later playback of them, according to section 14 (also referred to as NorDig PVR). |

A NorDig IRD requirement refers to a requirement that is applicable for all IRD capability and variant types. All NorDig IRDs shall support reception of MPEG-2 and MPEG-4/AVC based services, while MPEG-H/HEVC based services at present specification is optional for the basic IRD profile.

The NorDig Unified Requirements include the profiles; Basic TV and Hybrid.

The NorDig Basic profile corresponds to basic requirements for digital broadcasting, services that do not depend on enhancements by applications or interaction. This profile is a subset of the Enhanced Profile.

The NorDig Hybrid profile covers the NorDig requirements for enhanced services depend on a standardized API, based on the HbbTV v2.0.1 specification ETSI TS 102 796 [29].

Figure 1.1 indicates the relationship between the NorDig basic and additional optional capability profiles and the various building blocks.

In addition to the different versions of the NorDig IRDs above, the NorDig IRDs may include support for recording services and later playback of them, then becoming a NorDig PVR IRD (or simply a NorDig PVR). The basic PVR functionality that is specified for NorDig does not require support from the API (HbbTV).

An IRD may and in some cases shall include a two-way interface (e.g. WiFi, Ethernet, Eurodocsis etc, see section 8.3) typically with access to Internet, here referred to as a connectable IRD type (e.g. NorDig hybrid IRD is a connectable IRD with HbbTV API according to NorDig requirements in section 15, or a “Smart TV” using other techniques than HbbTV). A connectable IRD that have connected and activated the two-way interface is here referred to as connected IRD (i.e. a connected connectable IRD), while a connectable IRD that has not connected or activated the two-way interface is referred to as non-connected connectable IRD.

Figure 1.1 The NorDig IRD profiles and the main building blocks and examples of configurations.

NorDig has also specified NorDig Rules of Operation for NorDig compliant networks [63], and the Unified NorDig Test Specifications [64], in order to verify compliance with the NorDig Unified Requirements for new IRDs. These Rules of Operation and Test Specifications cover all NorDig profiles.

It should be noted that the NorDig Rules of Operation [63] and the NorDig Test [64] may provide additional detail to the specified requirements (in this specification), and should be used when additional background is needed in order to interpret the specified requirements.

The specifications of the NorDig IRDs are divided into two parts.

Part A:  Hardware and firmware

Part B: The Software system and Application Programming Interfaces (API) as relevant.

The specification parts A and B outline the desired hardware and software architectures. Based on this framework the mandatory interfaces, functionality and performance requirements of the IRD are specified. Part B deals also with requirements to the operating system. Optional requirements are specified for recommended, but not mandatory functions.

The NorDig group represents broadcasters, operators and service providers in the Nordic countries, see Annex A.

The various members of NorDig are independent of each other but intend to transmit to IRDs that satisfy the specified common requirements. In order to ensure compliance with the NorDig requirements, the NorDig IRDs will be subject to a set of verification tests, based on NorDig Test [64].

## Document History

Overview:

The NorDig I specification was first issued in 1998 when the common DVB API solution had not taken specific direction and based on the technical status of that time.

The NorDig Unified Requirements were updated in 2003, in 2004/2005 (Version 1.0.2) and in 2006 (version 1.0.3) when the IPTV-Addendum was merged into the text; in addition, some new requirements were introduced as mandatory after a specified grace period.

Version 2.0 was established in 2008 and includes the NorDig HDTV-Addendum.

Version 2.1 includes updates to version 2.0 up to July 2009 and the addition of basic requirements for NorDig PVR, which were available as an Addendum to NorDig Unified 2.0.

Version 2.2 includes updates to version 2.1 up to July 2010, including requirements for an enhanced terrestrial front-end (T2) and updates to the PVR requirements. All NorDig IRDs shall now support advanced codecs. The T2-front-end is mandatory for IRDs that target NorDig compliant signals based on the DVB-T2 specification.

The version 2.3 includes updates to version 2.2 up to May 2012, including updates to the terrestrial front-end (T2), the PVR and audio requirements. The version 2.3 included as an Addendum HbbTV as NorDig’s new API (replacing MHP)

The version 2.4 includes updates to version 2.3 up to November 2012. The main changes are inclusion of HbbTV requirements and removal of MPEG2 only IRD alternative “M2 Level”. All NorDig IRDs shall support both MPEG2 and MPEG4.

The version 2.5 and 2.5.1 was published in August 2014 and among things included updates for change of audio priority, Common Interface from CI+ v1.2 to v1.3, adding requirements for verification testing of HbbTV.

Theis version 2.6 was published January 2017 and main updates are adding 700 MHz LTE immunity for terrestrial IRDs, for Remote Control changing to require functions instead of keys and notification alternative for SSU etc.

The version 3.0 was published Month 20yy and main updates is the adding of requirements for HEVC capable IRDs (NorDig HEVC IRDs).

Details:

| Version | Date | Comments |
| --- | --- | --- |
| NorDig I ver. 1.1 | 12.05.98 | This is the first approved version of the complete NorDig I specification |
| NorDig I ver. 1.3 | 01.03.01 | Some editorial changes are performed, to bring the text in line with NorDig II (ver. 0.9). Some requirements are relaxed, when relaxed in NorDig II (ver. 0.9). Some new optional requirements are introduced in NorDig I that are mandatory requirements to NorDig II. References are updated to reflect the present status of the original references. |
| NorDig I ver. 1.4 | 01.10.2002 | This update of the NorDig I specification is contained in the NorDig Unified, ver 1.0, see below. This update includes relaxation of some specifications, partly due to experience from testing of IRDs, but mainly in order to keep the same minimum requirements for non-interactive services as for the Basic TV profile. Some text is modified, in order to improve clarity and unify text for identical requirements in the NorDig I and NorDig II specifications. Furthermore, some additional parameters/descriptors are specified in sections 12 and 13, in order to bring the specification in line with NorDig II, ver.1.1. Some requirements will be increased to mandatory after a grace period; these increases are due to technical progress and satisfied by most IRDs sold in 2002. |
| NorDig II ver. 0.9 | 08.06.2000 | This is the first approved version of the NorDig II specification, based on DVB-MHP-ver.1.0 until ver.1.1 becomes available |
| NorDig II ver. 1.0 | 13.06.2001 | This version includes an update to reflect the changes in DVB-MHP-ver1.1 compared to MHP-ver1.0, and some clarifications of the text. Furthermore, some additional parameters/descriptors are specified in sections 12 and 13 and the text is modified for better clarity. In addition some relaxations in line with the NorDig I ver. 1.3 specification have been included. |
| NorDig II ver. 1.1 | 01.10.2002 | This update of the NorDig II specification is contained in the NorDig Unified, ver. 1.0, see below. This update includes relaxation of some requirements, partly due to experience from testing of IRDs, but mainly in order to keep the same minimum requirements for non-interactive services as for the Basic TV profile. Some text is modified, in order to improve clarity. Some requirements, mainly related to the terrestrial front-end will be increased to mandatory after a grace period; these increases are due to technical progress and operational experience. |
| NorDig Basic TV | 01.10.2002 | This is the first approved version of the NorDig Basic TV profile. The specification text is based on NorDig II, ver 1.0 and harmonised with NorDig II, ver. 1.1 when relevant. The specification text is a subset of the NorDig Unified, ver. 1.0; see below. |
| NorDig Internet | 01.10.2002 | This is the first approved version of the NorDig Internet Access profile. It is based on the specification text for NorDig II, ver. 1.1, with necessary additions to include the DVB-MHP-Internet Access profile. The specification text is included in the NorDig Unified, ver. 1.0; see below. |
| NorDig Enhanced | 16.10.2002 | This is the first approved version of the NorDig Enhanced profile. The specification text is based on NorDig II, ver 1.1 and harmonised with NorDig Basic TV when relevant. The specification text is a subset of the NorDig Unified, ver. 1.0; see below. |
| NorDig Unified, ver. 1.0 | 16.10.2002 | This is the first approved version of the NorDig Unified requirements for IRDs and includes requirements for all NorDig profiles; including Basic, Enhanced, NorDig I, Interactive (NorDig II) and Internet. |
| NorDig Unified, ver. 1.0.1 | 01.07.2003 | This version includes updates of requirements that were introduced in version 1.0 with a grace period, and some new requirements with corresponding grace periods, mainly related to the terrestrial front-end, SI (chapters 12 and 13) and the user interface (chapters 16 and 17). Some text is modified in order to improve clarity. |
| NorDig Unified, ver. 1.0.2 | 30.4.2005 | This version includes updates of requirements that were introduced in version 1.0.1 with a grace period, some new requirements and modifications of the mandatory CA-requirements (see section 15.1). The specification is also expanded to include requirements for IP-front-ends (provided as a separate addendum to this specification) and requirements for terrestrial front-ends in the VHF-band. |
| NorDig Unified, ver. 1.0.3 | 28.2.2007 | This version includes updates of requirements that were introduced in version 1.0.2 with a grace period, while the NorDig I profile and requirements for Controllers and Memory are removed. Requirements for the IP based front-end are included (previously available as a separate addendum). |
| NorDig Unified ver. 2.0 | 01.07.2008 | This version includes NorDig Unified ver. 1.0.3 plus NorDig HDTV-Addendum ver 1.0 (previously available as a separate document). In addition it includes some updates to the existing requirements and introduction of some new requirements with a grace period. Some editorial changes are made, including change of chapter order compared with earlier versions. |
| NorDig Unified ver 2.1 | 01.07.2009 | This version includes NorDig Unified ver 2.0 plus the Additional requirements for NorDig PVR (previously available as a separate document). In addition it includes some updates to the existing requirements, including enhancements to the terrestrial and cable front-ends. Minimum requirements for reception of DVB-T2 signals are issued as an Addendum to this specification. New requirements are introduced with a grace period. |
| NorDig Unified ver 2.2 | 01.07.2010 | This version includes NorDig Unified ver. 2.1 plus the Additional Requirements for NorDig T2 IRDs (previously available as a separate document). In addition it includes some updates to the existing requirements, including enhancements to the front-ends and the PVR functions. All IRDs shall now support the MPEG 4-video compression. New requirements are introduced with a grace period |
| NorDig Unified ver 2.2.1 | 1.11.2010 | This version includes NorDig Unified ver. 2.2 requirements and corrections to the DVB-T2 FE and audio requirements. Also some references are updated. |
| NorDig Unified ver 2.3 | 15.5.2012 | This version includes NorDig Unified ver. 2.2 plus additional requirement for the PVR functionality and new requirements for the Audio functionality. |
| NorDig Unified ver 2.4 | 16.01.2013 | This version 2.4 includes updates to version 2.3 up to November 2012, including updates to the API (HbbTV), video and audio requirements. Most important changes are: - General: removal of M2 and M4 Levels, all NorDig IRDs shall now support MPEG2 and MPEG4. - General: DVB-T2, ETSI specification changed from v1.1.1 to v1.2.1 - General: HbbTV requirement, changed from v1.0 (ETSI 102 796 v1.1.1) to v1.5 (ETSI 102 796 v1.2.1) - Ch 3.4.10 Terrestrial – inclusion of immunity requirement against other signals, 800MHz LTE signals. - Ch 5 video – update and restructure of requirements - Ch 6 audio – update audio priority handling and supplementary audio - Ch 7 Teletext and Subtitling – coexist handling for subtitling and HbbTV - Ch 8.4 SCART – will become optional from 2014 - Ch 8.6 HDMI – update of HDMI requirements - Ch 8.7 remote control – updates related to HbbTV - Ch 12.1 SI – inclusion of Country and Language codes - Ch 12.6 SI PMT – updates for descriptors related to audio - Ch 14 PVR – updates Full service recording and Playback related to HbbTV - Ch 15.2 API – updated for HbbTV v1.5 - Ch 16.3 User preference settings – updates related to HbbTV and Supplementary Audio Annex I – new annex for examples of Signalling to be used for audio property |
| NorDig Unified ver 2.5 | 07.07.2014 | This version 2.5 includes updates of version 2.4 and the most important changes are: - RF connector, change of reference old IEC 60169-2 to IEC 61169-2. - Ch 3.4.7 terrestrial IRDs with RF bypass shall support control of gain - Ch 3.4 Immunity to “analogue” (PAL) signals, requirements removed - Ch 6 restructure of text and sub chapters - Ch 6.2.3 inclusion of DRC Presentation mode according to DVB/ESTI - Ch 6.5 Audio prioritising, change of audio priority from language highest priority to instead audio type highest priority. - 6.14 Loudness Levels, inclusion of DRC presentation mode 1 and 2 according to DVB ETSI TS 101 154. - Ch 7.1 Teletext and Subtitling, only display subtitling matching preferences and handling of temporary settings. - Ch 7.1 Teletext and Subtitling, handling of HbbTV Digital Teletext and EBU Teletext. NorDig HbbTV IRDsHybrids support toggling functions in TEXT key. - Ch 9 Conditional Access, change from CI+ v1.2 to CI+ v1.3. - Ch 10 SSU, complete restructure of text. Opens for more implementation alternatives. - Ch 11.4 zapping time, update of requirements (removal of zapping to/from analogue services). - Ch 12 SI, NorDig Logical Channel Descriptor version 1 changed back to mandatory. - Ch 13.5 (new) User information about service components - Ch 13.6 (new) Accessibility menus and settings - Ch 15.2 API HbbTV, add requirements on testing according with HbbTV test suits - Ch 16.1 Stored preferences, chapter restructured and updated. User preference requirements related to supplementary audio moved from 6.11 to 16.1.3.1. - Annex B Terrestrial IRDs and definition of SFN, correction of formula. |
| NorDig Unified ver 2.5.1 | 25.08.2014 | “Typo” corrections of the version 2.5 includes: - Ch 3.4.10.7, important correction for second bullet - Ch 6.2, first sentence removal of duplication text. - Ch 6.5.1, correction of time for grace period. - Ch 11.3, added missing grace period for new mandatory requirements. - Ch 13.5, added missing grace period for new mandatory requirements. |
| NorDig Unified ver 2.6 | DD20.01.2017 | This version 2.6 includes updates of version 2.5.1 and the most important changes are: - Removal of many grace periods that time has passed for - Ch 1 Introduction, adding description of connectable IRD - Ch 2.5.4, definition of Persistent and Temporary settings - Ch 3.4.5 change of modulation - Ch 3.4.10.7 update of requirements for 800MHz immunity and adding requirements for 700MHz immunity against LTE signals - Ch 8.7 changing from Remote Control (specific dedicated keys) to instead require User Control Functions - Ch 10 adding possibility for SSU Notification for Connectable IRDs and restructure of subchapter, moving/collecting broadcast specific requirements into ch 10.5 - Ch 12.7 adding descriptors for SSU Notification - Ch 13.3.2.4 re-writing and adding explanations for handling of Parental Control - Ch 15 migration to HbbTV v2.0.1 (with grace period until 1 July 2018) and test suite. - Ch 16 correction for settings related to SSU |

## Terminology

| Shall (Mandatory) | This word means that the item is mandatory. |
| --- | --- |
| Should (Recommended) | This word means that this item is not mandatory, but is highly recommended. |

## Definitions

Naming, NorDig is using following wordings to refer to a certain combination of capability and variant of IRD (including/excluding): InternetAcess (connectable/non-connectable) + FrontEnd (T/C/S/IP) + codec (HEVC/basic) + API (HbbTV/basic) + PVR (PVR/basic) + type (IRD/STB/iDTV). (A NorDig PVR IRD is often shorten to NorDig PVR).

NorDig Basic IRD (NorDig Basic):

The NorDig Basic IRDs (hereafter also denoted NorDig Basic) consist of a user terminal, including all possible low to high functionality implementations and its associated peripherals. NorDig Basic IRD is specified as a minimum NorDig IRD with without any optional capability a subset of this unified NorDig-specification.

The NorDig Basic IRD shall satisfy all requirements specified for a NorDig IRD, unless stated otherwise. Requirements that states a certain optional capability (e.g. NorDig HbbTV IRD or NorDig HEVC IRD), these requirement is only that IRD configuration and meaning that the requirement is optional for the NorDig Basic IRD.

NorDig HEVC IRD:

The NorDig HEVC IRDs (also denoted NorDig HEVC) is a NorDig IRD with capability for reception of HEVC services as defined by NorDig. The NorDig HEVC IRD shall satisfy all requirements specified for a NorDig IRD (unless stated otherwise) plus all requirements for NorDig HEVC.

NorDig HbbTV IRD Hybrid:

The NorDig HybridIRDs (hereafter denoted NorDig Hybrid) consist of a user terminal, including all possible low to high functionality implementations and its associated peripherals.

The NorDig HbbTV IRDs (also denoted NorDig HbbTV) is a NorDig IRD with capability for reception of HbbTV services as defined by NorDig. The NorDig HbbTV IRD shall satisfy all requirements specified for a NorDig IRD (unless stated otherwise) plus all requirements for NorDig HbbTV.

Integrated Receiver Decoder (IRD): 
Set-top-box (STB) or relevant parts of integrated digital TV (iDTV)-set.

NorDig PVR IRD (NorDig PVR):

The NorDig PVR IRD (also denoted NorDig PVR) is a NorDig IRD with the capability to record to internal media (for example a built-in hard disk drive) or removable media (for example a DVD or Blu-ray disk). The NorDig PVR (Personal Video Recorder) shall satisfy all requirements specified for a NorDig IRD, unless stated otherwise.

NorDig satellite, cable, terrestrial and IPTV IRD:

The satellite/cable/terrestrial/IPTV NorDig IRD refers to an IRD with a front-end that is capable of receiving satellite/cable/terrestrial/IPTV DVB signals according with section 3. For example the The terrestrial NorDig IRD-T2 refers to an IRD with a front-end that is capable of receiving DVB-T and DVB-T2 signals.

A NorDig IRD may support multiple FrontEnd variants (e.g. satellite, cable and terrestrial) and in this case the IRD shall support all the relevant requirements for all the supported ForntEnds as stated in section 3.

Connectable/non-connectable IRD:

An IRD may and in some cases shall include a two-way interface (e.g. WiFi, Ethernet, Eurodocsis etc, see section 8.3) typically with access to Internet, here referred to as a connectable IRD type (e.g. NorDig HbbTV IRD is a connectable IRD with HbbTV API according to NorDig requirements in section 15, or a “Smart TV” using other techniques than HbbTV). A connectable IRD that have connected and activated the two-way interface is here referred to as connected IRD (i.e. a connected connectable IRD), while a connectable IRD that has not connected or activated the two-way interface is referred to as non-connected connectable IRD.

Example multiple capabilities:

One example of naming for an IRD that supports multiple capabilities is a NorDig non-connectable terrestrial HbbTV PVR, which refers to all terrestrial types variants of HEVC and non-HEVC IRDs (STB and iDTV) that includes HbbTV and PVR capability but do not include any Internet access capability.

## References

| - Bluetooth 2.0 | Specification of the Bluetooth System, 4 November 2004, Bluetooth SIG. |
| --- | --- |
| - CEA-770.3 | Consumer Electronics Association (CEA): Standard Definition TV Analogue Component Video Interface, CEA -770.2C, November 2001. www.ce.org/Standards. |
| - CEA 861-D | Consumer Electronics Association (CEA): A DTV Profile for Uncompressed High Speed Digital Interfaces, July 18, 2006 |
| - DVB A 011 | Common Scrambling Algorithm. DVB Blue Book A011. |
| - Digital Europe HD Ready | “HD ready” Minimum Requirements, www.digitaleurope.org |
| - EN 50049-1 | Domestic and similar electronic equipment interconnection requirements: Peritelevision Connector |
| - EN 60728 and 50083-9 | Cenelec EN 60728-1, Cable networks for television signals, sound signals and interactive services - Part 1: System performance of forward paths. Cenelec 50083-9, Cabled distribution systems for television, sound and interactive multimedia signals – Part 9: Interfaces for CATV/SMATV headends and similar professional equipment for DVB/MPEG-2 transport streams. |
| - EN 50157-2-1 | Domestic and similar equipment interconnection requirements: AV.link-Part 2-1: Signal quality matching and automatic selection of source devices. |
| - EN 50221 | Common Interface for Conditional Access and other Digital Video Broadcasting Decoder Applications. |
| - EN 50494 | Satellite signal distribution over a single coaxial cable in single dwelling installations |
| - EN 55013 | Limits and methods of measurements of radio disturbance characteristics of broadcast receive equipment and associated equipment |
| - EN 61319-1 | Interconnections of satellite receiving equipment – Part 1: Europe. |
| - ETSI EN 300 421 v1.1.2 | Digital Video Broadcasting (DVB): Digital broadcasting systems for television, sound and data services: Framing structure, channel coding and modulation for 11/12 GHz Satellite services. (DVB-S) |
| - ETSI EN 300 429 v1.2.1 | Digital Video Broadcasting (DVB): Digital Broadcasting Systems for Television, Sound and Data Services; Framing Structure, Channel Coding and Modulation for Cable Systems. |
| - ETSI EN 300 468 v1.11.1 | Digital Video Broadcasting (DVB): Digital broadcasting systems for television, sound and data services: Specification for Service Information (SI) in Digital Video Broadcasting (DVB) systems |
| - ETSI EN 300 472 v1.3.1 | Digital Video Broadcasting (DVB): Digital Broadcasting Systems for Television, Sound and Data Services; Specification for Conveying ITU-R System B Teletext in Digital Video Broadcasting (DVB) Bitstreams. |
| - ETSI EN 300 473 v1.1.2 | Digital Satellite Master Antenna Television Distribution System (SMATV). |
| - ETSI EN 300 706 V1.2.1 | Digital Video Broadcasting (DVB): Enhanced Teletext specification |
| - ETSI EN 300 743 v1.3.1 | Digital Video Broadcasting (DVB): DVB Subtitling Systems. |
| - ETSI EN 300 744 v1.6.1 | Digital Video Broadcasting (DVB); DVB Framing structure, channel coding and modulation for digital terrestrial television. |
| - ETSI EN 302 755 v1.2.1 | Frame structure channel coding and modulation for a second generation digital terrestrial television broadcasting system (DVB-T2). |
| - ETSI EN 302 307 v1.2.1 | Digital Video Broadcasting (DVB): Second generation framing structure, channel coding and modulation systems for Broadcasting, Interactive Services, News Gathering and other broadband satellite applications (2009-08). (DVB-S2) |
| - ETSI ETR 162 | Digital Video Broadcasting (DVB);Allocation of Service Information (SI) codes for DVB systems |
| - ETSI ETR 289 | Digital Video Broadcasting (DVB); Support for use of scrambling and Conditional Access within digital broadcasting systems. |
| - HbbTV Test Suite | HbbTV Test Suite (see details of version in NorDig Test Plan related to HbbTV) |
| - ETSI TR 101 202 V1.2.1 | Digital Video Broadcasting (DVB); Implementation guidelines for Data Broadcasting |
| - ETSI TR 101 211 V1.9.1 | Digital Broadcasting Systems for Television, Sound and Data Services; Guidelines on the Implementation and Usage of DVB Service Information. |
| - ETSI TS 101 154 V2.3.1V1.11.1 | Digital Video Broadcasting (DVB); Implementation Guidelines for the use of MPEG-2 Systems, Video and Audio in Satellite Cable and Terrestrial Broadcasting Applications; 2012-11 2017-02. |
| - ETSI ES 102 796 V1.4.1 | Hybrid Broadcast Broadband TV (HbbTV), (referring to HbbTV version 2.0.1 functionality) |
| - ETSI TS 102 006 V1.4.1 | Digital Video Broadcasting (DVB); Specification for System Software Update (SSU) in DVB Systems |
| - ETSI TS 102 034 V1.4.1 | Digital Video Broadcasting (DVB); Transport of MPEG-2 Based DVB Services over IP based Networks |
| - ETSI TS 102 114 V1.2.1 | ETSI Technical Specification (TS): DTS coherent acoustics; Core and extensions; 2002-12 |
| - ETSI TS 102 201 V1.2.1 | Digital Video Broadcasting (DVB); Interfaces for DVB Integrated Receiver Decoder (DVB-IRD) |
| - ETSI TS 102 323 V1.4.1 | Digital Video Broadcasting (DVB); Carriage and signalling of TV-Anytime information in DVB transport streams, 2010-01 |
| - ETSI TS 102 366 V.1.2.1 | ETSI Technical Specification (TS): Digital Audio Compression (AC-3, Enhanced AC-3) Standard, August 2008 |
| - ETSI TS 102 822-4 V1.5.1 | Broadcast and On-line Services: Search, select, and rightful use of content on personal storage systems (“TV-anytime”); Part4: Phase 1 – Content referencing |
| - HDCP rev1.4 | Digital Content Protection LLC, “High-bandwidth Digital Content Protection System”, rev. 1.4, July 8, 2009. |
| - HDMI | HDMI Licensing, LLC: HDMI , “High- Definition Multimedia Interface”, rev. 1.4b, October 11th, 2011 |
| - IEC 61169-2 | Radio-frequency connectors – Part 2: Sectional specification – Radio frequency coaxial connectors of type 9,52. |
| - ISO/IEC 61169-24 (ISO 169-24) | Radio-frequency connectors – Part 24: Radio-frequency coaxial connectors with screw coupling, typically for use in 75 Ω cable distribution systems (Type F). |
| - IEC 60603-14 | Connectors for frequencies below 3 MHz for use with printed boards – Part 14: Detail specification for circular connectors for low-frequency audio and video applications such as audio, video and audio-visual equipment. |
| - IEC 60958 | Digital audio interface – Part 3: Consumer applications. |
| - IEC 61937 | Digital audio – Interface for non-linear PCM encoded audio bitstreams applying IEC 60958 [42]. |
| - IEEE 802.11 | The Working Group for Wireless Local Area Networks (WLANs). |
| - IEEE 802.3 | IEEE Standard for Information technology-Telecommunications and information exchange between systems-Local and metropolitan area networks–Specific requirements Part 3: Carrier Sense Multiple Access with Collision Detection (CSMA/CD) Access Method and Physical Layer Specifications |
| - IETF RFC 2131 | Dynamic Host Configuration Protocol, March 1997 |
| - IETF RFC 2132 | DHCP Options and BOOTP Vendor Extensions, March 1997 |
| - IETF RFC 3203 | DHCP reconfigure extension, December 2001 |
| - ISO 3166 – Part 1-3 | Codes for the representation of names of countries and their subdivisions: Part 1: Country codes, 1997 Part 2: Country subdivision code, 1998 Part 3: Code for formerly used names of countries, 1999 |
| - ISO 8859-1 | Information technology – 8-bit single-byte coded graphic character sets – Part 1: Latin alphabet No. 1, 1998 |
| - ISO/IEC 11172-3 | ISO/IEC: Information technology – Coding of moving pictures and associated audio for digital storage media at up to about 1,5 Mbit/s – Part 3: Audio |
| - ISO/IEC 13818 – 1 | Information Technology – Generic Coding of Moving Pictures and Associated Audio Information. Part 1: Systems. ISO/IEC International Standard IS 13818. |
| - ISO/IEC 13818-2 | Information technology – Generic coding of moving pictures and associated audio information: Video, ISO/IEC International Standard IS 13818 |
| - ISO/IEC 13818 – 3 | Information Technology – Generic Coding of Moving Pictures and Associated Audio Information. Part 3: Audio. ISO/IEC International Standard IS 13818. |
| - ISO/IEC 14496-3 | ISO/IEC: Information technology – Coding of audio-visual objects – Part 3: Audio, 2005 |
| - ISO/IEC 14496-10 | ISO/IEC: Information technology — Coding of audio visual objects — Part 10:Advanced Video Coding (MPEG-4 AAD and MPEG-4 HE-AAC), December 15, 2005 |
| - ISO/IEC 60958-3 Ed.3.0 | ISO/IEC: Digital audio interface – Part 3: Consumer applications, May 25, 2006 |
| - ISO/IEC 7816, 1-3 | Identification cards – Integrated circuit cards with contacts, Parts 1-3. ISO/IEC International Standard IS 7816. |
| - ITU-J.122 | (EuroDocsis 2.0): Second-generation Transmission Systems for Interactive Cable Television Services – IP Cable Modems. |
| - ITU-J.222.1 | (EuroDocsis 3.0): Third-generation Transmission Systems for Interactive Cable Television Services – IP Cable Modems: Physical Layer Specifications (Annex B) |
| - ITU/R Report 624-4 | ITU-R Report 624-4:1990, Characteristics of Television Systems. |
| - ITU-R BT.653-3 | ITU-R Recommendation BT653-2:1993, Teletext systems. |
| - NorDig RoO | a) NorDig Rules of Operation, ver 1.0, November, 2004 b) NorDig Rules of Operation, ver 2.5. October, 2016 |
| - NorDig Test | a) Unified NorDig Test Specification, ver. 2.0. July 2008. b) Unified NorDig Test Specification, ver. 2.2, (Not yet available) |
| - Universal Serial Bus | Universal Serial Bus (USB) Specification, Revision 2.0, April 27, 2000. |
| - CI Plus Specification | CI Plus Specification. Content Security Extensions to the Common Interface. Version 1.3.1 (2011-09), from, CI Plus LLP |
| - UK DTG D-Book | Digital Terrestrial Television (DTG), Requirements for interoperability, The D-Book 7 Part A, Version 1, March 2011 |
| - ETSI TS 102 831 v1.1.1 | Implementation guidelines for a second-generation digital terrestrial television broadcasting system (DVB-T2) |
| - ETSI TS 102 822-3-1 v1.6.1 | Broadcast and On-line Services: Search, select, and rightful use of content on personal storage systems ("TV-Anytime"); Part 3: Metadata; Sub-part 1: Phase 1 - Metadata schemas. Version 1.6.1 (2010-07) |
| - ISO 639-2 | Codes for the Representation of Names of Languages Part 2: Alpha-3 Code |
| - ETSI TS 102 822-3-2 v1.6.1 | Broadcast and On-line Services: Search, select, and rightful use of content on personal storage systems ("TV-Anytime"); Part 3: Metadata; Sub-part 2: System aspects in a uni-directional environment. Version 1.6.1 (2010-07) |
| - ETSI TS 102 851 v1.2.1 | Digital Video Broadcasting (DVB); Uniform Resource Identifiers (URI) for DVB Systems. Draft version 1.2.1 until it becomes official release. |
| - EBU, R-95 | EBU Recommendation R 095, Recommendation for Safe areas for 16:9 television production, July 2016 (or later). |
| - EBU R128 | EBU Recommendation R 128, Loudness normalisation and permitted maximum level of audio signals, August 2011. |
| - EBU TECH 3344 | EBU – TECH 3344, Practical guidelines for distribution systems in accordance with EBU R 128, (Supplementary information for EBU R 128), version 1.1, October 2011. |
| - EBU R68 | EBU Technical Recommendation R68-2000, Alignment level in digital audio production equipment and in digital audio recorders, revised year 2000. |
| - EBU TECH 3341 | EBU – TECH 3341, Loudness Metering: ‘EBU Mode’ metering to supplement loudness normalisation in accordance with EBU R 128, (Supplementary information for R 128) , August 2011. |
| - ITU-R BS.1770-2 | ITU Recommendation ITU-R BS.1770-2 (march 2011), Algorithms to measure audio programme loudness and true-peak audio level |
| - ITU-R BS.1771 | ITU Recommendation ITU-R BS.1771, Requirements for loudness and true-peak indicating meters |
| - IEC 62731 | IEC 6273, edition 1.0 (2013-01-29), Text-to-speech for television - General requirements. |
| - HbbTV test specification | Test Specification for HbbTV (latest version), Note: Available at http://www.hbbtv.org/resource-library/#testing-information-and-support |
| - ETSI ES 102 796 V1.2.1 | Hybrid Broadcast Broadband TV (HbbTV), (referring to HbbTV version 1.5 functionality, allowed option for NorDig HbbTV IRD Hybrid instead of v2.0.1 [29] until 1 July 2018) |
| - ISO/IEC 23008-2 | Information technology — High efficiency coding and media delivery in heterogeneous environments Part 2: High efficiency video coding |
| - ITU-R BT.601 | Studio encoding parameters of digital television for standard 4:3 and wide-screen 16:9 aspect ratios |
| - ITU-R BT.709 | Parameter values for the HDTV standards for production and international programme exchange |
| - ITU-R BT.1700 | Characteristics of composite video signals for conventional analogue television systems |
| - ITU-R BT.1847 | 1 280 × 720, 16:9 progressively-captured image format for production and international programme exchange in the 50 Hz environment |
| - ITU-R BT.1886 | Reference electro-optical transfer function for flat panel displays used in HDTV studio production |
| - ITU-R BT.2020 | Parameter values for ultra-high definition television systems for production and international programme exchange |
| - ITU-R BT.2100 | Image parameter values for high dynamic range television for use in production and international programme exchange |
| - ITU-R BT.2390 | High dynamic range television for production and international programme exchange |
| - HDCP rev2.2 | Digital Content Protection LLC, “High-bandwidth Digital Content Protection System - Mapping HDCP to HDMI”, rev. 2.2, February 13, 2013 and “Errata to HDCP 2.2 on HDMI Specification”, version 2, February 9, 2015. |
| - ANSI/CTA-861-G | A DTV Profile for Uncompressed High Speed Digital Interfaces |

## List of Abbreviations

0b	values written in binary (ie with base 2)

0x	values written in hexadecimal (ie with base 16)
AAC	Advanced Audio Codec

AAC-LC	Advanced Audio Codec Low Complexity

AC-3	Audio Codec 3

ACE	Active Constellation Extension

AFC	Automatic Frequency Control

AFD	Active Format Descriptor

AFNOR	Association Francaise de Normalisation

API	Application Programming Interface
ARC	Audio Return Channel (regarding HDMI interface)

AV	Audio (and) Video

AVC	Advanced Video Coding (MPEG-4 p.10/H.264)

BAT	Bouquet Association Table

BCD	Binary Coded Decimal

BDR	Broadcast Discovery Record (part of SD&S)

BER	Bit Error Ratio

BOOTP 	Bootstrap Protocol

bslbf	bit string, left bit first

C/N	Carrier to Noise ratio

CA	Conditional Access

CAM	Conditional Access Module

CAT	Conditional Access Table

CATV	Community Antenna Television

CEA	Consumer Electronics Association (North American Association)

CENELEC	Comité Européen de Normalisation Electrotechnique

CI	Common Interface

CID	Content Identifier descriptor

CIF	Common Intermediate Format

CI- CAM	CA-module that complies with the basic Common Interface specification [9]

CIP- CAM	CA-module that complies with the Common Interface Plus specification [66]

CRC	Cyclic Redundancy Check

CRID	Content Reference Identifier

CSO	Composite Second Order

CTB	Composite Triple Beat

CVBS	Composite Video Baseband Signal

D/A	Digital-to-Analogue converter

DAD	Default Authority Descriptor

DAVIC	Digital Audio-Visual Council
dB	decibel

dBFS	dB (relative to) Full Scale

DDS	Display definition segment

DDWG	Digital Display Working Group

DECT	Digital Enhanced Cordless Telecommunications

DHCP	Dynamic Host Configuration Protocol

DSB	Double SideBand

DSM-CC 	Digital Storage Media Command and Control
DTS	Digital Theater System (audio codec)

DVB	Digital Video Broadcasting

DVB-C	Digital Video Broadcasting – Cable
DVB-C2  	Digital Video Broadcasting – Cable system, second generation system

DVB-CAM	CA-module that complies with the DVB Common Interface specification

DVB-data	Digital Video Broadcasting – Data Broadcasting

DVB-S	Digital Video Broadcasting – Satellite
DVB-S2  	Digital Video Broadcasting – Satellite system, second generation system

DVB-T	Digital Video Broadcasting – Terrestrial system
DVB-T2  	Digital Video Broadcasting – Terrestrial system, second generation system 
E-AC-3	Enhanced Audio Codec 3

E-EDID	Enhanced Extended Display Identification Data (regarding HDMI interface)

EBU	European Broadcasting Union

ECCA	European Cable Communications Association

ECL	EuroCableLabs, technical cell of ECCA

EICTA	European Information & Communications Technology Industry Association

EIT	Event Information Table

EITp/f	Event Information Table, present/following tables

EITsch	Event Information Table, schedule tables

EITp	Event Information Table, present table/section of EITp/f

EITf	Event Information Table, following table/section of EITp/f

EPT	Effective Protection Target

EPG	Electronic Program Guide (based on API)

ESG	Event Schedule Guide (without any API)

FDD	(Mobile communication network) Frequency Division Duplex

FEF	Future Extension Frame

FFT	Fast Fourier Transform

GAP	Generic Access Protocol

GOP	Group Of Pictures

GPRS	General Packet Radio System

GS	Generic Stream

GSM	Group Special Mobile
HbbTV	Hybrid Broadcast Broadband TV

HDCP	High-bandwidth Digital Content Protection

HDMI	High-Definition Multimedia Interface
HDMI ARC	HDMI Audio Return Channel

HDTV	High Definition Television
HDR	High Dynamic Range

HEVC	High Efficiency Video Coding (MPEG-H p.2/H.265)

HE-AAC	High Efficiency Advanced Audio CodingCodec

HFR	High Frame Rate (here >60 frames/s)

HTTP	HyperText Transfer Protocol
HW	Hardware

iDTV	integrated Digital TV (IRD with display)

IEC	International Electrotechnical Commission

IEEE 	Institute for Electrical and Electronic Engineers

IEFT	Internet Engineering Task Force

IGMP	Internet Group Management Protocol

INA	Interactive Network Adapter

IP	Internet Protocol

IRD	Integrated Receiver Decoder

IMI	Instant Metadata Identifier

ISO	International Organisation for Standardisation

JTC	Joint Technical Committee

LCD	Logical Channel Descriptor

LCN	Logical Channel Number
LTE	(Mobile communication network) Long Term Evolution

LU	Loudness Units

LUFS	Loudness Units (relative to) Full Scale

L-PCM	Linear Pulse Code Modulation

MAC	Medium Access Control

MPEG	Moving Pictures Expert Group
MPTS	Multi Programme Transport Stream

MTU	Maximum Transfer Unit

NEM	Network Element Management

NIC	Network Interface Card

NIT	Network Information Table

NT	Network Termination in general

NVOD	Near Video On Demand

OSD	On Screen Display

PAL	Phase Alternating Line

PAPR	Peak-toAverage-Power Ratio

PAT	Program Association Table
PCM	Pulse Code Modulation

PLP	Physical Layer Pipe

PID	Packet Identifier

PMT	Program Map Table

PSI	Program Specific Information

PSTN	Public Switched Telephone Network
PCR	Programme Clock Referance

PVR	Personal Video Recorder, (same as PDR, Personal Digital Recorder, or DVR)

QAM	Quadrature Amplitude Modulation

QCIF	Quarter Common Intermediate Format

QEF	Quasi Error Free

QoS	Quality of Service

QPSK	Quaternary Phase Shift Keying

RF	Radio Frequency

RFC	Request For Comments

RMS	Root Mean Square

RoO	Rules of Operation

rpchof	remainder polynomial coefficients, highest order first

RS	Reed-Solomon

RST	Running Status Table

RTCP	Real-Time Transport Control Protocol

RTP	Real-Time Transport Protocol

RTSP	Real Time Streaming Protocol

S/PDIF	Sony Philips Digital Interface (for digital audio)
SAP	Session Announcement Protocol
SBR	Spectral Band Replication (regarding HE-AAC audio)

SCART	Syndicat des Constructeurs d'Appareils Radiorécepteurs et Téléviseurs

(video/audio interface)

SD&S	Service Discovery and Selection

SDL	(Mobile communication network) Supplemental Downlink

SDT	Service Description Table

SDTV	Standard Definition Television

SFN	Single Frequency Network

SFR	Standard Frame Rate (here up to 50 frames/s)

SDR	Standard Dynamic Range

SI	Service Information

SMATV	Satellite Master Antenna Television

SNTP	Simple Network Time Protocol
SPTS	Single Programme Transport Stream

ST	Stuffing Table

STB	Set-top box (IRD without display)

SW	Software

TCP	Transmission Control Protocol

TDT	Time and Date Table

TFS	Time Frequency Slicing

TFTP	Tunnelling File Transfer Protocol

TOT	Time Offset Table

TPS	Transmission Parameter Signalling
TRS	Tip Ring Sleeve

TR	Tone Reservation

TS	Transport Stream

TV	Television
TVA	TV Anytime

UHDTV	Ultra High Definition Television

UHF	Ultra-High Frequency

uimsbf	unsigned integer most significant bit first

UTC	Universal Time, Co-ordinated

VCR	Video Cassette Recorder

VHF	Very-High Frequency

VHS	Video Home System

VoIP	Voice over IP

VPN	Virtual Private Network

VSB	Vestigial SideBand

xDSL	x Digital Subscriber Line

XML	Extensible Markup Language

# General Features of the NorDig IRD

## General

The NorDig Unified specification text relates to all NorDig profiles and type of IRDs unless otherwise specified.

All requirements specified in this document are mandatory unless otherwise specified.

It should be noted that compliance with the NorDig requirements will require full compliance with at least one of the specified NorDig IRD configurations with a capability profiles (Basic, HbbTV, PVR, or HEVC or Hybrid) and a variant (frontend: types (satellite, cable, terrestrial or IPTV and type: STB or iDTV).

## Introduction

This chapter describes the overall structure of the NorDig IRD specification. The detailed requirements are specified in the chapters 3 – 16, except for general product requirements that are specified in section 2.5.

The IRD implements the services by a combination of hardware and software solutions. The IRD may correspond to a decoder (STB) or an integrated digital TV-set (iDTV), including a display. The main functional blocks are shown in Figure 2.1.

The IRD includes a bootloader as firmware. The bootloader can upgrade all resident system-software and application software in the IRD by new software loaded either via the distribution channel or locally.

The software solution is only restricted by the hardware programming interface, i.e. the hardware functionality, capacity and performance.

Figure 2.1 Basic IRD architecture

The IRD will be provided with an installed front-end, with a cable or satellite or terrestrial Tuner & Demodulator, and/or a front-end for IP-based networks, a Common Interface and/or a Smart Card Interface. The IP-based interface may be used for reception of broadcast signals and as an input/output for the interaction channel (not relevant for Basic IRDs), these and other external interfaces are shown in Figure 2.2.

The user shall be able to access the services from all the tuners by means of the remote control.

## IRD Hardware and Firmware

### Overview

The IRD hardware and firmware consists of a number of functional blocks as outlined in Figure 2.2. The IRD developer is free to decide on the hardware architecture as long as it fulfils the NorDig requirements for the relevant profile.

Figure 2.2 Functionality of Hardware and Firmware for NorDig IRD

### RF Interface and Tuner/Demodulator

The RF interface connects to the incoming modulated signal. The tuner/demodulator block performs channel (frequency) selection, demodulation and error correction of the incoming signal. Output from the tuner/demodulator block is a transport stream that is fed to the demultiplexer block, or – if present – the external plug-in conditional access (CA) module. One embedded tuner/demodulator block is required, for cable, satellite or terrestrial input. The satellite tuner/demodulator block controls the frequency band selection of the external RF unit and supplies power to it.

The RF-interface is not relevant for IRDs intended for IP-based networks, where the front-end functions are performed by the Interaction Channel Interface, see below.

All channel selections in the T/D blocks are controlled by the central Controller unit. See also
chapter 3.

### Rfin-Rfout Bypass (option)

Rfin-Rfout is an internal bypass from input to output of IRD. See also section 8.2.

### Two-way Interface

The two-way interface connects to the IP-based network. It allows the user to access (two-way) interactive services, see chapter 15 and IP-based services. See also section 8.3.

The two-way interface is not relevant for NorDig. However, the two-way interface will act as front-end interface for reception of multicast signals in case of IPTV, where IPTV is defined as a DVB-transport stream encapsulated in IP packages multicasted or unicasted over an IP-network, see sections Error! Reference source not found. and 8.3.

The two-way interface to the IP-based network may also be used by the NorDig IRD to download new software.  This is the most appropriate mechanism for software updates that are applicable to IP-based services, as they are immediately available to NorDig IRDs that are connected to the IP-based network.

### Demultiplexer

The demultiplexer block synchronises with the transport stream coming from the tuner/demodulator, the interaction channel (in case of IP front-end) or the CA module, and selects the appropriate audio, video and/or private data elementary streams according to the service selections made by the user. The demultiplexer block also contains functions related to descrambling of services that are subject to conditional access data in the smart card. The private data streams are managed by the IRD controller unit (main processor), while the audio and video streams are output to the Video/Audio decoder block. See also chapter 4.

### Video/Audio Decoding

The audio and video decoding units recover the audio and video signals from the input elementary packet streams. This involves processes like descrambling, de-packetisation, decompression, synchronisation with related services, digital to analogue conversion, etc. The analogue signals are output to external baseband connectors while the digital signals are output to the HDMI/ S/PDIF interfaces. See also chapters 5 and 6 and clause 7.3.1 of HbbTV specification ETSI TS 102 796 [29].

### Graphics processor

The graphics processor unit generates graphics and text to be displayed for the user, see chapters 7 and 15

### IRD Controller Unit and System Software Update (Bootloader)

The IRD controller unit is a microprocessor system that manages all the internal units and all attached external plug-in units. See also chapter 10.

The Bootloader is a system software download capability, implemented as a firmware module independent of the system software. It can be initiated via the Navigator. See also chapter 13.

### Common Interface and Plug-in CA Module

The Common Interface is a transport stream input/output.

The Plug-in CA module is an external plug-in conditional access (CA) module to be attached via the Common Interface. The main task of the CA module is to perform descrambling of services subject to conditional access. The CA module may be connected to an external smart card. See also section 9.2.

### Smart Card Interface(s) and Smart Card Reader(s)

The smart card readers allow external smart card(s) to be connected to the Controller unit. See also section 9.3.

### Remote Control

The remote control allows the user to remotely interact with the IRD and its applications such as for example move cursors and graphical pointers and to make selections in menus displayed by the graphics processor. See also section 8.7.

The remote keyboard (option) allows the user to enter alphanumeric symbols in addition to the functions provided by the remote controls.

### Video/Audio Interfaces

HDMI and SCART (optional from 2014) interfaces. See section 8.4 and 8.6.

Note 1: Not relevant for iDTV. 
Note 2: Optional

### Audio Output Interfaces (option)

One analogue stereo audio output interface, based on RCA or SCART connector, see also section 8.4. 
One digital audio interface, based on S/PDIF, see also section 8.5.3.

### Main hardware/firmware functions-Overview per configuration

Table 2.1 indicates some of the major hardware/firmware functions in the IRD. A more detailed overview, which also includes the NorDig profiles, is given in Annex J. Detailed requirements are specified in chapters 3-16.

|  |  |  | HEVC IRD |  |
| --- | --- | --- | --- | --- |
| NorDig IRD | STB | iDTV | STB | iDTV |
| Video decoding/processing |  |  |  |  |
| MPEG-2 MP@ML SDTV video | M | M | M | M |
| MPEG-4 AVC HL@L4 SDTV + HDTV video | M | M | M | M |
| MPEG-H HEVC HDR SFR UHDTV (HLG10 + PQ10) |  |  | M | M |
| Audio decoding/processing |  |  |  |  |
| MPEG-1 Layer II audio decoding | M | M | M | M |
| HE-AAC Level 4, including down-mix to stereo | Alt (5) | Alt (5) | Alt (5) | Alt (5) |
| HE-AAC-to-AC-3 or DTS for digital output (1) | Alt (5) | Alt (5) | Alt (5) | Alt (5) |
| AC-3 (AC-3 pass-through) digital output (1) | Alt (5) | Alt (5) | Alt (5) | Alt (5) |
| E-AC-3 , including down-mix to stereo | Alt (5) | Alt (5) | Alt (5) | Alt (5) |
| E-AC-3 (E-AC-3 to AC-3) digital output (1) (4) | Alt (5) | Alt (5) | Alt (5) | Alt (5) |
| Subtitling |  |  |  |  |
| DVB (SDTV) subtitling | M | M | M | M |
| DVB HDTV Subtitling | M | M | M | M |
| EBU Teletext subtitling (subtitling pages) | M | M | M | M |
| Teletext and API |  |  |  |  |
| EBU Teletext (normal pages) | M | M | M | M |
| HbbTV | O (6) | O (6) | M | M |
| CA |  |  |  |  |
| Embedded CA | (2) | (2) | (2) | (2) |
| Interfaces |  |  |  |  |
| DVB-C front-end for CATV IRDs | M | M | M | M |
| DVB-S front-end for DTH IRDs | M | M | M | M |
| DVB-S2 front-end for DTH IRDs | M | M | M | M |
| DVB-T front-end for DTT IRDs | M | M | M | M |
| DVB-T2 front-end for DTT IRDs | O(7) | O(7) | O(7) | O(7) |
| Two-way interface for IPTV IRDs | M | M | M | M |
| Analogue SD video output (SCART, component, composite, S-video) | R | O | R | O |
| HDMI with HDCP (3) | M | M/R(3) | M | M/R(3) |
| Digital Audio Output (e.g. SPDIF) (1) | R/O | R/O | R/O | R/O |
| Common Interface Plus for CA | (2) | M(2) | (2) | M(2) |
| Smartcard Interface for embedded CA (2) | (2) | (2) | (2) | (2) |
| M; Mandatory, R; (Highly) Recommended, O; Optional item to include, Alt; minimum one among several options 1) If IRD is equipped with a digital audio output (like S/PDIF), see section 8.5. 2) As specified by relevant network/CA-operator, see chapter 9. Common Interface Plus is mandatory for for iDTV-sets with screen diagonal larger than 30 cm and highly recommended for iDTV-sets with smaller screen diagonals, see section 9.2. 3) Interfacing an (DigitalEurope) HD Ready approved TV Display set and an external STB. HDMI input is mandatory for iDTV-sets with screen diameters larger than 30 cm and highly recommended for iDTV-sets with smaller screen diameters, see section 9.2. 4) E-AC-3 is not defined for S/PDIF output, instead an ‘E-AC-3 to AC-3’ conversion is expected for the S/PDIF output to ensure interoperability with legacy A/V receivers. For newer A/V receivers supporting E-AC-3, HDMI interface shall be used 5) At least one advanced audio decoder shall be included, see details in section 6.1. 6) Optional for NorDig Basic IRDs with a basic profile, mandatory for NorDig HbbTVHybrid IRDs and NorDig HEVC IRDs 7) May be mandatory in some terrestrial networks, ref operator’s requirements |  |  |  |  |

Table 2.1 Main hardware/firmware functions for the various IRD configurations

### Additional hardware/firmware for the PVR features

The NorDig PVR will include embedded or attached hardware/firmware (not shown in Figure 2.1) for recording of live services (TV, radio etc) in persistent memory (like HDD) for later playback, (even if the IRD has been completely powered off between the recording and the playback), see section 14.3.

## System Software and API

### Introduction

The NorDig software may contain two main parts, system software and applications (not relevant for NorDig Basic IRDs). The system software shall provide two main sets of functions. One set is accessible within the system software only and includes functions for control of hardware/firmware and handling of service information. Another set is available internally and externally for applications, and constitutes the Application Programming Interface, API (not relevant for NorDig Basic IRDs). See also chapter 15.

### Principal Software Architecture

An important feature in this software architecture is the possibility of replacing the whole software, with exception of the bootloader software itself. This allows the exchange or upgrade of the entire software 'over the air' or though the IP-based network according to the need for new functionality or for bug fixing (e.g. drivers).

The download of applications uses an internal function from the API, outside of the bootloader software.

Figure 2.3: Possible software architecture of a NorDig , to the left a NorDig Basic IRD without NorDig API and to the right a NorDig HbbTVHybrid IRD with NorDig API (i.e. HbbTV API).

Figure 2.3 illustrates only examples of NorDig IRD software architecture. The IRD manufacturers are free to implement system the way they want as long as it fulfils the NorDig IRD specification.

### System Software

The NorDig IRD includes a System Software in compliance with DVB specifications, i.e. APIs, PSI/SI (1), Navigator, teletext, subtitling and Common Interface. The system software can be completely upgraded via the bootloader (2).

Note 1: 	The NorDig IRD with an IP-based front-end will be based on a modified use of the DVB service information (SI), see section 12 and Annex C.

Note 2:	The bootloader is by definition a part of the hardware/firmware.

### NorDig APIs

The NorDig HbbTV IRD includes an open API in compliance with the HbbTV APIs (not relevant for NorDig Basic IRDs).

### PVR related software

The NorDig PVR includes additional software for handling of the PVR features, see chapters 12 and 14, and section 13.3.

## General Product Requirement

### General

The NorDig IRD shall satisfy all mandatory legal requirements, as specified for the European Union and by the relevant national authority.

### Energy Efficiency

The NorDig IRD should be energy efficient and minimise its power consumption during all modes of the IRD (Normal TV mode ("ON"), Standby mode etc). Manufacturers of NorDig IRDs are recommended to follow voluntary agreement(s) on energy consumption for complex set-top-boxes under the EU regulation and/or the European Commission's regulation (EC) No 1275/2008 implementing Directive 2005/32/EC of the European Parliament and of the Council with regard to ecodesign requirements for standby and off mode electric power consumption of electrical and electronic household and office equipment for their IRD products.

### Requirements that are optional for a time period (grace period)

NorDig normally introduce new mandatory requirements or changes to existing requirements with a certain time period from publication to when it becomes mandatory to support for new products, (typically via a note: “Optional for NorDig IRDs that are launched before <date>”). This time period is here referred to as a “grace period” for a requirement. The purpose with the grace period is to give IRD manufacturer a reasonable time to implement according to the new or modified requirements. A NorDig test lab refers here to a test lab that among things performs verification testing of an IRD’s implementation according to NorDig Unified IRD specification and NorDig Test Plan.

Unless an Operator or relevant network specifies anything else, the date in NorDig specification from when new IRD models must support a requirement, refers to the date relevant NorDig test lab(s) receive a new IRD (excl time for testing and any queue time). This means if an IRD is received at the relevant testlab(s) before a grace period date, the applicable requirement(s) is still optional even if the time for testing passes this date at the testlab.  Testlabs’ time for verification testing IRDs can sometimes vary significantly depending on the queue for testing etc.

### Definition of Persistent settings and Temporary changes to those settings

#### Definition and standard handling of Persistent setting

A persistent setting refers to a user preference setting (see Chapter 16) that is stored in persistent memory and therefore retains its value when changing service or when re-starting the NorDig IRD (e.g. power cycle). The setting’s persistent value becomes its default value.

Persistent setting may typically be accessed deeper in the IRDs menu structure and are typically generic for all installed services.

Note: as stated in Chapter 16.4 a reset to factory mode will overwrite any changes User(s) may have done to Persistent settings and reset settings to their initial factory default states.

#### Definition and standard handling of Temporary changes

A temporary change refers to a user preference (see Chapter 16) setting that retains its value only until changing selected service or re-starting the NorDig IRD. When a setting loses its temporary value, it shall (1) change back to its persistent value (i.e. default value).

Temporary changes are typically being intended to be accessed with a minimum of User Interaction steps (e.g. by use of dedicated remote control buttons).

Unless a specific temporary change states otherwise, settings shall (1) lose its temporary value (going back to its default value) at least when re-starting the NorDig IRD and should be lost when changing selected service (“zapping”).

Note 1: Optional for NorDig IRD released before 1 January 2018.

PART A: Hardware and Firmware

# The Frontend of the NorDig IRD

# MPEG-2 Demultiplexer

## General

The Demultiplexer shall be compliant to the MPEG-2 transport layer defined in ISO/IEC 13818-1[52]. The NorDig IRD shall support ETSI TS 101 154 [28] and the additional requirements stated below:

The NorDig IRD shall utilize the MPEG-2 Service Information as specified in Part B.

The NorDig IRD shall interpret the CA descriptor as defined in ETSI ETR 289 [24].

The NorDig SD IRD shall be able to decode an ISO/IEC 13818-1 [52] stream with data rates that include all rates up to that the front-end may deliver (1) as defined in chapter 3.

It should be possible to select one or many section-based data streams and output them as data on USB (if present).

The NorDig IRD shall be capable to utilise at least 32 elementary streams simultaneously, which requires 32 PID filters.

The NorDig IRD shall provide at least 32 section filters (2).

The NorDig IRD shall support variable bit rate elementary streams within a constant bit rate transport stream (excluding audio)

The NorDig IRD shall support a mixture of service types within the same ISO/IEC 13818-1[52] MPEG-2 transport stream (i.e. MPEG-2 SDTV service, MPEG-4 AVC SDTV and HDTV and Radio services may be multiplexed into the same transport stream).

Note 1: The satellite front-end may deliver up to 80.4 Mbps after error correction, see section Error! Reference source not found.,
note 1.

Note 2: This feature enables the NorDig IRD to utilise several components as video, audio teletext, SI, subtitling and data for additional services.

## DVB Descrambler Performance (for IRD with embedded descrambling)

The descrambler unit is based on the common scrambling algorithm as specified by DVB, see DVB A 011 [4]. Common Scrambling Algorithms versions 2 and 3 shall (1) be implemented in the NorDig IRD. The algorithms are available from ETSI (2). See also section 9. It shall (1) be able to descramble on transport level and on PES format. The NorDig IRD shall (1) be able to process in parallel up to at least 6 different streams (either PES or transport level) with different access conditions. Data streams without access control shall be bypassed by the descrambling unit.

Note 1:	See chapter 9. The DVB descrambler is mandatory for IRDs with a mandatory smart card interface for conditional access. The Common Scrambling Algorithm version 3 is mandatory for IRDs that are launched after 2011. Only mandatory for an IRD with embedded descrambling and a SmartCard Reader.

Note 2: ETSI acts as a neutral custodian for the distribution of the system information concerning the common scrambling system

## System Clock Recovery

During the system time clock (STC) acquisition audio and video shall be muted. (The transition shall be smooth and seamless when the customer changes the channel). The decoder shall be able to:

recover the STC using PCR with maximum jitter of  10 µs.

track long-term variations in the frequency of the encoder’s STC.

For each service, the demultiplexer shall recover the source clock by extracting the associated PCR values received within the incoming multiplex and insert them into the appropriate Phase Locked Loop.

# Video

## General requirements

The video decoder of the NorDig IRD’s video decoder shall fully comply with the DVB Implementation GuidelinesDVB specification for broadcasting applications for the use ofregarding “25 Hz MPEG-2 SDTV IRDs and Bitstreams”H.262/MPEG-2 SDTV, and “25 Hz H.264/AVC SDTV IRD and Bitstream” H.264/AVC SDTV and “25 Hz H.264/AVC HDTV IRD and Bitstream” HDTV video in satellite, cable and terrestrial broadcasting applicationsin, ETSI  TS 101 154 [28].

The video decoder of the NorDig HEVC IRD shall, in addition to above, comply with the DVB Implementation GuidelinesDVB specification for broadcasting applications for theregarding “HEVC HDR UHDTV IRD and Bitstream” in ETSI TS 101 154 [28].

Observe that in a future revision of the NorDig Unified Requirements, requirements regarding the “HEVC HDR HFR UHDTV IRD and Bitstream” will be added.

The following clauses of ETSI TS 101 154 [28] are relevant to this specification:

- 5.1; “25 Hz MPEG-2 SDTV IRDs and Bitstreams”

- 5.5; “Specifications Common to all H.264/AVC IRDs and Bitstreams”

- 5.6; “H.264/AVC SDTV IRDs and Bitstreams”. The minimum profile requirements for NorDig IRD for SDTV content is shall support High Profile at Level 3.0 bitstreams.

- 5.7.1; “Specifications common to all H.264/AVC HDTV IRDs and Bitstreams”.

- and 5.7.2; “25 Hz H.264/AVC HDTV IRDs and Bitstream”s,. The NorDig IRD shall support High Profile at Level 4.0 bitstreams.The minimum profile requirements for Nordig ID for HDTV content is High Profile at Level 4.0

- 5.14.1 “Specifications Common to all HEVC IRDs and Bitstreams”.

- 5.14.4 “HEVC HDR UHDTV IRDs and Bitstreams” (1). Support for both PQ10 and HLG10 is required for the NorDig HEVC IRD. Only support of 50 Hz and 25 Hz frame rates are required for the NorDig HEVC IRD.

Note 1: This implies support for the “HEVC HDR UHDTV Bitstream”, the “HEVC UHDTV Bitstream” and the “HEVC HDTV Bitstream”, excluding non-square pixel aspect ratios and excluding interlace scan.

### Video Decoder Reference Model for Video Decoder

External video inputs

Video Decoder

Decoder 
& Input Format Converter

+

SD
Format Converter

Decoder (Set-Top Box or decoder part of iDTV)

Decoder
Composition Output

Display Format Conversion

Display

Display (TV / iDTV)  iDTV

Graphics Plane

Graphics Plane

Graphics Planes

External video inputs

API Video Format Converter

Display

SD Display (TV)

HDMI

Graphic
Format
Converters

Video Decoder

Decoder 
& Input Format
Converter

+

Decoder - iDTV

Decoder
Composition Output

Display

Graphics Plane

Graphics Plane

Graphics Planes

API Video Format Converter

Graphic
Format
Converters

Video Decoder

Decoder 
Format Converter

+

Decoder - Set-Top Box

Decoder
Composition Output

Display

Graphics Plane

Graphics Plane

Graphics Planes

API Video Format Converter

HDMI

Graphic
Format
Converters

Figure 5.1 Video Decoder Reference Model for Video Decoder for the NorDig IRD

The Reference Model for Video Decoder Reference Model for NorDig IRD is illustrated in Figure 5.1. It is a logical reference model and does not imply any specific implementation of an actual iDTV or STB or iDTV.

## Supported resolutions and frame rates

The Video Decoder of the NorDig IRD shall (1) be able to receive and decode the resolutions according to ETSI TS 101 154 [28]: video formats specified in Table 5.1.

- Section 5.1 “25 Hz MPEG-2 SDTV IRDs and Bitstreams”, sub-section 5.1.4 “Luminance resolution”

- Section 5.6 “H.264/AVC SDTV IRDs and Bitstreams”, sub-section 5.6.2.3 “Luminance resolution”

- Section 5.7 “H.264/AVC HDTV IRDs and Bitstreams”, sub-section 5.7.1.4 “Luminance resolution”

The Video Decoder of the NorDig HEVC IRD shall, in addition to the capabilities of the NorDig IRD, be able to receive and decode the resolutions according to ETSI TS 101 154 [28]:

- Section 5.14.4 “HEVC HDR UHDTV IRDs and Bitstreams”, sub-section 5.14.4.3 “Luminance Resolutions”

- Section 5.14.3 “HEVC UHDTV IRDs and Bitstreams”, sub-section 5.14.3.2 “Luminance resolution” with the exception for non-square pixel aspect ratios

- Section 5.14.2 “HEVC HDTV IRDs and Bitstreams”, sub-section 5.14.2.2 “Luminance resolution” with the exception for non-square pixel aspect ratios and the exception for interlace scan.

## Supported frame rates

The Video Decoder of the NorDig IRD shall be able to receive and decode the frame rates according to ETSI TS 101 154 [28]:

- Section 5.1 “25 Hz MPEG-2 SDTV IRDs and Bitstreams”, sub-section 5.1.2 “Frame rate”

- Section 5.6 “H.264/AVC SDTV IRDs and Bitstreams”, sub-section 5.6.2.2 “Frame rate”

- Section 5.7 “H.264/AVC HDTV IRDs and Bitstreams”, sub-section 5.7.2.2 “Frame rate”

The Video Decoder of the NorDig HEVC IRD shall, in addition to the capabilities of the NorDig IRD, be able to receive and decode the frame rates according to ETSI TS 101 154 [28]:

- Section 5.14.1 “Specifications Common to all HEVC IRDs and Bitstreams”, sub-section 5.14.1.7 “Frame rate” (1).

- Section 5.14.4 “HEVC HDR UHDTV IRDs and Bitstreams”, sub-section 5.14.4.5 “Frame Rates” (1).

Support for other frame rates than 50 Hz and 25 Hz is optional in the NorDig HEVC IRD. For HEVC encoded video, only decoding of progressive scan video is mandatory.

Note 1: The specifications in section 5.14.1.7 shall apply with the restrictions in section 5.14.4.5, i.e. only progressive scan support is mandatory.

| Video Luminance Resolutions Hor. x Vert. (2) | Frame rate [Hz] | Progressive / Interlace | Source Aspect Ratio (Hor. : Vert) | Aspect _ratio _idc (AVC only) | Bit Stream (Profile@ Level) |
| --- | --- | --- | --- | --- | --- |
| 1920x1080 (3) | 25 | I | 16:9 | 1 | AVC HP@L4 |
| 1920x1080 | 25 | P | 16:9 | 1 | AVC HP@L4 |
| 1440x1080 | 25 | P / I | 16:9 | 14 | AVC HP@L4 |
| 1280x1080 | 25 | P / I | 16:9 | 15 | AVC HP@L4 |
| 960x1080 | 25 | P / I | 16:9 | 16 | AVC HP@L4 |
| 1280x720 (3) | 50 | P | 16:9 | 1 | AVC HP@L4 |
| 960x720 | 50 | P | 16:9 | 14 | AVC HP@L4 |
| 640x720 | 50 | P | 16:9 | 16 | AVC HP@L4 |
| 720x576 (3) | 25 | I | 4:3 16:9 | 2 4 | AVC HP@L3 & MPEG-2 MP@ML |
| 544x576 | 25 | I | 4:3 16:9 | 4 12 | AVC HP@L3 & MPEG-2 MP@ML |
| 480x576 | 25 | I | 4:3 16:9 | 10 6 | AVC HP@L3 & MPEG-2 MP@ML |
| 352x576 | 25 | I | 4:3 16:9 | 6 8 | AVC HP@L3 & MPEG-2 MP@ML |
| 352x288 (SIF) | 25 | I | 4:3 16:9 | 2 4 | AVC HP@L3 & MPEG-2 MP@ML |

Table 5.1 Mandatory Video Formats to receive and decode

## Video resolution scaling  Up-sampling/Up-converting

Up-samplingscaling of sub-sampled (sub-) resolutions of received video shall be made in accordance with ETSI TS 101 154 [28], i.e. sub-sampled (sub-) luminance resolutions in Reference Model Figure 5.1 shall be up-scaledconverted by the Decoder Format Converter into the full video raster of the Decoder Composition Output, see Reference Model Figure 5.1.

Regarding the NorDig STB, the video raster On the Decoder Composition Output, the outgoing signal's video raster shall eitherat all time be a manually chosen full video with a raster of  1920x1080, 1280x720 or 720x576 according to selected setting or a raster automatically selected via EDID-information as desired by the HDMI Sink (iDTV/display).

In addition to the raster resolutions above, the NorDig HEVC STB shall provide the raster 3840x2160.

Regarding NorDig For an iDTVs, all resolutions of received video shall internally be scaled to the native resolution of the screendisplay.

When up-convertingscaling video with an encoded luminance resolution of 720x576 or 704x576 to any 1:1square pixel aspect ratio format (e.g.i.e. 1280x720, or 1920x1080 or 3840x2160), only the centred 702 of the horizontal 720 / 704 pixels shall be used. Those 702 pixels correspond to the 52 microseconds of an active line, hence preserves correct geometry in the up-conversion process.

When up-convertingscaling other 576 line-basedvalid input line resolution formats to any 1:1 square pixel aspect (output) format (i.e. 1280x720, or 1920x1080 or 3840x2160), only the centred horizontal pixels shall be used; e.g. when up-converting (received) 544x576 line resolution format to any 1:1square pixel aspect ratio (output) format, only the centred 530 pixels of the horizontal 544 shall be used.

## Colorimetry

The NorDig IRD Decoder Format Converter shall use the VUI (Video Usability Information) parameters (ISO/IEC 14496-10) [56] colour_primaries, transfer_characteristics and matrix_coeffecients in received AVC encoded bitstreams and the Sequence Display Extension parameters (ISO/IEC 13818-2) [53] in MPEG-2 encoded bitstreams.

In addition to the NorDig IRD requirements above, the NorDig HEVC IRD Decoder Format Converter shall use the VUI (Video Usability Information) parameters (ISO/IEC 23008-2) [83] colour_primaries, transfer_characteristics and matrix_coeffecients in received HEVC encoded bitstreams.

It shall be assumed that bitstreams according to “HEVC HDR UHDTV Bitstreams using PQ10” (see chapter 5.14.4.4.3 in ETSI TS 101 154) [28] will provide the “Mastering Display Colour Volume SEI message” (see chapter 5.14.4.4.3.3.2 in ETSI TS 101 154) [28] and, for non-live programmes, the Content Light Level Information SEI message (see chapter 5.14.4.4.3.3.3 in ETSI TS 101 154) [28].

Hence, it is highly recommended that the NorDig HEVC IRD Decoder Format Converter makes use of the “Mastering Display Colour Volume SEI message” when adapting to the luminance and chrominance capability of the connected display. It is in addition recommended that the NorDig HEVC IRD makes use of the “Content Light Level Information SEI message” when available in the bitstream. (1)

### NorDig HEVC STB colorimetry

The Decoder Composition Output in NorDig’s Video Decoder Reference Model (see chapter 5.1.1) shall be advanced enough to perform all video format conversions (luminance-wise and chrominance-wise) needed to target legacy HDMI-sinks, as well as EDID-enabled adaption to the capability of the connected display, including HDR capability, described in ANSI/CTA-861-G [93]. The complete ANSI/CTA-861-G [93] shall be taken into account except the “6.10 Extended InfoFrame” and “6.10.1 HDR Dynamic Metadata Extended InfoFrame”. “HDR Dynamic Metadata” in general will be discussed in a future revision of the NorDig Unified Requirements, at the same time as requirements regarding the “HEVC HDR HFR UHDTV IRD and Bitstream” will be added.

When connected to a Sink (iDTV/display) of any HDMI-version, the HDMI 2.0b interface in-line with ANSI/CTA-861-G [93] will give the STB’s Video Format Converter necessary information regarding the desired colorimetry via EDID handshake. The EDID-information shall be used by the Decoder Format Converter of the NorDig HEVC STB to convert colorimetry. However, legacy SDR displays will signal their supported SDR video formats in priority without specifically signalling "Desired Content Max Luminance data" (see 7.5.13 “HDR Static Metadata Data Block” in ANSI/CTA-861-G [93]). Hence, shall the Decoder Format Converter of the NorDig HEVC STB output SDR video formats based on the HDR to SDR conversion methods described by the ITU, e.g. ITU-R BT.2390 and operational practises in HDR television production.

In the case of the High Dynamic Range (HDR) video format ITU-R BT. 2100/PQ [90], it is anticipated that the capabilities of professional reference monitors and consumer displays will evolve differently over time. Consumer displays may have lower luminance and chrominancecapabilities than professional reference monitors. Hence, there is a need for both an initial display adaption and a subsequent consumer viewing environment adaption. The latter for example via user control of overall brightness and contrast. Leads on how to best perform the initial display adaption is derived from the conversion methods described by the ITU, e.g. ITU-R BT.2390 and operational practises in HDR television production.

### Programme production colorimetry – informative

Table 5.21 below gives the documentation regarding programme production where to find appropriate chromaticity co-ordinates, opto-electronic transfer characteristics and matrix coefficients to be used for example when deriving luminance and chrominance signals from the red, green and blue primaries (or vice versa, i.e. YCbCr to RGB):

| Active composition resolution in the “Decoder Composition Output” (equal to the production resolution) (Horizontal x Vertical) | Documentation regarding programme production colour parametersfor appropriate Colour Processing | Comments |
| --- | --- | --- |
| 720x576 | ITU-R BT.1700 [86] (replaces ITU-R BT.470 System B, G) and ITU-R BT.601 [84] | Note that 576 lines in both interlaced scan (576i) and progressive scan (576p) shall be processed and output with equal colour parameters. Standard Dynamic Range production parameters. |
| 1280x720 | ITU-R BT.1847 [87] (SMPTE 296M) | The colour parameters in SMPTE 296M are the same as in ITU-R BT.709 [85]. Standard Dynamic Range production parameters. |
| 1920x1080 | ITU-R BT.709 [85] (SMPTE 274M) | The colour parameters in SMPTE 274M are the same as in ITU-R BT.709 [85]. Standard Dynamic Range production parameters. |
| 3840x2160 | ITU-R BT.2020 [89] | Standard Dynamic Range production parameters. |
| 3840x2160 | ITU-R BT.2100 [90] | High Dynamic Range production parameters used for PQ10 and HLG10 by DVB. (1) |

Table 5.12 Documentation for production appropriate Colour Processingcolour parameters

Note 1: "Narrow range" and "Full range" is defined in TABLE 9, Digital 10- and 12-bit integer representation, in ITU-R BT.2100. DVB, in TS 101 154, is only specifying the use of "Narrow range".

## Dynamic changes in the video stream

The NorDig IRD shall be able to handle dynamic changes to of either the video codec or the video modeformat that may occur dynamically within the transmitted stream.  In the case of a codec change, this is defined to mean a change between MPEG-2 and H.264/AVC.

After a change of video codec between MPEG-2 and H.264/AVC, the IRD should automatically resume decoding and output of valid video within five seconds.

The NorDig IRD shall be able to handle dynamic changes in transmission between different video formats and frame rates (e.g. 720p50 to 1080i25/1080p25 and 576i25 to 720p50), including changes in encoded sub resolution (e.g. 720x576 to 544x576) within one second after receiving Random Access Point. (Random Access Point equals H.264/AVC RAP for H.264/AVC and Sequence header for H.262/MPEG-2).

The NorDig IRD shall adapt to changes in transmitted aspect ratio (e.g. 16:9 / 4:3) within one second after the reception. The transition shall cause minimal disturbance of the decoded service.

The NorDig HEVC IRD shall, regarding HEVC encoded bitstreams, in addition be able to handle dynamic changes in transmission between encoded (sub-) resolutions (i.e. 3840x2160 in steps down to 960x540) within one second after receiving Random Access Point, ideally without interruption. (Random Access Point equals HEVC DVB_RAP for H.265/HEVC).

## 5.6      AVC still picture

By still picture means broadcast of only intra coded frames at very low frame rate (typical 1 frame per second). The NorDig IRD shall decode this still picture frame and repeat displaying this until next (still picture) frame is available to display.

The NorDig IRD shall support still picture for the h.264/AVC profiles main and high.

The AVC still picture video stream is defined to consist of the following NAL (Network Abstraction Layer) units:

- Access unit delimiter (NAL unit type 9)

- Sequence Parameter Set (NAL unit type 7)

  - Fixed_frame_rate_flag set to 0

- Picture Parameter Set (NAL unit type 8)

- Supplemental enhancement information (NAL unit type 6)

- Coded slice of an IDR picture (NAL unit type 5)

- End of sequence (NAL unit type 10)

- (If needed) Filler data (NAL unit type 12)

For the signalling of the AVC still picture the AVC video-descriptor will be used (in PMT) as specified in MPEG-2 Systems (ISO/IEC 13818-1 [52] and TS 101154 v1.11.1 [28]. The flag AVC_still_present will be set to 1.

## MPEG-2 Minimum video bandwidth

The NorDig IRD shall provide support for very low bandwidth video.

For MPEG-2 video the NorDig IRD decoder shall be able to decode at bit rates down to 1.0 Mbps for video resolutions up to full Standard Definition resolution video (720x576).

The NorDig IRD shall be able to decode H.264/AVC video at bitrates down to 250 kbps for all resolutions up to 1920x1080. For AVC still picture the NorDig IRD shall be able to decode down to 100 kbps.

## Frame Cropping

The NorDig IRD shall support frame cropping for H.264/AVC encoded video. Frame cropping signalling is used to indicate which area of the encoded video that should be displayed.

For 1080 line formats, the video is encoded with 1088 lines. To indicate which area of the encoded video that should be displayed, frame cropping signalling may be used. If frame cropping information is included in the encoded video, this shall be used to decide which 8 lines should be hidden in the Decoder Composition Output. If no frame cropping signalling is available, the IRD shall crop the bottom 8 lines.

The NorDig HEVC IRD shall support “conformance cropping window” for H.265/HEVC encoded video according to ETSI TS 101 154 sections 5.14.1.3 “Sequence Parameter Set” and 5.14.1.5.1 “Aspect Ratio and Overscan Information”.

## Overscan

For services carrying H.264/AVC video, the broadcaster may use the overscan_info_present and overscan_appropriate flags to indicate whether the IRD (NorDig IRD and NorDig HEVC IRD) receiver should apply this typical overscan (e.g. by masking with black pixels or by additional cropping plus scaling), or should display the complete broadcast video image (after appropriate Frame Cropping, see Chapter 5.8 Frame Cropping). The flags will be encoded according to Table 5.23.

| overscan_info_present_flag | Overscan_appropriate_flag | Usage |
| --- | --- | --- |
| 0x0 or not broadcasted | n/a | No preferred display method |
| 0x1 | 0x0 | Important information in entire video regionframe |
| 0x1 | 0x1 | Decoded picture suitable for applied overscan |

Table 5.23 Broadcast overscan flag

Unless the user requests otherwise, NorDig IRDs shall interpret and follow the overscan flags according to Table 5.35.4.

| overscan_info_present_flag | overscan_appropriate_flag | Behaviour |
| --- | --- | --- |
| 0x0 or not broadcasted | n/a | Implementation dependent |
| 0x1 | 0x0 | Overscan not applied |
| 0x1 | 0x1 | Overscan applied |

Table 5.35.4 Receiver NorDig IRD and NorDig HEVC overscan behaviour

The NorDig STBs shall pass the video unaltered, i. e. without overscan related reformatting to its HDMI output, setting the bits in the AVI Infoframe (see CEA 861 [3]) in accordance with Table 5.45.

| overscan_info_present_flag | overscan_appropriate_flag | <S1,S0> (in HDMI AVI Infoframe) |
| --- | --- | --- |
| 0x0 or not broadcasted | n/a | <0,0> |
| 0x1 | 0x0 | <1,0> |
| 0x1 | 0x1 | <0,1> |

Table 5.45 Overscan signalling on the HDMI

Most displays have a user option where it will display the full frame of 1080 line based video formats without any overscan applied. It is recommended that the Nordig iDTVs support such user option to achieve one-to-one pixel mapping on 1920 x 1080 resolution displays. Note that the user, if overriding the received overscan flags, may not see a clean aperture as content producers cannot promise artefact free areas outside the Action Safe Area described in “EBU R 095 – Recommendation R 095,Safe areas for 16:9 television production [73].

For the NorDig HEVC IRD, in addition to above, regarding Overscan Information via Video Usability Information for services carrying H.265/HEVC video, see ETSI TS 101 154 sections 5.14.1.5.0 “General” and 5.14.1.5.1 “Aspect Ratio and Overscan Information”.

### Safe area for overscan

The amount of applied overscan shall not be in conflict with the broadcasted Action Safe Areas. Please refer to “EBU R 095 – Recommendation R 095, Safe areas for 16:9 television production [73], for appropriate guidelines.

## High Definition Video Output and Display

The NorDig STB and the NorDig HEVC STB shall be able to use the HDMI EDID information provided by the Sink (iDTV/display) to automatically determine automatically the STB output as specified in section 5.4 and 5.5, and as an alternative to accept enable a manual setting of the STB output, as specified in section 8.6.

The NorDig STB and the NorDig HEVC STB shall ensure that it can always present video (where available) for all services in the installed service list, regardless of the capabilities of the connected display.

For NorDig iDTVs the output video shall always be convertedadapted to the display’s native resolution.

## Restrictions on analogue video output Down-conversion of High Definition Video for Standard Definition output

If SCART, or any other analogue video output (Y, Pb, Pr, RF-PAL or CVBS) is available (1), the decoded High Definition video with a resolution higher than Standard Definition (576i/25), shall always be down-converted by the SD Format Converter to interlaced Standard Definition resolution forbefore output via these interfaces.

The Ddown-conversion of pictures shall be implemented, from any received of the valid incoming encoded HD full screen luminance resolution values, see section 5.2. (1920x1080, 1440x1080, 1280x1080, 960x1080, 1280x720, 960x720 and 640x720) to SD resolution (720x576).

When down-converting any 1:1square pixel aspect ratio format (i.ee.g. 1280x720 or 1920x1080) in the Decoder Composition Output to 720x576 resolution, the target shall be 702x576 pixels to be centred in the 720x576 grid with nine black pixels inserted as the start of the 720 pixel active line and nine black pixels inserted as the end of the 720 pixel active line.

Down-converted HD or UHD (2) video shall be displayed as 16:9 letterbox on 4:3 displays. 4:3 centre-cut is not an allowed display option, since this would limit the Action Safe Area in HD program production.

The conversionSD Format Converter should apply appropriate re-interlacing (field mode integration re-interlacing). It shall process and output 720x576i25 in 4:3 frame aspect ratio or 16:9 frame aspect ratio video with colours according to section 5.55.4 Colorimertry.

Note 1: The NorDig IRD is not required to be equipped with any analogue video output.

Note 2: UHD video resolution is only applicable for the NorDig HEVC IRD, not the NorDig IRD.

## Display of 4:3 aspect ratio content Format for other Aspect Ratios

The NorDig IRDs shall have methods to display 16:9 transmitted content on a 4:3 monitor. Likewise The NorDig IRDs and iDTVs shall have methods to display 4:3 transmitted SDTV content on a 16:9 monitor (with any resolution and colorimetry capability). The NorDig IRD shall be able to maintain full height 4:3 picture aspect ratio (pillar box) on a 16:9 display. Other display modes for 4:3 content are optional.

If SCART is available (1), tThe user shall have the ability to select appropriate aspect ratio for the analogue video output (SCART), see section 8.4.

| Transmitted Aspect Ratio | 16:9 Monitor | 4:3 Monitor (historical and optional) |
| --- | --- | --- |
| 16:9 | As original | 16:9 aspect ratio maintained (Letterbox). |
| 4:3 | Full hight 4:3 aspect ratio maintained (Pillarbox) | As original |

Table 5.6 Mandatory dDisplay modes

Note 1: The NorDig IRD is not required to be equipped with any analogue video output.

## Rescaling for HbbTV application

A NorDig HbbTV IRD Hybrid shall support rescaling as defined in HbbTV under “video scaling” minimum requirements in clause 10.2.1 of ETSI TS 102 796 [29]. These shall be supported for any of the valid incoming encoded full screen luminance resolution (see 5.2 for full screen luminance resolution values). The video shall be scaled, preserving the aspect ratio, and when applicable converted colorimetry-wise, such that all of the decoded video is visible within the area of the AV Control object or HTML5 video object. (See HbbTV requirements in ETSI TS 102 796 [29] Appendix E4).

A NorDig HbbTV IRD Hybrid shall support rescaling as defined in HbbTV under “video scaling” minimum requirements in clause 10.2.1 of ETSI TS 102 796 [29]. These shall be supported for any of the valid incoming encoded full screen luminance resolution (see 5.2 for full screen luminance resolution values). The video shall be scaled, preserving the aspect ratio, and when applicable converted colorimetry-wise, such that all of the decoded video is visible within the area of the AV Control object or HTML5 video object. (See HbbTV requirements in ETSI TS 102 796 [29] Appendix E4).

## Graphic compositing with HDR video

When receiving graphics as separate components (e.g. subtitling, HbbTV) in an HDR-based TV service, these graphics may target a small, perhaps undefined, legacy SDR-based colour volume. Hence may ambiguity occur, primarily luminance-wise, to which HDR-based video colours to map the SDR-based colours of the graphics in the composition, see the Video Decoder Reference Model in section 5.1.1.

Hence, the SDR to HDR conversion methods (and vice versa) for video described by the ITU, e.g. ITU-R BT.2390 and operational practises in HDR television production, should be studied as guidance also regarding colour volume conversion (both luminance and chrominance) of graphics.

# Audio

## General

Former NorDig Unified Specifications (v 2.2.1 and earlier) was separated into M2 Level and M4 Level in the audio chapter. The new NorDig Unified Specification v 2.3 has unified the M2 and M4 Level into one audio specification.

NorDig has defined three audio decoder formats:

- MPEG 1 Layer II, which refers to MPEG-1 Layer II up to stereo (2.0) channel decoding [55]. (Might be used for MPEG-2 based services).

- E-AC-3, which refers to E-AC-3 streams (including AC-3) up to 5.1 multi-channel decoding [35]. (Might be used for MPEG-4 based services).

- HE-AAC, which refers to MPEG-4 HE-AAC Level 4 (including AAC-LC) up to 5.1 multi-channel decoding [55]. (Might be used for MPEG-4 based services).

Regarding the audio decoder (“codec”), the NorDig IRD shall meet all the audio codec requirements specified by the network(s) for where the NorDig IRD is intended to be used, see below (i.e. the audio codec requirements might slightly differ from one NorDig network to another)

(Informative: Some NorDig networks has aligned all (MPEG-4) services into always using HE-AAC and some other networks has aligned all (MPEG-4) services into always using E-AC-3 or AC-3, this among other things to limit the number of audio decoders in the IRDs and in some cases also to save broadcast capacity. However, some NorDig networks are using HE-AAC for some (MPEG-4) services and E-AC-3/AC-3 for some other of their (MPEG-4) services).

In NorDig networks/regions where there is no single operator for acceptance of the IRDs, the NorDig IRD shall support:

- MPEG-1 Layer II and E-AC-3 and HE-AAC audio decoding.

In NorDig networks/regions where there is a single operator/regulator in charge for specifying the functionality of the IRD and ensuring that the minimum requirements are met, the operator/regulator specifies one of following minimum audio decoding format alternatives for the NorDig IRD to minimum support for that relevant network (i.e. always minimum two audio decoders):

- MPEG-1 Layer II and E-AC-3 and HE-AAC audio decoding,

- MPEG-1 Layer II and E-AC-3 audio decoding,

- MPEG-1 Layer II and HE-AAC audio decoding.

The Audio decoders shall fully comply with the DVB specification for broadcasting applications for the use of MPEG-2 Systems, Video and Audio in satellite, cable and terrestrial Broadcasting Applications ETSI TS 101 154 [28].

### Audio User Preference Settings

The NorDig IRD shall have User Preference Settings for audio functions as stated in section 16.2 (for example primary and secondary audio language, audio format, audio type, audio delay, etc).

### Audio terminology

“Normal” audio refers in this specification to audio streams that are:

- intended for the majority of users and users that are not interested in any supplementary audio and

- signalled in the supplementary audio descriptor by mix_type ‘1’ and editorial description ‘0’ and in the ISO 639 language descriptor with audio_type ‘undefined’ and language not set to ‘nar’.

A Supplementary Audio (SA) service may be either:

- Audio description: audio that includes a narration describing the action of the scene and is targeted at users with visual or cognitive impairments (see section 6.11 for more information).

- Spoken subtitling: audio that includes a spoken rendition of the subtitles and is targeted at users with visual or cognitive impairments (see section 6.11 for more information).

- Clean audio: functionality that provides improved intelligibility. It is targeted at users with hearing impairments, but can as well serve as improvement for listening in noisy environments (see section 6.10 for more information).

## Audio Decoding

For NorDig IRD’s User Preference Settings for audio functions see section 6.1.1                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                     .

The majority of customer installed base of home theatre systems are not able to decode the newer E-AC-3 and HE-AAC formats. Therefore, the NorDig IRD shall be able to “transcode” audio streams as stated in sections 6.2.2.1 and 6.2.3.1.

### MPEG-1 Layer II:  Requirements on Audio Handling

#### MPEG-1 Layer II: Decoding

The NorDig IRD shall support:

- decode MPEG-1 Layer II streams at all bit rates and sample rates listed in ETSI TS 101 154 [28].

The NorDig IRD should support:

- decode MPEG-1 Layer II streams at half-sampling rates (22.05 and 24 kHz).

#### MPEG-1 Layer II:  Audio Output

The NorDig IRD shall be capable of providing the following formats on the HDMI output connector from an MPEG-1 Layer II bitstream (see section 16 for factory default settings):

- Decoded to PCM stereo bitstream.

The NorDig IRD including an S/PDIF output shall be capable of providing the following formats on the S/PDIF connector from an MPEG-1 Layer II bitstream:

- Decoded to PCM stereo bitstream

### E-AC-3 and AC-3:  Requirements on Audio Handling

#### E-AC-3 and AC-3: All Pass-through, Decoding and Transcoding

NorDig IRD supporting E-AC-3 and AC-3 shall

- decode AC-3 streams at all bit rates and sample rates listed in ETSI TS 102 366 [35] (not including Annex E).

- (additionally) decode E-AC-3 streams with data rates from 32 kbps to 3 024 kbps and support all sample rates listed in TS 102 366 [35] Annex E.

- be capable of transcoding E-AC-3 bitstreams to AC-3 bitstreams according to TS 102 366 [35]. Transcoding to AC-3 audio streams shall be at a fixed bit rate of 640 kbps.

#### E-AC-3 and AC-3: Metadata

The NorDig IRD supporting E-AC-3 and AC-3 shall support the use of a complete set of Dolby metadata [35] embedded in the audio stream when decoding AC-3 or E-AC-3 bitstreams, transcoding E-AC-3 bitstreams to AC-3, or creating a PCM stereo downmix from a decoded E-AC-3 or AC-3 bitstream. (1) (2)

Note 1:	The E-AC-3 and AC-3 encoders always adds default metadata if they are not fed with metadata.

Note 2:	NorDig has the intension to include more information in the future in the NorDig Rules of Operation about the inclusion of metadata for E-AC-3 and AC-3.

#### E-AC-3 and AC-3:  Audio Output

NorDig IRDs supporting E-AC-3 and AC-3 shall be capable of providing the following formats on the HDMI output connector connector from an E-AC-3 or AC-3 bitstream (see chapter 16 for factory default settings):

- Pass-through of native bitstream (AC-3 and E-AC-3).

- E-AC-3 bitstream transcoded to AC-3 bitstream.

- Decoded and downmixed (if > 2 channels) to PCM stereo bitstream

The following formats should be provided for the HDMI output connector from an E-AC-3 or AC-3 bitstream:

- Decoded to PCM multichannel bitstream.

The NorDig IRD supporting E-AC-3 and AC-3 and including an S/PDIF output shall be capable of providing the following formats on the S/PDIF connector from an E-AC-3 or AC-3 bitstream:

- E-AC-3 bitstream transcoded to AC-3 bitstream

- Decoded and downmixed (if > 2 channels) to PCM stereo bitstream

- Pass-through of AC-3 bitstream

### HE-AAC:  Requirements on Audio Handling

#### HE-AAC:  All Pass-through, Decoding and Transcoding

NorDig IRDs supporting HE-AAC (and thereby also AAC-LC) shall be capable of:

- decoding HE-AAC Version 1 at Level 2 at sampling rates of 48 kHz according to ETSI TS 101 154 [28].

  - decoding, including downmixing HE-AAC Version 1 at Level 4 (multi-channel, up to 5.1) at sampling rates of 48 kHz according to ETSI TS 101 154 [28], Annex C (downmix).

  - transcoding HE-AAC Version 1 at Level 4 (multi-channel, up to 5.1) at sampling rates of 48 kHz according to TS 101 154 [28], Annex H to AC-3 or DTS.

NorDig IRDs shall be able to skip bitstream elements that are not recognized, i.e. unknown Fill elements and Data Stream elements.

If NorDig IRD is supporting HE-AAC audio stream transcoding to AC-3 audio stream, it shall be done according to TS 102 366 [35]. Transcoding to AC-3 multichannel audio streams shall be at a fixed bit rate of 640 kbps.

If NorDig IRD is supporting HE-AAC audio stream transcoding to DTS audio stream, it shall be done according to TS 102 114 [32] at a fixed bit rate of 1,536 Mbps.

#### HE-AAC: Metadata

The NorDig IRD supporting HE-AAC shall support decoding of HE-AAC bitstreams both with and without audio metadata. It is highly recommended that the broadcast includes metadata for all HE-AAC bitstream, (NorDig has the intension to include more information in the future in the NorDig Rules of Operation about the inclusion of metadata for HE-AAC).

The NorDig IRD supporting HE-AAC shall support the use of the following MPEG-4 AAC metadata embedded in the audio stream when decoding HE-AAC and transcoding HE-AAC multi-channel to AC-3 or DTS:

- Program Reference Level according to ISO/IEC 14496-3 [55] (prog_ref_level)

- Downmix Parameters according to "Transmission of MPEG4 Ancillary Data" part of DVB specification ETSI TS 101 154 [28] (center_mix_level, surround_mix_level)

- DRC Presentation Mode according to ETSI TS 101 154 Annex C.5.2.2.3 (drc_presentation_mode) (see note 1)

- Dynamic Range Control (DRC) according to ISO/IEC 14496-3 [55] (dyn_rng_sgn, dyn_rng_ctl)

- Heavy Compression according to ETSI TS 101 154 Annex C.5.2.5 (compression_on, compression_value)

The NorDig IRD capable of transcoding metadata to their output format shall not alter the level of the audio contained within the bitstream, shall pass all audio channels and shall transcode all metadata to the output format.

The NorDig IRDs that are transcoding the incoming audio with metadata to an output format without metadata, shall apply the incoming metadata before the transcoding.

For HE-AAC bitstreams without metadata the NorDig IRD shall interpret that the bitstream uses default metadata values (for decoding and any transcoding) according to ETSI TS 101 154 chapter C.5.2.8 Table C.31 ‘Persistance of MPEG4 ancillary data’. In the absence of Program Reference Level metadata, it shall be assumed to be -23 dBFS (for mono, stereo and multichannel audio).

It is up to the broadcaster to ensure that sufficient headroom and/or dynamic range control values are included in the transmission to prevent any overload when downmixing.

Note: According to MPEG-4 HE-AAC specification (ISO/IEC 14496-3 [55] section 4.5.2.7.3 “Persistence of DRC information”) audio metadata is applied immediately during decoding whenever it is changed.

#### HE-AAC:  Audio Output

NorDig IRDs supporting HE-AAC shall be capable of providing the following formats on the HDMI output connector from a HE-AAC bitstream (see chapter 16 for factory default settings):

- Transcoded to AC-3 or DTS bitstream.

- Decoded and downmixed (if > 2 channels) to PCM stereo bitstream.

NorDig IRDs supporting HE-AAC should be capable of providing the following formats on the HDMI output connector from a HE-AAC bitstream (see chapter 16 for factory default settings):

- Pass-through of native bitstream (HE-AAC) (1).

- Decoded to PCM multichannel bitstream.

Note 1:	HE-AAC over HDMI is defined in HMDI 2.0 specification.

The NorDig IRD supporting HE-AAC and including an S/PDIF output shall be capable of providing the following formats on the S/PDIF connector from a HE-AAC bitstream:

- Decoded and downmixed (if > 2 channels) to PCM stereo bitstream

- Transcoded to AC-3 or DTS bitstream

## Multichannel Audio

Multichannel audio is defined as an audio stream with more than two channels of audio.

The NorDig IRD should be able to output an audio stream as non-PCM encoded audio if there is a multichannel audio stream present for the chosen service in the incoming transport stream.

The multi-channel audio shall be decoded to a stereo downmix under the control of the format indicated in the preferred stereo down-mix parameter in the audio metadata, i.e. a matrix encoded compatible stereo downmix (Lt/Rt signal) or to a normal stereo down-mix (Lo/Ro signal), and shall be fed to the (stereo) analogue output connectors (SCART, RCA, headphones output and/or built-in loudspeakers if present).

If audio does not include any preferred stereo down-mix parameter in the audio metadata, the factory default method should be according to ETSI TS 101 154 [28], Annex C.5.2.4 (Lo/Ro) for AAC. For AC-3 and E-AC-3, there is always audio metadata included together with the audio bitstream.

The choice and priority between different stereo audio streams, and downmixed multichannel audio streams for the analogue outputs shall then be as specified in section 6.5 (Audio prioritising). When multichannel audio (AC-3, E-AC-3, HE-AAC) is present, it is recommended to show a visual indication.

See more details in section 6.5 Audio Prioritising and section 6.8.2 Digital Audio Output (HDMI, HDMI ARC and S/PDIF).

## Stereo Audio

The NorDig IRD shall be able to output an audio stream as PCM encoded audio if there is a stereo or mono audio stream present for the chosen service in the incoming transport stream. It is optional to transcode the incoming audio stream to AC-3 or DTS.

See more details in section 6.5 Audio Prioritising and section 6.8.2 Digital Audio Output (HDMI, HDMI ARC and S/PDIF).

## Audio Prioritising

The NorDig IRD shall be able to select audio stream according to user selections. In addition, the user shall be able to manually select between all audio streams that are associated with the active service, these settings should be stored in the IRD´s memory separately for each service.

If manually selected audio is not able to be stored in the IRD´s memory per service, a global setting should be made possible to set manual prioritisation of stream type for all services. The priority for the selected audio source shall be based on the user selections and audio stream shall be selected according to the priority list in table table 6.1 below. (See section 0 for requirements of audio user preference settings for the NorDig IRD).

If the user selections are not matching with the audio streams, the NorDig IRD shall always select one of the audio streams which closest suites with the user selections and will hereby provide audio to end-user.

The following Table 6.1 shows audio source priority based on the property of the supported audio streams and for different user preference settings of audio format and audio type as selected by the user, see section 16.4 for factory default user preference settings related to audio.

| IRD settings → |  | “Normal” audio mode |  | Supplementary audio mode |  |
| --- | --- | --- | --- | --- | --- |
|  |  | stereo mode (factory default) | multichannel mode | stereo mode | multichannel mode |
| IRD behaviour depending on above IRD settings |  |  |  |  |  |
| Property of priority for audio | Priority | Order of priority |  |  |  |
| Audio type | 1 (highest) | 1.1 “normal” 1.2 supplementary audio | 1.1 “normal” 1.2 Supplementary audio | 1.1 supplementary audio 1.2 “normal” | 1.1 supplementary audio 1.2 “normal” |
| Language | 2 | 2.1 audio match primay audio language settings 2.2 audio match secondary audio language settings 2.3 (if no match) “normal” audio 2.4 (if no “normal” audio) any audio | 2.1 audio match primay audio language settings 2.2 audio match secondary audio language settings 2.3 (if no match) “normal” audio 2.4 (if no “normal” audio) any audio | 2.1 supplementary audio match primay audio language settings 2.2 supplementary audio match secondary audio language settings 2.3 (if no match) “normal” audio 2.4 (if no “normal” audio) any audio | 2.1 supplementary audio match primay audio language settings 2.2 supplementary audio match secondary audio language settings 2.3 (if no match) “normal” audio 2.4 (if no “normal” audio) any audio |
| Audio format | 3 | 3.1 stereo 3.2 Multichannel 3.3 mono | 3.1 Multichannel 3.2 stereo 3.3 mono | 3.1 stereo 3.2 Multichannel 3.3 mono | 3.1 Multichannel 3.2 stereo 3.3 mono |
| Stream type | 4 (lowest) | 4.1 MPEG-1 Layer II 4.2 HE-AAC 4.3 E-AC-3 4.4 AC-3 | 4.1 HE-AAC 4.2 E-AC-3 4.3 AC-3 4.4 MPEG-1 Layer II | 4.1 MPEG-1 Layer II 4.2 HE-AAC 4.3 E-AC-3 4.4 AC-3 | 4.1 HE-AAC 4.2 E-AC-3 4.3 AC-3 4.4 MPEG-1 Layer II |

Table 6.1 Audio Priority between incoming audio streams where a lower number refers to higher priority.

### Audio Prioritising, Type (“Normal” or supplementary)

If “Normal” audio type is selected, the NorDig IRD shall only use audio stream of types other than “Normal” if no “Normal” audio is available at input for the selected service (see 6.1.2 for definition of “Normal” audio).

If Supplementary audio type is selected, the NorDig IRD shall prioritise Supplementary Audio streams for the selected service that has language matching user preference settings for primary and secondary audio language. If the selected service has no Supplementary Audio with a language matching IRD’s user preference settings for either primary or secondary audio language, the NorDig IRD shall instead priotise “Normal” audio (before any Supplementary audio with a language not matching audio language preferences).

### Audio Prioritising, Languages

If an audio stream according to the primary audio language preference is not associated with the chosen service the NorDig IRD shall automatically choose the audio stream according to the secondary audio language preference, if present.

### Audio Prioritising, Format (multichannel or stereo)

If multichannel mode is selected and if both multichannel and stereo streams are available for the selected language and audio type, the NorDig IRD shall use the multichannel audio stream to provide downmixed audio in analogue audio output(s), if applicable, and suitable digital bitstream format in digital audio output(s) as e.g. in examples in Annex G (Example table when more than one audio codec is received).

If stereo mode is selected and if both multichannel and stereo streams are available for the selected language and audio type, the NorDig IRD shall use the stereo audio source to provide audio in analogue audio output(s), if applicable, and PCM stereo in digital audio output(s) as e.g. in examples in Annex G (Guidelines for NorDig IRD audio selection: “Example table when more than one audio stream is received”).

If the NorDig IRD has an optional external audio system mode, then when that external audio system mode is selected as the primary audio output, the NorDig IRD shall follow user preferences and the priorities in Table 6.1.

### Signalling to be used for audio property

The NorDig IRD shall use the signalling information in the PMT to determine the audio property (language, audio type, audio format and stream type).

| Audio Property | Supplementary audio descriptor | AAC descriptor, Enhanced AC-3 descriptor, AC-3 descriptor | ISO 639 descriptor | PMT stream type | if no signalling/descriptor |
| --- | --- | --- | --- | --- | --- |
| Audio Language | 1st | - | 2nd | - | - |
| Audio type | 1st | 2nd | 3rd |  | ”normal” |
| Audio format | - | 1st | - | - | stereo |
| Stream type | - | 2nd | - | 1st | - |

Table 6.2 NorDig IRD priority inside each audio property between different signalling for audio streams and right column the IRD’s interpretation if no signalling available for an audio stream.

See Annex I for some example cases of priority between different incoming signalling for the audio property.

Note: The behaviour of the NorDig IRD depends upon this signalling in the PMT to make the audio priority decision, so it is expected that all broadcasts will include this signalling.

#### Signalling to be used for audio language

For the selection of audio language (see Table 6.1 and Table 6.2), the NorDig IRD shall use the ISO 639 lanaguage code from the supplementary audio descriptor and/or the ISO 639 language descriptor.

If both descriptors are available for the same audio stream, then the supplementary audio descriptor shall have priority.

#### Signalling to be used for audio type

For the selection of audio type (see Table 6.1 and Table 6.2), the NorDig IRD shall use the signalling from supplementary audio descriptor (mix_type and editorial_classification), AAC descriptor (AAC_type), Enhanced AC-3_descriptor (service type flags), AC-3 descriptor (service type flags) and/or the ISO 639 langauge descriptor (audio_type).

If several of these descriptors are available for the same audio stream, then the supplementary audio descriptor shall have highest priority, second priority the AAC/Enhanced_AC-3/AC-3 descriptor and lowest priority the ISO 639 descriptor.

#### Signalling to be used for audio format

For the selection of audio format (mono, stereo or multichannel, see see Table 6.1 and Table 6.2), the NorDig IRD shall use:

- the AAC_type field in the AAC_descriptor for AAC audio,

- the number of channels flags in the AC-3 descriptor and Enhanced AC-3 descriptor for AC-3 and E-AC-3.

In any case where, for some reason, this information is not carried in the PMT for a particular audio stream, then the IRD shall prioritise based on the assumption that that audio stream contains “normal” stereo content.

#### Signalling to be used for audio stream type

For the selection of audio stream type (see see Table 6.1 and Table 6.2), the NorDig IRD shall use stream_type in the PMT and any complementing descriptor (AAC, Enhanced AC-3 or AC-3 descriptor) to decide which audio codec the stream has.

### Examples of priority

Below are two examples aimed at explaining how the priority table shall be used. In general, the IRD’s priority decision is completed once a single stream is identified when moving down the priority table. Hence it can occur that e.g. 4th priority rules do not have to be applied because the streams to decode have already been identified.

Example 1: IRD is in stereo mode, two audio streams are available to the IRD, both have same language, normal hearing, and are stereo, stream A is coded using HE-AAC and stream B is coded using MPEG-1 Layer-II. IRD shall decode and playback MPEG-1 Layer II since, priority 1, 2 and 3 does not provide a preference between the two streams, but MPEG-1 Layer-II has higher priority than HE-AAC.

Example 2: IRD is in multi-channel mode, two audio streams are available to the IRD, both have same language, normal hearing, stream A is multi-channel coded using E-AC-3 and stream B is stereo coded using HE-AAC. IRD shall decode and playback the E-AC-3 stream since, priority 1, 2 does not provide a preference between the two streams, but the IRD is multi-channel mode and there is only one multi-channel stream; the E-AC-3 stream. In this example the stream type priority is not used since already in the audio format priority a single stream could be identified.

In case of multiple audio streams being broadcasted, a clarifying example table can be found in Annex G “Example table when more than one audio codec is received”, for how to select which audio stream to decode in multichannel or stereo user selected situation. That table is given by the property rules of Table 6.1.

## Audio Output Formats

The following Table 6.3 shows the expected output format based on the currently decoded audio and per digital audio out setting (multichannel mode or stereo mode) as selected by the user.

Please note that Table 6.3 defines audio output formats after audio priority has been made between incoming audio streams (see section 6.5 and Table 6.2 for audio priority between incoming streams).

| Currently Decoded Audio | Output on S/PDIF and HDMI (incl. HDMI Audio Return Channel) (see note 4) |  | Output on 2 ch analogue output(s) and/or built in loudspeaker(s) (see note 5) |
| --- | --- | --- | --- |
|  | When Stereo mode is selected | When Multichannel mode is selected |  |
| MPEG-1 layer II | PCM stereo | PCM stereo | Decoded MPEG-1 Layer II |
| HE-AAC stereo | PCM stereo | PCM stereo or optionally HE-AAC stereo on HDMI (see note 3) HE-AAC transcoded to AC-3 or DTS on S/PDIF | Decoded HE-AAC |
| E-AC-3 stereo | PCM stereo | E-AC-3 on HDMI (see note 2) E-AC-3 transcoded to AC-3 on S/PDIF (see note 1) | Decoded E-AC-3 |
| AC-3 stereo | PCM stereo | AC-3 (see note 1) | Decoded AC-3 |
| AC-3 multichannel | Downmixed PCM stereo | AC-3 (see note 1) or PCM multichannel (see note 6) | Decoded downmixed AC-3 |
| E-AC-3 multichannel | Downmixed PCM stereo | E-AC-3 on HDMI (see note 2) E-AC-3 transcoded to AC-3 on S/PDIF (see note 1) PCM multichannel (see note 6) | Decoded downmixed E-AC-3 |
| HE-AAC multichannel | Downmixed PCM stereo | HE-AAC multichannel on HDMI (see note 3) HE-AAC transcoded to AC-3 or DTS on S/PDIF PCM multichannel (see note 6) | Decoded downmixed HE-AAC |

Table 6.3Audio Output formats

Note 1:	The S/PDIF output shall in any case comply with the content of the table above. For HDMI however, the following feature should be implemented:

When an HDMI Sink device indicates in its E-EDID structure that it only supports Basic Audio (i.e. two-channel L-PCM from the original stereo signal or from a stereo down-mix from the multi-channel signal), then the HDMI output will provide Basic Audio. This feature would then take precedence over the requirement of AC-3, E-AC-3, HE-AAC multi-channel and DTS in the table above whenever the Sink device indicates that only Basic Audio is supported. Observe however that the HDMI output could be different from S/PDIF output, since S/PDIF still has to comply with multi-channel format requirements as in the table above.

Note 2:	If an HDMI sink device indicates in its E-EDID structure that AC-3 decoding is supported, but E-AC-3 decoding is not supported, the IRD shall transcode E-AC-3 streams to AC-3 prior to HDMI transmission.

Note 3:	If an HDMI sink device indicates in its E-EDID structure that AC-3 or DTS is supported, but HE-AAC decoding is not supported, the IRD shall transcode HE-AAC streams to AC-3 or DTS prior to HDMI transmission.

Note 4:	For IDTVs, this shall apply when External Audio System (if supported) is selected as audio output. If built-in loudspeakers are selected for audio output the minimum requirement is to have PCM output signal in the digital output.

Note 5:	When external audio system (if supported) is selected as main audio output, IDTVs may
optionally mute the TV speakers.

Note 6:	If IRD supports PCM multichannel over HDMI (not applicable for HDMI ARC).

## Audio Video Synchronisation

The NorDig IRDs shall not introduce more than 5 ms of relative delay between the audio and video components on the primary output (1) and not more than 5 ms between the primary video output and a secondary audio output. (2)

The requirement for the relative delay shall be met from 5 seconds after the start of decoding a service. The relative delay between the audio and video components shall be synchronised continuously using the service’s PCR and PTS values in the MPEG transport stream, resulting in maintaining correct synchronisation until a change of service.

If the NorDig IRD, as a part of an integrated digital TV set (IDTV) has an audio output, the audio shall be in sync with the video display.

Where audio leaves the IRD in an encoded form (such as in IEC61937 [43] outputs and/or HDMI outputs and/or HDMI Audio Return Channel (ARC)), the IRD shall compensate for the decoding latency of the selected audio format, as specified for the relevant reference decoder for the selected format (e.g AC-3), such that the output of the reference decoder would be 5 ms with respect to the decoded video. This applies for all audio systems that the IRD supports.

Note 1:	In this case, primary output for STB is primary HDMI and TV SCART, and for IDTV is the video display and internal loudspekars (see sections 8.5 and 8.6)

Note 2:	In this case, primary video output is the same as note 1. Secondary audio outputs for STB are i.e. secondary HDMI, VCR SCART, analogue RCA, and S/PDIF (see section 8.5). Secondary audio outputs for IDTV are i.e. SCART, analogue RCA, S/PDIF, HDMI ARC, headphone output, and “external audio system”.

### Adjustment of Audio/Video delay

The NorDig IRD (1) shall support the possibility to adjust the audio-delay on the HDMI and S/PDIF output (if available) at least up to 250 ms and it should be adjustable in 5 ms steps or smaller steps, as the IRD may have several different user set-ups, resulting in different a/v delays; e.g. the IRD may be connected to several types of external audio-amplifiers and the IRD may be connected to several types of external screens.

The NorDig IRD should also be able to automatically adjust the audio/video synchronization via HDMI (minimum) version 1.3 interface to compensate for delay times of video displays.

Note 1: Optional (recommended) for NorDig IDTV.

## Audio Output Signals

### Analogue Audio Output (SCART, RCA)

If the IDTV has an analogue stereo audio output on a pair of RCA connectors and/or headphone connector, the audio shall be in sync with the video display, see section  0.

If the NorDig IRD has analogue stereo output(s), it shall be capable of decoding and down-mixing the supported audio formats for the analogue outputs.

The NorDig IRD with SCART and/or analogue outputs shall always have an audio signal present on the analogue outputs (SCART and stereo out, see section 8.5 and to built-in loudspeakers (see note 1)) if any of the supported formats is received.

For the NorDig IRD with analogue audio outputs (SCART and/or built-in loudspeakers), it shall be possible to control the audio level on the analogue outputs primarily used for TV viewing (TV SCART and built-in loudspeakers) with the remote control unit and buttons on the front panel (if present). It should be possible to control the audio level on other analogue audio outputs (like RCA, headphone etc) using the same controls.

Note 1: 	For IDTVs that provide a setting to choose the audio output between the built-in TV speakers and the external audio system, it is optional to have signal in the built-in TV speakers when the external audio system setting is selected. The reasons for this are that when the user selects output to an external audio system as the priority, then the audio levels and compression settings are optimized for an external audio system and may not be suitable for output to TV speakers (e.g. lower dialogue level, high dynamic range)

### Digital Audio Output (HDMI, HDMI ARC and S/PDIF)

When the NorDig IRD is part of an integrated digital TV set (IDTV) that cannot play out a complete multi-channel audio signal, it should have an S/PDIF output and/or an HDMI Audio Return Channel (HDMI ARC) output, see sections 8.5 and 8.6.

The NorDig STB shall have an HDMI output and should have an S/PDIF output, see sections 8.6.3 (HDMI audio) and 8.5.3 (S/PDIF).

For NorDig STB, where only PCM audio will be carried on the HDMI output primarily used for TV viewing (e.g. stereo only HDMI sink (2), selection of “Stereo mode” by the user), it shall be possible to control the audio level on this HDMI with the remote control unit and buttons on the front panel (if present). Where only PCM audio will be carried on other digital audio outputs (S/PDIF, secondary HDMI port etc), it should be possible to control the audio level on other digital audio outputs using the same controls.

Where audio leaves the NorDig IRD in an encoded format, the NorDig IRD shall compensate for the decoding latency of the selected form, as specified in section 0.

The digital audio output as defined in section 8.5.3 (S/PDIF) and 8.6.3 (HDMI) shall always give either a valid PCM-output according to IEC 60958 [42] or a non-PCM audio bit-stream according to IEC 61937 [43]. The user shall be able to choose between the following storable output modes on the digital audio output interface (1):

- Forced PCM output according to IEC 60958 [42] (Part1 General, Part 3 Consumer).

- Non-PCM audio bit-stream output according to IEC 61937 [43]  when present -and if not present output PCM according to IEC 60958 [42]. Non-PCM audio bitstream formats like AC-3, and DTS shall be possible to order and enable/disable according to priority set by the user.

This chapter is also valid for HDMI ARC, since this interface is specified the same as for S/PDIF interface. Observe that HDMI supports more stream types than S/PDIF.

See sections 6.3, 6.4 and 6.5 for more details.

Note 1: 	The IRD’s different digital audio outputs may use use different audio output formats, for example
 	HDMI using PCM stereo and S/PDIF using non-PCM audio bit-stream (like AC-3 or DTS).

Note 2:	A “stereo only HDMI sink” refers here to a downstream device which the STB is connected to and which can only render stereo audio, for example a TV set without a multichannel decoder.

## Dynamic Changes

The NorDig IRD shall be able to handle dynamic changes of audio component(s) (PID/PIDs) in a service. The IRD shall automatically identify if an audio component is added or removed between two programme events in the same service). The NorDig IRD should gracefully handle change of service or audio format at the audio outputs without significant disturbances to the end user.

The NorDig IRD shall be able to handle the following dynamic changes without user interaction and start decoding within one second after reception of a change (like PMT update, elementary stream header signalling);

- change of number of audio channels (still same audio codec), (for example from mono to stereo to multichannel, from multichannel to stereo to mono)

- change of bitrate for an audio component (for example from 192 kbps to 160 kbps)

- change of audio PID value (typical examples are during regional news insertions) (case SVT regional news insertion), the NorDig IRD shall use next preferred audio component/PID.

- removal of one audio component/PID, the NorDig IRD shall use next preferred audio component/PID (for example; a service broadcasts two or more audio components/PIDs and then removes one of them, like removal of AC-3 during regional news broadcast). (case SVT regional news insertion)

- addition of one audio component with higher preferred user settings (for example; a service adds an audio component/PID, like add AC-3 audio component, after a regional news broadcast). (case SVT regional news insertion)

The NorDig IRD shall handle the dynamic changes after change of selected service ("zapping") or dynamic PMT update (i.e. shall not require to re-install services) and shall be able to handle the following dynamic changes without user interaction and start decoding within one second after reception of change;

- change of the audio codec, (for example change from MPEG-1 Layer II into AC-3)

- change of ISO 639-2 [70] language for an audio component.

The NorDig IRD shall be able to read the audio information contained in the DVB_SI stream_content and component_type of the component descriptor as defined in EN 300 468 [15], see also chapter 12 and section 13.3.2. The NorDig IRD should be able to present the audio information, including the descriptors for audio description for the visually impaired and audio for the hard of hearing, contained in the component descriptor to the user for information and selection purposes.

## Clean Audio

The NorDig IRD should support “Clean Audio” which gives improved intelligibility by enhancing the dialogue audio of a multichannel audio service.

Today NorDig Clean Audio is limited to receiver mixing/processing of the “Normal” audio without broadcast control metadata (i.e. no AD_descriptor in the PES_private data). This is achieved by implementing an easy way to adjust the balance of the “Normal” multichannel audio between the centre channel and other channels in the audio stream before making a stereo down-mix (when used to boost the center channel compared to other channels via attenuation of the other channels). This also means that for this alternative the “Normal” audio does not include any supplementary PSI/SI signalling.

If implemented, clean audio shall be compliant to ETSI TS 101 154 Annex E4 [28], referring to the “Optional user control of mix of speech and background”

Note: 	NorDig will consider mandating this function when A/V decoder chipsets are available 	with this capability integrated within the audio decoder

## Supplementary Audio

### Informative for Supplementary Audio

A supplementary audio service (as defined in ETSI TS 101 154 [28]) is specified below for the “in-service delivery”, and applies when “normal” audio streams and the supplementary audio streams are available within the same service (i.e. listed within same PMT).

A Supplementary Audio (SA) service may be broadcasted as either:

- “Broadcast mixed” (1):  pre-mixed audio by the broadcaster where the Supplementary Audio stream is a complete self-standing audio which contains both the “normal” audio mixed together with the supplementary audio content.

- “Receiver mixed”: audio containing only the supplementary audio content which is not a complete self-standing audio and is not intended to be presented on its own. The receiver mixed supplementary audio and “normal” audio is typically mixed together inside the IRD, under some control of the broadcaster (mixing level).

IRDs currently in the market are known to handle supplementary audio in a variety of ways, and there are some cases where legacy IRDs are even “disturbed” by the presence of Supplementary Audio. To mitigate this and avoid unwanted behaviour, some Networks use special signalling for the Supplementary Audio. This means for example that in some networks a broadcast pre-mixed supplementary audio may be signalled in the ISO639 descriptor as “normal” (‘undefined’) audio type but with language ‘nar’, or a receiver mix supplementary audio may be signalled in the ISO639 descriptor as ‘visual impaired’ audio type but with a different language to that of the associated “normal” audio. Modern IRDs support the Supplementary Audio Descriptor however, and hence avoid the problems associated with legacy signalling.

Note 1: 	Broadcast mixed refers to when the audio stream is signalised as supplementary audio via
visual impaired and/or via other supplementary signalling in AC-3/E-AC-3/AAC
descriptor and/or supplementary audio descriptor. This means that for the case when the 
audio stream is signalised with ‘nar’ language code in ISO639 descriptor and no other supplementary signalling, IRD may have different settings for, for example via primary audio language preference setting to ‘narrative’.

### General requirements for supplementary audio

The Nordig IRD audio decoder shall be capable of supporting ‘visual impaired’ Supplementary audio (SA) services, as defined in ETS TS 101 154  [28] (however, control of pan and fade is optional).

The NorDig IRD shall (1) support both Broadcast mixed and Receiver mixed Supplementary Audio.

### IRD settings for Supplementary Audio

#### IRD user preference settings for Supplementary Audio (1)

The user shall be able to enable and disable Supplementary audio and to change default, (see section 16 for IRD factory default settings).

The NorDig IRD user selection of audio preferences for ‘normal’ and ‘Supplementary’ audio (see 6.1.1) shall be a fixed setting (i.e. remain when changing service and when re-starting the IRD). (The wording for supplementary audio may typically be ‘Audio Description’, ‘AD’, referring to all versions of supplementary audio types).

#### Temporary selection for Supplementary Audio

The NorDig IRD should have alternative to easy but temporary select supplementary audio, e.g. via an audio key or an AD key on the remote control (i.e. a temporary selection that does not change the IRD’s stored user preference settings). This temporary selection of ‘Audio Type’ shall not remain when changing service, language, stream type or when re-starting the IRD.

#### Audio priority for Supplementary Audio

Enabling a Supplementary Audio mode shall change the ‘Audio Type’ priority (see section 6.5). If no Supplementary audio streams are received, then the NorDig IRD shall use “normal” audio from the same selected language.

If several supplementary audio streams are available for the same language and the same IRD user supplementary audio preference settings (for example both Audio description and Spoken Subtitling), then the IRD shall by default prioritise the audio that is first listed in the PMT.

#### Display of available Supplementary audio streams

The IRD should present information to the user if the service has a supplementary audio available, for example in “info banner” after selecting such a service.

If the IRD offers the option to select a supplementary audio track as a temporary selection, then the IRD should indicate that this is of a supplementary audio type together with its language.

#### Audio mixing level

The NorDig IRD shall be able to adjust the relative audio mixing level between the incoming receiver mixed supplementary audio and the “normal” audio.

This relative adjustment of audio mixing level should be able to both enhance and reduce the supplementary audio level compared to the “normal” audio before mixing depending on user settings.

### Selection of audio streams

The NorDig IRD shall be able to select the correct audio type stream according to user preference settings, see section 6.5.

The NorDig IRD shall support to by default select the “normal” audio stream for services which has several audio streams with different audio type for selected language.

When the user has selected Supplementary Audio as preferred audio, the NorDig IRD shall support to by default select the supplementary audio stream for services which have several audio streams with different audio type for selected language.

Observe that the Supplementary Audio descriptor overrides the language and audio type stated in the ISO639 descriptor, see section 6.5.4.

### Signalling for Supplementary Audio

All Supplementary Audio streams (both Broadcast mixed and Receiver mixed) will be signalled by the broadcaster by means of the Supplementary Audio descriptors and for services with several audio streams (e.g one “normal”/ plus one Supplementary Audio stream) also an ISO 639 language descriptor.

Broadcast mixed Supplementary Audio streams will in ISO 639 descriptor use audio type 0x00 ‘undefined’ (to avoid issues in legacy IRDs).

Receiver mixed Supplementary Audio streams will in ISO 639 descriptor use audio type 0x03 ‘visual impaired’ or 0x00 ‘undefined’ (to avoid issues in legacy IRDs).

See section 6.5.1 for priority between different descriptors for audio streams and section 12.6 for more information about the descriptors for audio.

### Receiver mixing

The NorDig IRD shall support Receiver mixed Supplementary Audio when the two audio streams are of the same codec family, sampling rate and on two different PIDs. The NorDig IRD shall support receiver mixing of Supplementary Audio services encoded as MPEG-1 Layer II, HE-AAC and E-AC-3 coding standards, where those codecs are supported.

The NorDig IRD is not required to support receiver mixing when “normal” and Supplementary Audio are broadcasting within the same E-AC-3 stream/PID.

The NorDig IRD shall be able to mix the “normal” audio stream together with the supplementary audio stream (‘visual impaired’).

The NorDig IRD shall not decode a ‘receiver mixed’ supplementary audio stream without also decoding and presenting it with the associated “normal” audio stream either mixed to the same audio output, or optionally presenting Supplementary Audio on its own on the headphones output while presenting the “normal” audio on the other audio outputs, see section 6.10.7.

#### Receiver mixed, Pan and fade control

For MPEG-1 Layer II and HE-AAC, the NorDig IRD should support pan and fade control of the receiver mixing via inband control metadata in audio stream as described in ETSI TS 101 154 Annex E [28] and for E-AC-3 as in TS 102 366 [35].

For MPEG-1 Layer II and HE-AAC, the supplementary audio stream may carry the AD descriptor in its PES_private_data, as defined in TS 101 154 Annex E.2. This AD descriptor provides the audio decoder with the information needed to control the mixing the supplementary stream with the main stream (pan and fade). For MPEG supplementary audio stream without AD descriptor in PES header, the NorDig IRD shall use mixing as central forward presentation (pan 0x00) and no fade (fade = 0x00).

For the E-AC-3 codec, the mixing data is carried in the supplementary E-AC-3 stream, as defined in TS 102 366 [35], and, for this codec, any AD descriptor shall be ignored.

### Receiver mixed on its own on for head-phones output

The IRD with head-phones output should also support presenting the receiver mixed Supplementary Audio on its own on the head-phones interface and at the same time presenting the “normal” audio on its own on the other main audio outputs (HDMI, S/PDIF etc).

The reason behind this alternative is that:

- other family members may not want to be disturbed by the Supplementary Audio, but still be able to watch a programme together with family members who are helped by the Supplementary Audio and/or

- use of a separate loudspeaker for the Supplementary Audio compared to the “normal” audio, which can help to localise the different audio types and thereby give better intelligibility.

### Broadcast mixed

The NorDig IRD shall support broadcast mixed supplementary audio services encoded as MPEG-1 Layer II, HE-AAC, AC-3 and E-AC-3 coding standards, where those codecs are supported.

## IRD Internal Reference Level

The level for reference tones for transmission will be 18 dB below clipping level, in accordance with EBU Recommendation R.68 ”Alignment level in digital audio production equipment and in digital recorders” as recommended by ETSI TS 101 154 [23].

## Loudness Levels, – Dynamic Range Control and Downmixing

To achieve loudness and dynamic range consistency over codec formats, the Nordig IRD shall follow the following guidelines on audio levelling and dynamic range control. A typical implementation for loudness levels on an IDTV-implementation is found in Annex H, “Loudness levels, – Typical IDTV Audio Block diagram”.

### HE-AAC Audio Input and AC-3/E-AC-3 Audio Input

The Nordig IRD shall, by default, be able to present the decoded HE-AAC /AC-3/
E-AC-3 bitstream at an average loudness level of -23 LUFS. Where prog_ref_level as of ISO/IEC 14496-3 [55] or dialnorm as of ETSI TS 102 366 [35] is included in the stream, it shall indicate the loudness of the audio essence contained within the bitstream using the numerically equivalent dBFS value (e.g. a measured loudness of -23 LUFS shall be indicated by a prog_ref_level / dialnorm value of -23 dBFS).

#### HE-AAC Decoding

For decoding to a Target Reference Level of -23 dBFS, the following applies:

If program level data of the form prog_ref_level as of ISO/IEC 14496-3 [55] is present in the bitstream, it shall be used; otherwise a prog_ref_level of -23 dBFS shall be used.

If drc_presentation_mode as of ETSI TS 101 154 Annex C.5.2.2.3 [28] is present in the bitstream and indicates DRC Presentation Mode 2, then any dynamic range control data of the form dyn_rng_sgn/dyn_rng_ctlas of ISO/IEC 14496-3 [57] also present in the bitstream shall be used and may not be scaled if they indicate negative gains. Otherwise if dynamic range data of the form compression_value as of ETSI TS 101 154 Annex C 5.2 [28] or of the form compression_value as of ISO/IEC 14496-3 [55] is present in the bitstream, it shall be used and it may be scaled.

It is expected that the broadcaster shall ensure that sufficient headroom and/or dynamic range control values are included in the transmission to prevent any overload when downmixing.

For NorDig IRDs that supports a multichannel audio output, the Nordig IRD should be able to decode utilizing a Target Reference Level of -31 dBFS. See section 6.13.3 for audio output levels.

For decoding to a Target Reference Level of -31 dBFS, the following applies:

If program level data of the form prog_ref_level as of ISO/IEC 14496-3 [55] is present in the bitstream, it shall be used; otherwise a prog_ref_level of -23 dBFS shall be used.

If dynamic range control data of the form dyn_rng_sgn/dyn_rng_ctl as of ISO/IEC 14496-3 [55] is present in the bitstream, it shall be used and it may be scaled.

If downmixing is performed and dynamic range data of the form dyn_rng_sgn/dyn_rng_ctl as of ISO/IEC 14496-3 [55] is present in the bitstream, those shall be applied without scaling if they indicate negative gains.

#### E-AC-3 and AC-3 Decoding

For decoding to a Target Reference Level of -23 dBFS, the following applies:

If program level data of the form dialnorm as of ETSI 102 366 [35] is present in the bitstream, it shall be used, if dynamic range control data of the form compr as of  ETSI 102 366 [35] is present in the bitstream, it shall be used and it may not be scaled.

It is expected that the broadcaster shall ensure that sufficient headroom and/or dynamic range control values are included in the transmission to prevent any overload when downmixing. For NorDig IRDs that supports a multichannel audio output, the NorDig IRD should be able to decode utilizing a Target Reference Level of -31 dBFS.  See section 6.13.3 for audio output levels.

For decoding to a Target Reference Level of -31 dBFS, the following applies:

Program level data of the form dialnorm as of ETSI 102 366 [35] shall be used.

If dynamic range control data of the form dynrng as of of ETSI 102 366 [35] is present in the bitstream, it shall be used and it may be scaled.

If downmixing is performed and dynamic range data of the form dyn_rng_sgn/dyn_rng_ctl as of ISO/IEC 14496-3 [55] or of the form dynrng as of of ETSI 102 366 [35] is present in the bitstream, those shall be applied without scaling if they indicate negative gains.

#### Summary

A summary of which levelling data and dynamic range control parameters shall be used in conjunction with which stream type target reference level and DRC presentation mode can be found in Table 6.4 below.

| Stream type | Target reference level | Levelling data | DRC Presentation Mode | Dynamic range control parameter |
| --- | --- | --- | --- | --- |
| (E)-AC-3 | -23 dBFS | dialnorm per ETSI TS 102 366 | n/a | compr per ETSI TS 102 366 |
| (E)-AC-3 | -31 dBFS | dialnorm per ETSI TS 102 366 | n/a | dynrng per ETSI TS 102 366 |
| HE-AAC | -23 dBFS | prog_ref_level per ISO/IEC 14496-3 | Mode 1 | compression_value per ETSI 101 154 Annex C |
| HE-AAC | -31 dBFS | prog_ref_level per ISO/IEC 14496-3 | Mode 1 | dyn_rng_sgn/dyn_rng_ctl per ISO/IEC 14496-3 |
| HE-AAC | -23 dBFS | prog_ref_level per ISO/IEC 14496-3 | Mode 2 | dyn_rng_sgn/dyn_rng_ctl per ISO/IEC 14496-3 |
| HE-AAC | -31 dBFS | prog_ref_level per ISO/IEC 14496-3 | Mode 2 | dyn_rng_sgn/dyn_rng_ctl per ISO/IEC 14496-3 |

Table 6.4 Target reference levels

### MPEG-1 Layer II Audio Input

For MPEG-1 Layer II audio input, the default loudness is assumed to be -23 LUFS and therefore no level adjustments are expected to be made for decoding to a target reference level of -23 dBFS. It is expected that the broadcaster ensures that MPEG-1 Layer II audio stream has an equivalent loudness level of -23 LUFS.

For decoding to a Target Reference Level of -31 dBFS (i.e. digital audio output), the following applies:

An attenuation of 8 dB shall be made to the audio stream.

### Audio Output Levels

In order to match the loudness of PCM and coded audio streams in a downstream AV receiver, PCM streams at a target reference level of -23 dBFS shall be reduced in level by 8dB before being output on S/PDIF or HDMI to that device. PCM streams at a target reference level of -31 dBFS shall not be increased in level prior to output. When an HDMI Sink device indicates in its E-EDID structure that it supports multichannel audio, then the output should be at a target reference level of -31 dBFS.

When an HDMI Sink device indicates in its E-EDID structure that it only supports Basic Audio (i.e. two-channel L-PCM), then the output should be levelled to a target reference level of -23 dBFS.

For analogue (stereo) outputs, the target reference level should be levelled to -23 dBFS (as measured on a digital signal).

# Teletext and Subtitling

## General

DVB Subtitling and EBU Teletext Subtitling are mandatory in the NorDig IRDs. The user shall be able to enable and disable displaying of subtitles and to select primary and secondary subtitling language, see sections 7.2 and 7.3.

Note: 	The NorDig HbbTV IRD Hybrid shall support Subtitling preferences and requirements as stated below also for broadband distributed content where applicable.

### Subtitling user preferences

The IRD shall have user selection of subtitling preferences for ‘normal’ or ‘hard of hearing’ subtitles. The user preference settings for subtitling should be common for EBU Teletext subtitling and DVB Subtitling, see section 16 for factory default settings of the subtitling.

Note:	Correct functionality for the Hard of Hearing/hearing impaired service, requires that the Content 	Providers delivers this service as a mix of translated subtitling and Hard of Hearing/hearing 	impaired subtitling.

### Only display subtitling if match language in user preferences

The NorDig IRD shall only display subtitles, if a language of the received subtitle matches any of the NorDig IRD’s user preference settings for language for subtitling. (This means that if none of languages for the received subtitle(s) match any of the IRD’s user preference settings language for subtitling, then the NorDig IRD shall not display any subtitles).

### Temporary changes to subtitling settings

In case of the user has made temporary changes of the subtitling settings (i.e. without changing the IRD’s user preference setting), then this change shall (at least) remain until the user change service. (Clarification, this means the IRD shall not change back temporary subtitling setting based on EIT events for the service).

### Subtitling mode (Normal and Hard of hearing subtitling)

In case of ‘normal’ subtitling mode is selected, then the NorDig IRD shall only display ‘normal’ subtitles (signalised in subtitling descriptor and/or teletext descriptor). In this ‘normal’ subtitling mode the NorDig IRD shall not display any (hard or hearing) subtitling if the subtitling stream only includes ‘hard of hearing’/‘hearing impaired’ pages.

In case of ‘hard of hearing’ subtitling mode is selected and if no ‘hard of hearing’/‘hearing impaired’ pages are received (signalised in subtitling descriptor and/or teletext descriptor), then the NorDig IRD shall as a default use ‘normal’ subtitling pages from the same selected language. (For Teletext, see section 7.2.

### Simultaneous EBU and DVB subtitling

If both DVB Subtitling and EBU Teletext subtitling are received simultaneously with the same language code, the IRD shall only display the DVB Subtitling stream.

### Simultaneous EBU Teletext and HbbTV Digital Teletext

For services that have both an EBU Teletext service and an HbbTV Digital Teletext application signalled and available, the NorDig HbbTV IRD Hybrid shall be able to start and display the HbbTV Digital Teletext application as well as being able to start and display the EBU Teletext service (one at a time).

The NorDig HbbTV IRD Hybrid shall start teletext and be able to toggle between any HbbTV Digital Teletext and any EBU Teletext service as described in clause 5.3.4 of HbbTV specification ETSI TS 102 796 [29].

Note:	An EBU Teletext service refers here to classical Enhanced Teletext specification EN 300 472 [16], while a Digital Teletext Service refers here to an HbbTV application which is identified with an application_usage_descriptor in the AIT with usage_type equal to 1 as defined in the HbbTV specification ETSI TS 102 796 [29]; and typically launched by the TEXT key on Remote Control. The HbbTV based Digital Teletext could typically be a more modern version and replacing the EBU Teletext for HbbTVHybrid IRDs.

### Simultaneous Subtitling and HbbTV

A NorDig HbbTV IRD Hybrid shall support simultaneous display of HbbTV application and subtitles (DVB subtitling and EBU teletext subtitling), both for broadcast and at least for MPEG2 TS delivered via broadband [29]..

The NorDig HbbTV IRD Hybrid shall display the HbbTV application over the subtitles as described in clause 10.1.1 of HbbTV specification ETSI TS 102 796 [29]. This means that if the video is up or down-converted, to other than full screen video, the subtitles shall either be rescaled/repositioned appropriately or not displayed at all.

Note: 	The requirement for simultaneous display of HbbTV application and subtitles is optional (should) within HbbTV ETSI TS 102 796, while within NorDig HbbTV Hybrid (HbbTV) IRDs this is mandatory (shall) to support.

## EBU Teletext

### General

During normal operation, the NorDig IRD shall be able to demultiplex in parallel the EBU Teletext service transmitted in a packetised format according EN 300 472 [16].

The NorDig IRD shall include a teletext decoder to be able to display EBU Teletext using the OSD. The NorDig IRD shall be able to display EBU Teletext subtitling, both ‘normal’ Teletext subtitling pages of type 0x02 and Teletext subtitling pages for hearing impaired people of type 0x05, meeting the requirements for level 1.5 in ETSI EN 300 706 [18], "Enhanced Teletext Specification"

The Nordic characters defined in the Latin G2 supplementary set shall be supported

The NorDig IRD with OSD presentation shall be able to cache at least 200 decoded EBU Teletext pages in order to improve the access time for frequently used pages.

### Additional requirements for Analogue video Interface

The NorDig IRD shall for the analogue outputs also support insertion of the EBU Teletext data in the VBI of the analogue CVBS video output. In this case the teletext decoder of the TV-set might be used instead of the one in the STB. The VBI insertion shall be compliant with ITU-R BT.653-3 [62]. The EBU Teletext data shall be inserted in the lines 6 to 22 and lines 320 to 335.

## DVB Subtitling

### General

The NorDig IRD shall be capable of decoding, as a minimum, a subset of the DVB subtitle services as specified in section 7.3.2 and transmitted in conformance with ETS 300 743 [19], and displayed using the OSD capabilities whilst decoding the full television service (video and audio) to which it is associated.

The NorDig IRD shall be able to display both ‘normal’ and ‘hard of hearing’ subtitles, according to user preference settings.

Within DVB Subtitling it is possible to transmit common pages for all languages and subtitling streams inside one DVB subtitling PID, this is referred to as ‘ancillary pages’. Support for ancillary pages is optional for NorDig IRD. The enabling or disabling of the subtitle ancillary pages, if available, should be user controlled, with subtitle ancillary pages enabled as default option. The selection of subtitle ancillary pages shall be independent of the enabling of subtitle composition pages.

The precision of the presentation of the subtitles shall be within 2 frames.

### Subtitling subset

The NorDig IRD shall at least be capable of decoding the following DVB subtitling services:

| DDS | The Display Definition Segment for a subtitle service shall be supported for services that implement DDS, as defined in EN 300 743 [19]. Absence of a DDS implies that the display segment width shall be assumed as 720 pixels and the height as 576 lines. |
| --- | --- |
| Object types: | The handling of the object type (0x00) ‘basic object, bitmap’ shall be supported. The handling of the other object types (i.e. 0x01), ‘basic object, character’ and (0x02) ‘composite object, string of characters’) is optional. |
| Regions: | The number of regions shall be according to the ETS 300 743 [19] specification, however a limitation in the display area due to memory restrictions is allowed. The total number of regions to handle shall be able to cover four complete subtitle rows (per frame) where: One subtitle row shall be extendable to 1906 pixels * 60 pixels. The regions shall have the possibility to cover 457440 pixels per frame. |
| Number of objects: | The number of objects shall be at least 128. |
| CLUT: | The NorDig IRD shall be able to handle at least one colour look-up table (CLUT) with a minimum of 16 entries per region and the possibility to have one colour scheme applied in each of the regions. It shall be possible to choose any 24-bit RGB colour into the 16 entries. The decoder shall be able to handle the mapping to the closest colour match if the decoder has some limitation in the colour presentation. The use of the non_modifying_colour flag is optional. |
| Transparency: | The NorDig IRD shall implement at least 5 levels of transparency; 0% (opaque), 30%, 50%, 70% and 100% (completely transparent). Implementation of additional intermediate levels of transparency is optional. Where the NorDig IRD cannot complement a particular value of semi-transparency it shall replace it with the nearest value of transparency it can implement. However, if the encoded value of transparency is in the range 10%-90% it shall not be approximated as either 0% or 100% transparency. So, 9% may be approximated as 0% but 10% shall be represented with a value in the range 10% to 90%%, such as 30%. Similarly, 91% may be approximated as 100%. |
| Number of streams: | The NorDig IRD shall support at least one DVB-subtitling streams i.e. at least support decoding of one subtitling composition page while support of one simultaneously available ancillary page is optional. |

# Interfaces and Signal Levels

## Introduction

This chapter includes requirements to the various external interfaces, except for the frontends (tuner/demodulators) that are treated in chapter 3.

The main functional blocks are described in chapter 2 for the case with embedded tuner/demodulator.

## RF- bypass (option)

NorDig IRD should have an RF-bypass (RF in – RF out), see sections Error! Reference source not found. and Error! Reference source not found..

## Two-way Interface

The NorDig IRD with the HbbTV capabilityHybrid profile or an IP-based front-end shall support at least one of the following interaction channel interfaces:

- Ethernet (IEEE 802.3 [45] (100 Base-T, Auto-sense).

- EuroDocsis in accordance with ITU-J.122 [59] (ref IRDs with a cable front-end).

- Wireless LAN, Ethernet 802.11 n [44]

- Power line [HomePlug AV Specification, IEEE 1901.2010]

- USB 2.0 or higher

## SCART Analogue Video Interfaces (Option)

Video signals output via any type of analogue interface shall be of maximum 720x576 (SDTV) resolution, see section 5.11.

### SCART Interface (Option)

The NorDig STB should have one SCART Interface in accordance with EN 50049-1 [6] and EN 50157-2-1 [8].

The following table summarises the input/output signals available at all SCART interfaces:

| SCART | requirement | CVBS/AUDIO | RGB | PIN 8 | PIN 16 |
| --- | --- | --- | --- | --- | --- |
| 1 TV | Mandatory* | Out | Out | out (1) | out (2) |
| 2 VCR | Optional | in and out (3) | In | In | In (4) |

Table 8.1 SCART requirements

* Not relevant for iDTV

(1): 	the voltage shall be forwarded from in to out (12V or 6V)

(2):	the voltage shall be forwarded from in to out (0V or 1 - 3V)

(3): 	the OSD graphics should not be present on the VCR SCART output except for DVB subtitling
 	(if present and chosen)

(4):       the voltage should be forwarded from in to out (0V or 1 - 3V)

Control signal definitions:

PIN 8:	nom. 0 Volt/DC: 	internal source of the TV set

nom. 6 Volt: 		external  source, 16:9 format

nom. 12 Volt: 	external  source, 4:3 format

PIN 16: 	nom.0 Volt/DC:	CVBS active

1-3 Volt/DC:	RGB active

## Audio Output Interfaces (Option)

### VCR SCART connector for analogue audio

The NorDig IRD may provide analogue audio output signals via SCART interfaces. The audio interface of the VCR SCART shall deliver the same audio signal as available at TV SCART Interface. The internal volume control should only affect the audio signal at TV SCART interface, but not the audio signal of the VCR SCART audio interface.

### RCA connector for analogue audio

The NorDig IRD may provide analogue audio output signals via RCA connectors. The NorDig IRD analogue audio interface based on RCA connectors shall be:

- Two RCA connectors, female type IEC 60603-14 [41].

The audio signals shall be as specified in section 6.5 (Audio Prioritising).

### S/PDIF connector for digital audio

The NorDig IRD should include digital audio interface, based on S/PDIF, (IEC 60958 [42]) and a non-PCM encoded audio bit stream according to IEC 61937 [43], with a coaxial and/or an optical connector.

The audio signals shall be as specified in section 6.5 (Audio Prioritising).

### Headphone connector for analogue audio

The NorDig IRD should include analogue audio interface based on headphone connectors with 3.5 mm TRS stereo mini tele jack or ¼” TRS stereo tele jack.

## HDMI Interface

### General

#### NorDig iDTV and NorDig HEVC iDTV

The NorDig iDTV with screen diagonal 30 cm and above shall have a HDMI input interface in accordance with the DigitalEurope HD-Ready requirements [5] and the High Definition Multimedia Interface [38]. HDMI input interface is highly recommended for iDTV-sets with smaller screen diagonal. The HDMI output interface is recommended for iDTV-sets.

A NorDig iDTV or NorDig HEVC iDTV is, by definition, capable of receiving and decoding the relevant NorDig broadcast signals. Whilst NorDig recognises that specifying the HDMI input connector requirements on iDTVs is generally beyond its scope, for the avoidance of doubt the NorDig iDTV (1) shall include at least one HDMI 1.4b [38] or later version input while a NorDig HEVC iDTV (1) shall include at least one HDMI 2.0b [xx] or later version input.

Note: A NorDig iDTV or NorDig HEVC iDTV may have several other HDMI inputs with HDMI-versions which at the manufacturer’s discretion, may or may not be compliant with HDMI 1.4b or HDMI 2.0b, or later versions.

Note 1: <maybe add exceptions for smaller TV sets, portable, IRDs with external PSU or similar OR change the definition of a NorDig iDTV OR skip all exceptions inside the functional requirement sections and have exceptions listed in intro or such>

#### The NorDig STB and the NorDig HEVC STB

NorDig STBs shall have at least one High-Definition Multimedia Interface (HDMI) with type A output connector compliant with HDMI v1.4b or later [38] for the (non-HEVC) NorDig STB and compliant with HDMI v2.0b [xx] or later for the NorDig HEVC STB.

### Video Output and Display

#### The NorDig STB

The NorDig STB shall recognise E-EDID information provided by the display and subsequently follow the below requirements.

The NorDig STB shall use 1920x1080p@50 Hz as the default output format, if supported by the display. 

If 1920x1080p@50 Hz is not supported by the display, the NorDig STB should use 1280x720p@50Hz, rather than 1920x1080i@25Hz, as the output format – although this priority requirement may not comply with the specified priority order in the HDMI specifications regarding E-EDID information exchange.

The user shall be able to override the above behaviour in two different ways: 

1.        By choosing an “Original Format” option, i.e. to output the same format as received, if supported by the display. If the received format is not supported, the STB shall select the display mode providing the best possible video quality, as indicated by the E-EDID information. This is to avoid the STB output to go black, if there is a mismatch between received format and display capability.

Note: 	In the case of received 1080p@25Hz, and if the display does not accept this, the STB should perform 2:2 pulldown (a.k.a. frame-doubling) to reach 50 Hz and subsequently retry with the E-EDID information exchange.

2.        By choosing a “Fixed Format” option, i.e. to manually set, preferably with a dedicated knob on the remote control, the default output format from the NorDig STB to a fixed video format. The video format options shall include 1920x1080p@50Hz, 1280x720p@50Hz and 1920x1080i@25Hz.

#### The NorDig HEVC STB

The NorDig HEVC STB shall recognise E-EDID information provided by the display and subsequently follow the below requirements.

The NorDig HEVC STB shall use 3840x2160p@50 Hz as the default output format, if supported by the display. 

If 3840x2160p@50Hz PQ10 (including all decimated sub-resolutions) is not supported by the display, the NorDig HEVC STB should primarily convert to 3840x2160p@50Hz HLG10 and secondarily 3840x2160p@50Hz WCG+SDR (i.e. ITU-R BT.2020 [89]).

If 3840x2160p@50Hz HLG10 (including all decimated sub-resolutions) is not supported by the display, the NorDig HEVC STB should primarily convert to 3840x2160p@50Hz PQ10 and secondarily 3840x2160p@50Hz WCG+SDR (i.e. ITU-R BT.2020 [89]).

If 3840x2160p@50 Hz, no matter colorimetry, is not supported by the display, the NorDig HEVC STB should primarily convert to BT.709-based 1920x1080p@50Hz and secondarily BT.1847-based 1280x720p@50Hz, rather than BT.709-based 1920x1080i@25Hz, as the output format – although this priority requirement may not comply with the specified priority order in the HDMI specifications regarding E-EDID information exchange.

The user shall be able to override the above behaviour in two different ways: 

1.        By choosing an “Original Format” option, i.e. to output the same format as received, if supported by the display. If the received format is not supported, the STB shall select the display mode providing the best possible video quality, as indicated by the E-EDID information, and perform colorimetry conversion if needed because of display capability. This is to avoid the STB output to go black, if there is a mismatch between received format and display capability.

Note: 	In the case of received 25 Hz progressive scan, and if the display does not accept this, the STB should perform 2:2 pulldown (a.k.a. frame-doubling) to reach 50 Hz and subsequently retry with the E-EDID information exchange.

2.        By choosing a “Fixed Format” option, i.e. to manually set, preferably with a dedicated knob on the remote control, the default output format from the NorDig HEVC STB to a fixed video format. The video format options shall include 3840x2160p@50Hz PQ10, 3840x2160p@50Hz HLG10, 3840x2160p@50Hz WCG+SDR (i.e. ITU-R BT.2020 [89]), 1920x1080p@50Hz, 1280x720p@50Hz and 1920x1080i@25Hz.

### Audio Output

The HDMI Audio Output is specified in section 6.6 (Audio Output Formats). For IRDs integrated in IDTVs, a HDMI Audio Return Channel (HDMI ARC) should be implemented.

### Signal protection

NorDig IRDs with HDMI interfaces that can output content that is originated from a DVB input signal (or IP/OTT delivered content??)  shall support High-bandwidth Digital Content Protection (HDCP) rev. 1.4 [37].

In addition, NorDig HEVC IRDs with HDMI interfaces that can output content that is originated from a DVB input signal (or IP/OTT delivered content??) shall support High-bandwidth Digital Content Protection (HDCP) rev. 2.2 [92].

NorDig HEVC STB shall only fall back to HDCP rev. 1.4 when connected to an HDMI sink that doesn’t support HDCP rev. 2.2.

The received service may be flagged with a need for content protection or not (CP “ON” or “OFF”) via either the PMT-table or the CA-system or both, as specified by the relevant network/CA-operator. Signals that the IRD is entitled to receive shall be sent to the HDMI-sink (display) in accordance with the following conditions:

- In case the received service is flagged with no need for content protection, the signal may be sent to the sink with HDCP disabled (1).
In case both the PMT-table and the CA-system are used for signalling of such flag, and the HDCP is set to ”OFF”,  the signal shall only be sent to the sink when both the flag received via the PMT-table and the flag received via the CA-system indicate no need for content protection. 
Note 1: Disabling of HDCP is optional

- In case the received service is flagged with content protection required via either the PMT-table or the CA-system, the signal shall only be sent to the sink with the HDCP enabled, i.e. when the HDMI sink satisfies the HDCP requirements and HDCP protection is established on the HDMI-link.

The IRD should (1) provide an option for setting the preferred HDCP-state, (“HDCP-user setting”) to either ON or OFF. The HDCP-user setting shall apply to all services receivable by the IRD. Changes to this setting shall survive channel change, stand-by and power on/off.

Note 1:	This option – when available- shall be available via the IRD’s menu system, unless otherwise specified by the relevant network/CA-operator.

Table 8.2 defines the required actions of the IRD, based on the required content protection for the received service and the selected HDCP-user setting. The required content protection level and the required HDCP-state may be flagged via the PMT-table (see section 0) and/or the CA-system (as specified by the relevant network/CA-operator).

| Content Protection level flagged via |  |  | HDCP-user setting |  | Description (IRD actions) |
| --- | --- | --- | --- | --- | --- |
| Mode | CA-system (1) | PMT-table (2) | ”ON” | ”OFF” (3) |  |
| 1 | HDCP Not wanted | 0x00 (HDCP Not wanted) | DISABLE HDCP | OK (5) | HDCP shall be disabled for this service regardless of HDCP-user setting (Return to HDCP-user setting when leaving this service) |
| 2 | HDCP Wanted | 0x00 (See above) | OK (4) | ENABLE HDCP | HDCP shall be set to ON for this service regardless of HDCP-user setting (Return to HDCP-user setting when leaving this service) |
| 3 | CP Not needed | 0x01* Not needed *) Default value if no signalling | OK (4) | OK (5) | Content protection is not required. HDCP may be set to ON or OFF |
| 4 | CP Required | 0x01 (See above) | OK (4) | Display message (6) | Content protection is required. HDCP ON is required for viewing this service |
| 5 | CP Not needed | 0x02 (cond’al require-ment) | OK (4) | OK (5) if SD content) Display message (6) if protected content is received | HDCP may be set to ON or OFF for SD (see table 12.20) HDCP ON is required for HD for viewing this service |
| 6 | CP Required | 0x02 (See above) | OK (4) | Display message (6) | Content protection is required. HDCP ON is required for viewing this service |
| 7 | CP Not needed | 0x03 (CP required) | OK (4) | Display message (6) | Content protection is required. HDCP ON is required for viewing this service |
| 8 | CP Required | 0x03 (See above) | OK (4) | Display message (6) | Content protection is required. HDCP ON is required for viewing this service |
| Note 1: The specified modes may be omitted or redefined by the relevant network/CA-operator. Modes 2, 4, 6 and 8 are not relevant when the CA-system is not used for signalling of required content protection level. Note 2: The Content protection levels are fully defined in Table 12.28. Note 3: “HDCP OFF” is not recommended for the HDCP user setting in networks where some programmes will require “HDCP ON”, because it may lead to excessive zapping times. Note 4: No need to override HDCP user setting (HDCP is ON) Note 5: No need to override HDCP user setting (HDCP is OFF) Note 6: The IRD shall inform the end user that the HDCP user setting must be turned ON in order to view protected content. |  |  |  |  |  |

Table 8.2 IRD actions versus required (signalled) Content Protection level and HDCP user setting

### Additional analogue interfaces for NorDig IRD signals (option)

The NorDig STB shall provide down-converted versions of the received HDTV signals via analogue video interfaces, where the output signal is down-converted to SDTV format, see section 5.11.

The NorDig IRD may provide analogue audio output signals via the SCART and/or the RCA connectors, as specified in sections 8.4, 8.5.1 and 8.5.2.

## User Control functions (Remote Control)

### General

The functions described in the below tables consist of those operations which are considered to be necessary to implement a fully operational IRD (dependant on the class of device like: Basic IRD, HbbTV IRDHybrid or PVR). This operations may be implemented as logical functions, as physical buttons on a remote control or a combination of the two.

NorDig strongly recommends that the mandatory functions defined here are implemented in a manner which makes them easily accessible for a user.

Where these functions are implemented as dedicated physical buttons they will implicitly be accessible.

Where the functions are implemented as logical functions, NorDig strongly recommends that the User Interface renders these functions at a high enough menu level that a user will be able to easily locate and select them. Ideally this would be at the first or second level menu structure.

### Handling of Persistent and Temporary settings

Remote control functions can be used to change persistent or temporary IRD settings, see chapter 2.4.4.2 for definitions. NorDig does not mandate which remote control functions are persistent and which are temporary, but Table 8.7.5 states the recommended type for each function. Where appropriate, temporary changes shall in addition be able to be made persistent.

### Accessibility

Functionality which is directly related to accessibility features (Subtitles, Supplementary audio, Talking menus etc.) should also be implemented in an easily accessible area of any IRD User Interface.  See chapter 13.6.

### Grouping of User Control settings

NorDig recommends device manufacturers to use good practice when considering the layout of core remote control functions. This applies whether the implementation is as a set of physical buttons or rendered as an on-screen display. Good practice would typically dictate that common functions are grouped in reasonable, useable proximity to each other. This recommendation would, for example, include:

- Number buttons being grouped

- PVR buttons being grouped

- Colour buttons being grouped

- Arrow buttons, OK/select, Back and Exit being grouped

All buttons within a group should/shall be presented in equal way for the user to select between all buttons in the group (for example during an IRD status mode of presenting EBU Teletext page all colour buttons should be presented and not just one colour button at the time).

For clarity, this recommendation does not mean all the above functions need to be grouped.

### User Control, basic functions

| Basic Functions | STB | iDTV | Recommendation for Temporary/ Persistent |
| --- | --- | --- | --- |
| Power On/Off Turns the IRD on and off | M | M | - |
| Programme Up/Down Function to switch between programmes. It is recommended to start at the same channel after a power off/on. | M | M | P |
| Volume Up/Down Function to adjust the volume output level. Optional for receivers without display. It is recommended to keep the volume level when changing channels and power off/on. | O | M | P |
| TV/ Radio If this function is provided this is what it should do: Function that puts the IRD directly into conventional television state, i.e. only audio, video and subtitling or radio state (i.e. toggle between TV and Radio category list of services). | O | O | - |
| Subtitles Options for subtitling (On/Off/Hard of hearing, Languages) | M | M | P |
| Audio Description/Spoken Subtitles | M | M | Tp |
| Audio Selection of audio language and/or audio stream | M | M | Tp |
| Mute Mutes audio | O | O | T |
| Program Guide This function displays an Electronic Programme Guide. | M | M | - |
| Info Provides additional event information | M | M | - |
| Teletext This function launches the EBU Teletext or HbbTV Digital Teletext if present, see section 7.1.6. | M | M | - |
| Numerals 0-9 | M | M | - |
| Menu This function starts the main menu for all the persistent settings (ref chapter 16) and the navigator (ref chapter 13) functionality. | M | M | - |
| Navigation (e.g. Arrow keys) A navigation or pointing system for navigation on the OSD | M | M | - |
| OK or Select A function that selects or confirms current choice or statement | M | M | - |
| Back This function exits from the current menu or “page” and returns to the previous state. In TV/Teletext it should return to previous channel/page. | O | O | - |
| Exit Exits the current menu/info/program guide (back to TV screen) | O | O | - |
| Colours Four color-coded controls for non-dedicated functions. The colours shall be red, green, yellow and blue | M | M | - |
| High Contrast User Interface see section 13.6.3 | O | O | P |
| Talking Menus see section 13.6.2 | O | O | - |
| Key: M = Mandatory, O = Optional, T = temporary change, P = persistent change, - = not applicable, Tp = Temporary change with a persistent system setting |  |  |  |

### User Control, NorDig HbbTVhybrid functions

| NorDig HbbTVHybrid Functions in addition to basic functionality | HbbTVHybrid |
| --- | --- |
| HbbTV function activation Please refer to the HbbTV specification. This function activates the HbbTV Digital Teletext application. Typically this function is covered by the Teletext or Red control. | M |
| Back Please refer to the HbbTV specification. This function is always available to applications once an HbbTV application is active | M |
| Exit This function terminates the currently running HbbTV application. It does not disable the HbbTV feature, therefore if the current service has an auto-start application then it shall be re-launched and broadcast video shall be reset to its default position. | M |
| Key: M = Mandatory, O = Optional, T = temporary change, P = persistent change, - = not applicable, Tp = Temporary change with a persistent system setting |  |
| ### User Control, PVR functions |  |
| PVR Functions in addition to basic functionality | PVR |
| List of Recordings Opens a screen with list of recordings (should be both existing and scheduled). | M |
| Record/One Touch Record Start manual recording / start recording of present event. | M |
| Play Start playing timeshift TV / start playback of recording. | M |
| Pause Pause playback of recording or timeshift TV | M |
| Stop Stops playback/timeshift/recording | M |
| Fast Forward/Fast Rewind Fast forward/rewind of the timeshift or recording (with different speeds). | Mc |
| Skip Back/Skip Forward (Previously JUMP) go to a specific time in the recording / fast jump to a manufacturer defined fixed time or to next index point (also have to update section14.4.2) | Mc |
| Key: M = Mandatory, O = Optional, T = temporary change, P = persistent change, - = not applicable, Tp = Temporary change with a persistent system setting Mc = Conditional Mandatory that either forward/rewind or Skip back/forward functions are available in the NorDig IRD. If skip back/forward function is not available, then NorDig recommends implementation of higher forward/rewind speeds |  |

### User Control, other optional functions

Other functions are left to the discretion of manufacturers.

It is expected that a production IRD will include additional functions not described here which may be implemented entirely at the device manufacturer discretion. See table below.

Other Optional Functions

Display resolution Toggles through all available display resolutions (original format, 720p, 1080i and more)

Aspect ratio Normally find on TVs. Toggles through different aspect ratios, 16:9, 4:3, Zoom,

Option / short menu A way of accessing menu functions that are used more often than the set-up menu, but not as often to need separate function.

User manual If the IRD has an interactive on-screen user manual

Help Shows help about where You are in the menu

3D Toggles through different 3D functions

Smart TV / Internet Gives access to smart TV functions and/or Internet web browsing functions

Channel list Gives access to a list of available services. Often combined with OK-button.

Search Search for content, web search etc.

Other Teletext functions Mix modes, next/prep page, enlarge Teletext, show hidden text, etc.

Digital / Analogue Toggles between digital and analogue services

### Design and Labelling for physical Remote Control

The manufacturer is responsible for the design of the remote control and the labelling of the remote control functions.

Manufacturers should consider the needs of visually impaired users when designing remote control handsets.  Items to consider include the size/texture/tactile response of buttons and the colour/typeface/size of labelling. Frequently used remote control functions such as Volume Up/Down, Programme Up/Down should be placed in an easy accessible manner.

### Mapping of Key Events for NorDig HbbTV IRDHybrid profile

The NorDig HbbTVHybrid IRD shall generate (HbbTV) events according to Table 8.3 in response to user control functions, (e.g. when a key is pressed on the NorDig IRD remote control).

| User Control functionsKey | HbbTV Key eventEvent |
| --- | --- |
| Numerals 0-9 | VK_0 to VK_9 |
| Navigation Up | VK_UP |
| Navigation Down | VK_DOWN |
| Navigation Left | VK_LEFT |
| Navigation Right | VK_RIGHT |
| OK | VK_ENTER |
| Back | VK_BACK |
| Colour Red | VK_RED |
| Colour Green | VK_GREEN |
| Colour Yellow | VK_YELLOW |
| Colour Blue | VK_BLUE |
| Stop | VK_STOP |
| Play | VK_PLAY or VK_PLAY_PAUSE (1) |
| Pause | VK_PAUSE or VK_PLAY_PAUSE (1) |
| Fast Forward | VK_FAST_FWD |
| Fast Rewind | VK_REWIND |

Table 8.3 Mapping of NorDig HbbTVHybrid IRD Key Events to HbbTV.

Note 1:	VK_PLAY and VK_PAUSE are used for PVRs with separate remote control keys for these two functions, while VK_PLAY_PAUSE is used for PVRs with one common toggling multifunctional key for these two functions.

| User Control functions | Key Event |
| --- | --- |
| Power On/Off | Not available to HbbTV applications |
| Programme Up | Not available to HbbTV applications |
| Programme Down | Not available to HbbTV applications |
| Volume Up | Not available to HbbTV applications |
| Volume Down | Not available to HbbTV applications |
| Teletext | Not available to HbbTV applications |
| Subtitling/Option | Not available to HbbTV applications |
| Guide | Not available to HbbTV applications |
| Info | Not available to HbbTV applications |
| TV/Radio | Not available to HbbTV applications |

Table 8.4 User Control functions that are not available to HbbTV applications.

# Interfaces for Conditional Access

## General

The NorDig IRD shall (1) support at least one Common Interface Plus (for CA module) for conditional access and/or it shall support at least one smart card interface (2) for conditional access. The smart card interface with associated embedded functions should support use of external smart card(s) for at least one CA-system.

All NorDig iDTV sets with screen diagonal larger than 30 cm shall be equipped with at least one Common Interface that comply with the Common Interface Plus specification, see section 9.2. The Common Interface is optional/recommended for other NorDig IRD types (e.g. STB).

Note 1: 	Mandatory for the NorDig IRDs that are intended for use in networks broadcasting signals that are accessed controlled, as well as not access controlled.

Note 2: The requirements for conditional access interfaces are specified by the relevant network/CA 	operator.

## Use of the Common Interface

### General

The Common Interface can be used for conditional access and other purposes. A Conditional Access (CA) Module (CAM) may be connected to the Common Interface of the NorDig IRD in order to provide access control of the incoming services.

The Common Interface shall be able to be used with modules that comply with the DVB Common Interface Plus specification (version 1.3 or later), see ref. CI Plus specification [66]; such modules are referred to as CIP-CAM.

The Common Interface shall also be able to be used with CA-modules that comply with the DVB Common Interface specification, see EN 50221 [9]; such modules are referred to as CI-CAM.

### Minimum requirements for the Common Interface

Each CI-slot of the NorDig IRD shall (1) be in compliance with the Common Interface Plus (CIP) specification [66]. Each CI-slot shall support both CIP-CAMs and CI-CAMs in accordance with with the interoperability matrix that is specified in the CI Plus specification [66], table 4.1.

Note 1:	As stated in section 9.1 CI is mandatory for NorDig iDTV-sets (with screen diagonal above 30 cm) (and is optional for other NorDig IRD types).

The CI Plus interface for the NorDig IRD shall support at least the maximum bitstream that can be provided via the front-end, see section 4.1 (1). The CIP-CAM shall support 96Mbit/s.

Note 1: 	The CI Plus specification [66] states that the IRD shall support 72Mbit/s and may support 96Mbit/s on the PCMCIA interface. 96Mbit/s shall be supported if the front-end can provide higher bitrates than 72Mbit/s. This applies e.g. for the satellite front-end, which can provide up to 80.4 Mbps.

### Minimum requirements for the NorDig CA-Module

#### General – the CA-modules

The CA-module may contain the CA security device (“CA-module with fully embedded CA-system”) or a smart card interface for connection to an external smart card (“CA-module with partly embedded CA-system”).

#### CA-module with fully embedded CA-system

The CA-module will be CA-system specific and contain all CA-functions, including the security device. For this case the relevant specifications have to be obtained from the relevant CA-system vendor.

#### CA-module with partly embedded CA-system

Proprietary CA-module (CIP-CAM or CI-CAM):

- The CIP-CAM or the CI-CAM will be connected to a security device (smart card).

- The CIP-CAM shall provide the CI-functions specified in the CI Plus specification [66] and the additional functions specified by the relevant CA-system vendor for the smart card interface.

- The CI-CAM (1) shall provide the CI-functions specified in EN 50221[9] and the additional functions specified by the relevant CA-system vendor for the smart card interface.

Note 1:	Use of CI-CAM is not supported in all NorDig networks and may be phased out in most 	networks.

## Use of Smart Card Reader

### General

The smart card hardware with associated software can be used for conditional access and other purposes. This section will only consider use related to conditional access.

The smart card reader shall support an interface as partially specified in section 9.3.2 below and hardware/firmware for descrambling as specified in chapter 4. In addition, there shall be filtering of ECM/EMM streams and program interfaces as specified below for conditional access.

The IRD shall be capable of replacing the CA-system software by download of new IRD and CA-system software via the bootloader, over air or locally.

### The Smart Card Interface

#### All NorDig profiles

The embedded smart card reader is used with conditional access and/or other applications.

The smart card interface shall comply with ISO/IEC 7816 Part 1-3 [58]. The NorDig IRD does not need to support synchronous cards. The NorDig IRD shall implement all aspects related to asynchronous cards with the following exceptions:

support for Vpp is not required

support for AFNOR pin-out is not required

Vcc range is 5V+/- 5%

Icc max is 65 mA

The clock frequency shall be at least 5 MHz.

The possibility of using the data exchange protocol T=0 shall be supported. It shall be possible to include support for the data exchange protocol T=1 through an IRD software upgrade.

### ECM and EMM Filtering

The NorDig IRD shall implement ECM and EMM acquisition in accordance with ETSI ETR 289 [24].

The NorDig IRD shall be able to simultaneously acquire at least two ECM streams. The ECMs shall be filtered based on PID, TID and toggle bit.

The NorDig IRD shall be able to acquire EMMs from at least one EMM stream (one PID). The EMMs shall be filtered based on PID, TID and section address field. The section address field is CA system specific, and described as part of the smart card application interface. The IRD shall be able to filter on three TID and address field combinations simultaneously.

### Descrambling of selected services

The NorDig IRD shall implement descrambling of selected services, see section 4.2.

### Application Level Interface for Conditional Access.

The application level smart card interface for conditional access is CA-system specific. The application level interface definitions are restricted information that can be obtained from relevant CA-system vendors.

# The System Software Update

# Performance

## Introduction

In this chapter the performance of decoded digital video and audio signals are specified (only relevant for IDTV in case of external interfaces). It also includes zapping performance regarding the time to recover when changing services. The performance for demodulated analogue video and audio signals (optional for NorDig IRD with embedded analogue cable front-end) is also specified.

Other performance issues are treated in other chapters.

## Video Performance of RGB and PAL Signals

The RGB- and CVBS-signals at the appropriate interfaces of the NorDig IRD shall meet the characteristics given in ITU report 624-4 [61].

## Audio Performance of the Decoded Digital Signal

Reference for the performance of all audio measurement is full scale minus 12 dB and the measurement shall be made at a sampling rate of 48 kHz.

The NorDig IRD shall at least satisfy the performance as stated below:

| Measurement item | Min | Typical | Max. |
| --- | --- | --- | --- |
| Output impedance (Ohm) |  | 600 | 1000 |
| Output level for sine wave at 1kHz of 0 dBTP (mV RMS) | 1590 (i.e. -1 dB) | 2000 (note 1) | 2520 (i.e. +1 dB) |
| Output level for sine wave at 1kHz of -12 dBTP (mV RMS) | 397 (i.e. -1 dB) | 500 | 630 (i.e. +1 dB) |
| Flatness of amplitude response: (dB)(at 40 Hz to 80 Hz) 80 Hz to 13,5 kHz 13,5 kHz to 20 kHz | -2 -1 -2 |  | +2 +1 +2 |
| Dynamic range (dB) | 80 |  |  |
| Harmonic distortion ratio (%) |  |  | 0.1 |
| Cross-talk between channels (dB, at 20 Hz to 20 kHz) |  |  | -60 |
| Hum suppression (dB) | 60 |  |  |
| S/N (dB, weighted, quasi peak, ITU/R rec. 468) | 66 |  |  |
| Phase difference between channels (°), 40 Hz to 13,5 kHz 13,5 kHz to 15 kHz |  |  | 10 15 |
| Amplitude difference between channels (dB, 20 Hz to 20 kHz) |  |  | 1 |
| Volume control (affected steps with 3 dB/step) |  | 6 |  |
| Signal attenuation at mute (dB) | 70 |  |  |

Table 11.1 Audio performance

Note:	Full scale is defined, for a digital signal, as the maximum signal in accordance with the encoding system specification. Full scale amplitude is defined after pre-emphasis and is the same for all frequencies after encoding.

## Zapping Time for TV Services

The NorDig IRD’s zapping time for the services shall satisfy the requirements given in Table 11.2.

The figures in Table 11.2 shall be met for an input signal which has:

- video GOP length of 12,

- a repetition rate of ECM of 2 per second (for scrambled services)

- repetition rate of PAT and PMT of 10 times per second and

- maximum PTS-to-PCR relative delay shall be 700ms.

The picture on the display during the zapping time shall be either frozen or black and the sound shall be muted until the new session has been stabilised.

The figures in the table are valid for two services on one multiplex as well as for two multiplexes and for both scrambled and unscrambled (FTA) services.

| IRD Type | Average max zapping time |
| --- | --- |
| IRD with embedded CAS | 2.5 seconds |
| IRD with CI and using a CAM | 3.5 seconds |

Table 11.2 Maximum zapping time

Note: 	An IRD may have several methods of changing selected service to be decoded (zapping), for example via P+/P-, service list, numeric keys, from EPG/ESG menu etc. The different methods of zapping may have slightly different zapping time, for example due to response time, time-out for keying etc. Figures above are for the IRD’s fastest method, other method should not introduce more than 1s of extra zapping time. Observe that CAMs from different vendors can have different performance and this can have an impact on the zapping time. Figures above are for CAMs with well proven performance. Different CAS broadcast settings (e.g. CI+ messages) can also impact on the zapping time, which must be taken into account when evaluating the result. Figures above are for CAS broadcast settings with fastest zapping time.

Part B: 	The system software with application

# Service Information

## General

### General Requirements

The NorDig IRD shall be able to process, i.e. sort out, store and make available through the Man-Machine Interface (NorDig Basic IRD) the incoming SI data (descriptors) as tabulated in sections 0-12.8, i.e. these are (minimum) mandatory descriptors for the receiver to decode and interpret, (see also Table 12.2 for an overview over minimum broadcast and receiver requirements). The processing shall be compliant with EN 300 468 [15] and ETSI TR 101 211 [27].

Descriptors or other data structures that are currently undefined or are unknown to NorDig IRD shall be skipped and shall not cause any harm. This means for example that NorDig IRD shall ignore/skip the complete text string that is using DVB character tables that the IRD does not support.

The NorDig IRD shall be able to process the PSI/SI tables, both for the ‘Actual’ and for ‘Other’ transport streams. SI tables for the ‘Other’ transport streams, SIother, should be seen as informative and shall always be double checked with the corresponding SI tables for the ‘Actual’ transport stream, SIactual.

The NorDig IRD shall at least start updating for any changes in the received “quasi-static” SI data after it returns to active from stand-by mode. “Quasi static“SI-data includes NIT and SDT, i.e. SI that is typically stored in the flash memory for service navigations, such as service name, service_ID, number of services. (The ‘running status’ is not included in the quasi-static SI data. As a guideline for the implementation, this updating may be performed in the background, to shorten the start-up of the basic video and audio).

The NorDig IRD shall at least start action for any changes in the received “dynamic” PSI and SI data, (PMT, EIT, TDT/TOT, running status and CA mode) within 1 second. (As a guideline for the implementation, the trigger for changes in received tables can be based on comparing the ‘version id’ in the tables).

NorDig IRDs with IP-based front-end shall support “TS Full SI” and should support “TS Optional SI”, as specified in ETSI TS 102 034 [31]. With respect to DVB SI as specified in ETSI EN 300 468 [15], the following general requirements and comments apply:

    - The NorDig IRD with IP-based front-end shall process the following DVB SI tables if present in the transport stream (see also Table12.1):

- Service Description Table (table_id = 0x42 – Actual transport stream)

- Event Information Table, Present/Following and Schedule

- Time and Date Table/Time Offset Table 
See section 12.5 for complete procedure to retrieve network time.

- Conditional Access Table (CAT)

- Programme Map Table (PMT)

    - For NorDig IRDs with IP-based frontends the NIT is not used. Instead the IRDs shall (1) look for the Service Provider Discovery Information as defined in ETSI TS 102 034 [31]. The entry point(s) for Service Provider Discovery Information shall be according to the mechanisms defined in ETSI TS 102 034 [31]. A service list shall be built based on the information in the Service Provider Discovery Information. See also Error! Reference source not found..

    - In order to locate possible bootloader streams retransmitted from e.g. satellite, the NorDig IP IRD shall (1) look in the Broadcast Discovery Record (according to ETSI TS 102 034 [31]). A bootloader service shall be signalled as a particular service with service_type set to 0x81.

Note 1:	 Use of ETSI TS 102 034 is suspended, as it is currently not used in most IP-based networks. For 	information about required performance related to this item, contact the relevant network 	operator.

As the NorDig IRD needs information like manufacturer, HW version, SW version etc., the NorDig private Linkage Descriptor shall be included in the Broadcast Discovery Records. The XML scheme of the private Linkage Descriptor is given in section 13.4.

### PSI/SI classification

Static PSI/SI data is defined as the PSI/SI data that must be updated by the IRD in Installation mode (channel search or first time initialization).

Quasi static PSI/SI data is defined as the PSI/SI data that must be updated by the IRD in (automatic) Update mode (i.e. when it is toggled between stand-by mode and active mode or vice versa).

Dynamic PSI/SI data is defined as the PSI/SI data that must be updated by the IRD in active/TV viewing mode (i.e. within 1s after a change in the data occurs).

### Private data specifier value

NorDig defined private descriptors and data inside PSI and/or SI tables are recognised with private_data_specifier_value set to 0x00000029, used according to ETSI TR 101 211 [27] and ETSI ETR 162 [23].

### Service Types

The NorDig IRD shall minimum handle the service types listed in Table12.1 below.

Service types that are not supported by the NorDig IRD should be ignored.

|  | Service | Component | Status | Category | Priority |
| --- | --- | --- | --- | --- | --- |
| Class of service (description of service type) | type | type |  | type | within category |
| basic TV service (mainly MPEG-2/H.262 based SDTV) | 0x01 | n/a | M | TV | 53 |
| Radio service (mainly MPEG-1 Layer II based) | 0x02 | n/a | M | Radio | 2 |
| Teletext service | 0x03 | n/a | M | Others |  |
| Advanced codec based Radio service (MPEG-4 AAC including HE-AAC and E-AC-3) | 0x0A | n/a | M Alt (2) | Radio | 1 |
| Data broadcast service (e.g. for SSU service) and HbbTV standalone services | 0x0C | n/a | M | Others |  |
| AVC/H.264 Advanced codec based SDTV service (MPEG-4 AVC) | 0x16 | n/a | M | TV | 42 |
| AVC/H.264 Advanced codec based HDTV service (MPEG-4 AVC) | 0x19 | n/a | M | TV | 31 |
| NGA based Radio | 0xHH | n/a | M (1) | Radio |  |
| HEVC/H.265 based TV service (up to UHD, SFR and SDR compatible, incl HDR/HLG10) | 0x1F |  | M (1) | TV | 2 |
| HEVC/H.265 based TV service with HDR (up to UHD, SFR/HFR and HDR/PQ10), non-backward compatible | 0x20 |  | M (1) | TV | 1 |
| Others | others |  | O | Others | lowest |
| M; Mandatory, R; (Highly) Recommended, O; Optional item to support, Alt; minimum one among several options. Priority; lower value refers to higher priority. Note 1: Mandatory for NorDig HEVC IRDs, optional IRDs not supporting HEVC. |  |  |  |  |  |

Table12.1 Overview of service types, service category groups and priority between the service types

Informative: During migration period simulcasting of the content in (MPEG2) SDTV and in (MPEG4 AVC) HDTV may occur. Simulcasting may be under the same service (service_id) or on separate services (separate service_ids).

For service simulcasting on separate service_ids a linkage ‘NorDig Simulcast replacement service’ (linkage type 0x82) will be broadcasted from the SDTV version pointing to the HDTV version of the same service, in order to help the IRD to know that these services are two versions of the “same” service/content,

The service types ‘Advanced codec based Radio service’ (0x0A), ‘advanced codec based SDTV service’ (0x16) and ‘advanced codec based HDTV service’ (0x19) will be used for services main component is an advanced codec stream. For TV services the main component is the video stream while for Radio services the main component is the audio stream. These service types will be used when it is not desirable that an old MPEG2 only IRD install and list a MPEG4 service.

The service type ‘digital TV service’‘(0x01) will be used for services including MPEG-2 video stream. It may also be used for service simulcasting MPEG2 and MPEG4 AVC video and for service only including MPEG-4 AVC video. All IRDs will install service type ‘digital TV service’‘(0x01). This service types (0x01) will be used for a service that only includes MPEG-4 AVC video when it is desirable that an old MPEG2 only IRD installs and lists a service (even if old MPEG2 only IRD cannot decode the video, used for promotion purpose).

The logic channel number shall, as far as possible, be unique within each original network id for each service type category (TV, Radio and Others).

### Service Categories

The services are grouped into three service type categories; TV (1), Radio (2) and Others/data (3) services:

- TV category includes services with service type; 0x01 digital (MPEG-2) TV service, 0x16  AVC/H.264 advanced codec SDTV service and 0x19 AVC/H.264 advanced codec HDTV service, 0x1F HEVC/H.265 based TV service and 0x20 HEVC/H.265 based TV HDR service.

- Radio category includes services with service type; 0x02 digital radio sound service and 0x0A advanced codec digital radio sound service.

- Others/(data) category includes all other service types that are not included in TV (1) and Radio (2) categories.

The NorDig IRD shall during installation of services create a common service list for each category (i.e. all 0x01, 0x16, and 0x19, 0x1F and 0x20 within same TV category list and so on for the Radio and Other/data categories).

Informative: These categories enable the IRD to create a common TV category service list for all TV service types (0x01, 0x16, and 0x19, 0x1F and 0x20) and similar for Radio and Other/data service lists.

### Used PSI/SI descriptors

| Descriptor | Tag value | NIT (3) | BAT | SDT | EIT | TOT/ TDT | CAT | PMT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| video_stream_descriptor | 0x02 | - | - | - | - | - | - | mb Mr |
| audio_stream_descriptor | 0x03 | - | - | - | - | - | - | mb Or |
| target_background_grid_descriptor | 0x07 | - | - | - | - | - | - | Ob Or |
| video_window_descriptor | 0x08 | - | - | - | - | - | - | Ob Or |
| CA_descriptor | 0x09 | - | - | - | - | - | mb Mr | mb Mr |
| ISO_639_language_descriptor | 0x0A | - | - | - | - | - | - | mb Mr |
| carousel_identifier_descriptor | 0x13 | - | - | - | - | - | - | mb Mr (1) |
| Metadata_pointer_descriptor | 0x25 | Ob Or (4) | - | Ob Or (4) | - | - | - | - |
| Metadata_descriptor | 0x26 | - | - | - | - | - | - | Ob Or (4) |
| network_name_descriptor (3) | 0x40 | Mb Mr | - | - | - | - | - | - |
| service_list_descriptor (3) | 0x41 | Ob Mr | - | - | - | - | - | - |
| satellite_delivery_system_descriptor (3) | 0x43 | mb Mr | - | - | - | - | - | - |
| cable_delivery_system_descriptor (3) | 0x44 | mb Mr | - | - | - | - | - | - |
| service_descriptor | 0x48 | - | - | Mb Mr | - | - | - | - |
| linkage_descriptor (3) | 0x4A | mb Mr | - | Ob Mr | * | - | - | - |
| short_event_descriptor | 0x4D | - | - | - | mb Mr | - | - | - |
| extended_event_descriptor | 0x4E | - | - | - | Ob Mr | - | - | - |
| component_descriptor (7) | 0x50 | - | - | Mb Mr | Ob Mr | - | - | - |
| component_descriptor | 0x50 | - | - | - | Ob Mr | - | - | - |
| stream_identifier_descriptor | 0x52 | - | - | - | - | - | - | Ob Mr |
| CA_identifier_descriptor | 0x53 | - | - | Ob Mr | Ob Mr* | - | - | - |
| content_descriptor | 0x54 | - | - | - | mb Mr | - | - | - |
| parental_rating_descriptor | 0x55 | - | - | - | Ob Mr | - | - | - |
| teletext_descriptor | 0x56 | - | - | - | - | - | - | mb Mr |
| local_time_offset_descriptor | 0x58 | - | - | - | - | Mb Mr | - | - |
| subtitling_descriptor | 0x59 | - | - | - | - | - | - | mb Mr |
| terrestrial_delivery_system_descriptor (3) | 0x5A | mb Mr (2) | - | - | - | - | - | - |
| private_data_specifier_descriptor (3) | 0x5F | mb Mr | - | mb Or | mb Or | - | - | mb Mr |
| frequency_list_descriptor (3) | 0x62 | Ob Mr | - | - | - | - | - | - |
| data_broadcast_id_descriptor | 0x66 | - | - | - | - | - | - | mb Mr |
| AC-3 descriptor (7) | 0x6A | - | - | - | - | - | - | mb Mr |
| application_signalling_descriptor | 0x6F | - | - | - | - | - | - | mb Mr (1) |
| default_authority_descriptor (4) | 0x73 | Ob Mr (4) | - | Ob Mr (4) | - | - | - | - |
| Related_content_descriptor (4) | 0x74 |  |  |  |  |  |  | Ob,Or (4) |
| content_identifier_descriptor (4) | 0x76 | - | - | - | Ob Mr (4) | - | - | - |
| Enhanced_AC-3_descriptor (7) | 0x7A |  |  |  |  |  |  | mb Mr |
| AAC_audio_descriptor (7) | 0x7C |  |  |  |  |  |  | mb Mr |
| extension descriptor (5) | 0x7F | mb Mr | - | mb Mr | mb Mr | - | - | mb Mr |
| user defined | 0x80-0xFE | - | - | - | - | - | - | - |
| NorDig private: logic_channel_descriptor (Version 1) (3) | 0x83 | Ob Mr | - | - | - | - | - | - |
| NorDig private: logic_channel_descriptor (Version 2) (3) | 0x87 | Ob Mr | - | - | - | - | - | - |
| NorDig private: content_protection_descriptor | 0xA0 | - | - | - | - | - | - | Ob Mr |
| CI_protection_descriptor | 0xCE |  |  | Mr(6) |  |  |  |  |
| Forbidden | 0xFF | Fb | Fb | Fb | Fb | Fb | Fb | Fb |
| - Descriptor not applicable or not yet used as minimum within NorDig Mb Mandatory to Broadcast, always/all time mb Mandatory to Broadcast if applicable, i.e. if certain criteria is met (e.g. if scrambling is used) Ob Optional to broadcast, but recommended (if applicable) Fb Forbidden to broadcast (may cause misinterpretation) Mr Mandatory to receive and interpret if broadcast Or Optional to receive and interpret (if broadcasted) * Optional for satellite and cable IRDs. |  |  |  |  |  |  |  |  |
| Note 1: Only mMandatory for IRD with HbbTV capability API based profiles Note 2: Mandatory to broadcast, in accordance with ETSI TR 101 211 [27]. Note 3: Descriptors carried in the NIT are not relevant for IRDs with IP-based Front-end, see Annex C. Delivery descriptors requirements depends of which tuning and demodulation the IRD supports, see table 12.7. Note 4: Only applicable for NorDig PVR IRDs only. Note 5: Only applicable for NorDig IRD-T2 Note 6: Mandatory to receive from SDT-actual for IRDs that support use of CIP-CAMs, see section 9.2 Note 7: The value of component_type to be used within the component_descriptor shall be equal to the value of component_type held in the AC-3_descriptor or Enhanced_AC-3_descriptor or AAC_descriptor Comment: Descriptors used for the UNT of the DVB SSU Enhanced profile are given in table 12.30 Descriptors used for the RCT (only applicable for PVRs) are given in table 12.21 |  |  |  |  |  |  |  |  |

Table 12.2 Overview over minimum used descriptors in NorDig broadcast and receivers

| Descriptor | Tag extension value | NIT (1) | BAT | SDT | EIT | TOT/ TDT | CAT | PMT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| T2_delivery_system_descriptor | 0x04 | mb Mr | - | - | - | - | - | - |
| Supplementary_audio_descriptor | 0x06 |  |  |  |  |  |  | mb Mr |
| reserved for future use | 0x09-0x7F | - | - | - | - | - | - | - |
| user defined | 0x80-0xFF | - | - | - | - | - | - | - |
| Descriptor not applicable or not yet used as minimum within NorDig mb Mandatory to Broadcast if applicable, i.e. if certain criteria are met (e.g. if scrambling is used) Mr Mandatory to receive and interpret if broadcast |  |  |  |  |  |  |  |  |
| Note 1: Descriptors carried in the NIT are not relevant for IRDs with IP-based Front-end, see Annex C |  |  |  |  |  |  |  |  |

Table 12.3 Overview over minimum used descriptors in the extension_descriptor in NorDig broadcast and receivers

### Character sets in text strings

The NorDig IRD shall support the character tables specified in Table 12.4. Respective character table in NorDig transmission is signalled by using bytes in the beginning of text field according to ETSI EN 300 468 Annex A.2 [15] and as reproduced for convenience in Table 12.4.

| Table description | Character code table | First byte | Second byte | Third byte |
| --- | --- | --- | --- | --- |
| Latin Alphabet | ISO/IEC 6937+ € (2) | N/A | N/A | N/A |
| Latin Alphabet No. 5 | ISO/IEC 8859-9 | 0x05 | N/A | N/A |
| Western Europe | ISO/IEC 8859-1 | 0x10 | 0x00 | 0x01 |
| North and North-East European | ISO/IEC 8859-4 | 0x10 | 0x00 | 0x04 |
| Latin Alphabet No. 9 | ISO/IEC 8859-15 | 0x10 | 0x00 | 0x0F |

Table 12.4 Character tables and signalling bytes in the beginning of text string

Note 1: The table above is relevant for text strings in SI and ESG.

Note 2: This table is referred in ETSI EN 300468 as Figure A1: Character code table 00-Latin Alphabet. 	This table is ISO/IEC 6937 plus Euro-sign (€). This is the default character set to be used if no 	particular character set is given (ref ETSI EN 300468, Annex A.2 [15]).

Note 3: The requirements for character table support can also be specified by the network/CA 	operator.

### Country and Language Codes within PSI/SI

Preferably all (main) country codes in ISO 3166 and all Alpha-3 language codes in ISO 639-2 should be handled. Due to the quite large number of codes in these specifications, Table 12.5 and Table 12.6  specifies the minimum types of codes that shall be handled by the NorDig IRD including the recommended translations. NorDig IRDs intended only for a specific network, the operator(s)/regulator(s) in charge for specifying the functionality of the IRD for that network and ensuring that the minimum requirements are met, may exclude some of the mandatory country and language codes.

NorDig has defined the language code ‘nar’ as “narrative”, that may be used for supplementary audio streams (audio description etc).

| Country (in English) | ISO 3166 code | Translation to be used (to native) | Possible to select as user’s preferred country |
| --- | --- | --- | --- |
| SWEDEN | SWE | Sverige | Mandatory |
| DENMARK | DNK | Danmark | Mandatory |
| IRELAND | IRL | Éire | Mandatory |
| FINLAND | FIN | Suomi | Mandatory |
| NORWAY | NOR | Norge | Mandatory |

Table 12.5 ISO 3166, Country codes for NorDig IRDs

| Language (in English) | ISO 639-2 | Translation to be used in DTT | Possible to select as user’s preferred languages | Comments |
| --- | --- | --- | --- | --- |
|  | Code | To native |  |  |
| Danish | dan | Dansk | mandatory |  |
| English | eng | English | mandatory |  |
| Finnish | fin | Suomi | mandatory |  |
| Irish / Gaelic | iri | Gaeilge | mandatory | ISO 639-2 Bibliographic |
| Irish / Gaelic | gle | Gaeilge | mandatory | ISO 639-2 Terminological |
| Norwegian | nor | Norsk | mandatory |  |
| Narrative | nar |  | recommended (optional) | may be used for supplementary audio streams |
| Original language | qaa | Original (dan) Original (eng) Alkuperäinen (fin) Bunaidh (gle) Original (nor) Original (swe) | recommended (optional) | See DVB SI spec ETSI 300468 Annex F for “original audio” |
| Sami | smi | Sámegiella | mandatory |  |
| Swedish | swe | Svenska | mandatory |  |
| Undefined | und | Udefineret (dan) Undefined (eng) Määrittelemätön (fin) Neamhshainithe (gle) Undefined (nor) Odefinierat (swe) | optional | treated same as original language (qaa) |

Table 12.6 ISO 639-2 Language codes for NorDig IRDs

## Network Information Table (NIT)

### The Network Information Table Descriptors

| NIT descriptors | Cable IRD | Satellite IRD | Terrestrial IRD |
| --- | --- | --- | --- |
| Metadata_pointer_descriptor (3) | Optional | Optional | Optional |
| Network_name_descriptor | Mandatory | Mandatory | Mandatory |
| Service_list_descriptor | Mandatory | Mandatory | Mandatory |
| Satellite_delivery_system_descriptor | n/a | Mandatory | n/a |
| Cable_delivery_system_descriptor | Mandatory | n/a | n/a |
| Terrestrial_delivery_system_descriptor | n/a | n/a | Mandatory |
| T2_Terrestrial_delivery_system_descriptor (2) | n/a | n/a | Mandatory (2) |
| Linkage_descriptor | Mandatory | Mandatory | Mandatory |
| Private_data_specifier_descriptor | Mandatory | Mandatory | Mandatory |
| Frequency_list_descriptor | Optional | Optional | Mandatory |
| default_authority_descriptor (3) | Mandatory (3) | Mandatory (3) | Mandatory (3) |
| (NorDig) logic_channel_descriptor (Version 1) | Mandatory | Mandatory | Mandatory |
| (NorDig) logic_channel_descriptor (Version 2) | Mandatory | Mandatory | Mandatory |

Table 12.7 NIT descriptors

Note 1:	The NIT is not used with NorDig IRDs with IP-based frontends. Hence if NIT is transmitted,
   	the NorDig IP IRDs shall ignore this table. Instead, the information provided by the NIT will be
 	replaced by the Service Discovery and Selection mechanisms, specified in Section 13.4.

Note 2: Descriptor is signalled in the extension_descriptor and only applicable for NorDig IRD T2.

Note 3:	NorDig PVR only.

### Metadata Pointer Descriptor (NorDig PVR only, Broadcast Record Lists)

This metadata pointer descriptor (see syntax ISO/IEC 13818-1 and additional description DVB/ETSI TS 102 323) is applicable for IRD supporting NorDig Broadcast record Lists.

For metadata pointer descriptor that is delivered in the NIT, refers that the metadata (NorDig Broadcast record Lists) is valid for the network (ie original_network_id). Located in NIT first common descriptor loop metadata pointer descriptor.

The value of the fields in the metadata pointer descriptor will be set as following signifying the metadata contained conforms to the NorDig BRL:

| Metadata pointer descriptor fields | Value/description | Remark |
| --- | --- | --- |
| metadata_application_format | 0x0100 | NorDig uses TV Anytime and DVB standard value 0x0100, (while DTG uses value 0x0101) |
| metadata_format | 0x3F | the IRD shall use the metadata_application_format value to interpret the format of the carried metadata |
| metadata_locator_record | 0b0 |  |
| program_number | service_id |  |
| transport_stream_location | original_network_id |  |

Table 12.8 NorDig BRL’s Metadata pointer descriptor field usage

#### Metadata Descriptors Extension

The metadata descriptor extension is included in both the metadata pointer descriptor and metadata descriptor, according with DVB/ETSI TS 102 323. However, the metadata descriptors extension found in the metadata descriptor shall have priority in the NorDig IRD over the metadata descriptors extension found in the metadata pointer descriptor.

The value of the fields in the metadata descriptor will be set as following signifying the metadata contained conforms to the NorDig BRL:

| Metadata pointer descriptor fields | Value | Remark |
| --- | --- | --- |
| DVB carriage format | 0x0 | align with TS102 323, section 9.2 Delivery of Containers |
| metadata service identifier flag | 0b0 |  |

Table 12.9 NorDig BRL’s Metadata pointer descriptor field usage

The fragment types list may appear in the broadcast. NorDig PVR support NorDig BRL shall operate correctly in the presence of them.

The metadata_service_identifier will be included and is used to identify the NorDig Broadcast Record List metadata service as profiled in DVB/ETSI TS 102 323 clause 9.6.

The user_data_bytes in the metadata descriptor extension shall carry a default group authority structure as defined in 12.2.2.2 below.

#### Default Group Authority Structure

The default group authority structure shall convey the authority for the default Record List group CRID.

| Syntax | No. Of Bits | Identifier |
| --- | --- | --- |
| Default_group_authority_structure(){ |  |  |
| authority_string_length | 8 | uimsbf |
| for (i-0;i<authority_string_length;i++){ |  |  |
| authority_byte | 8 | uimsbf |
| } |  |  |
| for (i-0;i<N;i++)[ |  |  |
| user_data_byte | 8 | bslbf |
| } |  |  |
| } |  |  |

Table 12.10 Default Group authority structure

authority_string_length: The number of authority_bytes in the authority_string.

authority_byte: This byte forms part of a string representing the default group authority.

### Cable Delivery System Descriptor

Reference to analogue services (PAL) may be used in NorDig digital (cable) networks. These services will be signalised in the SI as an own “transport stream”. The cable_delivery_system_descriptor for these analogue services will contain the correct centre frequency for the (PAL) vision carrier, while the other delivery parameters will be set to zero (i.e. not defined; FEC Outer = 0, Modulation = 0 etc). The service_list_descriptor for this “transport stream” (analogue service) will list only one TV service, with service_type set according to Table 72 in EN 300 468 [15] (0x07, PAL coded signal).

### Terrestrial Delivery System Descriptor

NorDig IRDs should use the modulation parameters (see below) in the terrestrial_delivery_system_descriptor as a recommendation when trying to tune to a multiplex. The NorDig IRD should, however, always be able to detect the modulation from the transmission itself (e.g. assisted by TPS bits).

Operators can broadcast the same transport stream in the same network using different modulation parameter settings. This allows for optimization of the network coverage in frequency planning involving SFN and MFN combination networks.

The modulation parameters carried in the terrestrial_network_descriptor are recommended to be the one applicable to the majority of receivers in that network.

### T2 Delivery System Descriptor

T2_delivery_system_descriptor is signaled in the extension_descriptor.

The NorDig IRD-T2 shall use the system parameters in the T2_delivery_system_descriptor to determine the mapping between original_network_id/network_id/transport_stream_id and T2_system_id/plp_id.

The NorDig IRD-T2 should use the other system parameters in the T2_delivery_system_descriptor as a recommendation when trying to tune to a multiplex. The NorDig IRD-T2 should, however, always be able to detect these system parameters from the transmission itself (i.e. assisted by L1 signalling).

Operators can broadcast the same transport stream in the same network using different system parameter settings, reflected in a different T2_system_id. This allows for optimization of the network coverage in frequency planning involving SFN and MFN combination networks.

### Linkage Descriptor

The following linkage_type values shall be interpreted by a NorDig IRD, when used inside the NIT

0x01, linkage to a service that contain information about the network

0x02, linkage to an EPG service (1)

0x04, linkage to transport stream which carries EIT schedule information for all of the services in the network (i.e. “barker channel” service).

0x09, linkage to DVB System Software Update service (bootloader), see section 10

Note 1: 	Not relevant for NorDig Basic IRD

### Frequency List Descriptor

The Frequency List Descriptor lists frequencies where the transport stream occurs, in addition to the frequency given by the system delivery descriptor. (e.g. the transport stream is broadcast on the frequency given by the system delivery descriptor or broadcast on one of the frequencies given by the frequency list descriptor. This feature can be used in terrestrial networks where the same transport stream can be received on more than one frequency.)

### Default authority descriptor (in NIT) (NorDig PVR only)

The Default Authority Descriptor (DAD), defined in DVB document Carriage and signalling of TV-Anytime (TVA) information ETSI TS 102 323 [34], may be used to shorten the CRIDs carried within EIT by defining an appropriate CRID default authority over a defined scope.

The DAD may be used in first loop of NIT to set a common Default Authority (DA) for all services within that Network. It may also be used in second loop (TS loop) to set a common Default Authority for all services within a Transport Stream.

The prefix “crid://” may be omitted from the start of the text string in the Default Authority in the NIT (both first or second loop). See separate section about CRID usage in 12.4.6

As described in ETSI TS 102 323 [34], where an event in the EIT does not have a complete URL within the Content Identifier Descriptor (CID) (i.e. a CRID starting with ‘/’), the NorDig PVR IRD shall:

- Use default authority (DA) defined for this service in the SDT.

- If no default authority is defined in the SDT, the PVR shall use the default authority in the second TS loop of the NIT for the actual transport stream this service belongs to.

- If no default authority is defined for the actual transport stream in second loop of NIT, the receiver shall use default authority in first loop in NIT for the network this service belongs too.

Example of handling of CRID together with Default authority values.

| Broadcast signalling |  |  |
| --- | --- | --- |
| DAD, Default Authority Descriptor | Default authority: |  |
| in NIT, first loop | ‘network.se’ |  |
| in NIT, second loop for TS1 | ‘ts1.network.se’ | (no other TS has default authority in NIT TS loop) |
| in SDT, service 1 of TS 1 | ‘provider_A’ | (no other services has default authority in SDT) |
| Service 1 in TS 1,Service 2 in TS 1 and Service 3 in TS 2 have all events with above Event1 and Event2 |  |  |
| CID, Content Identifier descriptor | CRID in EIT for services: |  |
| Event1 | ‘/abc/soap_event12345 | Example with non-complete CRID |
| Event2 | ‘abc/soap_event6789’ | Example with complete CRID |
| Receiver CRID compilation |  |  |
|  |  | Interpretation within IRD of CID + DAD |
| Service 1 belonging to TS 1 | Event 1 | crid://Provider_A/abc/soap_event12345 |
| Service 1 belonging to TS 1 | Event 2 | crid://abc/soap_event6789 |
| Service 2 belonging to TS 1 | Event 1 | crid://ts1.network.se/abc/soap_event12345 |
| Service 2 belonging to TS 1 | Event 2 | crid://abc/soap_event6789 |
| Service 3 belonging to TS 2 | Event 1 | crid://network.se/abc/soap_event12345 |
| Service 3 belonging to TS 2 | Event 2 | crid://abc/soap_event6789 |

### NorDig private; Logic_Channel_descriptor (LCD)

#### General

The logic channel descriptor is used in the second descriptor loop in the NIT, i.e. in each “TS loop” (1). Several LCDs may be listed in each TS loop.

Note 1:	 In one NorDig satellite network, the NorDig LCD will be carried in the SDT.

Data in this descriptor shall be treated as quasi-static and is used to order services in the IRD’s default service lists. The descriptor enables an IRD to create a (first time) default order of the services in the IRD’s service lists controlled by the operator, observe that this shall not affect the end-user defined lists in the IRD, if any.

Comment: The wording ‘Channel List’ refers here to the transmission side and this LCD is used to transmit a Channel List, while the ‘Service List’ refers to the IRD’s stored list of services. The IRD uses the transmitted LCD Channel List data among other SI data to create, update and sort services of its own default Service List).

#### NorDig private; Logical Channel Descriptor (version 1)

Note:	This older version of the NorDig Logical Channel Descriptor is in some NorDig Networks replaced by the newer version 2 below.

##### Syntax of LCD version 1

The syntax of the Logical Channel Descriptor (version 1) is shown in Table 12.11.

| Syntax | No. of bits | Identifier |
| --- | --- | --- |
| logical_channel_descriptor(){ descriptor_tag descriptor_length for (i=0;i<number_of_services;i++){ service_id visible_service_flag reserved logical_channel_number } } | 8 8 16 1 1 14 | uimsbf uimsbf uimsbf bslbf bslbf uimsbf |

Table 12.11 Logical_Channel_descriptor (LCD v1)

descriptor_tag: This shall be assigned to be 0x83 (decimal 131)

visible_service_flag: This 1-bit ﬁeld when set to ‘1 ’/’true’ indicates that the service is normally visible and selectable (subject to the service type being suitable etc.) via the NorDig  IRD service list. When set to ‘0’/’false’ this indicates that the IRD is not expected to offer the service to the user in normal navigation modes however theNorDig IRD should provide a mechanism to access these services (for example by direct entry of the logical channel number).

reserved: All “reserved” bits shall be set to ‘1’(observe, however, that the IRD shall be able to handle (neglect) future use of reserved bits).

logic_channel_number: this is a 14-bit field which indicates the broadcaster preference for ordering services. It shall be working together with service_type. The logic channel number shall be grouped into three service type categories; TV, Radio and Others/data services as specified in 12.1.5. Each broadcaster shall, as far as possible, allocate unique logic_channel_number within his original_network for each service type category (TV, Radio and Others). The logic_channel_number use is defined in Table 12.12.

| visible service flag | Logic channel number (decimal value) | Description |
| --- | --- | --- |
| 0 | 0 | Service not suitable for selection by the user. For example, the value zero may be used for data services only intended for selection from interactive applications or for firmware download services etc. |
| 1 | 0 | Reserved for future use |
| 0 | 1 – 16383 | Reserved for future use |
| 1 | 1 – 9999 | Service displayed in service list and ESG. Accessible via P+/- keys or from numeric keys (same value as decimal value of logic_channel_number) |
| 1 | > 9999 | Reserved for future use |

Table 12.12 Logic_channel_number allocation (LCD v1)

##### Sorting of services inside a Channel list

All “visible” services shall be displayed in the service list(s), sorted according to logic_channel_number and be addressed with a number in the service list equal to the logic_channel_number, as far as possible. The NorDig IRD may have several default service lists (or sections inside one) for the different service_types, for example one for each service_type or typically three main categories; TV, Radio and Others (“Others” is not applicable for IRDs without API, see chapter: 14: NorDig Basic IRD) If the IRD has several service_lists, the addressing of each service in each list shall match, as much as possible the logic_channel_number value (if no collision within a list).

Services shall first be ordered depending on their original_network_id, secondly to their service category, thirdly to their logic_channel_number and last on their service_type (independently of several services have collision in the logic_channel_number or if they are listed or not in the logic_channel_descriptor). I.e. first all services from one original network and within that original_network first all TV category services, after that all Radio category services and last all Other category services. After that original network, any next original network that the IRD is able to receive and so on.

Services listed in the logic_channel_descriptor, shall have higher priority when ordering the services in the default service list, than services that are not listed. With other words, broadcast services may not be listed in any logic_channel_descriptor and these shall be displayed and accessible in the default service list, but be located last in the service list, in order to their service_type.

##### Conflict handling of Logic_channel_number (Informative)

If several services are allocated to the same logic_channel_number, (within the same channel list, as may be the case if several terrestrial regions can be received at the same location or several satellite networks are received), one service shall be ordered according to the logic_channel_number and the others shall be placed last in that list. Empty spaces in the broadcast logic channel numbering shall then not be used, instead they shall be located last, after the service with highest logic_channel_number of that service_type (The broadcaster may quite consciously choose to leave empty spaces in the logic channel numbering, for future services, etc, in order to avoid a complete rearrangement of the list). How to choose which service within same service type that should be placed according to the channel list is up to the IRD manufacturer.

##### Example of Logic_Channel_descriptor (LCD) (version 1)

The examples in Table 12.13 and Table 12.14 below illustrates how broadcasted services shall be ordered in the IRD’s service lists according to (a terrestrial) broadcast.

| ONID | TSID | SID | NID | VSF | LCN | Service type | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 100 | 10 | 100 | 101 | 1 | 10 | 0x01 (TV) | SD service that in SDT incl Linkage to NorDig Simulcast replacement service at SID 140 |
| 200 | 10 | 100 | 200 | 1 | 10 | 0x01 (TV) | other network provider |
| 100 | 10 | 110 | 101 | 1 | 11 | 0x01 (TV) |  |
| 100 | 10 | 90 | 101 | - | - | 0x01 (TV) | no logic_channel_descr attached to this service |
| 100 | 20 | 120 | 101 | 1 | 23 | 0x01 (TV) |  |
| 100 | 20 | 200 | 101 | 1 | 23 | 0x02 (Radio) | other type of service |
| 100 | 20 | 120 | 102 | 1 | 23 | 0x01 (TV) | same service from other terrestrial region (and other transmitter point) |
| 100 | 20 | 130 | 101 | 1 | 24 | 0x01 (TV) |  |
| 100 | 30 | 140 | 101 | 1 | 10 | 0x19 (HDTV) | HD Simulcast service |
| 100 | 30 | 150 | 101 | 1 | 11 | 0x19 (HDTV) | HD service (no simulcast, only prio to LCN 11 due to its service_type) |
| 100 | 10 | 500 | 101 | 0 | 0 | 0x0C (Data) | e.g. SSU/Bootloader or EPG service |

Table 12.13 Example of broadcast of SI and services (LCD v1). The abbreviations are defined as: 
S_ID; service_id, ON_ID; original_network_id; TS_ID; transport_stream_id, NID; network_id,
VSF; visible_service_flag, LCN; logic_channel_number.

Table 12.14 exemplifies how services shall be sorted and listed in the IRD’s service list (from broadcast example above) in a NorDig IRD with at least two service_lists, one for TV and one for Radio services. Displayed for the viewer in each service list, will typically be the number (LCN) and the service_name.

| NorDig IRD, service list installation example |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TV service list |  |  |  |  |  | Radio service list |  |  |  |  |
| Number | ONID | TSID | SID | NID |  | Number | ONID | TSID | SID | NID |
| 10 | 100 | 30 | 140 | 101 |  | 23 | 100 | 20 | 200 | 101 |
| 11 | 100 | 30 | 150 | 101 |  |  |  |  |  |  |
| 23 | 100 | 20 | 120 | 102 |  |  |  |  |  |  |
| 24 | 100 | 20 | 130 | 101 |  |  |  |  |  |  |
| 25 | 100 | 10 | 110 | 101 |  |  |  |  |  |  |
| 26 | 100 | 10 | 90 | 101 |  |  |  |  |  |  |
| 27 | 200 | 10 | 100 | 200 |  |  |  |  |  |  |

Table 12.14 NorDig IRD service list example using LCD v1.

The service [ONID, TSID, SID] = 100, 20, 120 is listed only once (even though that service is transmitted twice). This due to that the IRD in this example above has a stronger and a better reception (quality) of the TS where service [ONID, TSID, SID, NID] = 100, 20, 120, 102 belongs to, than for the TS where the service [ONID, TSID, SID, NID] = 100, 20, 120, 101 belongs to.

#### NorDig private; Logical Channel Descriptor (version 2)

##### Syntax of LCD version 2

The syntax of the Logical Channel Descriptor (version 2) is shown in Table 12.15.

| Syntax | No. of bits | Identifier |
| --- | --- | --- |
| Logical_channel_descriptor(){ |  |  |
| descriptor_tag | 8 | uimsbf |
| descriptor_length | 8 | uimsbf |
| for (i=0;i<N;i++){ |  |  |
| channel_list_id | 8 | uimsbf |
| channel_list_name_length | 8 | uimsbf |
| for (i=0;i<N;i++) { |  |  |
| char | 8 | uimsbf |
| } |  |  |
| country_code | 24 | uimsbf |
| descriptor_length | 8 | uimsbf |
| for (i=0;i<number_of_services;i++){ |  |  |
| service_id | 16 | uimsbf |
| visible_service_flag | 1 | bslbf |
| reserved_future_use | 5 | bslbf |
| logical_channel_number | 10 | uimsbf |
| } |  |  |
| } |  |  |
| } |  |  |

Table 12.15 Logical_Channel_descriptor (version 2)

descriptor_tag: This shall be assigned to be 0x87 (decimal 135)

channel_list_id: This is an 8-bit field which serves as a label to identify the channel list (uniquely allocated within each original_network_id). The user should be able to select a preferred channel list to be used, when several are available during the first-time installation (or complete re-installation).

channel_list_name_length: This 8-bit field specifies the number of bytes that follow the channel_list_name_length field for describing characters of the name of the Channel List. Maximal length is 23 bytes for the channel_list_name.

char: This is an 8-bit field. A string of character fields specifies the name of the channel list, the channel_list_name. (Maximal length is 23 bytes for the channel_list_name). Text information is coded using the character sets and methods described in EN 300 468, annex A. The IRD is recommended to use the channel_list_name to present information on the OSD, for example when the user chooses a preferred channel list among several to create the IRD’s service list(s).

country_code: This 24-bit field identifies a country using the 3-character code as specified in ISO 3166 [49]. Each character is coded into 8-bits according to ISO 8859-1 [50] and inserted in order into the 24-bit field. In the case that the 3 characters represent a number in the range 900 to 999, then country_code specifies an ETSI defined group of countries. These allocations are found in ETSI ETR 162 [23].

EXAMPLE:	Sweden has 3-character code "SWE", which is coded as:

'0101 0011 0101 0111 0100 0101'.

The IRD may use this field (in combination with the IRD’s user preference settings) to propose a channel list to be chosen as preferred when several are available.

service_id: A service_id that belongs to the TS (i.e. services from other TS shall not be listed). One service may only be listed once in each channel list, but may belong to/be listed in more than one channel list.

visible_service_flag: This 1-bit ﬁeld when set to ‘1’/’true’ indicates that the service is normally visible and selectable (subject to the service type being suitable etc.) via the NorDig IRD’s service list. When set to ‘0’/’false’ this indicates that the IRD is not expected to offer the service to the user in normal navigation modes however the NorDig IRD should provide a mechanism to access these services (for example by direct entry of the logical channel number).

Reserved: All “reserved” bits shall be set to ’1’ (observe, however, that the IRD shall be able to handle (neglect) future use of reserved bits).

logic_channel_number: This is a 10-bit field which indicates the broadcaster preference for ordering services. It shall be working together with service_type. The logic channel number shall be grouped into three service type categories; TV, Radio and Others/data services as specified in section 12.1.5. Each broadcaster shall, as far as possible, allocate unique logic_channel_number within his original_network for each service type category (TV, Radio and Others). The logic_channel_number use is defined in Table 12.16.

| Visible service flag | Logic channel number (decimal value) | Description |
| --- | --- | --- |
| 0 | 0 | Service not suitable for selection by the user. For example, the value zero may be used for data services only intended for selection from interactive applications or for firmware download services etc. |
| 1 | 0 | Reserved for future use |
| 0 | 1 – 1024 | Reserved for future use |
| 1 | 1 – 999 | Service displayed in service list and ESG. Accessible via P+/- keys or from numeric keys (same value as decimal value of logic_channel_number) |
| 1 | > 999 | Reserved for future use |

Table 12.16 Logic_channel_number allocation

##### Handling of multiple Channel lists from same network

The Logical Channel Descriptor version 2 enables transmission within same network of multiple Channel lists, meaning that there might it be several channel lists available for a IRD to choose between. The NorDig IRD may treat each channel list as complete with all intended services for that network (original network id) and it is up to the broadcaster to ensure that all intended services are included in all lists.

The NorDig IRD shall at least store the sorting from one of the available Channel lists as default, but it is recommended that the NorDig  IRD store all the transmitted Channel Lists sorting that matches the IRD’s country code settings (especially for IRDs that are not letting the user choose list during installation).

When several Channel Lists are available from same network (original network id) for the IRD during first time installation (or complete re-installation), the NorDig IRD shall choose the channel list as the default one with following priority:

- The list with same country code as the IRD’s user preference setting’s country code. If several lists available with same matching country code, the IRD shall choose the one with lowest list_id value OR let the viewer choose from a list, (typically using the channel_list_name)

- If no Channel list has a country code that matches the user preference setting’s country code, the NorDig IRD shall let the viewer choose from a list (recommended) OR choose the one with lowest list_id value.

##### Sorting of services inside a Channel list

All “visible” services shall be displayed in the service list(s), sorted according to logic_channel_number and be addressed with a number in the service list equal to the logic_channel_number, as far as possible. The IRD may have several default service lists (or sections inside one) for the different service_types, for example one for each service_type or typically three main categories; TV, Radio and Others (“Others” is not applicable for IRDs without API). If the NorDig IRD has several service lists, the addressing of each service in each list shall match, as much as possible the logic_channel_number value (if no collision within a list).

Services shall first be ordered depending on their original_network_id, secondly to their service category, thirdly to their logic_channel_number and last on their service_type (independently of several services have collision in the logic_channel_number or if they are listed or not in the logic_channel_descriptor). I.e. first all services from one original network and within that original_network first all TV category services, after that all Radio category services and last all Other category services. After that original network any next original network that the IRD is able to receive and so on.

Services listed in the logic_channel_descriptor, shall have higher priority when ordering the services in the default service list, than services that are not listed. With other words, broadcast services may not be listed in any logic_channel_descriptor and these shall be displayed and accessible in the default service list, but be located last in the service list, in order to their service_type.

##### Conflict handling of Logical_channel_number

If several services are allocated to the same logical_channel_number, (within the same channel list, as may be the case if several terrestrial regions can be received at the same location), one service shall be ordered according to the logical_channel_number and the others shall be placed last in that list.

Empty spaces in the broadcast logic channel numbering shall then not be used; instead they shall be located last, after the service with highest logic channel number of that service_type. (The broadcaster may quite consciously choose to leave empty spaces in the logic channel numbering, for future services, etc, in order to avoid a complete rearrangement of the list).

Whenever two or more services within same category are allocated to the same logical_channel_number, the HD IRD shall priorities the advanced codec services as following for TV category; first 0x19 adv codec HDTV service, secondly 0x16 adv codec SDTV service and last 0x01 (MPEG-2 SD) TV service and for radio category; first 0x0A adv codec radio and secondly 0x02 radio service types.

How to choose which service within same service type and same service priority that should be placed according to the channel list is up to the IRD manufacturer.

##### Example of Logic_Channel_descriptor (LCD) (version 2)

Table 12.17 below illustrates how broadcast services shall be ordered in the NorDig IRD’s service lists, by use of LCD Version 2; with an example from a terrestrial broadcast.

| CLID | ONID | TSID | SID | NID | VSF | LCN | Service type | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 100 | 10 | 100 | 101 | 1 | 10 | 0x01 (TV) | SD service that in SDT incl Linkage to NorDig Simulcast replacement service at SID 140 |
| 1 | 200 | 10 | 100 | 200 | 1 | 10 | 0x01 (TV) | other network provider and other combination channel_list_id, ONID |
| 1 | 100 | 10 | 110 | 101 | 1 | 11 | 0x01 (TV) |  |
| - | 100 | 10 | 90 | 101 | - | - | 0x01 (TV) | no logic_channel_descr attached to this service |
| 1 | 100 | 20 | 120 | 101 | 1 | 23 | 0x01 (TV) |  |
| 1 | 100 | 20 | 200 | 101 | 1 | 23 | 0x02 (Radio) | other type of service |
| 1 | 100 | 20 | 120 | 102 | 1 | 23 | 0x01 (TV) | same service from other terrestrial region (and other transmitter point) |
| 1 | 100 | 20 | 130 | 101 | 1 | 24 | 0x01 (TV) |  |
| 1 | 100 | 30 | 140 | 101 | 1 | 10 | 0x19 (HDTV) | HD Simulcast service |
| 2 | 100 | 10 | 100 | 101 | 0 | 0 | 0x01 (TV) | Same service listed in a other channel list |
| 1 | 100 | 30 | 150 | 101 | 1 | 11 | 0x19 (HDTV) | HD service (no simulcast, only prio to LCN 11 due to its service_type) |
| 1 | 100 | 10 | 500 | 101 | 0 | 0 | 0x0C (Data) | e.g. SSU/Bootloader or EPG service |

Table 12.17 Example of broadcast of SI and services, LCN Version 2. The abbreviations are defined as: 
CL_ID; Channel_list_id, ON_ID; original_network_id; TS_ID; transport_stream_id,
NID; network_id, S_ID; service_ID, VSF; visible_service_flag, LCN; logic_channel_number.

Table 12.18 exemplifies how services shall be sorted and listed in the IRD’s service list (from broadcast example above) in a NorDig IRD with at least two service_lists, one for TV and one for Radio services and here with the channel list CLID 1 (country code match) and ONID 100 as the chosen preferred channel list. Displayed for the viewer in each service list, will typically be the number (LCN) and the service_name.

| NorDig IRD, service list installation example |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TV service list |  |  |  |  |  | Radio service list |  |  |  |  |
| Number | ONID | TSID | SID | NID |  | Number | ONID | TSID | SID | NID |
| 10 | 100 | 30 | 140 | 101 |  | 23 | 100 | 20 | 200 | 101 |
| 11 | 100 | 30 | 150 | 101 |  |  |  |  |  |  |
| 23 | 100 | 20 | 120 | 102 |  |  |  |  |  |  |
| 24 | 100 | 20 | 130 | 101 |  |  |  |  |  |  |
| 25 | 100 | 10 | 110 | 101 |  |  |  |  |  |  |
| 26 | 100 | 10 | 90 | 101 |  |  |  |  |  |  |
| 27 | 200 | 10 | 100 | 200 |  |  |  |  |  |  |

Table 12.18 NorDig IRD service list example using LCD v2.

The service [ONID, TSID, SID] = 100, 20, 120 is listed only once (even though that service is transmitted twice). This due to that the IRD in this example above has a stronger and a better reception (quality) of the TS where service [ONID, TSID, SID, NID] = 100, 20, 120, 102 belongs to, than for the TS where the service [ONID, TSID, SID, NID] = 100, 20, 120, 101 belongs to.

#### NorDig LCD simultaneous version 1 and version 2 transmissions

When broadcasting both LCD version 1 and version 2 within one Original Network ID, the NorDig IRD supporting both descriptors shall only sort according to the version 2 (i.e. NorDig LCD version 2 has higher priority).

#### Reception of multiple (DTT) networks and NorDig LCD

Comment: There are several areas within the Nordic region where DTT networks from the neighbouring countries can be received (for example southern part of Sweden, where Danish and German DTT can be received). Below follows a clarification to the use of the NorDig logical channel descriptor (LCD), regarding reception from multiple DTT networks

The NorDig IRD with terrestrial front-end shall be able to install several (DTT) original networks (with different original network ids).

For multiple original networks (original network ids) the NorDig IRD shall first sort/list all services from one original network (original network id) according to that LCD, before sorting/listing the next original network. The first original network is the primary network and any additional received original networks are referred to as secondary network(s).

The user shall be able to set which original network that shall be the primary, either via the user preferences, e.g. matching country setting (preferred) or via user selectable list of available original networks or similar mechanism. In order to simplify this, the NorDig IRD should map/translate the original network id into the country name. This means that for IRD where the user has set the country setting, the primary network should automatic be the country matching the original network id (and its services shall be listed first in the NorDig IRD’s service list).

(Automatic) updates within the NorDig IRD shall not change within the IRD’s service list the relative order between the installed primary network and secondary network(s).

For IRDs with terrestrial front-end, intended for the Nordic area it is recommended to include a translation list of all Northern Europe DTT original network ids. For original networks transmitting LCD version 2, the country code will be found directly within the descriptor without any mapping.

The primary DTT network shall be listed according to its LCD (version 2 or version 1), then additional (secondary) network(s) shall be listed, one-by-one, with its services after the primary network’s last listed services (i.e. not use empty logical numbers within first network). This means that the services from the additional DTT network(s) will not be listed according to its LCN values. Important is to only include visible marked services from additional (secondary) networks and not any service that is marked as non-visible. It is recommended - if possible- to keep the relative order between the listed services within any secondary network(s).

If the IRD manufacture chose to have multiple service lists, (one for each original network id or similar), then the primary network shall be the IRD’s default service list after the installation.

Note: Within DVB’s SI code allocation (ETR162), there is normally an un-written code of practise for digital terrestrial networks that the original network id has been allocated by the DVB office to the value of 0x2000 plus the country’s ISO 3166 Country code value. This is true for almost all countries, but not for e.g.  the Swedish DTT, which has original network id value (0x22F1).

#### Guidelines of number of services to be handled (Informative)

It is recommended that NorDig IRD with terrestrial front-end are able to handle up to 400 services identities during installation mode and 200 services. It is recommended that NorDig IRD with satellite, cable and IP front-end are able to handle up to 600 services identities during installation mode and 400 services afterwards. (More services are recommended during installation mode due to the possibilities to receive from several transmitter sites and use of multiple Channel lists).

## Service Description Table (SDT)

### The Service Descriptor Table Descriptors

SDT descriptors

Metadata_pointer_descriptor (2)

Service_descriptor

CA_identifier_descriptor

Linkage_descriptor

Service_identifier_descriptor

Default_authority_descriptor (2)

CI_protection _descriptor (3)

Table 12.19 SDT descriptors

Note 1: 	NorDig IRDs with IP-based front-end: SDT is only used for actual transport stream 	(table_id = 0x42).

Note 2: NorDig PVR only.

Note 3: Mandatory for IRDs that support use of CIP-CAMs, see section 9.2

#### Metadata Pointer Descriptor (NorDig PVR only, Broadcast Record Lists)

This metadata pointer descriptor is applicable for IRD supporting NorDig Broadcast record Lists.

For metadata pointer descriptor that is delivered in the SDT, refers that the metadata (NorDig Broadcast record Lists) is valid for that service(s) and will be placed in the descriptor loop of all services for which this metadata service is relevant.

For the rest see NorDig chapter 12.2.2 (including the metadata pointer extension).

### Service Descriptor

The service_type (under the service_descriptor) value 0x81 is reserved for the NorDig legacy bootloader use (see section 12.2.9.2).

### CA Identifier Descriptor

This descriptor may be present in the SDT when at least one service component is scrambled. The CA_system_id is allocated by ETSI and is given by ETSI ETR 162 [23]. The descriptor may be used statically (recommended). It will in that case be set according to the services regular/normal scrambling status. Alternatively, it may be used dynamically, in accordance with the current services scrambling status.

This static use enables IRDs to “grey mark” services that cannot be descrambled due to lack of the required CA-system for the relevant service(s). It allows the IRD to display services that are only temporary (event based) scrambled.

### Linkage Descriptor

The following linkage_type value shall be interpreted by a NorDig IRD when used inside the SDT:

0x05, linkage to a service replacement service. When present, the NorDig IRD should automatically switch to the replacement service if the ‘running_status’ is set to “1” (not running) and if the NorDig IRD are able to receive the SDT containing the original service during the replacement, also switch back when ‘running_status” is set to “4” (running).

0x82, NorDig Simulcast replacement service, linkage from an (MPEG2) SDTV based service to an (MPEG4 AVC) HDTV replacement service with the same content. It may be used during simulcasting of a service in both an SDTV and an HDTV version on separate service ids with same content within the same original network id. This linkage may be included in the (MPEG2) SDTV service (service_type 0x01) within the SDT pointing to the HDTV version (service_type 0x19) of the service. Whenever it is used, it will be used quasi-static.

When an SDTV service includes this NorDig simulcast replacement service linkage (0x82) pointing to the HDTV version of the service, the NorDig IRDs shall only include the HDTV version/(service) of the two services within its TV service list. The SDTV may be omitted or hidden at the end of the list, dependant of IRD implementation.

Clarification: If no ‘NorDig Simulcast replacement service’ linkage is included, both services shall be included. If only the (MPEG-2) SDTV version (service_type 0x01) is possible to receive (due to e.g. reception problems/limitations), the (HDTV) IRD shall include this service even if it carries a linkage ‘NorDig simulcast replacement service’.

### Default authority descriptor (in SDT) (NorDig PVR only)

The default authority descriptor, defined in DVB document Carriage and signalling of TV-Anytime (TVA) information ETSI TS 102 323 [34], may be used to shorten the CRIDs carried within EIT by defining an appropriate CRID authority over a defined scope.

The DAD may be used in the descriptor loop of each service in the SDT to set a Default Authority for all events in that service which do not have a complete URL (see example in chapter 12.2.8 Default authority descriptor in NIT.

The prefix “crid://” may be omitted from the start of the text string in the Default Authority in the SDT (normally the “crid://” will be omitted within the transmission). See separate chapter for CRID usage in section 12.4.6.

## Event Information Table

### General

The NorDig IRD shall support EIT present/following (p/f) for both actual and other tables (1).

The NorDig IRD shall support EIT schedule (sch) for both actual and other tables (1) up to at least 8 days of schedules.

Note 1:	DVB SI ‘Other’ tables are optional/not applicable for NorDig IRDs with IP-based Front-end.

### The Event Information Table Descriptors

| Event descriptors | EIT p/f | EIT sch |
| --- | --- | --- |
| short_event_descriptor | M | M (1) |
| Component_descriptor | M | M (1) |
| Extended_event_descriptor | M | O |
| Content_descriptor | M | M (1) |
| Parental_rating_descriptor | M | M (1) |
| CA_identifier_descriptor (optional) | O | O |
| Content_identifier_descriptor | M (2) | M (2) |

Table 12.20 EIT p/f descriptors

Note 1:	EIT schedule is recommended (optional) for NorDig IRDs with IP-based Front-end

Note 2:	NorDig PVR only.

### CA Identifier Descriptor

This descriptor is optional, however, it may be present in the EIT whenever at least one service component is scrambled. The CA_system_id is allocated by ETSI and is given by ETSI ETR 162 [23]. When used, it will be used dynamically, i.e. following the services scrambling status, mainly targeting the ESG/EPG applications.

### Content Descriptor

The NorDig IRD should handle all content nibbles listed in the DVB SI specification (ETSI EN 300 468 [15]), but shall at least be able to handle all content nibble level 1 classes. If there is no content coding in conformance with table present for an event, the default content description ”unclassified” shall be assumed by the receiver.

### Content Identifier Descriptor (NorDig PVR only)

The Content Identifier Descriptor (CID), defined in DVB document Carriage and signalling of TV-Anytime (TVA) information ETSI TS 102 323 [34], section 12.1, is used to associate a CRID to an event and is placed within the event loop of EIT. One or more instances of the descriptor may be present in the EIT (schedule and p/f) and a single descriptor may contain multiple CRIDs. Usage shall be consistent between EIT schedule actual and other within a Network. If a Content Identifier Descriptor is present in EITp/f, a NorDig PVR should use this in preference to the Content Identifier Descriptor for the same event in EIT schedule.

NorDig uses TV Anytime standard values for the crid_type, which are:

- 0x01, TVA programme CRID

- 0x02, TVA series CRID

- 0x03, TVA recommendation CRID

Only a single TVA programme CRID (crid_type 0x01) shall be associated with an EIT event.

The Content Identifier Table is not used in NorDig networks, (i.e. only crid_location == ‘00’ is used).

All events having the same programme CRID (type 0x01), regardless of IMI (see ETSI TS 102 822-4 [36], Section 10), refer to the same programme content.

All events having the same series CRID (type 0x02) belong to the same series.  An event may be associated with more than one series CRID. A CRID value may be reused after 91 days for other content.

### CRID encoding and reuse (NorDig PVR only)

The CRID shall be according to ETSI TS 102 822-4 [36], Section 8. The use of abbreviated CRIDs shall follow the rules set out in ETSI TS 102 323 [34] Section 6.3.1.

The NorDig PVR shall support CRIDs that is encoded according to the following rules:

- The CRID is further restricted to only contain characters encoded over the range from ISO 6937 0x20 to 0x7F.

The length of the CRID plus IMI (if any) shall not exceed 64 characters as a combined total for the crid’s authority, data and instance metadata identifier (including the separator ‘#’).

The CRIDs are not intended to be human readable and shall not be displayed on-screen.  The CRID is simply an identifier.

The authority part of a CRID shall be a registered internet domain name and therefore globally unique. The data part of a CRID is only unique within the scope of the associated CRID authority. An IMI is only unique within the scope of the complete CRID.

Broadcasters shall endeavour to use the same CRID whenever a programme is repeated.  However, this cannot be guaranteed.  A repeat of any content by a different service provider may result in a different CRID being assigned.

CRIDs and IMIs may be reused to refer to different programme concepts with the following restrictions:

Series CRIDs shall not be re-used for 91 days after the scheduled end-time of the last event that referenced this CRID.

Programme CRIDs shall “never” be reused for different programme content, (i.e. Broadcasters shall do their outmost to keep the CRID unique for all time).

IMI shall not be reused for a different instance of the same CRID within 3 hours of the scheduled end time (start_time plus duration). Two events greater than or equal to 3 hours apart but with the same CRID & IMI shall not be considered to be split parts of the same instance.

#### CRID type 0x01 – programme CRID (NorDig PVR only)

Programme CRIDs are used to identify two or more EIT events as being the same programme. This prevents duplicate programmes being recorded from within the same series and also allows alternative programme instances to be recorded (or offered for recording) if a booking clash occurs.

It is not necessary for all EIT events to have a programme CRID. An event may only include a maximum of one programme CRID. In the current context they are only useful where alternative instances or split programmes are being identified.

#### CRID type 0x02 – series CRID (NorDig PVR only)

Where a series CRID is conveyed in a CID according to the signalling outlined, it is to be used to only refer to an editorial concept of a series.

An event may be associated with more than one series, i.e. an event may include several series CRIDs. Where an event is associated with more than one series, an invitation to record ‘programmes in the same series as this event’ would book to record all events in all series associated with the selected event (see more section 14.3.3).

#### CRID type 0x03 – recommendation CRID (NorDig PVR only)

This identifies a looser linkage to another programme or series. A recommendation may point to a single event (programme CRID) or a series (series CRID).

A CRID in the CID shall be marked as crid type 0x03 (recommendation) and be a programme or series CRID.

It is not required that the recommendation CRID be present in the current scope of EIT. If the event referenced by the recommendation CRID is not present in the current scope of EIT, the recommendation may be presented to the user when it appears in EIT. If a recommendation CRID does not appear in EIT within 91 days of the referencing event, it shall be discarded.

A recommendation may reference an event earlier in the EIT schedule than the linked-from event, e.g. to link to a preview programme.

#### Split programme (split content) (NorDig PVR only)

A programme may consist of multiple EIT events within the same service or over several services. For example, a film might be divided into two parts (blocks) interrupted by a news programme in the middle or a longer sport event might be split into several parts over several services.

To be able to signal a split content programme the events shall include a CRID in the Content Identifier Descriptor (CID) that includes a programme CRID (crid type 0x01) with an Instance Metadata Identifier (IMI) extension. A “split programme” is defined as several events which have the same programme CRID and IMI value and the gap between each event is less than 3 hours (measured from the end of the preceding event to the start of the next event). Such events shall be considered to be segments of a single item of content. An item of content may be split across more than two events as long as the gap between each event remains less than 3 hours.

Where a broadcaster changes a single programme into a split programme (using IMIs) the broadcaster should ensure that one of the events of the new split programme maintains the event_id of the original single event. Failure to do this will result in lost or incomplete recordings.

### Event Information Table Schedule

Upon user request for EIT schedule information, the IRD shall (1) look for the reference using linkage descriptor mechanism in the NIT and perform a frequency re-tuning if necessary. Linkage_type 0x04 (“Transport Stream containing complete network/bouquet SI”) shall be used to refer to EIT schedule information.

Note 1:	 Recommended for NorDig Basic IRD

## Time and Date Table and Time Offset Table

The NorDig IRD shall have a real-time clock and time/date (calendar) running continuously. The time/date (calendar) shall be updated by incoming TDT and TOT from SI. NorDig IRD shall display the correct time for each country based on TDT, TOT, and the country name selected by the user.

NorDig IRDs with an IP-based front-end shall be able to establish real time and date (calendar) by any of the following methods:

- Updates by incoming TDT and TOT from the SI.

- Retrieval from the IP-network, based on RFC 1305 (Network Time Protocol (Version 3)). The address of the NTP server shall be retrieved from the Network Time Server DHCP option (42).

Note: 	This protocol may be used for Network time services for the transport stream with 
	accuracy better than 50 ms.

- Retrieval from the IP-network, based on RFC 2030 (Simple Network Time Protocol (SNTP) Version 4 for IPv4, IPv6 and OSI). The address of the SNTP server shall be retrieved from the Time Server DHCP option (3).

Note: 	This protocol may be used for Network time services for applications with an accuracy 	of 100 ms.

The NorDig IP IRD shall prefer the TDT/TOT, followed by the NTP protocol, in case no TDT/TOT is available in the stream. In case neither TDT/TOT nor a NTP server is available, the IRD shall retrieve time and date from the SNTP server.

In order to support offset from UTC time when using NTP or SNTP servers, the IRD shall support the DHCP Time Offset option (option number 2), specified in RFC 2132, IETF RFC 2132 [47].

The NorDig IRD should have an internal timer for the possibility to automatically switch from stand by mode to the operational mode. This timer shall (1) be initiated locally (accepted by the end user).

Note 1:	During this kind of start up or during any pre-programmed zapping, it is advisable that the 	NorDig IRD does not try to start anything that requires user acknowledgement or similar, for 	example updating of service list data or bootloader software.

### Time Offset Table Descriptor

Time Offset Table

local_time_offset_descriptor

Table 12.21 TOT descriptors

## Conditional Access and Program Map Tables

### Conditional Access Table Descriptors

Conditional Access Table

CA_descriptor

Table 12.22 CAT descriptors

### Program Map Table Descriptors

Program map Table

metadata_descriptor (3)

teletext_descriptor

Subtitling_descriptor

stream_identifier_descriptor

video_stream_descriptor

CA_descriptor

ISO_639_language_descriptor

AC-3 descriptor

Enhanced_AC-3_descriptor

AAC_descriptor

Supplementary_audio_descriptor

Private_data_specifier_descriptor

data_broadcast_id_descriptor (1)

application_signalling_descriptor (2)

carousel_id_descriptor (1)

(NorDig) Content_Protection_descriptor

related_content_descriptor (3)

Table 12.23 PMT descriptors

Note 1: Use of the data_broadcast_id_descriptor and the carousel_identifier_descriptor for signalling relevant for the SSU is specified in ref ETSI TS 102 006 [30], see also section 12.7

Note 2: 	This descriptor is only mandatory for the NorDig HbbTV IRDs-based receivers (i.e. Hybrid Profile)

Note 3: 	 NorDig PVR only.

### Component priority multiple video or audio streams

Component priority when multiple video or audio streams are received

The following applies for services that transmit in parallel more than one type of video/audio stream under the same service_id (e.g. simulcasting within the same service):

For video decoding, NorDig IRDs shall select the service’s video component, and set the default setting in accordance with the priority list in Table 12.24.

| Video codec | Stream_type | Priority |
| --- | --- | --- |
| MPEG-4 AVC HP@L4 HD video stream | 0x1B | 1 (highest) |
| MPEG-4 AVC HP@L4 / L3 SD video stream | 0x1B | 2 |
| Basic, MPEG-2 MP@ML video stream (or MPEG1) | 0x02 (0x01) | 3 (lowest) |

Table 12.24 Default priority order for the IRD between different video streams

For Audio decoding, NorDig IRDs shall prioritise the service´s components according to chapter 6.5, Audio prioritising.

NorDig IRDs shall ignore advanced audio streams when it does not support such decoding those streams. For example, an IRD that do not include any AC-3 (down-mix) decoder, (maybe only supports pass-through of AC-3 to the digital audio output), shall not choose the AC-3 audio stream as default. Instead it shall choose among the IRDs supported audio stream types according to chapter 6.5, Audio prioritising.

When several streams of the same type are received, the primary stream shall be selected in accordance with the definitions in section 6.5 (Audio Prioritising).

Note 1: Some typical dynamic changes in audio streams are listed in section 6.9.

Note 2: Hard of hearing audio stream is defined in ISO 639_language_descriptor in [70].

### ISO 639 language descriptor

The NorDig IRD shall use the ISO 639 descriptor to select the preferred audio stream according to user preference settings, see section 6.5 Audio Prioritising. Observe that other audio descriptors have priority compared to this as stated in section 6.5.4. The descriptor will at least be used when a service contains more than one audio stream of same codec. (Used for all NorDig audio codecs streams, ie MPEG-1 Layer II, HE-AAC, AC-3 and E-AC-3 audio streams).

The NorDig IRD shall use the field ‘audio type’ to select the preferred audio stream. The NorDig IRD shall minimum support audio types: 0x00 ‘undefined’ (referred here to as “normal”) and 0x03 ‘visual impaired’ (note 1) audio.

To avoid issues in legacy IRDs, language code ‘nar’ may be used for some supplementary audio streams (audio description etc), (‘nar’ is a non-allocated code in ISO 639 Part 2, intended here to represent “narrative”). Some networks may even use European language that is not used in that country, e.g. in Finland the language Dutch may be used for the supplementary audio streams in some networks. Due to legacy IRDs, audio type 0x00 ‘undefined’ may be used for some Supplementary Audio streams. See section 6.11 for further information about Supplementary Audio.

The NorDig IRD shall minimum support language codes as stated in section 12.1.8.

### AC-3 descriptor

The NorDig IRD supporting AC-3 shall use the AC-3 descriptor (‘Number of channels flags’ value) when selecting appropriate audio format (stereo or multichannel), when several audio streams is available for the service, as described in section 6.5.4 Signalling to be used for audio property.

For AC-3 audio streams with no supplementary audio descriptor but the ‘service type flags’ set to Visually Impaired (VI) in the AC-3 descriptor, this shall be the trigger for the NorDig IRD that the audio stream is a broadcast mixed Supplementary Audio.

Note: AC-3 is not suitable for receiver mixed Supplementary Audio and NorDig IRDs are not required to support this.

### Enhanced AC-3 descriptor

The NorDig IRD supporting E-AC-3 shall use the Enhanced AC-3 descriptor (‘Number of channels flags’ value) when selecting appropriate audio format (stereo or multichannel), when several audio streams is available for the service, as described in section 6.5.4 Signalling to be used for audio property.

For E-AC-3 audio streams with no supplementary audio descriptor but the ‘service type flags’ set to Visually Impaired (VI) in the Enhanced AC-3 descriptor, this shall be the trigger for the NorDig IRD that the audio stream is a Supplementary Audio. In this case the NorDig IRD shall use the ‘full service flag’ to determine whether the stream is broadcast mixed audio (value ‘1’) or if it is a receiver mixed audio (value ‘0’).

### AAC descriptor

The NorDig IRD supporting HE-AAC shall use the AAC descriptor (‘audio type’ value) when selecting appropriate audio format (stereo or multichannel), when several audio streams is available for the service, as described in section 6.5.4 Signalling to be used for audio property.

The broadcaster may typically signalize the maximum ‘profile and level’ and ‘AAC type’ that the audio stream may have in transmission. An audio stream that is signalized as multichannel may for certain periods only be a stereo channel audio stream.

The NorDig IRD supporting HE-AAC shall support profile and level values stated in Table 12.25 below

| profile and level value | Description | NorDig comment |
| --- | --- | --- |
| 0x51 | LC-AAC Level 2 | no SBR used, up to stereo (mono or stereo) |
| 0x52 | LC-AAC Level 4 | no SBR used, up to 5.1 (e.g. mono, stereo, 5.1) |
| 0x58 | HE-AAC Level 2 | SBR may be used, up to stereo |
| 0x5A | HE-AAC Level 4 | SBR may be used , up to 5.1(e.g. mono, stereo, 5.1) |

Table 12.25 AAC descriptor’s Profile and level values for NorDig IRDs

The NorDig IRD supporting HE-AAC shall minimum support AAC types stated in Table 12.26 below.

| AAC type value | Description | NorDig comment |
| --- | --- | --- |
| 0x01 | HE-AAC audio, single mono channel | trigger for normal (mono) audio |
| 0x03 | HE-AAC audio, stereo | trigger for normal (stereo) audio |
| 0x05 | HE-AAC audio, surround sound | trigger for normal (multichannel) audio |
| 0x42 | HE-AAC Receiver mixed Supplementary Audio as per annex E of TS 101 154 | If no supplementary audio descriptor is included then this shall be trigger for receiver mixed Supplementary Audio and this audio stream may include any AD_descriptor in PES_private_data for pan and fade control. |
| 0x47 | HE-AAC receiver mix audio description for the visually impaired | If no supplementary audio descriptor is included then this shall be trigger for receiver mixed Supplementary Audio and this audio stream may will not include any AD_descriotr in PES_private_data. |
| 0x48 | HE-AAC broadcaster mix audio description for the visually impaired | If no supplementary audio descriptor is included then this shall be trigger for broadcast pre-mixed Supplementary Audio |

Table 12.26 AAC descriptor’s AAC type values for NorDig IRDs.

As stated in section 6.5.4.3, for AAC audio streams without AAC descriptors shall be assumed to be of AAC type 0x03 (level 2, “normal” stereo).

The AAC descriptor shall have priority to any MPEG-4 audio descriptor.

### Supplementary_audio_descriptor

The NorDig IRD shall support the supplementary audio descriptor for both types of Supplementary Audio streams (i.e. Broadcast mixed and Receiver mixed).

The NorDig IRD shall for the supplementary audio descriptor minimum support the combinations of mix type and editorial classification as listed in Table 12.27 below.

| Audio type | Audio purpose | Mix type | editorial_classification |
| --- | --- | --- | --- |
| “normal” | Main audio | 1 | 0 |
| SA broadcast mixed | Audio description | 1 | 1 |
| SA receiver mixed | Audio description | 0 | 1 |
| SA broadcast mixed | Spoken Subitlies | 1 | 3 |
| SA receiver mixed | Spoken Subtitles | 0 (note 1) | 3 (note 1) |

Table 12.27 Minimum combinations of the supplementary audio descriptor for the NorDig IRD. SA refers to Supplementary Audio.

The supplementary audio descriptor may also be used for the “normal” audio streams, the IRD has to parse the descriptor to determine the audio type.

Note 1: 	Up to some version of the ETSI EN 300 468 (v1.13.1) it has been missed to include the important combination mix_type ‘0’ with editorial classification ‘3’ (referring to receiver mixed spoken subtitles) and mentions that this is invalid and may not be used. NorDig however overwrite this and make this combination valid and shall be supported by the NorDig IRDs.

### Metadata descriptor (NorDig PVR only, Broadcast Record Lists)

This metadata descriptor (as described in ISO/IEC 13818-1 [52]) is applicable for IRD supporting NorDig Broadcast record Lists.

Located in PMT ES descriptor loop the metadata descriptor which relates to the carousel carrying the NorDig BRL metadata service.

The metadata descriptor (PMT) and the metadata pointer descriptor (NIT/SDT) for the NorDig BRL will have matching values.

The metadata descriptor for NorDig BRL will also include metadata descriptor extension and Default Group Authority Structure as described in NorDig chapter 12.2.2.

The value of the fields in the metadata descriptor will be set as following signifying the metadata contained conforms to the NorDig BRL:

| Metadata pointer descriptor fields | Value | Remark |
| --- | --- | --- |
| metadata_application_format | 0x0100 | Matching value set in metadata_pointer_descriptor. NorDig uses TV Anytime and DVB standard value 0x0100, (while DTG uses value 0x0101) |
| metadata_format | 0x3F | Matching value set in metadata_pointer_descriptor |
| metadata_service_id |  | Matching value set in metadata_pointer_descriptor |
| decoder_config_flags | 0b000 | Referring to that no configuration is required |
| DSCM-CC_flag | 0x1 | Referring to that the metadata is delivered in a carousel |
| service_identification_record |  | shall carry the path to the metadata in the carousel to which this descriptor refers. This path follows the convention set out in TS 102 323 [34] clause 5.3.4.2. |

The metadata descriptor will include Metadata Descriptors Extention in accordance with DVB/ETSI TS 102 323[34], as described in NorDig section 12.2.2.1.

### Content_ Protection_descriptor

This descriptor is used to signal the content protection level for the received service, see Table 12.29.

The IRD shall use the signalled Content Protection level together with the IRD’s HDCP user setting to determine if HDCP shall be enabled or disabled on the HDMI output interface, see section 8.6.4.

The Content Protection descriptor shall be conveyed in the descriptor loop immediately following the program_info_length field in the Program Map Table. The descriptor only applies to the service to which the program map table is applicable. If the descriptor is missing for a service, it shall be interpreted as content protection level 0x01 (see Table 12.29).

| Syntax | Number of bits | Identifier |
| --- | --- | --- |
| Content_Protection_descriptor(){ descriptor_tag descriptor_length Content_Protection_level } | 8 8 8 | uimsbf uimsbf uimsbf |

Table 12.28 Semantics for the Content_Protection_ descriptor

descriptor_tag: The descriptor tag is an 8-bit field which identifies the descriptor. The value for the Content_Protection_descriptor is 0xA0 (decimal 160, i.e. within the “user defined” range).

descriptor_length: The descriptor length is an 8-bit field specifying the total number of bytes of the data portion of the descriptor following the byte defining the value of this field. The value is always 0x01 for the Content_Protection_descriptor.

Content_Protection_level: The descriptor length is 8-bits, and represents the level of Content Protection required for a program/service. The currently defined levels and their interpretation are defined in Table 12.29:

| Content Protection level | Description |
| --- | --- |
| 0x00 | Outputs shall not be protected. Any protection mechanism, such as HDCP, shall be switched off, regardless of video format and resolution |
| 0x01* *) Default value if no signalling | Content protection is not required. The IRD’s content protection mechanism may be set to ON or OFF, regardless of video format and resolution. |
| 0x02 | Content protection is mandatory for video with higher resolution than 576 horizontal lines, but not for resolutions with 576 or less lines. The IRD’s content protection mechanism shall be set to ON for video with higher resolution than 576 horizontal lines, and may be set to ON or OFF for video with resolution lower than or equal to than 576 horizontal lines. |
| 0x03 | Content protection is mandatory regardless of video format and resolution. The IRD’s content protection mechanism shall be set to ON. |
| Note: Additional levels may be defined in the future. If the IRD receives other values for the Content Protection level than defined in Table 12.29, it shall neglect all bits except the last two and use these bits for interpreting the Content Protection level. The IRD shall in addition to the information provided in the Content_Protection_descriptor also use Content Protection information provided via the Conditional Access system when determining the required content protection, see section 8.6.4 |  |

Table 12.29 Specified content protection levels

IRD actions for the various required Content Protection levels, as depending on the HDCP user setting, are specified in section 8.6.4 (Table 8.2).

### Related Content Descriptor (NorDig PVR only)

The NorDig PVR that supports Trailer booking (see 14.3.10), shall be able to handle the related content descriptor (as specified in ETSI TS 102 323 [34]).The RCT is signalled in the service’s PMT dynamically. The related content descriptor will typically only be referenced in the PMT as long as RCT is availble.

## SSU UNT Descriptors

## Related Content Table (NorDig PVR only)

## NorDig Broadcast Record List syntax (NorDig PVR only)

# Navigator

## General

The NorDig IRD shall implement a basic Navigator, which provides user access to system information, and allows the user to control the operation of the IRD. The Navigator is by definition part of the system software. A minimum functionality is required as specified below.

The Navigator shall include a service list function and a basic Event Schedule Guide (ESG), see EN 300 468 [15]. The Navigator shall also initiate bootloading, as described in chapter 10.

The Navigator shall support the Nordic and English languages.

## Service List

### Service List Requirements

#### Service List Requirements for IRDs, except for IP-based front-end

The NorDig IRD shall maintain a service list based on SI-information. The NorDig IRD identifies a service uniquely through the combination of original_network_id, transport_stream_id and service_id. (The broadcaster however shall make services uniquely identified in the broadcast through the combination of only original_network_id and service_id).

The service list shall include the services and should also include the corresponding network names. The service list can be completely updated by the user by initiating the tuning/scanning procedure(s) for the connected tuners (see section Error! Reference source not found.). The corresponding part of the service list shall be updated within 1 second after reception of an updated SI table; updates should be made each time the NorDig IRD is switched from active to stand-by and shall be made each time the NorDig IRD is switched from stand-by to active.

The IRD shall build up different sections inside one service list or build up several service lists, one for each different service category as the default IRD service list(s). Minimum three different sections/lists shall be supported for three different categories of service_types and they are ‘TV’, ‘Radio’ and ‘Data’/’other’ services, (see 12.1.5 for service categories).

Whenever two or more services within same category are allocated to the same logical_channel_number, the NorDig HD IRD shall first prioritise the advanced codec services as stated in Table12.1 above (see chapter 12.1.4 for priority between different services within same service category).

The service list shall be displayed to the user. The user shall be able to select a service from the displayed service list. The selected service shall appear immediately (see section 11.4).

The IRD should provide functionality for the viewer to build up additional personal service lists with the viewer’s own preferred services (like mixed service_type) and own preferred order or manually re-order the default service list(s). If any network operator makes changes in his part of the service list, the NorDig should place new entries at the end of the corresponding part of the user service list.

The information in the descriptors specified in Table 13.1 and Table 13.2 shall be displayed. The original network operator name may be omitted in case only one network is available.

#### Service List Requirements for IRDs with IP-based front-end

NorDig IRDs with IP-based front-end shall support the Service Discovery mechanism specified in ETSI TS 102 034 [31] and the additions specified in section 13.4.

Based upon this mechanism, NorDig IRDs with IP-based front-end shall be able to generate and maintain a service list of all available services at any time.

### Service list functions for the Network Information Table (NIT)

The NorDig IRD shall (1) make use of the descriptors listed in table 13.1 in all NIT_actual (the transport stream the NorDig IRD is tuned to) and NIT_other (other transport stream) tables available in order to update the service list (system delivery data, number of transport streams, logic channel number etc).

Note 1: 	NorDig IRDs with a terrestrial front-end shall be able to install and update the service list components even if the transport stream does not contain the terrestrial_ delivery_ system_descriptor in the NIT_actual and the NIT_other streams (NIT_actual: the transport stream the IRD is tuned to. NIT_other: other transport stream). 

NorDig IRDs with a terrestrial front-end dedicated for stationary reception may receive TS including NIT_actual and NIT_other tables. Due to the nature of the terrestrial networks all the transport streams listed in the NIT_other can be impossible to be received. Therefore, before using the information in NIT_other tables, carefulness shall be taken. 

NorDig IRDs with a terrestrial front-end dedicated for mobile and portable reception may also receive TS including NIT_actual and NIT_other tables. In that case information in NIT_other tables may have informative background use for faster service acquisition when receiver is moved from one coverage area to another coverage area. 

NorDig IRDs with a IP-based front-end: Not relevant. See Error! Reference source not found.

A cable NorDig IRD should provide functionality for fast installation of services by typing the network_ID into the receiver. In such a case, the IRD shall process only that specific NIT (actual and other) table (with corresponding network_ID) from current/actual transport stream and only install/display services listed in that table’s service_list_descriptors.

A Navigator shall never display services that the IRD is not able to receive or decode except for de-scrambling (i.e. a pure satellite IRD shall not display services which are described in NIT_other tables for secondary cable networks).

A NorDig IRD shall not install, be able to reach or display services or networks with original_network_ID and/or network_ID which are marked as ‘private_temporary_use’ as defined in ETSI ETR 162 [23] (i.e. an original_network_ID 0xFF00 – 0xFFFF and/or network_ID 0xFF01 – 0xFFFF). (This descriptor may be used by broadcasters to avoid confusing consumers with (shorter) test and demonstration transmissions).

Services that are not listed in NorDig Logic_channel_descriptor, shall be displayed in the service list(s) and shall be located last in the list (for that service_type).

NIT descriptors

Network_name_descriptor

Satellite_delivery_system_descriptor

Cable_delivery_system_descriptor

Terrestrial_delivery_system_descriptor

T2_delivery_system_descriptor (1)

Service_list_descriptor

(NorDig) Logic_channel_descriptor

Note 1: Descriptor is signaled in the extension_descriptor and is only applicable for NorDig IRD-T2.

Table 13.1 NIT descriptors

### Service List functions for the Service Description Table (SDT)

The IRD shall (1) use the descriptors listed in table 13.2 from both SDT_actual and SDT_other tables to update the service list (service names etc.).

Note 1: Not relevant for NorDig IP IRD. See Error! Reference source not found.

SDT descriptors

Service_descriptor

CA_identifier_descriptor

Table 13.2 SDT descriptors

### Network Evolution and Service Changes

The NorDig IRD shall (1) dynamically update the Service List whenever changes occur in the NIT and SDT tables (i.e. typically handling the version numbers of the tables).

Initiation of update in the Service List that the IRD is not able to perform in the ‘background’ without disturbances or user action/confirmation, shall (only) be made after manual power up or after user selection to an affected service/transport stream (e.g. when re-scanning is needed). Initiation of update in the Service List for services signaled as invisible should not require action/confirmation from user, (see section 12.2.9 NorDig private; Logic_Channel_descriptor (LCD) for invisible services).

Note 1: 	For NorDig IP IRDs this funtion is handled by updating the Service Provider Discovery
 	Information and the DVB-IP ServiceOffering Records. The version_number shall be
 	incremented whenever the content of these records changes, hence the NorDig IP IRD
 	shall continuously monitor the version_number.

## Event Schedule Guide (ESG)

The Event Schedule Guide (ESG) is part of the Navigator in the IRD and presents program event information for the user about its installed services via a Graphical User Interface (GUI) as defined by the IRD manufacturer.

### ESG Requirements

#### ESG and length

The NorDig IRD shall be able to display an ESG for the user with a minimum of eight days (1) of schedule data, defined as whole days from present day and ahead according to EN 300 468 [15]. The ESG shall be based on the information from the EIT tables, see section 12.4 and EN 300 468 [15].

Comment: Eight days of schedule data for the services within one NorDig network (original network) consists of typically of up to 2-4 MB of data per language.

Note 1:	Support for EIT schedule is recommended (optional) for NorDig IRDs with IP-based Front-end

#### Proper handling of EIT data

The NorDig IRD shall maintain proper behaviour in case of the incoming event information data for the services exceeds the available free memory for the ESG and not affect the IRD’s basic service decoding and navigation.

If the NorDig IRD’s memory for the ESG is exceeded, then the NorDig IRD shall prioritize the event information nearest in time and first reduce the data most far ahead in time for all service, for example via using EIT table filtering (instead of reducing service by service). (If the user has made personalized favourite service list consisting of a subset of available services, then the NorDig IRD should first priorities favourite services and then events most nearest in time).

The NorDig IRD shall be able to handle situations when the EIT is not present.

#### ESG performance

The NorDig IRD shall maintain the full ESG up to date and be able to display the ESG within 10 seconds after selection, even if not all EIT sections have been received (in which case gaps may occur in timeline for some services). The NorDig PVR shall be able to present the ESG regardless of recording status (i.e. while recording or timeshifting an event, it shall be possible to present the ESG).

The NorDig IRD should cache EIT data during normal service viewing to speed up time to present a full ESG after selection.

The ESG shall be non-discriminatory and display all services on an equal basis.

The ESG shall process and display the relevant content of the following tables (including start-time, end-time/duration and content of all descriptors specified below in 13.3.2 and 13.3.3).

### Event Information Table (EIT)

NorDig IRD shall make use of the EIT p/f tables from both EIT_actual and EIT_other tables.

| Event descriptors | EIT p/f | EIT sch |
| --- | --- | --- |
| Short_event_descriptor | M | M (1) |
| Extended_event_descriptor | M | M (1) |
| Component_descriptor | M | O |
| Content_descriptor | M | M (1) |
| Parental_rating_descriptor | M | M (1) |
| CA_identifier_descriptor (optional) | O | O |
| Content_identifier_descriptor | M (2) | M (2) |

Table 13.3 EIT p/f descriptors

Note 1:	EIT schedule is optional for NorDig IRDs with IP-based Front-end.

Note 2:	 NorDig PVR only.

#### Dynamic update of EIT data

The EIT data shall be treated as dynamic information which means that the EIT data is often updated by the broadcaster several times during a day, for example

- The description of events may be changed/updated from when the event was first “published”/broadcasted,

- Some events may be re-scheduled,

- Past events from current day may be removed from broadcast etc.

As factory default, the NorDig IRD shall continuously monitor and update the ESG without user request to update (for example by monitoring the tables’ version ids). Information in the ESG shall be updated within 10 second after reception of the updated tables.

#### Multiple languages in EIT data

Some NorDig networks transmit EIT data in multiple languages; the NorDig IRD shall be able to display the EIT data from chosen language (according to user preferences).

#### Time periods with no EIT data or missing EIT data

If services have gaps in the EIT data or if services have no or missing EIT data the ESG shall not display an error message (i.e. it shall not give the impression for the user that IRD or the EIT data is faulty), instead the text information field should stay empty or display informative text (like “No event information”), see recommendation of informative text in Annex F. A gap in the EIT data refers here to a gap in time between one event ends (start_time + duration) to the next event starts (start_time) for a service.

An example could be that when the IRD is still caching the EIT data (e.g. after startup) do not display any text information for events and services that are missing EIT data (typically due to that the IRD has yet not received that EIT data) and after a reasonable long time of caching (for example after analysing last_section_number etc) for missing EIT data then display informative text.

#### Parental Control from EIT data

The parental rating descriptor is used to give a rating of programme events based on age or other criteria and is used to prevent children from viewing unsuitable programme events. Parental rating may differ from one country to another, according with DVB SI Guidelines (ETSI TS 101 211 [27]). The typically case is that parental rating is dynamic and varies from one program event to another for one service.

The NorDig IRD shall provide a parental rating functionality that, when enabled, blanks video and mutes sound out of the IRD whenever the incoming rating value in the parental rating descriptor of the EIT data of current viewed programme event (present event) is higher than IRD’s user setting. The user shall be able to enable and disable the parental functionality and when enabled to configure a minimum age/level (in years). It should make use of 4 digits pin code or similar to access and change settings (a technique that prevent easy access for a child). See section 16 for factory default user preference settings.

The IRD may/should in addition offer the viewer a fast way of temporary disable blanking and muting of a service with higher rating than settings via user entering pin code or similar. This temporary disable of parental rating should continue as long as IRD stays on selected service and go back to user preference settings after change of service or re-start of IRD.

The IRD should start/(stop) its blanking video and muting audio within 1 second after reception of selected service’s present event information containing parental rating higher/(lower) than its user settings but shall at least within 10 seconds react after reception of parental rating information in the EIT data.

For NorDig PVR see also 14.3.9 Full Service Recording and 14.4.5 Full service playback.

#### ESG presentation filtering of EIT data

The NorDig IRD should provide a function which allows the user to filter events in the ESG with the same content type (from content descriptor), events belonging to the same series (from content identifier descriptor), recommended events referred to by an event and to search events using keywords (from description).

### Time and Date Table (TDT) and Time Offset Table (TOT)

The ESG shall display correct event times as conveyed by the TDT, adjusted by the offset relayed in the TOT and using the country name selected by the user.

Time Offset Table

Local_time_offset_descriptor

Table 13.4 TOT  descriptors

Note:	TDT contains UTC time, but no descriptors.

Additional requirements for NorDig PVRs (NorDig PVR only):

The ESG shall display all events using the correct time offset applicable at the event start time and date signalled in the EIT. The offset applied to the events UTC time shall be determined first on time of booking and subsequently updated if there is a new next_time_offset received. If there is more than one time_offset_section, ESG shall use the section that is applicable for event start time and date, see illustrative example in fig 13.1.

Figure 13.1 Example of time displayed via the ESG when the broadcast time information includes a shift from one to two hours time offset

## Service Discovery and Selection for IRDs with IP-based front-end

Comment: NorDig has specified a set of requirements for Service Discovery and Selection for IRDs with IP-based front-ends, based on ETSI TS 102 034 [31]. This text is currently suspended, because it is not used in Nordic networks that carry IPTV signals; service selection and discovery in these networks are all based on browser technology. A revised specification for SD&S, based on use of browser technology is being considered by NorDig

## User Information (UI) about service components (audio and subtitling)

### UI for Audio information

For services with multiple audio streams (PIDs), the NorDig IRD shall be able to display information about available incoming audio streams, for the user to temporary select audio stream (see 6.5and 6.11.4). Then the NorDig IRD shall minimum include following information about the currently available incoming audios (listed in the PMT for the current selected service):

- Audio Language (if available)

- Audio Type (it shall be possible for the user to differentiate available supplementary audio stream(s) from “normal” audio stream(s) even when these have the same language)

- Incoming Audio format (multi/stereo)

This can for example be done via wordings and/or symbols etc. (As stated in 6.11.6 ‘receiver mixed’ audio shall not be decoded on its own).

The NorDig IRD should also display information about:

- Incoming Audio stream type (audio codec)

The NorDig IRD should for currently selected audio also display information about:

- Outgoing audio format (multi or stereo)

- Outgoing Audio stream type (uncompressed, DTS, DolbyDigital (AC-3) etc)

### UI for Subtitling information

The NorDig IRD first automatically selects subtitling stream/PID if several subtitling streams are available (DVB Subtitling stream/PID prior over EBU Teletext subtitling, see section 7.1).

The NorDig IRD shall be able to display information about different incoming subtitling pages within the selected subtitle stream/PID, for the user to be able to temporary select subtitling page or disable displaying subtitling. Then NorDig IRD shall be able to display following information about the subtitling:

- Subtitle language

- Subtitling type (it shall be possible for the user to differentiate available hearing impaired/hard of hearing subtitling pages from “normal” subtitling page(s) even when these have same language)

## Accessibility menus and settings

### Accessibility settings

The NorDig IRD should support easy access to Settings for Accessibility Services by:

- Ensuring that Accessibility settings are made available using a minimum number of User Interaction steps (e.g. by use of dedicated remote control buttons),

- Grouping Accessibility settings together within the IRD's User Interface

### Talking menus (optional)

Talking menus refers here to Text-to-Speech functionality in the IRD that use speech synthesis that typically reads the IRD’s UI menus etc, this is to improve usage for blind or partially sighted users.

The NorDig IRD is recommended to support Text-to-Speech functionality and that it is implemented according with IEC 62731 [80]. If supported, then the NorDig IRD shall be able to enable and disable this Text-to-Speech feature see 16.2and see chapter 16.4 for the IRD’s factory default setting.

# NorDig PVR feature requirements (NorDig PVR only)

## Introduction - PVR

This chapter (together with PVR-related requirements specified in chapter 12 and section 13.3) specifies the minimum requirements for a NorDig PVR, which may record live services (TV, radio etc) in persistent memory (like HDD) for later playback, (even if the IRD has been completely powered off between the recording and the playback).

A NorDig PVR is a recordable IRD that fulfils all mandatory requirements specified in this chapter 14 (and relevant part in chapter 12 and 13), which among other things includes support for series recording, accurate recording, split recording etc. (A NorDig IRD with some recording capability but which do not meet all mandatory NorDig PVR requirements is just a “NorDig IRD with recording capability”).

NorDig recordable iDTVs that can not fulfil the requirements for Simultaneous Recording (14.3.8), is anyway recommended to support the other NorDig PVR requirements (like series recording, accurate recording, split recording, playback features etc).

Programming a recording (or booking) in the PVR refers to the user action of making a booking to record a live event, series and/or other broadcast content, either to be scheduled in the future or for immediately recording.

## General - PVR

### Recording File System

The NorDig PVR shall at all times keep a file system of the PVR’s recordings and make them available upon request for the user to select and playback.

The user shall be able to list the recordings as:

- all recordings, as ordered by date&time

The user should be able to list the recordings as:

- all series (where all episodes of a series are group into same item in the list) and all non-series recordings

- all episodes of a specific series

For all recordings that have been programmed via the ESG or EPG, each recorded item in the NorDig PVR’s list of recordings shall display for the user at least information about the recorded event’s date of recording and event_name extracted from EIT data during the recording. If no event information is available for a specific recording, then the service_name shall be used. For manual recording that span several events (excluding split events, see below), it is recommended to use the service_name instead.

In addition, the NorDig PVR’s list of recordings should display information about the item’s time and duration of the recording and the description taken from the EIT (preferably all EIT data for the event, like short and extended description, etc). The description of the event (preferably from the EIT p/f data) could typically be presented when highlighting the recorded item in the list of recordings.

Due to the latency within all transmission of EIT data, it is recommended to wait 1 minute after the event’s start_time or until the event’s running status has become ‘running’ before acquire the event’s EIT data (if EIT p/f is used).

The time and date in the list of recordings shall use the local time offset (based on the user’s preferences settings), as applicable at the time of recording.

A NorDig PVR with IP front-end may use equivalent data to EIT data to display information about recorded items, if no EIT data is available inside the IP Network (as specified by the Operator).

### Recording capacity

The NorDig PVR shall be able to indicate its momentary available recording capacity. The basis for the indication shall be explained in the instruction manual and should be in terms of capacity (e.g. GB), percentage or time (e.g. hours). (PVRs should consider when indicating available capacity in terms of time that many services often uses variable bitrate and that the capacity will vary between different services types, like SDTV, HDTV, radio etc)

The Manufacture shall clearly state the recording capacity for the NorDig PVR in marketing specification and in the instruction manual. It shall as a minimum be specified in terms of bytes (like GigaByte, GB etc).

### Deletion of recordings

The user shall be able to manually delete any recorded event in the NorDig PVR by deleting one recording at the time. The user should be able to manually delete all recorded events in the NorDig PVR. The user should be able to manually delete all recorded events belong to the same Series in the NorDig PVR.

The NorDig PVR shall have a mode (set as factory default) where the NorDig PVR shall ask for user confirmation before deletion of recordings (i.e. the NorDig PVR may in addition have alternative mode where the NorDig PVR will delete recordings without any extra confirmation).

### Failed and incomplete recordings

The NorDig PVR shall have a mechanism for informing the user of failed or incomplete (partial) recordings. For incomplete (partial) recordings it should inform the user how much of the booked event has not been successfully recorded.

### Save only the last number of episodes

The NorDig PVR should be able to let the user set the PVR to save/keep a configurable number of the latest events (episodes) within a Series. If the user has set the NorDig PVR to keep a specific number of events in a series and the NorDig PVR has recorded more, then the NorDig PVR shall automatically remove the “oldest” event (without any additional user confirmation).

The criteria to decide which event is the “oldest” within a Series, shall be based on which event has the lowest TVA programme CRID value. If the NorDig PVR can not easily decide which event that has the lowest TVA programme CRID value (for example due to lack of digits inside the TVA programme CRID), then the PVR shall keep all recordings from that Series.

### File system intact after update

The NorDig PVR’s file systems of recorded events shall be intact after

- updating of the PVR IRD’s System Software and/or

- updating of CA system and/or

- re-installation or update of installed services

### Limitations in local storage, interfaces, extraction and removable media for recordings

Some of the broadcasted content is signalled as protected, for example via the CA-system, copyright and/or copy protection signalling as specified by the relevant network/CA operator.

The requirements for external interfaces of recordings, internal storage, limitation for extraction of protected content and for removable media for the NorDig PVR and other NorDig recordable IRDs are specified by the relevant network/CA operator.

For protected content (unless otherwise specified by the relevant network/Operator), it shall not be possible to extract or output content from the NorDig PVR and other NorDig recordable IRDs in un-protected format, therefore all recordings shall be stored in a protected format.

Some networks and operators require local scrambling for all recording, some other allows either local scrambling or original DVB scrambling etc.

NorDig PVRs’ and other NorDig recordable IRDs using standardised removable media, such as DVD or Blu-ray for recording of protected content shall downscale any HD content with higher resolution than SD (i.e. higher than 720x576i25 or 960x540p50) to SD resolution (maximum 720x576 or 960x540) before storing it to the removable media. HD content Content with HD resolution or higher may be recorded in its original resolution if the recording retains the original broadcast scrambling or any other local device scrambling approved by the Network/Operator. The downscaling should be made as specified in 5.11.

### Disk management / de-fragmentation

The NorDig PVR shall have appropriate disk management (including de-fragementation handling for Hard Disk Drive based PVRs) to minimise need for re-formatting disk during its lifetime.

### Safe margins

The NorDig PVR should have the possibility to add extra recording time before and after the event’s scheduled time, as a safe margin. Typically this could be done as a pre-defined default user preference setting value, configurable by the user via the user settings.

### NorDig Record Lists functionality

This NorDig Broadcast Record List (BRL) is optional for NorDig IRDs/PVRs to support, but if supported then all specified requirements for this NorDig Broadcast Record List feature in this document (recording, playback, management, presentation etc) shall be supported.

#### Recording capacity and disk management for Record Lists

For the avoidance of doubt, there are no additional requirements on recording capacity and disk space management specific to Broadcast Record List.

#### Broadcast Record List Entry Point

NorDig PVR supporting BRL shall provide a static entry point for displaying the available broadcast record lists for the user, e.g. through a native User Interface menu.

#### Presentation of Broadcast Record Lists

NorDig PVR supporting BRL shall have a means to present all Broadcast Record Lists in the default broadcast record list group (see section Error! Reference source not found.). NorDig PVR supporting BRL shall not present broadcast record lists from groups with a different root unless specifically recognised.

NorDig PVR supporting BRL shall only display broadcast record lists that are:

- included in the last received version of the metadata, and

- lists to which the user is currently subscribed, even when that list is no longer in the metadata, subject to the rules in 14.3.20.10.

It is possible that the metadata associated with a broadcast record list will change from time to time. When presenting broadcast record lists, the NorDig PVR supporting BRL shall always present the last received metadata. The NorDig PVR supporting BRL shall present at least the title and synopsis of each broadcast record list.

#### Broadcast Record List Subscription and Un-subscription

Users shall be able to subscribe to any broadcast record list (as profiled by section 14.2.10.3) and unsubscribe from any previously subscribed broadcast record list. Lists that are currently subscribed to shall be identified.

#### Presentation of Future scheduled BRL Recordings

The NorDig PVR supporting BRL should mark events in ESG and EPG that are booked/schedule to be recorded in the future that is part of any active Records Lists the PVR currently subscribed to.

#### Presentation of Acquired BRL Recordings

The NorDig PVR supporting BRL shall have a means to present a list of available recordings acquired using the broadcast record list functionality. The NorDig PVR supporting BRL shall have a means to present the title and synopsis for all such recordings. It is strongly recommended that NorDig PVR supporting BRL indicate to the user the subscribed broadcast record list(s) in which the acquired event was contained at the time of recording.

The user should be able to list the BRL recordings as:

- only all recordings of a specific Record List

- only all recordings from a BRL group

To avoid confusing viewers since for example BRL recordings may automatically be deleted (via expiry date) or embargoed, a PVR may have to different entry points for listing manual set (single or series) recordings compared to BRL recodings.

If the event has an expiry time, information regarding expiry of that recording shall be available. Recordings that have passed their mandatory expiry time (see 14.2.10.9) shall be clearly identified as expired and shall not be playable.

If the event has an embargo time (see 14.2.10.8), information regarding the embargo time shall be available, unless the embargo time has passed.

#### Limitations for External Media

In addition to any content management signalling and what is specified in NorDig section 14.2.7, storage of broadcast record list content to optical or other removable media shall be permissible provided that:

- any embargo time has passed and

- the content is not signalled with a mandatory expiry time

#### Management of Embargoed Content

For content with ActivationTime, until this date and time is reached the recorded content shall not be playable although associated metadata (synopsis etc.) may be displayed if required.

The NorDig PVR supporting BRL shall use the broadcast time as the underlying timebase to determine whether the embargo time has passed.

For acquired recordings with ActivationTime that are (or have been) listed in multiple Record Lists that the PVR is subscribing to, the one with the latest ActivationTime shall be used for the embargo.

#### Management of Content Expiry

For recorded content initiated by the Broadcast Record Lists and which includes a ExpiresTime element, (applicable for both “visible” scheduled events in EIT and for “non-visible” off-scheduled events), the ExpiresTime shall define the date and time after which the associated instance of the content shall no longer be playable by the user (ie recording shall be deleted), persistent memory used for the recording shall be released and the recorded event shall no longer be listed for the user be among available recordings.

The NorDig PVR supporting BRL shall be able to display the time of when the recorded content will expire, either in absolute time or in relative time (like “tomorrow”).

The NorDig PVR supporting BRL shall use the broadcast time as the underlying timebase to determine whether the expiry time has passed.

For acquired recordings with ExpiresTime that are (or have been) listed in multiple Record Lists that the PVR is subscribing to, the one with the earliest ExpiresTime shall be used to determine when the content shall no longer be playable by the user.

## PVR Recording

### General PVR recording

The NorDig PVR shall as a minimum support recording up to 20 Mbps per (SD) service and shall as a minimum support recording up to 30 Mbps per (HD) service. The NorDig HEVC PVR shall in addition as a minimum support recording up to 25 Mbps per (Full HD) service and shall as a minimum support recording up to 45 Mbps per (UHD) service.

The NorDig PVR shall be able to record for at least 60 minutes with the above transmission figures.

The NorDig PVR shall be able to record all supported service types (TV, radio etc) and its components (as described in 14.3.9).

On-screen informational messages or menus generated by the NorDig PVR shall not be recorded with the programme content.

### ESG/EPG recording programming

The NorDig PVR shall make it possible for the user to select individual events and series to be recorded from the ESG or EPG display (based on information from EIT data).

The NorDig PVR shall be able to make a booking from the ESG/EPG and later record this event both for events that do not include any CRID (i.e. only based on service and event_id) and events that include CRID’s.

The selected event(s) for recording shall be marked as selected for recording on the ESG and EPG display.

If the user selects an event for recording from the ESG/EPG which has the same programme CRID value as an earlier recording within the NorDig PVR list of recordings, the NorDig PVR shall inform the user at the time of booking that this new selected event might already have been recorded and offer the option for the user to record anyway or not (1). The NorDig PVR should display information about this earlier recording (like the event name, date of recording and description).

### Series recording

All events that have the same series CRID belongs to the same Series. An individual event inside a Series is referenced here as an Episode. (For definition of CRID see section 12.4.6.2).

The NorDig PVR shall be able to record a complete Series via the CRID.

The NorDig PVR shall store and track series CRIDs that are programmed for recording for up to 91 days between occurrences in EIT schedule. To allow broadcasters to reuse a series CRID for a different editorial concept, the NorDig PVR shall discard any series CRIDs not seen in EIT for 91 days.

The display of programmes selected for recording shall include an indication if the programme is included as a consequence of being one of a series.

The IRD should be aware that the default authority may be changed over time (for example a service might have default authority added in SDT), the NorDig PVR should automatically update its stored default authorities (not only during installation).

#### Series, record all episodes

The NorDig PVR shall support recording of all episodes of a specific series via series CRID’s in the broadcast.

It shall be possible from ESG/EPG to program the NorDig PVR to record a series of events. The NorDig PVR shall indicate in the ESG/EPG that an event is part of a series. The NorDig PVR shall, if the user selects to record the event that belongs to a series, request the user what to record:

- Only the single event selected.

- Several or All events (episodes) of the series

#### Series, record limited number of episodes for a series

The NorDig PVR should support recording of a (limited) number of episodes of a specific series via series tagging in the broadcast.  The limitation should either be a period of time or a number of episodes.

#### Series, only one instance/copy of each episode

The NorDig PVR should support the feature to only record one instance/copy of each episode in a series for series recording (to handle re-runs).

### Split recordings

A programme may consist of multiple EIT events within the same service or over several services. For example, a film might be divided into two parts/blocks interrupted by a news programme in the middle (see Figure 14.1 A) or a longer sport event might be split into several parts/blocks over several services, (see Figure 14.1 B).

Signalling carried in the SI allows the PVR to identify and record all the events containing the parts of a single programme. A “split programme” is a single piece of content which comprises of two or more EIT events having the same CRID and IMI value with the gap from the scheduled end time (start_time plus duration) to the scheduled start time of any two of those events is less than 3 hours (see section 12.4.5).

The NorDig PVR shall consider a split programme to be segments of a single item of content. When selecting a split programme for recording, the NorDig PVR shall select and record all constituent events so that the complete programme content is recorded.

Figure 14.1 (Illustration) Handling of split recordings for the NorDig PVR. Split programme events (events with the same CRID value that are broadcast close in time to each others) shall be recorded with one and the same programming by the NorDig PVR.

A) max gap time between events with the same programme CRID value that shall still be treated to

belong to the same programme for recording.

B) Split programme over several services.

C) Split programme with gap and over several services

There are cases where a NorDig PVR may during the time of programming a recording only see a single event with the booked CRID and IMI combination (for example initially only the first part/block of the split programme has so far been included in the EIT). The NorDig PVR shall continue to monitor the EIT for additional events with the same CRID and IMI combination and include them to the selected recording.

In case of overlap between the split events and if the NorDig PVR has limitation in recording capacity when back-to-back recording, then the NorDig PVR shall first finalise recording of the first part/event of the split programme (according to the events start time and duration) before starting recording the next part of the split programme, this is the same behaviour as back-to-back recordings.

During the lifecycle of EIT schedule broadcasters may change programmes from split to single or vice versa.

In the NorDig PVR split recordings shall clearly be marked in the list of recordings as constituent parts belonging to the same programme, for example as one and the same entity or similar (1). It shall be enough to select only one entity from the file list of recording to get a playback of the complete programme (including all its all constituent events).

### Recommended events

When the event selected has one or more recommendation(s) associated with it (signalised from original event with crid_type 0x03), the NorDig PVR should offer the option to record the recommendations (programme or series) as well as the selected programme or series.

Once selected, the appropriate recommended event(s) shall also be marked as selected to be recorded on the EPG display.

The recommended event(s) may also have recommendation(s) of its own. When user chooses to select to include the recommendation(s) into the recording, the NorDig PVR shall not include more than the original event’s recommendation(s) (i.e. the NorDig PVR shall not follow more than the original event’s initial recommendation and a recommendation should not be used to create a linked list of events to be recorded).

### Alternative instance

When scheduled recordings overlap, the NorDig PVR should use the alternate instance information (1), when provided, to record one or more of the programmes at their alternate times thereby minimising the conflict, subject to any device limitations (e.g. available space).

Where a programme is repeated in its entirety a broadcaster may assign the same programme CRID to both EIT events. The NorDig PVR should detect an alternative instance of a programme (as when two events have same programme CRID). This can be used to assist in resolution of booking conflicts. Where alternate instances belong to the same series this allows the NorDig PVR to only record a single showing of each episode, usually the first.

### Accurate Recording

The NorDig PVR shall determine the timing of the recording through monitoring of the EITschedule and EIT present/following information.

The NorDig PVR shall record at least for the duration where the event ID in the EIT present table matches the event ID of the event selected from the EIT schedule to a precision of 10 Seconds, unless there is a conflict with another recording event.

The NorDig PVR may monitor the running status of the event in EIT present table and record only that part where the running status is set as running. (Note as specified in DVB SI Guidelines TR101 211 [27], ‘undefined’ running status in EIT present table shall treat the present event as running).

Where the Event ID is signalled in EIT present table early (in advance of the schedule start_time) the NorDig PVR shall start recording. As a minimum, the NorDig PVR shall handle early starts of at least 10 minutes, provided there are no other recordings in progress.

The NorDig PVR shall monitor the EIT schedule and EIT present/following for updates to the start time and duration such that any event will be captured should the schedule be updated no later than 2 minutes prior to the current scheduled time of broadcast.

Where the Event ID does not appear within EITp/f (in neither the present nor following tables) within the expected schedule time and duration the NorDig PVR should record according to the scheduled start time and duration. If the event id appears in the EIT following table at the scheduled start time, it means that the event is delayed and the NorDig PVR should wait with the start of the recording until the event ID appears in the EIT present table.

The duration of the recording shall be changed even if the EITp/f is updated after the start time has elapsed, until the event is no longer present in the EIT present table.

If the NorDig PVR starts to record at the expected scheduled start time even if the event does not appear within EITp/f, the recording shall be considered as incomplete.

Where there is a loss of signal or EIT present table is no longer being received, the NorDig PVR will continue to record at least until the end time of the event (defined by start_time plus duration) in the last received EITp/f. If the signal is restored the NorDig PVR will continue to record according to its normal operation.

In standby mode (where the NorDig PVR IRD is not decoding any transport stream) the NorDig PVR shall have the capability to power on automatically twice per day to update the EIT and scheduled recordings. There may be an option to amend the time of power on or to switch off the facility as a user option, but factory default for this shall be that it is on.

Figure 14.2 (Illustration) Handling of accurate recording. In red the minimum requirement, record only using as long as event is present in EIT present section table. In purple the improved version, via using event is present in EIT present sections and running status is “running”.

### Simultaneous recording

The NorDig PVRs shall be able to record one service while viewing another, independently even if the services are on different transport streams.

The NorDig PVR should be able to record a background service (that is not viewed) at the same time as timeshift record the viewing service, independently if the services are on different transport streams.

### Full service recording

The NorDig PVR shall be able (factory default) for all recordings to include all supported components/PIDs for the basic TV viewing listed in the PMT of the recorded service (e.g. video, audio 1, audio 2, EBU Teletext, DVB subtitles, PCR etc) and other relevant metadata from the PSI and SI (like parental rating, signal protection/HDCP etc), excluding any HbbTV or other API related streams (Any HbbTV related streams are optional to be included in the recording).

Note:	 For a NorDig PVR using removable media formats (such as DVD or Blu-ray) for recordings, 	such devices shall include all supported components/PIDs for that format and any subtitling shall 	(according to the user preference settings) be burnt in to the video or converted into a supported 	subtitling format. Observe the limitation specified above for removable media.

### Trailer booking/Promotional Linking (optional)

#### General

The trailer booking (or promotional linking) is typically used during a promotion trailer to give the viewer the opportunity to easy and directly program/book their PVR to record the event the trailer is referring to.

The NorDig PVR supporting Trailer booking shall have the ability to decode and process Related Content Signalling as defined in chapter 12.8 (RCT) and 12.6.11 (related content descriptor) in order to drive broadcast-triggered native or API based applications typical example Trailer Booking (Promotional Linking). (This refers to that in the future other usage of the RCT may be added).

The decoding and display of information referenced by descriptors carried in the RCT shall be supported as defined in chapter Error! Reference source not found. (short event descriptor in RCT) and Error! Reference source not found. (image icon descriptor).

The event name shall be displayed together with any promotional text at time of booking (when displaying the Trailer booking menu on screen). At the time of booking, the NorDig PVR shall not include any event description text from the short event descriptor.

The short event descriptor’s event name (from the RCT or EIT) shall be used to provide information about the event in the PVR’s list of booked recordings. The short event descriptor’s event description text from EIT may also be used in the PVR’s list of booked recordings to provide more information.

The NorDig PVR supporting Trailer booking shall display all combinations of the broadcast icon and default icon in accordance with chapter 14.3.10.2 below.

The NorDig PVR supporting Trailer booking shall display all combinations of the broadcast icon and default icon in accordance with chapter 14.3.10.2 below.

#### Icon activation and deactivation

The NorDig PVR supporting Trailer booking shall display and remove from display the icon according to following rules:

- a new RCT table (version number change) with a link count greater than zero shall cause the icon to appear

- a new RCT table (version number change) with a link count equal to zero shall cause the icon to disappear.

The display/removal of the on-screen icon and allocation/release of remote key for trailor booking shall occur within 2 seconds of the RCT table changes described above.

Note: 	“link count” is the number of links the receiver can understand and use, not the total links in the 	RCT. For example, a receiver without an IP connection may ignore all links that reference online 	content, hence, such a link by itself would not cause an icon to pop-up on screen.

When the NorDig PVR supporting Trailer booking is in normal TV viewing mode and a promotional link becomes active the green button on the remote control shall temporary be redirected away from its other usage (like HbbTV application). As soon the trailer link is no longer active, the trailer booking key shall be released.

Exceptions: If user has entered into another TV mode (e.g. entered Teletext page, HbbTV application, menu mode or similar modes) before the promotional link becomes active, the green button shall not be redirected. If user enters into another TV mode (e.g. entered Teletext page, MHP application, menu mode or similar modes) during an active promotional link, the green button shall be released from trailer booking usage. If the user re-enters back to normal TV viewing mode during an active promotional link, the NorDig PVR should redirect trailer booking key (green button) and display the trailer booking icon.

Normal TV viewing mode refers here to when user has not actively entered Teletext page viewing mode, “opened” HbbTV application, entered menu or similar modes. (Observe that the broadcaster may control in the broadcast change of HbbTV application state and allocation of remote keys. Broadcaster should be aware that trailer booking may not work properly if used keys for trailer booking on remote control are allocated for the HbbTV application).

An icon may be deactivated by pressing the receiver‘s usual cancel key e.g. “back”, “TV”, .

It should be possible in the user preference setting to pre-defined a default alternative if NorDig PVR supporting Trailer booking shall be active and react to broadcasted Trailer booking messages (in RCT) or not. (Factory default setting shall be on (active) for trailer booking).

#### Default Icon

For NorDig PVR supporting Trailer booking, the icon that is displayed on screen during an active link can be from a receiver inbuilt icon or a broadcaster signalling icon. ETSI TS 102 323 [34] in Link info structure regarding default icon signalling defines four combinations of how these icons can be used: all combinations may be used.

The default icon shall be a representation of the green button on the remote control – so the icon shall look like a green button.

The popup shall also contain a text in local language – telling the user to press the green button to program/book the described event (example: ’Press Green To Book’)

Manufacturers should be aware that the green button on the remote control will be used to select the trailer booking and take account of other on-screen objects (such as HbbTV applications).

Figure 14.3 Illustrative example of a trailer booking default icon

Note: 	For services that have both HbbTV application and trailer booking, a HbbTV Hybrid IRD will typically allocate the green button for HbbTV usage and prevent this button for trailer booking control unless the HbbTV application have released the green button for its usage. It is up to the broadcaster to control and release the green button from HbbTV application usage to enable HbbTV Hybrid IRD to use the green button during broadcast of trailer booking.

#### Image icon position

The NorDig PVR supporting Trailer booking shall use the signalled position information to position the icon if no native UserInterface items are being displayed. If native UserInterface items (like zapper banner, menu) or subtitling are displayed simultaneous as image icon, then the signalled position information should be used as a guide.

#### Subtitling and display of image icon

##### Default Icon

The NorDig PVR supporting Trailer booking shall be able to continue decode and display subtitling (EBU Teletext subtitling or DVB Subtitling, see chapter 7) also when displaying the image icon. (When the IRD is displaying the trailor booking menu, NorDig PVR should continue displaying subtitling).

### Back-to-back recording

The NorDig PVR shall be able to record back-to-back events both on same and on different services.

Note: 	Back-to-back events refer to two events that immediately follow each other, i.e. the following event start_time is immediately after the previous events stop time, (start_time plus duration), see figure below.

For overlapping events see 14.3.16.2. If the first event has same or higher priority as the second event (see 14.3.16.2) and the first event will overrun, if the NorDig PVR has limitation in recording capacity, it shall first finalise recording of the first event (including its overrun part) before starting recording the next event. The overrun shall be treated as the more important part of the events, see figure below.

If the NorDig PVR has been set to add additional recording time (“safe margins”) before and after recorded events’ start and stop time, any overlapping “safe margins” between the back-to-back events shall be removed (if the NorDig PVR has limitation in recording capacity for this), see figure below.

Figure 14.4 (Illustration) Handling back-to back recordings

### Timeshift recording

The NorDig PVR shall be able to pause or timeshift live TV for at least 60 minutes. It should be possible to save time-shifted events into the PVR list of recordings.

### Late Recording

The NorDig PVR should support a continuous time-shift buffer for late catch-up recording. If supported, then the NorDig PVR (based on Hard Disk Drive) shall be able to disable the continuous time-shift buffer in settings. It should be possible to record a complete event after the event has started including any portion already in the time-shift buffer.

A late recording is when the user programs the PVR to record an event after it has started (according to the events start_time and that it is within EIT present table). If the time-shift buffer includes the start of the event, the recording is treated to be a complete late recording. If the time-shift buffer does not include the start of the event or if the PVR does not have any time-shift buffer, the recording is treated to be an incomplete late recording.

For incomplete late recording the NorDig PVR shall check if there is an alternative instance available within the EIT data (i.e. an event with same CRID value). If an alternative instance of the event is found within the EIT data, the NorDig PVR should offer for the user to automatically replace the incomplete recording later with a complete recording using the alternative instance (the NorDig PVR should however anyway first finalise the rest of the present incomplete recording). If there is conflict because of the alternative instance, the NorDig PVR shall not use alternative instance, it shall then keep the late incomplete recording.

If there is no alternative instance information available at the time of recording, the NorDig PVR shall monitor EIT tables for alternative instance until incomplete recording is removed from the NorDig PVR.

The Late recording should not include recordings from previous event, even if the continuous time-shift buffer did include that at the time of programming.

The manufacture should inform the user in the NorDig PVR’s manual about limitation in late recording, for example it could typically be that it is only possible to record/store as long to back in time for one service as the service has been selected for viewing or as long time as the timeshift buffer is.

### Manual recording

The NorDig PVR shall make it possible for the viewer to set a manual recording, without using the EPG/ESG/EIT data, by setting the service, start-time and end-time (or duration).

The time and date for the user when programming shall be the local time, including any offset, at the time of recording according to the IRD’s settings, and not the local time at the time of programming. This means that if there is a change in local time offset (e.g. change in daylight-saving time) between the time of programming and the time of recording, the time and date shall refer to the new local time at the time of recording.

It should be possible to set weakly repeated manual recording (like every Monday 19:00:00 to 20:00:00 or every weekday between 12:00:00 to 12:15:00)

### One touch recording (OTR)

The NorDig PVR shall include a direct recording setting as a One-touch recording (OTR) function which allows the user to start a recording, while watching live TV, with one button press on the remote control.

This One-touch recording shall not be delayed by further requests for user interaction unless to proceed would affect a recording that is either already underway or scheduled to start before the end of the OTR recording.

The duration of the One-touch recording operation shall be based on either a pre-set time or current viewed event.

In addition, the NorDig PVR may have another setting alternative that when pressing the OTR button the NorDig PVR asks for user confirmation whether the direct recording shall be based on current viewed event (from EIT data) or on a pre-set time.

### Automatic conflict handling

A conflict arises when the NorDig PVR is restricted to perform a recording due to limitation in recording capacity.

#### Conflict during the time of programming a (individual, series or manual) recording

If a conflict is detected it shall be indicated immediately to the user, together with details of the cause, so that the user can take appropriate action.

When programming a recording, which comes in conflict with an earlier programmed recording and when the NorDig PVR can detect an alternative instance in one or both of them, the NorDig PVR shall either automatically re-program one of the to the alternative instance or propose that viewer solve the conflict by moving one of the recordings to the alternative instance and asking for confirmation.

#### Conflict occurring after the time of programming recording(s)

If the NorDig PVR has a number of active programmed series recordings and if there occurs a request of more simultaneous recordings than the NorDig PVR is capable of handling, the NorDig PVR shall be able to handle this without user confirmation at the time of actual recording, i.e. the IRD may inform of the conflict via the OSD but shall automatically solve the conflict at the time of actual recording if the user does not manually change the conflict handling. Any information on OSD about conflict shall not be included in recording and shall have a time-out if no user reaction. All requests for user confirmation shall be done during the time of programming or during the setting of user preferences.

The conflict(s) shall be solved with higher priority recordings having preference before recordings with a lower priority. It is up to the NorDig PVR manufacture to define the PVR’s priority list, however it may typically be as prioritised in Table 14.1. Conflict(s) of recording with same priority level shall also be automatically solved (at least one of them shall be recorded), but it is up to the NorDig PVR manufacture to define a mechanism. The NorDig PVR should offer for the user the ability to change the conflict priority in the user preferences.

Factory default Priority list of handling recording conflict in NorDig PVR

| Priority | Recording type |
| --- | --- |
| 1 (high) | Manual single recordings |
| 2 | Manual repeated recordings |
| 3 | Individual event recording (single shot) without alternative instance |
| 4 | Individual event recording (single shot) with alternative instance |
| 5 | Series recordings without alternative instance |
| 6 | Series recordings with alternative instance |
| 7 | Broadcast record list recordings without alternative instance |
| 8 | Broadcast record list recordings with alternative instance |
| 9 (low) | Automatic Keyword recordings |

Table 14.1(Illustration) Priority list for PVR

If a conflict occurs in a partially or completely overlapping recording after the time of programming the NorDig PVR shall prioritises the recording with the highest priority, as illustrated in figures below. For conflict with events with same priority, the NorDig PVR shall first finalise the first recording (including any late over-run) before starting with next. (See also back-to-back recording).

Any recording that may only be partially recorded (incomplete) due to overlapping of other recordings (including late over-run) should be recorded anyway (see Failed and incomplete recordings).

Figure 14.5 Examples of conflict handling when PVR identifies events that have different priority

### Maximum length of recordings

If the is a failure within the transmission of the EIT and other transmission errors, the NorDig PVR shall stop recording 4 hours after scheduled duration of the event has passed (even if the event still appears in EIT present table).

For events that have a duration that is longer than 8 hours, the NorDig PVR may stop recording after 8 hours.

### Recording of recently removed recordings

Recently removed programme CRID recordings are here defined as recordings that included programme CRID data and that have been deleted and there has not passed 91 days from its recorded start_time or 31 days from its deletion time (whatever occurs last of these two).

For already booked series recordings, the NorDig PVR should not re-acquire recently removed programme CRID recordings (identified via match of programme CRID). After the time has passed for the recently removed programme CRID recording, the NorDig PVR shall re-acquire the event.

When the viewer/user during programming a recording, ie making a booking, (manual or series) and viewer/user selects an event or series within the received EIT data which contain an event that is a recently deleted programme CRID recording (identified via match of programme CRID), the NorDig PVR should prompt the viewer that the event has been recorded and recently deleted by the user and ask for confirmation to continue booking it for recording (ie if viewer confirms re-acquire the content, the NorDig PVR shall be able to record the event).

### Recording of parallel broadcast and simulcast

Some services and events are parallel broadcasted in multiple versions. In this context, a parallel broadcast of same programme event is identified that all service’s events have the same programme_CRID. Examples of parallel broadcasts are services with same national content but with different regional content and services with same content but with different resolution ie e.g. SD and HD simulcast broadcast services.  The NorDig IRD that has been programmed to record an event with a specific programme_crid or series_crid which a programme_crid is part of, shall only record one instance of the event from one of the services when the event is parallel broadcasted over several services at the same time.

The NorDig PVR should, by factory default, prioritise recording of the event from HD service over the SD service the service with higher service priority according to chapter 12.1.4 (service type) during SD and HD simulcast broadcast with different resolutions and/or different formats of the same programme event. (See SI chapter 12 for more details of the service types for SD and HD services).

The NorDig PVR may allow an end user to change the user preference setting regarding or during the time of programming offer the end user to choose which service type has a recording priority during SD and HD simulcast broadcast with different resolutions and/or different formats.

### NorDig Record Lists recording functionality

This NorDig Broadcast Record List is optional for NorDig IRDs/PVRs to support, but if supported then all requirements for this NorDig Broadcast Record List feature shall be supported.

#### Management of Content Grouping

A NorDig PVR supporting BRL shall receive and manage all TV-Anytime groups that are part of the Record List metadata service.

#### Content Acquisition

A NorDig PVR supporting BRL shall automatically acquire, without user intervention, all events that correspond to PushDownloadProgram fragments which are, members of Record List groups to which the user has subscribed.

Events requiring acquisition from the schedule shall be acquired according to the current recording acquisition rules.

A PushDownloadProgram CRID that the program CRID is not yet part of the current EIT, shall only be valid for recording up to 91 days from the last Broadcast Record List it has been published (after that the NorDig PVR shall remove that event from its recording schedule).

A PushDownloadProgram ProgramURL to an event_id of a specific service that is not yet part of the current EIT, shall only be valid for recording up to 10 days from the last Broadcast Record List it has been published (after that the NorDig PVR shall remove that event from its recording schedule).

#### Recording List Hierarchies

When booking a record list, the NorDig PVR supporting BRL shall book all children of that record list see Error! Reference source not found.. There is no requirement for the NorDig PVR to detect loops in any group hierarchy.

#### Management of Content Versioning

If the version of a content that has already been acquired is seen to change, then NorDig PVR supporting BRL should re-acquire the event, replacing the old version in storage.

#### Metadata Changes Post Broadcast

A NorDig PVR supporting BRL may monitor the metadata for changes up to 7 days from the original broadcast of the content, or up to the event's availability to the viewer (if signalled). If a change is detected to an already acquired event, then the most recently transmitted version should be applied.

#### Unresolvable PushDownloadPrograms

If a CRID signalled in a PushDownloadProgram cannot be resolved to an EIT event, the NorDig PVR supporting BRL shall continue attempts to resolve the signalled CRID according to Section 8.7.2 rules.

#### Use of Program Information

In the case where information is available both from EIT and programme information table, the programme information table shall be used.

#### Recording Conflicts

Any user initiated activity in the NorDig PVR supporting BRL shall take precedence over recording of Broadcast Record List events.

It is strongly recommended that, in the case where a broadcast record list event could not be acquired due to a lack of resources, alternate instances of the event be located and rebooked.

The behaviour defined in this subsection may be modified through previously set user preferences.

#### Broadcast Record List Removal

A broadcast record list that has been subscribed to shall be automatically unsubscribed from when it has not been received in the broadcast metadata for 91 days.

#### Broadcast Record List Updates

A NorDig PVR supporting BRL shall within 10 minutes act upon updates to the received metadata.

## Playback

### General

The NorDig PVR shall be able to playback recordings of all supported service types (TV, radio etc) and all belonging components/PIDs (as described in 14.4.5).

Only the service related interactive applications from the current viewed service (live or playback) are required to be active, this means that during playback all interactive applications from the live service in the background may be terminated.

### Replay/Playback – trick modes

The NorDig PVR shall support the following trick modes during playback of recorded events (incl time-shift) for all supported video formats/codecs:

- Play (playback at normal speed)

- Pause

- Stop (stop may be combined with pause, but must enable an easy way to stop playback and return to list of recordings and live viewing mode)

- Fast forward and shall support fast forward at minimum 3 different speeds, (like x3, x6, x15 and x30).

- Fast reverse and should support fast reverse at minimum 3 different speeds, (like x3, x6, x15 and x30).

The audio may be muted during trick modes (except during normal playback). The subtitling (EBU Teletext subtitling or DVB Subtitling) and other event based data application may be skipped during trick modes (except during normal playback). The NorDig PVR should make use of the AU_information according to ETSI TS 101 154 [28], if that information is available in the recorded video stream. The slower fast forward and fast reverse modes should be smoother rather than trying to keep the exact selected trick mode speed.

The NorDig PVR should support the following trick modes during playback of recorded events (incl time-shift) for all supported video formats/codecs:

- Slow forward (like x1/2 and x1/4 of the normal speed)

- Slow reverse (like x1/2 and x1/4 of the normal speed)

- Next, go to next recorded event within same series (i.e. to a event within same series with a newer date or “episode_id”, if the current event belongs to a series and the NorDig PVR has recorded several events) otherwise go to next recording within the list of recordings.

- Previous, go to a previous recorded event within same series (i.e. to a event within same series with an older date or lower episode_id, if the current event belongs to a series and the NorDig PVR has recorded several events) otherwise go to previous recording within the list of recordings.

- Jump, go to a specific time in the recording and/or fast jump a manufacture defined fixed time (e.g. 4 min forward).

The NorDig PVR should be able to insert indexes into the recordings to enable fast access to different parts of the recording.

### Relative Synchronisation

The NorDig PVRs shall not introduce more relative delay (reduced “lipsync”) during playback between the audio, video and other PES packetised components (like subtitling) compared to decoding of live content, measured 5s or later after start of normal playback (see chapter 6.1.2.1 and 7).

After using trick mode, the relative delay shall meet the requirements within 5s after resuming back to normal playback speed.

### Simultaneous recording and playback

The NorDig PVR shall be able to record and playback simultaneously. It shall be possible to record one service from the live transmissions while playback another earlier recording.

The user shall be also able to start the playback of a recording for which the recording has not yet completed (“chase playback”).

### Full service playback

During playback of recorded content the user shall be able to perform the same full service selection as would have been possible during basic live viewing, such as select audio and/or subtitling language (if several components with same type are available), switch subtitling on or off, select audio format etc (with the limitation outlined in section 14.3.9). The basic live viewing refers to all streams excluding any HbbTV related streams. Dynamic changes in the services (such as a change of video aspect ratio or change of audio format) that occur during the recording shall be processed in the same way as during live viewing.

During playback, the NorDig PVR shall be able to set the same control as during live viewing, for example blanking of video and muting of sound depending on the event’s parental rating values (see 14.3.2) and signal protection (HDCP) on its digital output interface (see 9.9.4). For cases where the information is coming from EIT data (like parental rating descriptor), the playback shall at least act on the EIT signalling at the start of the recording (see 15.2.1). For the cases where the information is coming from PMT or the elementary streams (like signal protection and aspect ratio), the playback shall perform the same as live viewing and following any changes therein (i.e. PMT and elementary stream header information shall be stored and processed during playback).

### Resume Playback

When resuming back to a partially viewed recording, the NorDig PVR should resume back to the point of the recording (or a moment before that point) where the previous playback was stopped.

It should be possible to set index points of interest during playback, where later playback may be jumped to.

### NorDig Record Lists management and playback functionality

The requirement for playback of NorDig Record Lists recordings are the same as for playback for other types of NorDig recordings (manual recordings, series recordings), see above. (In addition, there are also requirements for embargo and expiry of recordings, see 14.2.10.8 and 14.2.10.9).

# IRD System Software and API

## NorDig Basic IRD

The NorDig Basic TV IRD shall have a system software for interpretation and handling of the active service information and control of the local hardware/software.

## NorDig HbbTV IRDHybrid

The NorDig HbbTVHybrid IRD shall support all mandatory features and requirements of HbbTV v.2.0.1 (1) as specified in ETSI TS 102 796 v1.4.1 (1) specification [29] including updates by published errata (2) from the HbbTV Association.

The NorDig HbbTVHybrid IRD shall have a broadband interface in accordance with NorDig Section 8.3 (two-way interface).

The NorDig HbbTVHybrid IRD shall have HbbTV feature as enabled by default (see Section 16.3). It shall have a menu option to allow user to enable / disable the HbbTV feature as a whole and it should have a menu option to allow user to enable / disable the HbbTV feature service by service.

The implementation of the HbbTV in the NorDig HbbTVHybrid IRD shall be verified by the manufacturer, the verification testing is based on self-testing by manufacturer (unless otherwise specified by the relevant network/Operator). Verification testing of the HbbTV parts of NorDig HbbTV IRDHybrid shall be based on HbbTV test specification [81] and HbbTV test suite [25] (3) plus the extra HbbTV test cases defined by NorDig, for more information see NorDig Test Plan.

NorDig mandates some of the requirements that are optional in HbbTV [29], this also means that relevant test cases in the HbbTV test suite [25] also have to be performed successfully (for more information see NorDig Test Plan).

NorDig broadcast member may request to get a copy of the manufacture’s HbbTV Verification Test report with test results, before approving HbbTVHybrid IRD on the market.

Note 1: Optionally NorDig HbbTVHybrid IRDs released before July 1 2018 may instead support all mandatory features and requirements of HbbTV 1.5 as specified in ETSI TS 102 796 V1.2.1 [82] including updates by published errata from the HbbTV Association. At the time of writing (January 2017), the most recent published errata to HbbTV 1.5 is errata #3 but an errata #4 may be published before July 1 2018).

Note 2: At the time of writing (January 2017) there are no published errata to HbbTV 2.0.1 however a first errata document is expected to be published during 2017 and others may be published before July 1 2018.

Note 3: NorDig HbbTVHybrid IRDs implementing HbbTV 1.5 [82] as permitted by the grace period defined above in note 1 also need to pass the same HbbTV test suite [25] as those implementing HbbTV 2.0.1. The test suite lists which test cases are applicable to IRDs implementing different specification versions.

Note: NorDig members will typically use Nordic and Irish languages in text strings in HbbTV and some cases use multiple languages within one and the same HbbTV application, observe that the NorDig HbbTV IRDHybrid shall as stated in HbbTV specification [29] support all characters in these languages and multiple languages.

# User Preferences

## General

The user shall be able to store preference settings in persistent memory. All user preference settings listed below shall remain when changing service and when re-starting the IRD. The following user preferences shall be implemented in the NorDig IRD, unless it is stated below as optional (should) requirement. (See section 16.4 for factory default values).

## User Preference Settings

### General User Preference Settings

The user shall be able to select storable user preferences (stored as persistent settings) for following General related functions (see section 16.4 for factory default values):

- Service list as defined in section 13.2.

- Country setting based on country code [49] for pre-selection of the primary menu, primary audio language, primary subtitle language settings and channel list selection as defined in 12.2.9.3.

- Talking menus/Text-to-Speech preferences as defined in section 13.6.2,  if this optional feature is supported by the IRD.

- SSU setting(s) to select whether system software upgrades are allowed or not if the IRD supports Fully Automatic mode. See chapter 10 more details and 16.4 for factory default for this.

- HbbTV setting(s) for NorDig HbbTV IRD Hybrid as defined in section 15.2.

The following user preferences should be provided for NorDig IRDs:

- HDCP preferences as specified in sections 8.6.4.

### Video User Preference Settings

The user shall be able to select storable user preferences (stored as persistent settings) for following video related functions (see chapter 16.4 for factory default values):

- HDMI Video preferences for Output video format, as set by the user:

- Automatic mode, based on use of E-EDID, as specified in section 8.6.2

- Fixed format, as specified in section  8.6.2

### Audio User Preference Settings

The user shall be able to select storable user preferences (stored as persistent settings) for following audio related functions (see chapter 16.4 for factory default values):

- Primary and secondary audio language (see section 6.5.1).

- Audio Format between stereo or multi-channel for the digital outputs, when the outputs are equipped for multichannel audio (see section 6.3, 6.4, 6.5.2, 6,6 and 8.6.3).

- Audio Type between “Normal” and Supplementary Audio (see section 6.11 and 16.1.3.1)

- Audio delay value for the digital audio output (mandatory for NorDig STB with HDMI and/or S/PDIF output, optional/recommended for iDTV, see section 6.7 and 6.7.1).

- Audio output method for HDMI output, HDMI ARC and S/PDIF (if several alternatives is supported by the IRD), e.g. pass-through of native bitstream, decoded to PCM, transcoded to DTS and AC-3, (see section 6.2.2.3, 6.2.3.3, 8.5.3 and 8.6.3).

- Mixing level between the receiver mixed supplementary audio and “Normal” audio, (see section 6.10.3.5).

The following user preferences should be provided for NorDig IRDs:

- Clean Audio selection as specified in section 6.10.

#### Additional User Preference Settings for Supplementary Audio

The NorDig IRD should have user selection of Supplementary Audio preferences for between the different sub-variants as; ‘Audio Description’, ‘Spoken Subtitles’ and ‘all types of Supplementary Audio’.

The user preference settings for enabled/disable default Supplementary audio shall be common for Broadcast mixed (1) and Receiver mixed alternatives.

The IRD should have additional setting for Receiver mixed alternative, which presents the Supplementary Audio on its own on the head-phones output interface, while the “normal” audio is presented on its own on the main audio output, see section 6.10.7.

The IRD may have additional setting for Receiver mixed alternative.

Note 1: This requirement for Broadcast mixed refers to when the audio stream is signalised as supplementary audio via visual impaired and/or via other supplementary signalling in AC-3/E-AC-3/AAC descriptor and/or supplementary audio descriptor. This means that for the case when the audio stream is signalised with ‘nar’ language code in ISO639 descriptor and no other supplementary signalling, IRD may have different settings for, for example via primary audio language preference setting to ‘narrative’.

### Subtitling User Preference Settings

The user shall be able to select storable user preferences (stored as persistent settings) for following subtitling related functions:

- Primary and secondary subtitling language as defined in chapter 7.

## Deletion of service lists

The IRD shall provide a function to remove all service lists (default and user defined); this function should not affect other parameters (e.g. user preferences).

Note. 	Removal of service list can be implemented as part of factory reset, see below.

## Reset to factory mode

The IRD shall provide a function to reset all parameters to factory mode, thus removing all service lists, user preferences, etc. After reset, the IRD shall enter installation state.

The factory mode should be set to the following:

| RF input DC power supply source for satellite front-end: | On |
| --- | --- |
| RF input DC power supply source for terrestrial front-end: | off (1) |
| RF-output preset channel: | Channel 43 (PAL-G) (2) |
| RF bypass gain | Disabled |
| Menu language: | equal to country settings |
| Audio (normal) | On (3) |
| Primary audio language: | equal to country settings |
| Audio; Supplementary Audio / Audio Description (visual impaired): | Off (4) |
| Audio format setting: | Stereo |
| Subtitling (normal): | On |
| Primary subtitling language: | as country settings |
| Subtitling; hard of hearing/hearing impaired: | Off |
| HbbTV Interactivity | On |
| HDMI Audio output | Automatic, using E-EDID information |
| HDMI Video output | Automatic using E-EDID information |
| HDCP | ON or as specified by the relevant network/ CA operator see section 8.6.4 (5) |
| SSU | If this menu item is supported, a value that disables Fully Automatic mode (6) |
| PVR recording priority, SD vs HD | HD |
| Talking menus (Text-to-Speech), optional | Off |
| Note 1: In the first time installation and resetting to factory default settings, the DC power supply shall be switched off. It is recommended that the receiver ask if the DC power supply is turned on in the first time installation and in the installation after resetting to factory settings, to speed up the initialisation procedure. Note 2: Applicable for IRDs with RF PAL modulator Note 3: Audio type 0x00 'Undefined' Note 4: Refers to all kinds of supplementary audio (Audio Description, Spoken Subtitling) and both distribution formats: Broadcast mixed and receiver mixing of visual impaired audio stream. Note 5: The IRD should provide an option to manually set the HDCP default to “ON” or “OFF”, see section 8.6.4. Note 6: Settings for SSU (System Software Update) may be implemented as one combined setting or several settings for different parts. The settings required will depend on the combination of modes supported (see 10.2) but could, for example, take the form of “Enable/Disable” or “Fully Automatic / Semi Automatic”. |  |

Table 16.1 Factory default settings for IRDs

: NorDig Members and Acknowledgement

: Background and options for IRDs with a terrestrial front-end

: Placeholder for changes in 13.4 for IP-based IRDs.

: Implementations Guidelines for best service selection in automatic channel search in terrestrial networks

: Raw carrier to noise values, (C/N)RAW

: Recommendation for menu wordings and translations

Here follows guidelines/recommendation of wordings to be used in menus etc in the IRD for a number of functions with translation to NorDig’s different native languages. This as a help to the IRD manufacture, to easier communicate and explain for the users/viewers the usage of a function, both from broadcasters and IRD manufacture point of view. The list of receommended wordings to functions is yet not complete and mainly includes functions that earlier experience has shown are most in need of advice (empty fields is intended to included in future releases).

In some cases, guidelines include Helping text in different languages that may be used to explain function in more detail, typically when user highlight/mark this function before selecting/changing this function’s setting(s).

Subtitling

| Normal Subtitles (subtitles) |  |  |
| --- | --- | --- |
| Language | Function wording | Helping text /description |
| English | ‘Subtitles’ or ‘Normal subtitles’ | Normal subtitles that include translation of the spoken language into one or several languages for the viewer, in the case this subtitling is coming as a separate stream. |
| Swedish |  |  |
| Norwegian | Teksting | Teksting på norsk av utenlandsk tale. |
| Danish | Tekstning | Tekstning på dansk af udenlandsk tale |
| Finnish | Tekstitys | Vieraskielisten ohjelmien tekstitys |
| Sami | Teaksta | Hállan tekstejuvvon sámegillii |
| Irish | Fotheidil nó Fotheideal teilitéacs | Fotheideal teilitéacs dúnta |

Normal subtitles (see section 7 Subtitling) recommended wordings for IRD menus. (Normal subtitlies includes typically textual translation of audio with foreign language for normal viewers. For Hard of hearing subtitling see below).

Accessibility

| Audio description (audio) |  |  |
| --- | --- | --- |
| Language | Wording in language | Helping text /description in language |
| English | ‘Audio description’ or sometimes ‘Video description’ or sometimes ‘Narrative description’ | A voice describes what is happening in the picture. It is intended for those who have visual impirments. |
| Swedish | ‘Ljudbeskrivning’ or ‘Syntolkning’ | En röst beskriver händelser i bilden för de som har svårigheter att se. |
| Norwegian | ‘Synstolkning’ | Talebeskrivelse av visuelle sanseinntrykk for de som har problem med å se. |
| Danish | ‘Synstolkning’ | Synstolkning er et særligt lydspor, som sendes samtidig med billedsiden til et tv-program som beskriver hvad der sker i billede |
| Finnish | ‘Kuvailutulkkaus’ | Puhuttu kuvailuääni kertoo mitä ohjelmassa tapahtuu. |
| Sami | Gova tulkon | Njálmmálaččat muitaluvvo čalmmehemiide dat, mii govas oidno. |
| Irish | Reacaireachta nó Físeán reacaireachta |  |

Accessibility - Audio - Audio description (see section 6.11 Supplementary audio) recommended wordings for IRD menus.

| Spoken subtitles (audio) |  |  |
| --- | --- | --- |
| Language | Function wording | Helping text /description |
| English | ‘Spoken subtitles’ | A voice reads the subtitles for you. It is intended for those who have visual impairments. |
| Swedish | ‘Talande textremsa’ | En röst läser upp textremsan för de som har svårigheter att läsa textremsan. |
| Norwegian | ‘Lydteksting’ | En stemme som leser tekstingen for de som har synsproblemer. |
| Danish | ‘Oplæste undertekster’ | Oplæste undertekster, oplæster automatisk underteksterne på fremmed-sprogede programmer |
| Finnish | ‘Puhetekstitys’ | Käännöstekstitys kuuluu puheena yhdessä ohjelmaäänen kanssa. |
| Sami | Teaksta lohkkojuvvo | Teksten lohkkojuvvo čalmmehemiide. |
| Irish | Fotheideal labhartha |  |

Accessibility - Audio – Spoken subtitles (see section 6.11 Supplementary audio) recommended wordings for IRD menus. This is in the case the IRD has settings to differentiate normal audio description from spoken subtitling. If the IRD has one and the same setting for both normal audio description and spoken subtitling, then recommend using the wordings for Audio description above.

| Supplementary Audio Mixing Level (relative volume control of audio receiver mixing) |  |  |
| --- | --- | --- |
| Language | Function wording | Helping text /description |
| English | ‘Supplementary Audio Mixing Level’ or ‘Audio mixing’ | Select the balance between supplementary audio and normal audio. |
| Swedish | ‘Ljudmixning’ | Bestäm balansen mellan kompletterande ljud och vanligt ljud |
| Norwegian | ’Lydnivå på lydteksting og synstolking’ | Bestemme ballanse mellom lydteksting eller synstolking og originallyd |
| Danish |  |  |
| Finnish | ‘Lisä-äänen voimakkuus’ | Pää- ja lisä-äänen välisen voimakkuuden säätö. |
| Sami | Jietnatekstema ja govvadulkoma jietnadássi | Jietnatekstema dahje govvadulkoma ja originálajienaid dássádallan |
| Irish | Meascadh fuaime nó Meascadh leibhéal |  |

Accessibility - Audio - relative volume control of audio receiver mixing (see section 6.11.3.5 in Supplementary audio) recommended wordings for IRD menus.

| Sign language (video) |  |  |
| --- | --- | --- |
| Language | Function wording | Helping text /description |
| English | ‘Sign language’ | For those who have hearing impairments |
| Swedish | ‘Teckenspråkstolkning’ | För de som har svårigheter att höra. |
| Norwegian | ‘Tegnspråk’ | Tegnspråktolking for de som har hørselsproblemer |
| Danish | ‘Tegnsprogstolkning’ | Tegnsprog er et tilbud til døve, der ellers ikke kan følge med i udsendelser |
| Finnish | ‘Viittomakieli’ |  |
| Sami | Seavagiella | Seavagillii dulkojuvvo bealjehemiide. |
| Irish | Teanga chomharthaíochta |  |

Accessibility, Video, Sign Language recommended wordings for IRD menus.

| Subtitles for Hard Of Hearing (subtitles) |  |  |
| --- | --- | --- |
| Language | Function wording | Helping text /description |
| English | ‘Subtitles for Hard Of Hearing’ |  |
| Swedish | ’Textremsa för hörselnedsatta’ or ’Undertext för hörselnedsatta’ |  |
| Norwegian | ‘Teksting for hørselshemmede’ | Teksting av norsk tale |
| Danish | ‘Tekstet’ | Undertekster af dansk tale |
| Finnish | ‘Tekstitys huonokuuloisille’ |  |
| Sami | Teksten bealjehemiide | Njálmmálaš dárogiella tekstejuvvo |
| Irish | Fotheidil |  |

Subtitling – Hard of hearing subtitles (see section 7 Subtitling) recommended wordings for IRD menus.

SI – recommendation wordings

| Country name in native |  |  |  |
| --- | --- | --- | --- |
| Country (in English) | ISO 3166 country code | Languge | Wording in language (translation to be used) |
| SWEDEN | SWE | Swedish | ‘Sverige’ |
| DENMARK | DNK | Danish | ’Danmark’ |
| IRELAND | IRL | Irish | ’Éire’ |
| FINLAND | FIN | Finnish | ’Suomi’ |
| NORWAY | NOR | Norwegian | ‘Norge’ |

SI - country name (see section 12.1.8) recommended wordings for IRD menus.

| Language name in native |  |  |  |
| --- | --- | --- | --- |
| Language (in English) | ISO 639-2 language code | Languge | Wording in language (translation to be used) |
| Danish | dan | Danish | ‘Dansk’ |
| English | eng | English | ‘English’ |
| Finnish | fin | Finnish | ‘Suomi’ |
| Irish / Gaelic | iri | Irish / Gaelic | ‘Gaeilge’ |
| Irish / Gaelic | gle | Irish / Gaelic | ‘Gaeilge’ |
| Norwegian | nor | Norwegian | ‘Norsk’ |
| Narrative | nar | English | ‘Narrative’ |
| Original language | qaa | Danish | ’Original’ |
| Original language | qaa | English | ’Original’ |
| Original language | qaa | Finnish | ’Alkuperäinen’ |
| Original language | qaa | Irish / Gaelic | ’Bunaidh’ |
| Original language | qaa | Norwegian | ’Original’ |
| Original language | qaa | Swedish | ’Original’ |
| Sami | smi | Sami | ’Sámegiella’ |
| Swedish | swe | Svenska | ’Svenska’ |
| Undefined | und | Danish | ’Udefineret’ |
| Undefined | und | English | ’Undefined’ |
| Undefined | und | Finnish | ’Määrittelemätön’ |
| Undefined | und | Irish / Gaelic | ’Neamhshainithe’ |
| Undefined | und | Norwegian | ’Undefined’ |
| Undefined | und | Swedish | ’Odefinierat’ |

SI - Language name (see section 12.1.8) recommended wordings for IRD menus.

| SI EIT/Navigator, time slots with no event information |  |  |
| --- | --- | --- |
| Language | Function wording | Helping text /description |
| English | ‘No event information’ |  |
| Swedish | ‘Ingen information’ |  |
| Norwegian | ‘Ingen programinformasjon’ |  |
| Danish | ‘Ingen programinformation’ |  |
| Finnish | ‘Ei ohjelmatietoja’ |  |
| Irish |  |  |

SI/Navigator – SI – EIT – in ESG/EPG for time slots with no event information available (see section 13.3.2.3).

:

: Guidelines for NorDig IRD audio selection

: Loudness levels, – Typical IDTV Audio Block diagram

: Examples of Signalling to be used for audio property

: Comparison of NorDig profiles