NorDig Unified Test Plan

for

Integrated Receiver Decoders

for use in

cable, satellite, terrestrial and managed IPTV based networks

Draft of Audio Section for v3.2.2

Date: August 2025

Following text is only during drafting and will be removed before final NorDig RoO specification

DRAFTING GUIDELINES / Explanation from the editors related to DRAFT versions:

This NorDig Test Plan for v3.2.2 draft document is based on the official NorDig Test Plan v3.2.1.

Yellow highlight marking marks changes in text compared to NorDig Test Plan v3.2.1.

  - New modified text: without strikethrough marks new additional text,

  - Removed text: with strikethrough marks old text proposed to be removed

- Green marked text: highlighting text that under extra scrutiny during this update (not yet agreed).

- Blue marked text: comments or other raw text that will be removed before final version.

- Grey marked text: refers to text that not are relevant to this review/update.

Guide: To improve version handling and readability, old text from NorDig Test Plan v3.2.1 that is proposed to be deleted in future v3.2.2 should not be removed from draft version.

## Task 7: Audio

Audio

| Test Case | Audio User Preference Settings |  |  |
| --- | --- | --- | --- |
| Section | NorDig Unified 6.1.1 |  |  |
| Requirement | The NorDig IRD shall have User Preference Settings for audio functions as stated in section 16.2 (for example primary and secondary audio language, audio format, audio type, audio delay, etc). |  |  |
| IRD variants and capability | all IRDs |  |  |
| Test procedure | This is a general requirement that will be used in the following tests. |  |  |
| Test result(s) |  |  |  |
| Conformity | OK Fault Major Minor, define fail reason in comments |  |  |
| Comments | If possible describe if fault can be fixed with software update: YES NO Describe more specific faults and/or other information |  |  |
| Date |  | Sign |  |

| Test Case | MPEG-1 Layer II: Requirements |  |  |
| --- | --- | --- | --- |
| Section | NorDig Unified 6.2.1.1, 6.8.1 |  |  |
| Requirement | The NorDig IRD shall support: - decode MPEG-1 Layer II streams at all bit rates and sample rates listed in ETSI TS 101 154. The NorDig IRD should support: - decode MPEG-1 Layer II streams at half-sampling rates (22.05 and 24 kHz). |  |  |
| IRD variants and capability | all IRDs |  |  |
| Test procedure | This is general requirement that will be tested in following tests. |  |  |
| Test result(s) |  |  |  |
| Conformity | OK Fault Major Minor, define fail reason in comments |  |  |
| Comments | If possible describe if fault can be fixed with software update: YES NO Describe more specific faults and/or other information |  |  |
| Date |  | Sign |  |

| Test Case | MPEG-1 Layer II: Analogue audio output |  |  |
| --- | --- | --- | --- |
| Section | NorDig Unified 6.2.1.1, 6.8.1 |  |  |
| Requirement | The NorDig IRD with analogue outputs shall always have an audio signal present on the analogue outputs (to built-in loudspeakers (see note 1)) if any of the supported formats is received. The NorDig IRD shall support: - decode MPEG-1 Layer II streams at all bit rates and sample rates listed in ETSI TS 101 154. |  |  |
| IRD variants and capability | all IRDs |  |  |
| Test procedure | Purpose of test: To verify that receiver decodes MPEG-1 Layer II streams at all bit rates and sample rates and audio level can be adjusted. Equipment: Source MUX Exciter DVB receiver Monitor A transport stream with following MPEG-1 Layer II audio content: - audio_mode = ‘0’ (stereo), data rate = 192 kbit/s, at least one of the sampling frequencies: 32, 44.1 or 48 kHz - audio_mode = ‘1’, ‘2’ (dual channel), data rate = 192 kbit/s, at least one of the sampling frequencies: 32, 44.1 or 48 kHz - audio_mode = ‘3’ (single channel), data rate: 96 kbit/s, sampling frequency: 48 kHz Test procedure: The IRD is tuned to a service containg all the combinations of the MPEG-1 Layer II audio parameters above. The audio level is adjusted in the audio output. Expected result: IRD supports MPEG-1 Layer II audio decoding and audio is available properly in analoque audio output(s). The audio output level can be adjusted. |  |  |
| Test result(s) |  |  |  |
| Conformity | OK Fault Major Minor, define fail reason in comments |  |  |
| Comments | If possible describe if fault can be fixed with software update: YES NO Describe more specific faults and/or other information |  |  |
| Date |  | Sign |  |

| Test Case | MPEG-1 Layer II: HDMI output interface |  |  |
| --- | --- | --- | --- |
| Section | NorDig Unified 6.2.1.2, 6.2.1.1 |  |  |
| Requirement | The NorDig IRD shall be capable of providing the following formats on the HDMI output connector from an MPEG-1 Layer II bitstream (see section 16 for factory default settings): - Decoded to PCM stereo bitstream The NorDig IRD shall support: - decode MPEG-1 Layer II streams at all bit rates and sample rates listed in ETSI TS 101 154. |  |  |
| IRD variants and capability | all IRDs |  |  |
| Test procedure | Purpose of test: To verify that receiver decodes MPEG-1 Layer II streams at all bit rates and sample rates to HDMI output interface and audio level can be adjusted. This test is mandatory for STBs and IDTVs supporting HDMI output or HDMI ARC. Equipment: Source MUX Exciter DVB receiver Monitor A transport stream with following MPEG-1 Layer II audio content: - audio_mode = ‘0’ (stereo), data rate = 192 kbit/s, at least one of the sampling frequencies: 32, 44.1 or 48 kHz - audio_mode = ‘1’, ‘2’ (dual channel), data rate = 192 kbit/s, at least one of the sampling frequencies: 32, 44.1 or 48 kHz - audio_mode = ‘3’ (single channel), data rate: 96 kbit/s, sampling frequency: 48 kHz Test procedure: The IRD is tuned to a service containg all the combinations of the MPEG-1 Layer II audio parameters above. This task can be done parallel with Task 7:2. The audio level is adjusted in the audio output. Expected result: IRD supports MPEG-1 Layer II audio decoding and audio is available properly in HDMI output in PCM stereo bit stream format independently of the user setting for stereo or multichannel. The audio output level can be adjusted. |  |  |
| Test result(s) |  |  |  |
| Conformity | OK Fault Major Minor, define fail reason in comments |  |  |
| Comments | If possible describe if fault can be fixed with software update: YES NO Describe more specific faults and/or other information |  |  |
| Date |  | Sign |  |

| Test Case | MPEG-1 Layer II: S/PDIF output interface |  |  |
| --- | --- | --- | --- |
| Section | NorDig Unified 6.2.1.2, 6.2.1.1 |  |  |
| Requirement | The NorDig IRD including an S/PDIF output shall be capable of providing the following formats on the S/PDIF connector from an MPEG-1 Layer II bitstream: • Decoded to PCM stereo bitstream The NorDig IRD shall support: • decode MPEG-1 Layer II streams at all bit rates and sample rates listed in ETSI TS 101 154. |  |  |
| IRD variants and capability | all IRDs |  |  |
| Test procedure | Purpose of test: To verify that receiver decodes MPEG-1 Layer II streams at all bit rates and sample rates to S/PDIF output interface. Equipment: Source MUX Exciter DVB receiver Monitor A transport stream with following MPEG-1 Layer II audio content: - audio_mode = ‘0’ (stereo), data rate = 192 kbit/s, at least one of the sampling frequencies: 32, 44.1 or 48 kHz - audio_mode = ‘1’, ‘2’ (dual channel), data rate = 192 kbit/s, at least one of the sampling frequencies: 32, 44.1 or 48 kHz - audio_mode = ‘3’ (single channel), data rate: 96 kbit/s, sampling frequency: 48 kHz Test procedure: The IRD is tuned to a service containg all the combinations of the MPEG-1 Layer II audio parameters above. This task can be done parallel with Task 7:2. Expected result: IRD supports MPEG-1 Layer II audio decoding and audio is available properly in S/PDIF output in PCM stereo bit stream format independently of the user setting for stereo or multichannel. |  |  |
| Test result(s) |  |  |  |
| Conformity | OK Fault Major Minor, define fail reason in comments |  |  |
| Comments | If possible describe if fault can be fixed with software update: YES NO Describe more specific faults and/or other information |  |  |
| Date |  | Sign |  |

| Test Case | AC-3: Requirements |  |  |
| --- | --- | --- | --- |
| Section | NorDig Unified 6.2.2 |  |  |
| Requirement | NorDig IRD supporting E-AC-3 according to ETSI TS 102 366 and AC-3 shall • decode AC-3 streams at all bit rates and sample rates (not including Annex E). • (additionally) decode E-AC-3 streams with data rates from 32 kbps to 3 024 kbps and support all sample rates Annex E. • be capable of transcoding E-AC-3 bitstreams to AC-3. Transcoding to AC-3 audio streams shall be at a fixed bit rate of 640 kbps. |  |  |
| IRD variants and capability | all IRDs |  |  |
| Test procedure | Purpose of test: To verify that the IRD supports different AC-3 bit rates and sampling rates. Equipment: Source MUX Exciter DVB Receiver Audio decoder The TS shall contain services, which has - a AC-3 (mono,stereo) audio component with relevant signaling at bit rates and sampling rates: - Sampling rates 32, 44,1 and 48 kHz - Bit rates 64, 192, 384 and 640 kbit/s - a AC-3 (multichannel) audio component with relevant signaling at bit rates and sampling rates: - Sampling rates 32, 44,1 and 48 kHz - Bit rate 64, 192, 384 and 640 kbit/s Test procedure: - Connect the IRD to an audio receiver, e.g. Home Theater System with HDMI. - Verify that service has a multichannel AC-3 audio available. - In receiver menu, select MPEGstereo audio. - Verify that AC-3 multichannel is decoded and downmixed to PCM stereo bitstream for HDMI output and if supported for HDMI ARC. - Verify the audio output level can be adjusted. - In receiver menu, select multichannel audio. - Verify that AC-3 multichannel is passed-through as native bitstream for HDMI output. - Verify that service has a stereo AC-3 audio available. - In receiver menu, select MPEGstereo audio. - Verify that AC-3 stereol is decoded and downmixed to PCM stereo bitstream for HDMI output and if supported for HDMI ARC. - Verify the audio output level can be adjusted. - In receiver menu, select multichannel audio. - Verify that AC-3 stereo is passed-through as native bitstream for HDMI output. Expected result: IRD supports for AC-3 input audio signal and down-mix to stereo. |  |  |
| Test result(s) | Bit rate; Sampling rate; OK or NOK. mono. stereo. Multichannel 5.1. Down-mix to stereo; N/A; N/A. |
| Conformity | OK Fault Major Minor, define fail reason in comments |  |  |
| Comments | If possible describe if fault can be fixed with software update: YES NO Describe more specific faults and/or other information |  |  |
| Date |  | Sign |  |

| Test Case | AC-3: Analogue audio output |  |  |
| --- | --- | --- | --- |
| Section | NorDig Unified 6.2.2.1, 6.8.1 |  |  |
| Requirement | The NorDig IRD with analogue outputs shall always have an audio signal present on the analogue outputs (to built-in loudspeakers (see note 1)) if any of the supported formats is received. If the NorDig IRD has analogue stereo output(s), it shall be capable of decoding and downmixing the supported audio formats for the analogue outputs. NorDig IRD supporting E-AC-3 and AC-3 according to ETSI TS 102 366 shall - decode AC-3 streams at all bit rates and sample rates (not including Annex E) |  |  |
| IRD variants and capability | all IRDs |  |  |
| Test procedure | Purpose of test: To verify that receiver decodes AC-3 bitstream and audio output level can be adjusted. This test in only mandatory for STB or IDTV with SCART or analogue audio output interface. Equipment: Source MUX Exciter DVB receiver Monitor A transport stream containing aservice with AC-3 multichannel audio Test procedure: - Verify that service has a multichannel AC-3 audio available. - In receiver menu, select stereo audio audio. - Verify that service containing AC-3 multichannel audio is downmixed to stereo at analog audio output. - In receiver menu, select multichannel audio. - Verify that AC-3 multichannel is downmixed to stereo at analog audio output. - Verify it is possible to adjust audio output level. Expected result: IRD supports AC-3 audio decoding and audio is downmixed to stereo in analog audio output(s). The audio output level can be adjusted. |  |  |
| Test result(s) |  |  |  |
| Conformity | OK Fault Major Minor, define fail reason in comments |  |  |
| Comments | If possible describe if fault can be fixed with software update: YES NO Describe more specific faults and/or other information |  |  |
| Date |  | Sign |  |

| Test Case | AC-3: HDMI output and HDMI ARC interface |  |  |
| --- | --- | --- | --- |
| Section | NorDig Unified 6.2.2.1, 6.2.2.3 |  |  |
| Requirement | Where HDMI, HDMI ARC (or eARC) is implemented NorDig IRDs supporting E-AC-3 and AC-3 shall be capable of providing the following formats from an E-AC-3 or AC-3 bitstream (see chapter 16 for factory default settings): - Pass-through of native bitstream (AC-3 and E-AC-3) - E-AC-3 bitstream transcoded to AC-3 bitstream - Decoded and downmixed (if > 2 channels) to PCM stereo bitstream NorDig IRD supporting E-AC-3 and AC-3 according to ETSI TS 102 366 shall - decode AC-3 streams at all bit rates and sample rates (not including Annex E). |  |  |
| IRD variants and capability | all IRDs |  |  |
| Test procedure | Purpose of test: To verify that receiver decodes and passes-through AC-3 bitstream and the audio output level for decoded AC-3 bitstream can be adjusted. This test in only relevant for IRD with HDMI output or HDMI ARC. Equipment: MPEG-2 source MUX Exciter DVB receiver HDMI receiver Monitor A transport stream containing aservice with AC-3 multichannel audio. Test procedure: - Connect the IRD to an HDMI receiver, e.g. Home Theater System with HDMI. - Verify that service has a multichannel AC-3 audio available. - In receiver menu, select MPEG stereo audio. - Verify that AC-3 multichannel is decoded and downmixed to PCM stereo bitstream for HDMI output and if supported for HDMI ARC. - Verify the audio output level can be adjusted. - In receiver menu, select multichannel audio. - Verify that AC-3 multichannel is passed-through as native bitstream for HDMI output. Expected result: IRD supports AC-3 audio decoding and downmixing to PCM stereo when stereo audio is selected and the audio output level can be adjusted. IRD supports AC-3 audio pass-through native bitstream when multichannel audio is selected. |  |  |
| Test result(s) |  |  |  |
| Conformity | OK Fault Major Minor, define fail reason in comments |  |  |
| Comments | If possible describe if fault can be fixed with software update: YES NO Describe more specific faults and/or other information |  |  |
| Date |  | Sign |  |

| Test Case | AC-3: S/PDIF output interface |  |  |
| --- | --- | --- | --- |
| Section | NorDig Unified 6.2.2.1, 6.2.2.3 |  |  |
| Requirement | The NorDig IRD supporting E-AC-3 and AC-3 and including an S/PDIF output shall be capable of providing the following formats on the S/PDIF connector from an E-AC-3 or AC-3 bitstream: - E-AC-3 bitstream transcoded to AC-3 bitstream - Decoded and downmixed (if > 2 channels) to PCM stereo bitstream - Pass-through of AC-3 bitstream NorDig IRD supporting E-AC-3 and AC-3 according to ETSI TS 102 366 shall - decode AC-3 streams at all bit rates and (not including Annex E) |  |  |
| IRD variants and capability | all IRDs |  |  |
| Test procedure | Purpose of test: To verify that receiver decodes and passes-through AC-3 bitstream. This test is only relevant for IRD with S/PDIF output. Equipment: MPEG-2 source MUX Exciter DVB receiver S/PDIF receiver Monitor A transport stream containing aservice with AC-3 multichannel audio. Test procedure: - Connect the IRD to a S/PDIF receiver e.g. Home Theater System with S/PDIF. - Verify that service has a multichannel AC-3 audio available. - In receiver menu, select stereo audio. - Verify that AC-3 multichannel is decoded and downmixed to PCM stereo bitstream for S/PDIF output. - Verify the audio output level can be adjusted. - In receiver menu, select multichannel audio. - Verify that AC-3 multichannel is passed-through as native bitstream for S/PDIF output. Expected result: IRD supports AC-3 audio decoding and downmixing to PCM stereo when stereo audio is selected. IRD supports AC-3 audio pass-through native bitstream when multichannel audio is selected. |  |  |
| Test result(s) |  |  |  |
| Conformity | OK Fault Major Minor, define fail reason in comments |  |  |
| Comments | If possible describe if fault can be fixed with software update: YES NO Describe more specific faults and/or other information |  |  |
| Date |  | Sign |  |

| Test Case | AC-3: Metadata |  |  |
| --- | --- | --- | --- |
| Section | NorDig Unified 6.2.2.2 |  |  |
| Requirement | The NorDig IRD supporting E-AC-3 and AC-3 shall support the use of a complete set of Dolby metadata embedded in the audio stream - when decoding AC-3 or E-AC-3 bitstreams - transcoding E-AC-3 bitstreams to AC-3 - or creating a PCM stereo downmix from a decoded EAC-3 or AC-3 bitstream. |  |  |
| IRD variants and capability | all IRDs |  |  |
| Test procedure | TBD |  |  |
| Test result(s) |  |  |  |
| Conformity | OK Fault Major Minor, define fail reason in comments |  |  |
| Comments | If possible describe if fault can be fixed with software update: YES NO Describe more specific faults and/or other information |  |  |
| Date |  | Sign |  |

| Test Case | E-AC-3: Requirements |  |  |
| --- | --- | --- | --- |
| Section | NorDig Unified 6.2.2.1 |  |  |
| Requirement | NorDig IRD supporting E-AC-3 and AC-3 according to ETSI TS 102 366 shall: - decode AC-3 streams at all bit rates and sample rates (not including Annex E) - (additionally) decode E-AC-3 streams with data rates from 32 kbps to 3 024 kbps and support all sample rates Annex E - be capable of transcoding E-AC-3 bitstreams to AC-3 bitstreams. Transcoding to AC-3 audio streams shall be at a fixed bit rate of 640 kbps. |  |  |
| IRD variants and capability | all IRDs |  |  |
| Test procedure | Purpose of test: To verify that the IRD supports different E-AC-3 bit rates and sampling rates. Equipment: Audio decoder MUX Exciter IRD TS Source The TS shall contain services, which has - a E-AC-3 (mono,stereo) audio component with relevant signaling at bit rates and sampling rates: - Sampling rates 32, 44,1 and 48 kHz - Bit rates 64, 192, 384 and 640 kbit/s a E-AC-3 (multichannel) audio component with relevant signaling at bit rates and sampling rates: - Sampling rates 32, 44,1 and 48 kHz - Bit rates 64, 192, 384 and 640 kbit/s Test procedure: - Connect the IRD to an audio receiver, e.g. Home Theater System with HDMI. - Verify that service has a multichannel E-AC-3 audio available. - In receiver menu, select MPEGstereo audio. - Verify that E-AC-3 multichannel is decoded and downmixed to PCM stereo bitstream for HDMI output and if supported for HDMI ARC. - Verify the audio output level can be adjusted. - In receiver menu, select multichannel audio. - Verify that E-AC-3 multichannel is passed-through as native bitstream for HDMI output. - Verify that service has a stereo E-AC-3 audio available. - In receiver menu, select MPEGstereo audio. - Verify that E-AC-3 stereo is decoded and downmixed to PCM stereo bitstream for HDMI output and if supported for HDMI ARC. - Verify the audio output level can be adjusted. - In receiver menu, select multichannel audio. - Verify that E-AC-3 stereo is passed-through as native bitstream for HDMI output. Expected result: IRD supports for E-AC-3 input audio signal and down-mix to stereo. |  |  |
| Test result(s) | Bit rate; Sampling rate; OK or NOK. mono. stereo. Multichannel 5.1. Down-mix to stereo; N/A; N/A. |
| Conformity | OK Fault Major Minor, define fail reason in comments |  |  |
| Comments | If possible describe if fault can be fixed with software update: YES NO Describe more specific faults and/or other information |  |  |
| Date |  | Sign |  |

| Test Case | E-AC-3: Analogue audio output |  |  |
| --- | --- | --- | --- |
| Section | NorDig Unified 6.2.2.1, 6.8.1 |  |  |
| Requirement | The NorDig IRD with analogue outputs shall always have an audio signal present on the analogue outputs (to built-in loudspeakers (see note 1)) if any of the supported formats is received. If the NorDig IRD has analogue stereo output(s), it shall be capable of decoding and downmixing the supported audio formats for the analogue outputs. NorDig IRD supporting E-AC-3 and AC-3 according to ETSI TS 102 366 shall • (additionally) decode E-AC-3 streams with data rates from 32 kbps to 3 024 kbps and support all sample rates. |  |  |
| IRD variants and capability | all IRDs |  |  |
| Test procedure | Purpose of test: To verify that receiver decodes E-AC-3 bitstream and audio output level can be adjusted. This test in only mandatory for STB or IDTV with SCART or analogue audio output interface. Equipment: Source MUX Exciter DVB receiver Monitor A transport stream containing aservice with E-AC-3 multichannel audio. Test procedure: - Verify that service has a multichannel E-AC-3 audio available. - In receiver menu, select MPEG-1 Layer II audio, (Sometimes called PCM). - Verify that E-AC-3 multichannel is downmixed to stereo at analog audio output. - In receiver menu, select multichannel audio. - Verify that E-AC-3 multichannel is downmixed to stereo at analog audio output. Expected result: IRD supports E-AC-3 audio decoding and audio is downmixed to stereo in analog audio output(s). The audio output level can be adjusted. |  |  |
| Test result(s) |  |  |  |
| Conformity | OK Fault Major Minor, define fail reason in comments |  |  |
| Comments | If possible describe if fault can be fixed with software update: YES NO Describe more specific faults and/or other information |  |  |
| Date |  | Sign |  |

| Test Case | E-AC-3: required output formats for HDMI output HDMI ARC and HDMI eARC |  |  |
| --- | --- | --- | --- |
| Section | NorDig Unified 6.2.2.3 |  |  |
| Requirement | Where HDMI, HDMI ARC (or eARC) is implemented, NorDig IRDs supporting E-AC-3 and AC-3 shall be capable of providing the following formats from an E-AC-3 or AC-3 bitstream (see chapter 16 for factory default settings): - Pass-through of native bitstream (AC-3 and E-AC-3) - E-AC-3 bitstream transcoded to AC-3 bitstream - Decoded and downmixed (if > 2 channels) to PCM stereo bitstream |  |  |
| IRD variants and capability | all IRDs with option for HDMI, or HDMI ARC, or HDMI eARC |  |  |
| Test procedure | Purpose of test: To verify that IRD decodes/transcodes and passes-through E-AC-3 bitstream. To verify for the decoded E-AC-3 bitstream that the audio output level can be adjusted. This test is only relevant for STB IRDs with HDMI output and iDTV IRDs with HDMI ARC and/or HDMI eARC output. Equipment: TS source MUX Exciter STB IRD HDMI receiver Monitor HDMI ARC receiver HDMI eARC receiver iDTV IRD A transport stream with E-AC-3 multichannel audio. Test procedure: - Connect the IRD to an HDMI (ARC, eARC) receiver e.g. Home Theater System with HDM (ARC, eARC) input that supports E-AC-3 decoding. - Verify that service has a multichannel E-AC-3 audio available. - In receiver menu, select stereo audio. - Verify that E-AC-3 multichannel is decoded and downmixed to PCM stereo bitstream for HDMI output. - Verify the audio output level can be adjusted. - In receiver menu, select multichannel audio. - Verify that E-AC-3 multichannel is passed-through as native bitstream* for HDMI output or transcoded to AC-3. *HDMI (ARC, eARC) receiver. Expected result: IRD supports E-AC-3 audio decoding and downmixing to PCM stereo when stereo audio is selected. IRD supports AC-3 audio pass-through native bitstream for HDMI output and/or HDMI ARC or transcode it to AC-3 if receiving device does not support E-AC-3 when multichannel audio is selected. |  |  |
| Test result(s) |  |  |  |
| Conformity | OK Fault Major Minor, define fail reason in comments |  |  |
| Comments | If possible describe if fault can be fixed with software update: YES NO Describe more specific faults and/or other information |  |  |
| Date |  | Sign |  |

| Test Case | E-AC-3: S/PDIF output interface |  |  |
| --- | --- | --- | --- |
| Section | NorDig Unified 6.2.2.1, 6.2.2.3 |  |  |
| Requirement | The NorDig IRD supporting E-AC-3 and AC-3 and including an S/PDIF output shall be capable of providing the following formats on the S/PDIF connector from an E-AC-3 or AC-3 bitstream: - E-AC-3 bitstream transcoded to AC-3 bitstream - Decoded and downmixed (if > 2 channels) to PCM stereo bitstream - Pass-through of AC-3 bitstream NorDig IRD supporting E-AC-3 and AC-3 according to ETSI TS 102 366 shall - (additionally) decode E-AC-3 streams with data rates from 32 kbps to 3 024 kbps and support all sample rates. - be capable of transcoding E-AC-3 bitstreams to AC-3. Transcoding to AC-3 audio streams shall be at a fixed bit rate of 640 kbps. |  |  |
| IRD variants and capability | all IRDs |  |  |
| Test procedure | Purpose of test: To verify that receiver decodes/transcodes E-AC-3 bitstream. This test is only relevant for IRD with S/PDIF output. Equipment: MPEG-2 source MUX Exciter DVB receiver S/PDIF receiver Monitor A transport stream with E-AC-3 multichannel audio. Test procedure: - Connect the IRD to a S/PDIF receiver e.g. Home Theater System with S/PDIF. - Verify that service has a multichannel E-AC-3 audio available. - In receiver menu, select stereo audio. - Verify that E-AC-3 multichannel is decoded and downmixed to PCM stereo bitstream for S/PDIF output. - In receiver menu, select multichannel audio. - Verify that E-AC-3 multichannel is passed-through as native bitstream for S/PDIF output. Expected result: IRD supports E-AC-3 audio decoding to PCM stereo bitstream when stereo audio is selected. IRD supports E-AC-3 transcode to AC-3 when multichannel audio is selected. |  |  |
| Test result(s) |  |  |  |
| Conformity | OK Fault Major Minor, define fail reason in comments |  |  |
| Comments | If possible describe if fault can be fixed with software update: YES NO Describe more specific faults and/or other information |  |  |
| Date |  | Sign |  |

| Test Case | E-AC-3: Metadata |  |  |
| --- | --- | --- | --- |
| Section | NorDig Unified 6.2.2.2 |  |  |
| Requirement | The NorDig IRD supporting E-AC-3 and AC-3 shall support the use of a complete set of Dolby metadata [36] embedded in the audio stream when: - decoding AC-3 or E-AC-3 bitstreams - transcoding E-AC-3 bitstreams to AC-3 - or creating a PCM stereo downmix from a decoded EAC-3 or AC-3 bitstream |  |  |
| IRD variants and capability | all IRDs |  |  |
| Test procedure | Purpose of test: To verify that the IRD supports Dolby metadata. Equipment: Audio decoder MUX Exciter IRD TS Source The TS shall contain a service E-AC-3, which has the following metadata included in audio component: - DolbyDynamicRange Control - Dolby Dialogue Normalization according to ISO/IEC 14496-3 : 2005 (Audio 3rd edition) - Down Mix parameters Test procedure: - Setup the system - Connect audio decoder to HDMI output. - Verify that the IRD supports metadata correctly for decoding of the E-AC-3 stereo - Verify that the IRD supports metadata correctly for transcoding E-AC-3 multichannel to AC-3. - Verify that the IRD supports metadata correctly for creating PCM stereo downmix. Expected result: IRD shall support E-AC-3 metadata according to requirement. |  |  |
| Test result(s) |  |  |  |
| Conformity | OK Fault Major Minor, define fail reason in comments |  |  |
| Comments | If possible describe if fault can be fixed with software update: YES NO Describe more specific faults and/or other information |  |  |
| Date |  | Sign |  |

| Test Case | HE AAC: Requirements |  |  |
| --- | --- | --- | --- |
| Section | NorDig Unified 6.2.3.1 |  |  |
| Requirement | NorDig IRDs supporting HE-AAC (and thereby also AAC-LC) shall be capable of: • decoding HE-AAC Version 1 at Level 2 at sampling rates of 48 kHz according to ETSI TS 101 154. • decoding, including downmixing HE-AAC Version 1 at Level 4 (multi-channel, up to 5.1) at sampling rates of 48 kHz according to ETSI TS 101 154, Annex C (downmix). • transcoding HE-AAC Version 1 at Level 4 (multi-channel, up to 5.1) at sampling rates of 48 kHz according to TS 101 154, Annex H to AC-3 or DTS. Nordig IRDs shall be able to skip bitstream elements that are not recognized, i.e. unknown Fill elements and Data Stream elements. If NorDig IRD is supporting HE-AAC audio stream transcoding to AC-3 audio stream, it shall be done according to TS 102 366. Transcoding to AC-3 multichannel audio streams shall be at a fixed bit rate of 640 kbps. If NorDig IRD is supporting HE-AAC audio stream transcoding to DTS audio stream, it shall be done according to TS 102 114 at a fixed bit rate of 1,536 Mbps. |  |  |
| IRD variants and capability | all IRDs |  |  |
| Test procedure | Purpose of test: To verify that the IRD supports HE AAC requirements. Equipment: Audio decoder MUX Exciter IRD TS Source The TS shall contain - a service with HE AAC Level 2 @48kHz (mono,stereo) audio component with relevant signaling. - a service with HE AAC Level 4 @ 48kHz (multichannel)audio component with relevant signaling. Test procedure: - Connect the IRD to an audio receiver, e.g. Home Theater System with HDMI. - Verify that service has a multichannel HE AAC audio available. - In receiver menu, select MPEG stereo audio. - Verify that HE AAC multichannel is decoded and downmixed to PCM stereo bitstream for HDMI output and if supported for HDMI ARC. - Verify the audio output level can be adjusted. - In receiver menu, select multichannel audio. - Verify that HE AAC multichannel is trascoded to DTS or AC-3 bitstream for HDMI output. - Verify that service has a stereo HE AAC audio available. - In receiver menu, select MPEGstereo audio. - Verify that HE AAC stereo is decoded and downmixed to PCM stereo bitstream for HDMI output and if supported for HDMI ARC. - Verify the audio output level can be adjusted. - In receiver menu, select multichannel audio. - Verify that HE AAC stereo is transcoded to DTS or AC-3 bitstream for HDMI output. Expected result: IRD supports decoding of HE AAC Level 2 and 4 @ 48 kHz and transcoding of it to AC-3 or DTS and supports down-mixing. |  |  |
| Test result(s) | Feature; OK or NOK. Decoding of HE AAC L2 @48kHz. Decoding of HE AAC L4 @48kHz. Transcoding of HE AAC L4 @48kHz to AC-3 at 640kbps. Transcoding of HE AAC L4 @48kHz to DTS 1.536Mbps. Down-mixing of HE AAC L4 @48kHz. |
| Conformity | OK Fault Major Minor, define fail reason in comments |  |  |
| Comments | If possible describe if fault can be fixed with software update: YES NO Describe more specific faults and/or other information |  |  |
| Date |  | Sign |  |

| Test Case | HE AAC: Analogue audio output |  |  |
| --- | --- | --- | --- |
| Section | NorDig Unified 6.2.3.1, 6.8 |  |  |
| Requirement | The NorDig IRD with analogue outputs shall always have an audio signal present on the analogue outputs (to built-in loudspeakers (see note 1)) if any of the supported formats is received. If the NorDig IRD has analogue stereo output(s), it shall be capable of decoding and downmixing the supported audio formats for the analogue outputs. NorDig IRDs supporting HE-AAC (and thereby also AAC-LC) shall be capable of - decoding HE-AAC Version 1 at Level 2 at sampling rates of 48 kHz according to ETSI TS 101 154. - decoding, including downmixing HE-AAC Version 1 at Level 4 (multi-channel, up to 5.1) at sampling rates of 48 kHz according to ETSI TS 101 154, Annex C (downmix). |  |  |
| IRD variants and capability | all IRDs |  |  |
| Test procedure | Purpose of test: To verify that receiver decodes HE AAC bitstream and the audio output level can be adjusted. This test in only mandatory for STB or IDTV with SCART or analogue audio output interface. Equipment: Source MUX Exciter DVB receiver Monitor A transport stream shall contain: - a service with HE AAC Leve l2 @ 48kHz (stereo) audio component with relevant signaling. - a service with HE AAC Level 4 @ 48kHz (multichannel) audio component with relevant signaling. Test procedure: - Verify that services are available. - In receiver menu, selectstereo audio. - Select service with HE AAC Level2@48kHz (stereo). - Verify that stereo audio is available at analog audio output. - Verify the audio output level can be adjusted. - Select service with HE AAC Level4@48kHz (multichannel) - Verify that HE AAC multichannel is downmixed to stereo at analog audio output and HE AAC stereo is available at analog audio output. - Verify the audio output level can be adjusted. - In receiver menu, select multichannel audio. - Verify that HE AAC multichannel is downmixed to stereo at analog audio output output and HE AAC stereo is available at analog audio output. Expected result: IRD supports HE AAC stereo audio decoding to stereo audio and the audio output level can be adjusted. IRD supports HE AAC multichannel audio downmixing to stereo in analog audio output(s) and the audio output level can be adjusted. |  |  |
| Test result(s) |  |  |  |
| Conformity | OK Fault Major Minor, define fail reason in comments |  |  |
| Comments | If possible describe if fault can be fixed with software update: YES NO Describe more specific faults and/or other information |  |  |
| Date |  | Sign |  |

| Test Case | HE AAC: required output formats for HDMI output, HDMI ARC and HDMI eARC |  |  |
| --- | --- | --- | --- |
| Section | NorDig Unified 6.2.3.3 |  |  |
| Requirement | Where HDMI, HDMI ARC (or eARC) is implemented, NorDig IRDs supporting HE-AAC shall be capable of providing the following formats on the HDMI output connector from a HE-AAC bitstream (see chapter 16 for factory default settings): - Transcoded to AC-3 or DTS bitstream - Decoded and downmixed (if > 2 channels) to PCM stereo bitstream |  |  |
| IRD variants and capability | all IRDs with option for HDMI, or HDMI ARC, or HDMI eARC |  |  |
| Test procedure | Purpose of test: To verify that IRD decodes/transcodes and passes-through HE AAC bitstream. When HE AAC bitstream is decoded and transcoded the audio output level can be adjusted. This test in only relevant for STB IRDs with HDMI output and iDTV IRDs with HDMI ARC and/or HDMI eARC output. Equipment: TS source MUX Exciter STB IRD HDMI receiver Monitor HDMI ARC receiver HDMI eARC receiver iDTV IRD A transport stream contains: - a service with HE AAC Level2 @48kHz (mono,stereo)audio component with relevant signaling. - a service with HE AAC Level 4 @48kHz (multichannel)audio component with relevant signaling. Test procedure: - Connect the IRD to HDMI (ARC, eARC) receiver e.g. Home Theater System with HDMI(ARC, eARC). - Verify that services are available. - In receiver menu, select stereo audio. - Select service with HE AAC Level2@48kHz (stereo). - Verify that HE AAC stereo is transcoded to PCM stereo audio bitstream at HDMI (ARC, eARC) output. - Verify the audio output level can be adjusted. - Select service with HE AAC Level4@48kHz (multichannel) - Verify that HE AAC multichannel is downmixed to PCM stereo at HDMI (ARC, eARC) output. - Verify the audio output level can be adjusted. - In receiver menu, select multichannel audio. - Verify that HE AAC multichannel is transcoded to AC-3 or DTS multichannel at HDMI (ARC, eARC) output. Expected result: IRD supports HE AAC stereo audio decoding to PCM stereo bitstream at its HDMI output and HDMI ARC when stereo audio is selected. The audio output level can be adjusted. IRD supports HE AAC multichannel audio transcoding to AC-3 and/or DTS at its HDMI output and HDMI ARC when multichannel is selected. |  |  |
| Test result(s) |  |  |  |
| Conformity | OK Fault Major Minor, define fail reason in comments |  |  |
| Comments | If possible describe if fault can be fixed with software update: YES NO Describe more specific faults and/or other information |  |  |
| Date |  | Sign |  |

| Test Case | HE AAC: S/PDIF output interface |  |  |
| --- | --- | --- | --- |
| Section | NorDig Unified 6.2.3.3 |  |  |
| Requirement | The NorDig IRD supporting HE-AAC and including an S/PDIF output shall be capable of providing the following formats on the S/PDIF connector from a HE-AAC bitstream: • Decoded and downmixed (if > 2 channels) to PCM stereo bitstream • Transcoded to AC-3 or DTS bitstream |  |  |
| IRD variants and capability | all IRDs |  |  |
| Test procedure | Purpose of test: To verify that receiver decodes/transcodes and passes-through HE AAC bitstream. This test in only relevant for IRD with S/PDIFoutput. Equipment: MPEG-2 source MUX Exciter DVB receiver S/PDIF receiver Monitor A transport stream contains: - a service with HE AAC Level 2 @48kHz (mono,stereo)audio component with relevant signaling. - a service with HE AAC Level 4 @48kHz (multichannel)audio component with relevant signaling. Test procedure: - Connect the IRD to S/PDIF receiver e.g. Home Theater System with S/PDIF. - Verify that servicesare available. - In receiver menu, select stereo audio. - Verify that HE AAC multichannel is transcoded to PCM stereo at S/PDIF output and HE AAC stereo is transcoded to PCM stereo at S/PDIF output. - In receiver menu, select multichannel audio. - Verify that HE AAC multichannel is transcoded to AC-3 or DTS multichannel at S/PDIF output and HE-AAC stereo is transcoded to AC-3 2.0 or PCM stereo at S/PDIF output. Expected result: IRD supports HE AAC stereo audio decoding to PCM stereo when stereo is selected at its S/P DIF output. IRD supports HE AAC multichannel transcode to AC-3 and/or DTS at its S/PDIF output when multichannel is selected. |  |  |
| Test result(s) |  |  |  |
| Conformity | OK Fault Major Minor, define fail reason in comments |  |  |
| Comments | If possible describe if fault can be fixed with software update: YES NO Describe more specific faults and/or other information |  |  |
| Date |  | Sign |  |

| Test Case | HE AAC: Metadata |  |  |
| --- | --- | --- | --- |
| Section | NorDig Unified 6.2.3.2 |  |  |
| Requirement | The NorDig IRD supporting HE-AAC shall support the use of the following MPEG-4 AAC metadata embedded in the audio stream when decoding HE-AAC and transcoding HE-AAC multi-channel to AC-3 or DTS: - Program Reference Level according to ISO/IEC 14496-3 (prog_ref_level) - Downmix Parameters according to "Transmission of MPEG4 Ancillary Data" part of DVB specification ETSI TS 101 154 (center_mix_level, surround_mix_level) - Dynamic Range Control (DRC) according to ISO/IEC 14496-3 (dyn_rng_sgn, dyn_rng_ctl) - Heavy Compression according to ETSI TS 101 154 Annex C.5.2.5 (compression_on, compression_value) |  |  |
| IRD variants and capability | all IRDs |  |  |
| Test procedure | Purpose of test: To verify that the IRD supports HE AAC metadata. Equipment: MUX Exciter IRD TS Source The TS containing: - A service with program reference level (prog_ref_level) - A service with downmix parameters (center_mix_level, surround_mix_level) - A service with dynamic range control (dyn_rng_sgn, dyn_rng_ctl) - A service with heavy compression (compression_on, compression_value) Test procedure: - Connect the IRD under the test to an amplifier that supports the metadata decoding - Play the stream and tune the IRD to the service that contains the metadata - Check that the IRD transforms the HE AAC metadata in the stream to the matching Dolby metadata. Note: A Dolby Reference decoder can be used to read the Dolby metadata values (if IRD transcodes to AC-3). NOTE: For IRD´s that transcode to DTS, metadata might be more difficult to verify. DTS Reference decoder can be used Expected result: IRD shall support HE AAC metadata according to requirement. |  |  |
| Test result(s) |  |  |  |
| Conformity | OK Fault Major Minor, define fail reason in comments |  |  |
| Comments | If possible describe if fault can be fixed with software update: YES NO Describe more specific faults and/or other information |  |  |
| Date |  | Sign |  |

| Test Case | Audio prioritizing for non-NGA streams - Audio language support |  |  |
| --- | --- | --- | --- |
| Section | NorDig Unified 6.5.1, 6.5.3, 16.2.3 |  |  |
| Requirement | The user shall be able to select storable preferences for primary and secondary audio language. If an audio-stream according to the primary audio language preference is not associated with the chosen service the NorDig IRD shall automatically choose the audio stream according to the secondary audio language preference, if present. In addition the user shall be able to manually select between all audio-streams that are associated with the active service. For prioritisation between non-NGA audio PIDs/streams (for non-NGA capable NorDig IRDs or NGA capable NorDig IRDs that has not prioritised NGA stream(s) according to section 6.5.2), the audio source priority shall be based on the property of the supported audio streams and for different user preference settings of audio format and audio type as selected by the user according with table 6.1 below. See section 16.4 for factory default user preference settings related to audio. |  |  |
| IRD variants and capability | all IRDs |  |  |
| Test procedure | Purpose of test: To verify the support for primary and secondary audio language. Equipment: Transport stream containing at least one service with two audio languages. Suitable languages are Swedish, Finnish, Norwegian, Danish, Icelandic, Sami, Irish/Gaelic and English. Test procedure: - Verify a service with two different audio components is broadcasted dedicated for test of Nordic languages - Verify the audio components are signaled correctly. - Verify it is possible to select and setup primary and secondary audio language in the receiver setup. - Verify the receiver selects the correct audio for primary language. - Drop the audio component selected as primary audio language in receiver. - Verify the receiver selects correct audio component, i.e. secondary audio language. Expected result: It shall be possible to select primary and secondary language. If the selected primary language is not broadcasted, the selected secondary audio language shall be selected automatically. |  |  |
| Test result(s) |  |  |  |
| Conformity | OK Fault Major Minor, define fail reason in comments |  |  |
| Comments | If possible describe if fault can be fixed with software update: YES NO Describe more specific faults and/or other information |  |  |
| Date |  | Sign |  |

| Test Case | Audio Prioritising for non-NGA streams – audio format and stream type |  |  |
| --- | --- | --- | --- |
| Section | NorDig Unified 6.5, 16.2.3. |  |  |
| Requirement | The NorDig IRD shall be able to select audio stream according to user selections, these settings should be stored in the IRD´s memory separately for each service. If manually selected audio is not able to be stored in the IRD´s memory per service, a global setting should be made possible to set manual prioritisation of stream type for all services. The priority for the selected audio source shall be based on the user selections and audio stream shall be selected according to the priority list in table table 6.1 below. If the user selections are not matching with the audio streams, the NorDig IRD shall always select one of the audio streams which closest suites with the user selections and will hereby provide audio to end-user. In the event that the NorDig IRD finds more than one stream with same audio property parameters (type, language, format and stream type) that fully or partially matches the IRD user preference settings, of the remaining streams (which have the same audio properties), the stream with the lowest PID shall be selected. The user shall be able to select multi-channel audio for the digital outputs, when the outputs are equipped for multichannel audio. If multichannel mode is selected and if both multichannel and stereo streams are available for the selected language and audio type, the NorDig IRD shall use the multichannel audio stream to provide downmixed audio in analog audio output(s), if applicable, and suitable digital bitstream format in digital audio output(s) as e.g. in examples in Annex G (Example table when more than one audio codec is received). If stereo mode is selected and if both multichannel and stereo streams are available for the selected language and audio type, the NorDig IRD shall use the stereo audio source to provide audio in analog audio output(s), if applicable, and PCM stereo in digital audio output(s) as e.g. in examples in Annex G (Guidelines for NorDig IRD audio selection: “Example table when more than one audio stream is received”). |  |  |
| IRD variants and capability | all IRDs |  |  |
| Test procedure | Purpose of test: To verify the audio priority selection and output audio format. Test Equipment: Audio decoder MUX Exciter STB TS Source Transport stream containing services with audio components (same language and audio type) listed in table below in test results. Suitable languages are Swedish, Finnish, Norwegian, Danish, Icelandic, Sami, Irish/Gaelic and English. Audio type is “normal”. Audio format and stream type shall be signaled in AAC_descriptor, AC-3_descriptor or E-AC-3_descriptor according to following: - the AAC_type field in the AAC_descriptor for AAC audio, - the number of channels flags in the AC-3 descriptor and Enhanced AC-3 descriptor for AC-3 and E-AC-3. Correct signaling is broadcasted depending of the audio format and stream type in test results table. IRD is assumed to prioritise audio format and stream type according to list above. Test procedure: - Setup the system - Verify that the audio selection is done correctly according the input formats in table below in test results - Repeat test with all input format in table below - Fill in test results Expected result: The IRD shall select the audio format and stream type correctly. |  |  |
| Test result(s) | Available inputs formats (same language and audio type); Output on analog interface or integrated loudspeakers; OK/NOK. When Stereo is selected (default); When Multichannel is selected. MPEG1 layer II & AC-3; Decoded from MPEG1 LII; Downmixed from AC-3. MPEG1 layer II & E-AC-3; Decoded from MPEG1 LII; Downmixed from E-AC-3. HE AAC stereo & HE AAC multichannel; Decoded from HE AAC stereo; Downmixed from HE AAC multichannel. HE AAC stereo & AC-3 multichannel; Decoded from HE AAC stereo; Downmixed from AC-3 multichannel. Available inputs formats (same language and audio type); Output on S/PDIF; OK/NOK. When Stereo is selected (default); When Multichannel is selected. MPEG1 layer II & AC-3; PCM (from MPEG1 LII); AC-3. MPEG1 layer II & E-AC-3; PCM (from MPEG1 LII); Transcoded to AC-3 (from E-AC-3). HE AAC stereo & HE AAC multichannel; PCM (from HE AAC stereo); Transcoded to AC-3, or DTS (from HE AAC multichannel). HE AAC stereo & AC-3 multichannel; PCM (from HE AAC stereo); AC-3. Available inputs formats (same language and audio type); Output on HDMI; OK/NOK. When Stereo is selected (default); When Multichannel is selected. MPEG1 layer II & AC-3 multichannel; PCM (from MPEG1 LII); AC-3. MPEG1 layer II & E-AC-3 multichannel; PCM (from MPEG1 LII); AC-3 1). MPEG1 layer II & E-AC-3; PCM (from MPEG1 LII); E-AC-3. HE AAC stereo & HE AAC multichannel; PCM (from HE AAC stereo); HE AAC multichannel. HE AAC stereo & HE AAC multichannel; PCM (from HE AAC stereo); Transcoded 2) to AC-3, or DTS. HE AAC stereo & AC-3 multichannel; PCM (from HE AAC stereo); AC-3. 1) transcoded to AC-3 if E-AC-3 is not supported by receiving device 2) transcoded to AC-3 or DTS if HE AAC not supported by receiving device |
| Conformity | OK Fault Major Minor, define fail reason in comments |  |  |
| Comments | If possible describe if fault can be fixed with software update: YES NO escribe more specific faults and/or other information |  |  |
| Date |  | Sign |  |

| Test Case | Audio Prioritising for non-NGA streams – audio type |  |  |
| --- | --- | --- | --- |
| Section | NorDig Unified 6.5 16.2.3 |  |  |
| Requirement | The NorDig IRD shall be able to select audio stream according to user selections, these settings should be stored in the IRD´s memory separately for each service. If manually selected audio is not able to be stored in the IRD´s memory per service, a global setting should be made possible to set manual prioritisation of stream type for all services. The priority for the selected audio source shall be based on the user selections and audio stream shall be selected according to the priority list in table table 6.1 below. If the user selections are not matching with the audio streams, the NorDig IRD shall always select one of the audio streams which closest suites with the user selections and will hereby provide audio to end-user. In the event that the NorDig IRD finds more than one stream with same audio property parameters (type, language, format and stream type) that fully or partially matches the IRD user preference settings, of the remaining streams (which have the same audio properties), the stream with the lowest PID shall be selected. |  |  |
| IRD variants and capability | all IRDs |  |  |
| Test procedure | Purpose of test: Verify that IRD selects normal/undefined (0x00) audio component by default. Equipment: TS source MUX Exciter DVB receiver The TS must contain two services with several audio components for the same language, audio formats and stream type but different audio types. Suitable languages are Swedish, Finnish, Norwegian, Danish, Icelandic, Sami, Irish/Gaelic and English. Audio type is signaled in PMT in one or all of the following descriptors: - Supplementary_audio_descriptor for any stream type. - AAC_descriptor for AAC audio. - AC-3 descriptor for AC-3 audio. - Enhanced AC-3 descriptor for E-AC-3 audio. - ISO639_descriptor for any stream type. IRD is assumed to prioritise the audio type signalization information in that order. Audio format is stereo. Stream type for all audio components is either MPEG-4 HE.AAC version 1 or E-AC-3 including metadata corresponding audio_type. Service1; Service2; Frequency. MUX TS_id 1 Network_id 1 ON_id 1); SID 1100 S_name Test11 PMT PID 1100 V PID 1109 A PID 1108 ISO639_language_code 2) Audio_type hearing impaired A PID 1107 ISO639_language_code 2) Audio_type visual impaired commentary A PID 1106 ISO639_language_code 2) Audio_type Normal/Undef LCD 1 visible; SID 1200 S_name Test12 PMT PID 1200 V PID 1209 A PID 1208 ISO639_language_code 2) Audio_type hearing impaired A PID 1207 ISO639_language_code 2) Audio_type visual impaired commentary A PID 1206 ISO639_language_code 2) Audio_type Normal/Undef LCD 2 visible; Can be chosen depending of the distribution media. - ON_id (Original_network_id) can be chosen in range 0x0001-0xfe00 (operational network) and it shall be same for both muxes. - Language must the same for all audio components and it can be one of the swe/fin/nor/ice/dan/smi/gle/iri/eng/und Test procedure: - Verify the receiver selects audio component which has normal/undefined audio by default - Verify user is able to select different audio components. - Verify that the user is able to store selected audio component in the IRD´s non-volatile memory, or can be set by changing the priority defined in list in table 6.1. - Put IRD into standby mode and unplug the power cord - Turn on the IRD - Zap to different service and verify that the IRD is selecting wanted audio. Expected result: IRD selects the audio component that is signalled as Normal/Undefined (0x00) by default when several audio components with the same language code, audio format and stream type but different audio type are available within one service. The user shall also be able to select between the different audio components. The user should also be able to store the selected audio component prioristation to non-volatile memory. |
| Test result(s) |  |  |  |
| Conformity | OK Fault Major Minor, define fail reason in comments |  |  |
| Comments | If possible describe if fault can be fixed with software update: YES NO Describe more specific faults and/or other information |  |  |
| Date |  | Sign |  |

| Test Case | Audio Prioritising for non-NGA streams – audio format signaling missing |  |  |
| --- | --- | --- | --- |
| Section | NorDig Unified 6.5.3 |  |  |
| Requirement | For the selection of audio format (mono, stereo or multichannel, see Table 6.1 and Table 6.2, the NorDig IRD shall use: - the AAC_type field in the AAC_descriptor for AAC audio, - the number of channels flags in the AC-3 descriptor and Enhanced AC-3 descriptor for AC-3 and E-AC-3. In any case where, for some reason, this information is not carried in the PMT for a particular audio stream, then the IRD shall prioritise based on the assumption that audio stream contains “normal” stereo content. In the event that the NorDig IRD finds more than one stream with same audio property parameters (type, language, format and stream type) that fully or partially matches the IRD user preference settings, of the remaining streams (which have the same audio properties), the stream with the lowest PID shall be selected. |  |  |
| IRD variants and capability | all IRDs |  |  |
| Test procedure | Purpose of test: Verify which audio component the IRD selects when no additional audio format signalization is added to audio component. Equipment: TS source MUX Exciter DVB receiver The TS shall contain two services with two audio components. The PMTs shall contain the elementary streams with appropriate stream_types. All audio tracks shall be signaled with a same language and audio_type (0x0) in ISO_639_language_descriptor. No other descriptors shall be signaled for the audio streams. Service1; Service2; Frequency. MUX TS_id 1 Network_id 1 ON_id 1); SID 1100 S_name Test11 PMT PID 1100 V PID 1109 A PID 1108 (HE AAC L4) A PID 1107 (MPEG1 LII stereo) LCD 1 visible; SID 1200 S_name Test12 PMT PID 1200 V PID 1209 A PID 1208 (AC-3 multichannel) A PID 1207 (MPEG1 LII stereo) LCD 2 visible; Can be chosen depending of the distribution media. - ON_id (Original_network_id) can be chosen in range 0x0001-0xfe00 (operational network) and it shall be same for both muxes. Test procedure: - Configure the IRD to select ‘normal’ stereo audio. - Zap through the services in the stream. - Verify that the IRD selects the MPEG1-LII audio component and audio is played out correctly. - Configure the IRD to select ‘normal’ multichannel audio. - Zap through the services in the stream. - Verify that the IRD selects the advanced audio component and audio is played out correctly. Expected result: IRD selects the audio component that is signalled as Normal/Undefined (0x00) by default when several audio components without signaling is available within one service. |
| Test result(s) |  |  |  |
| Conformity | OK Fault Major Minor, define fail reason in comments |  |  |
| Comments | If possible describe if fault can be fixed with software update: YES NO Describe more specific faults and/or other information |  |  |
| Date |  | Sign |  |

| Test Case | Audio video synchronization |  |  |
| --- | --- | --- | --- |
| Section | NorDig Unified 6.7.1 |  |  |
| Requirement | The NorDig IRDs shall not introduce more than ± 5 ms of relative delay between the audio and video components on the primary output (1) and not more than ± 25 ms between the primary video output and a secondary audio output. (2) The relative delay between the audio and video components shall be continously synchronized. If the NorDig IRD, as a part of an integrated digital TV set (iDTV) has an audio output, and/or decoded PCM digital audio output, the audio shall be in sync with the video display. Where audio leaves the IRD in an encoded form (such as in IEC61937 bitstream from S/PDIF, HDMI, HDMI ARC or HDMI eARC outputs, the IRD shall compensate for the decoding latency of the selected audio format, as specified for the relevant reference decoder for the selected format (e.g AC-3), such that the output of the reference decoder would be ± 5 ms with respect to the decoded video. This applies for all audio systems that the IRD supports. |  |  |
| IRD variants and capability | all IRDs |  |  |
| Test procedure | Purpose of test: To verify that the relative time delay difference between video and audio on primary output satisfies the specification. Mandatory for iDTVs with integrated loudspeakers. Relevant for STB and iDTVs with audio output interfaces for analog audio,HDMI output and HDMI ARC. Equipment: Source MUX Exciter Reference device for video and audio inspection Reference device for audio listening Video/audio synchronisation measurement device IDTV STB Transport stream containing: - a ‘Bounce’ video signal (fully black and white pictures). - The audio (sound bursts) is synchronized with the video. Test procedure: If the IRD is iDTV: - Subjectively verify the video content in the integrated display is in synchronization with the audio content on the integrated loudspeakers. - If any doubt is arisen for the synchronisation accuracy, measure the synchronization with the dedicated measurement equipment. - If the iDTV supports analog audio output or digital audio output on HDMI output, HDMI ARC or S/PDIF, connect a receiving reference device to such an output interface. - Verify subjectively if the audio on the reference device is in synchronization with the video displayed on the iDTV. - If any doubt is arisen for the synchronisation accuracy, measure the synchronization with the dedicated measurement equipment. If the IRD is STB: - Connect a receiving reference device to primary HDMI output interface. - Verify subjectively if the audio on the reference device is in synchronization with the video displayed on the reference device. - If any doubt is arisen for the synchronisation accuracy, measure the synchronization with the dedicated measurement equipment. - If the STB supports analog audio output or digital audio output on S/PDIF, connect a receiving reference device to such an output interface. - Verify subjectively if the audio on the reference device is in synchronization with the video displayed on the reference device. Expected result: If the IRD is an iDTV - the relative delay between video on integrated display and audio on integrated loudspeakers is less than ±5 ms. - the relative delay between video on integrated display and audio on the analog audio output or digital audio outputs HDMI ARC, HDMI eARC and S/PDIF is less than ±5 ms. If the IRD is an STB, - the relative delay between video and audio on HDMI output is less than ±5 ms. - the relative delay between video on reference display and audio on analog audio output or digital audio output S/PDIF is less than ±5 ms. The secondary display output is not measured. |  |  |
| Test result(s) |  |  |  |
| Conformity | OK Fault Major Minor, define fail reason in comments |  |  |
| Comments | If possible describe if fault can be fixed with software update: YES NO Describe more specific faults and/or other information |  |  |
| Date |  | Sign |  |

| Test Case | Adjustement of Video/audio-delay |  |  |
| --- | --- | --- | --- |
| Section | NorDig Unified 6.7.1 |  |  |
| Requirement | The NorDig IRD shall support the possibility to adjust the audio-delay on the HDMI and S/PDIF output (if available) up to 250 ms and it should be adjustable in 5 ms steps, as the IRD may have several different user set-ups, resulting in different a/v delays; e.g. the IRD may be connected to several types of external audio-amplifiers and the IRD may be connected to several types of external screens. |  |  |
| IRD variants and capability | all IRDs |  |  |
| Test procedure | Purpose of test: To verify that the IRD has settings for video/audio delay. This requirement is only applicable to IRDs that are not part of an iDTV. Equipment: Audio decoder MUX Exciter STB TS Source Display Test procedure: - Verify the IRD has a setting to setup video/audio delay up to 250ms. - Verify the setting has effect in delay between video and audio. Expected result: It is possible to change static delay between video and audio decoding and it has effect. |  |  |
| Test result(s) |  |  |  |
| Conformity | OK Fault Major Minor, define fail reason in comments |  |  |
| Comments | If possible describe if fault can be fixed with software update: YES NO Describe more specific faults and/or other information |  |  |
| Date |  | Sign |  |

| Test Case | Audio handling when changing service or audio format |  |  |
| --- | --- | --- | --- |
| Section | NorDig Unified 6.9 |  |  |
| Requirement | The NorDig IRD should gracefully handle change of service or audio format at the audio outputs without significant disturbances to the end user. |  |  |
| IRD variants and capability | all IRDs |  |  |
| Test procedure | Purpose of test: To verify that the IRD handles audio format changes in zapping and audio format change. Equipment: MUX Exciter IRD TS Source The TS shall contain several services, but at least following services with following components - a HE AAC Level2 @48kHz and/or E-AC-3 (stereo)audio component with relevant signaling. - a HE AAC Level 4 @48kHz and/or E-AC-3 (multichannel)audio component with relevant signaling. - a MPEG-1 Layer II (stereo) with relevant signaling. Test procedure: - Zap to different services - Verify the IRD is able to settle to different audio formats - Decide if the format settling could disturb end-user. Expected result: Audio format change is handled gracefully. Audio handling does not disturb the end-user. |  |  |
| Test result(s) |  |  |  |
| Conformity | OK Fault Major Minor, define fail reason in comments |  |  |
| Comments | If possible describe if fault can be fixed with software update: YES NO Describe more specific faults and/or other information |  |  |
| Date |  | Sign |  |

| Test Case | Dynamic changes in audio components |  |  |
| --- | --- | --- | --- |
| Section | NorDig Unified 6.9 |  |  |
| Requirement | The NorDig IRD shall be able to handle dynamic changes of audio component(s) (PID/PIDs) in a service. The IRD shall automatically identify if an audio component is added or removed between two programme events in the same service). The NorDig IRD should gracefully handle change of service or audio format at the audio outputs without significant disturbances to the end user. The NorDig IRD shall handle the dynamic changes after change of selected service ("zapping") or dynamic PMT update (i.e. shall not require to re-install services) and shall be able to handle the following dynamic changes without user interaction and start decoding within one second after reception of change; • change of the audio codec, (for example change from MPEG-1 Layer II into AC-3) • change of ISO 639-2 language for an audio component. |  |  |
| IRD variants and capability | all IRDs |  |  |
| Test procedure | Purpose of test: To verify the IRD is able to handle dynamic changes of audio components in transmission. - change of number of audio channels within same audio codec (eg. AC-3 2.0 to AC-3 5.1 and vice versa. - change of bitrate for an audio component (eg. MPEG1-L2 bitrate from 192 kbps to 160 kbps) - change of audio PID value (any codec) - change from dual-channel audio into stereo audio and vice versa - removal of selected audio component and using the next preferred one - addition of one audio component with higher preferred user setting - change of the audio codec (eg. from MPEG1-L2 to AC-3 - change of ISO 639-2 language for an audio component Equipment: MPEG-2 source MUX Exciter DVB receiver MPEG-2 source must have capability to support dynamic changes in audio components within service(s). In case of dual-channel audiothe first language_descriptor definition corresponds physical left channel and second language_descriptor definition corresponds physical right channel. Test procedure: - Use MPEG-2 source to generate dynamic changes in audio components as listed in test results - Fill in the test results. Expected result: IRD is able to handle dynamic changes in audio components. UR 6.10 says “within 1 second” for some changes. |  |  |
| Test result(s) | OK / NOK. Change of number of audio channels within same audio codec (eg. AC-3 2.0 to AC-3 5.1 and vice versa.. Change of bitrate for an audio component (eg. MPEG1-L2 bitrate from 192 kbps to 160 kbps). Change of audio PID value (any codec). Change from dual-channel audio into stereo audio and vice versa. Removal of selected audio component and using the next preferred one. Addition of one audio component with higher preferred user setting. Change of the audio codec (eg. from MPEG1-L2 to AC-3. Change of ISO 639-2 language for an audio component. |
| Conformity | OK Fault Major Minor, define fail reason in comments |  |  |
| Comments | If possible describe if fault can be fixed with software update: YES NO Describe more specific faults and/or other information |  |  |
| Date |  | Sign |  |

| Test Case | Dynamic update of PMT- Component priority (stream type) |  |  |
| --- | --- | --- | --- |
| Section | NorDig Unified 12.6.3, 6.5.2, 6.5.3 |  |  |
| Requirement | From 12.6.3: Component priority when multiple video or audio streams are received The following applies for services that transmit in parallel more than one type of video/audio stream under the same service_id (e.g. simulcasting within the same service): For Audio decoding, NorDig IRDs shall prioritise the service´s components according to chapter 6.5, Audio Prioritisation. NorDig IRDs shall ignore advanced audio streams when it does not support decoding those streams. For example, an IRD that do not include any AC-3 (downmix) decoder, (maybe only supports pass-through of AC-3 to the digital audio output), shall not choose the AC-3 audio stream as default. Instead it shall choose among the IRDs supported audio stream types according to chapter 6.5, Audio prioritisation. When several audio streams of the same type are received, the primary stream shall be selected in accordance with the definitions in section 6.5 (Audio Prioritisation). From 6.5.2: NGA capable NorDig HEVC IRD shall prioritise the selection of NGA/AC-4 audio PID/stream over other audio PIDs/streams using other audio codecs when: - NGA stream signalised a language that matches IRD user preference settings for primary audio language. (See section 6.5.4.1 how IRD shall detect language for PMT signalling), or - NGA stream does not signalise any language (in PMT), or - No NGA stream matches the user preference settings for primary audio language and no non-NGA stream matches the IRD user preference settings for primary and secondary audio language. Otherwise, the non-NGA audio PIDs/streams shall be selected in accordance with section 6.5.3 below. From 6.5.3: For prioritisation between non-NGA audio PIDs/streams (for non-NGA capable NorDig IRDs or NGA capable NorDig IRDs that has not prioritised NGA stream(s) according to section 6.5.2), the audio source priority shall be based on the property of the supported audio streams and for different user preference settings of audio format and audio type as selected by the user according with table 6.1 below. |  |  |
| IRD variants and capability | HEVC, all IRDs |  |  |
| Test procedure | Purpose of test: To verify that the NorDig IRD is able select audio components in priority order and handle dynamic changes in service (PMT and PIDs). The idea in this test is that there is always (video and) audio available in the NorDig IRD output (STB output or iDTV’s integrated display and speakers) independently which input audio format(s) is(are) available in the service. Equipment: MPEG source MUX1 Exciter DVB receiver The IRD shall be configured with stereo output mode and normal audio mode. The IRD shall output audio in the correct format in the physical audio output or iDTV’s integrated speakers. MPEG source must have capability to produce video simultaneously with * audio component broadcast for following audio formats: - MPEG-1 Layer II - MPEG-4 HE AAC - AC-3 - E-AC-3 - AC-4 (for HEVC IRDs) Audio streams shall be stereo, normal audio and the same language. Service1; Service2; Frequency. MUX1 TS_id 1 Network_id 1 ON_id 1); SID 1100 S_name Test11 PMT PID 1100 V PID 1109 (MPEG 2 SD) V PID 1119 (AVC HD) V PID 1129 (AVC SD) A PID 1108 (MPEG1 L2) A PID 1118 (HE AAC) A PID 1128 (E-AC-3) A PID 1138 (AC-3) A PID 1148 (AC-4) Logical_chan_desc 1 visible; SID 1200 S_name Test12 PMT PID 1200 V PID 1209 incl PCR A PID 1208 Teletext PID 1207 DVB Subt PID 1106 Logical_chan_desc 2 visible; Can be chosen depending of the distribution media. 1) ON_id (Original_network_id) can be chosen in range 0x0001-0xfe00 (operational network) The service PCR is broadcasted in separate PID. Test procedure: - Control that the MUX1 have services signaled as defined in table above - Configure the IRD for stereo output and normal audio mode - Zap all the services and check quickly that they works. - Zap to service1 in MUX1 - Drop PIDs (and update PMT accordingly) in following order: - Audio PID 1118 (HE-AAC) - Audio PID 1128 (AC-3) - Audio PID 1138 (E-AC-3) - Audio PID 1148 (AC4) - Add PIDs (and update PMT accordingly) in following order: - Audio PID 1148 (AC4) - Audio PID 1138 (E-AC3) - Audio PID 1128 (AC3) - Audio PID 1118 (HE-AAC) - Verify that the service is decoded correctly by watching the video, listening the audio Expected result: NorDig HEVC IRD: After remove and addition of the PIDs, all the component in the service are decoded correctly. Basic (non-HEVC) IRD: stream_types not supported by the receiver are not affecting the receiver operability. |
| Test result(s) | Audio codec priority for stereo output and normal audio mode: Audio codec; Stream_type; Priority; OK/NOK. AC-4; 0x06; 1 (highest). Basic, MPEG-1 Layer II audio stream; 0x03; 2. MPEG-4 HE.AAC (or LC.AAC) v2 audio stream; 0x11; 3. E-AC-3 (Enhanced AC-3) audio stream; 0x06; 4. AC-3 (AC-3) audio stream; 0x06; 5 (lowest). |
| Conformity | OK Fault Major Minor, define fail reason in comments |  |  |
| Comments | If possible describe if fault can be fixed with software update: YES NO Describe more specific faults and/or other information |  |  |
| Date |  | Sign |  |

| Test Case | Audio descriptors |  |  |
| --- | --- | --- | --- |
| Section | NorDig Unified 6.9 |  |  |
| Requirement | The NorDig IRD shall be able to read the audio information contained in the DVB_SI stream_content and component_type of the component descriptor as defined in EN 300 468, see also chapter 12 and section 13.3.2. The NorDig IRD should be able to present the audio information, including the descriptors for audio description for the visually impaired and audio for the hard of hearing, contained in the component descriptor to the user for information and selection purposes. |  |  |
| IRD variants and capability | all IRDs |  |  |
| Test procedure | The IRD manufacturer shall describe the used test procedure and the used test setup. |  |  |
| Test result(s) | The IRD manufacturer shall describe the used test procedure and the used test setup. |  |  |
| Conformity | OK Fault Major Minor, define fail reason in comments |  |  |
| Comments | If possible describe if fault can be fixed with software update: YES NO Describe more specific faults and/or other information |  |  |
| Date |  | Sign |  |

| Test Case | Supplementary audio |  |  |
| --- | --- | --- | --- |
| Section | NorDig Unified 6.11 |  |  |
| Requirement | The Nordig IRD audio decoder shall (1) be capable of supporting ‘visual impaired’ Supplementary audio (SA) services, as defined in ETS TS 101 154 (however, control of pan and fade is optional). The NorDig IRD shall (1) support both Broadcast mixed and Receiver mixed Supplementary Audio. The NorDig IRD shall (1) have user selection of audio preferences for ‘normal’ and ‘Supplementary’ audio and which is a fixed setting (i.e. remain when changing service and when re-starting the IRD). The user preference settings for enabled/disable default Supplementary audio shall (1) be common for Broadcast mixed (2) and Receiver mixed alternatives. The NorDig IRD shall be able to select the correct audio type stream according to user preference settings, see section 6.5. When the user has selected Normal audio as preferred audio, the NorDig IRD shall by default select the Normal audio stream for services which have several audio streams with different audio type for selected language. When the user has selected Supplementary Audio as preferred audio, the NorDig IRD shall by default select the supplementary audio stream for services which have several audio streams with different audio type for selected language. The NorDig IRD with separate user preference settings for Audio Type selection of Audio Description and Spoken Subtitles (see section 16.2.3) shall select the appropriate Supplementary audio stream based on supplementary_audio_descriptor signalling. Furthermore, if the Supplementary Audio is signaled via ISO 639(1) language descriptor but no supplementary_audio_descriptor (i.e. supplementary_audio_descriptor is missing, meaning that the IRD can not distinguish if Supplementary Audio is Audio Description or Spoken Subtitles), the Supplementary Audio shall be selected if any of the user preferences settings Audio Description or Spoken Subtitles are turned on. Note 1: ISO_639_language_descriptor audio_type 0x03 (visually impaired commentary) is used in combination with one particular language, while Normal Audio is signalled as audio_type 0x00. |  |  |
| IRD variants and capability | all IRDs |  |  |
| Test procedure | Purpose of test: To verify the IRD handles supplementary audio for narrator and spoken subtitling within audio streams. Equipment: DVB Receiver Exciter MUX TS Source A TS containing several services with several audio components for broadcast and receiver mixing. Supplementary_audio_descriptor is signalled for all audio components according to content definition below. The normal and supplementary audio for receiving mixing within a service are encoded as MPEG-1 Layer II, HE-AAC and E-AC-3 depending of which audio codecs are supported by the IRD. The audio streams sampling rate must be the same and the audio streams are separated to two different PIDs. Transmitter; Service 1; Service 2; Frequency. MUX 1 TS_ID 1 Network_ID 1 ON_ID 1); SID 1100 S_Name Test11 PMT PID 1100 V PID 1109 A PID 1108ISO639_language_code 2) Audio_type 0x00 A PID 1107 ISO639_descriptor: language_code 2) audio_ type 0x00 supplementary audio desc: “broadcast mixed” LCD 1 visible; SID 1200 S_Name Test12 PMT PID 1200 V PID 1209 A PID 1208 ISO639_language_code 2) Audio_type 0x00 A PID 1207 ISO639_descriptor: language_code 2) Audio_type 0x03 supplementary audio desc “receiver mixed) LCD 2 visible; Can be chosen depending of the distribution media.. 1) ON_id (Original_network_id) can be chosen in range 0x0001-0xfe00 (operational network) 2) Language must the same for all audio components and it can be one of the swe/fin/nor/ice/dan/smi/gle/iri/eng/und. Test procedure: The selected language for primary language code is referred as Lang1 below. The selected language for secondary language code is referred as Lang2 below. - Set the primary audio selection to Lang1 and secondary to Lang2. - Set audio type preference to “normal” audio. - Tune to Service 1 and verify that audio PID 1108, Lang1 0x00 audio type is selected. - Tune to Service 2 and verify that audio PID 1208, Lang1 0x00 audio type is selected. - Set audio type preference to “supplementary audio”. - Tune to Service 1 and verify that audio PID 1107, Lang2 0x00 audio type is selected. - Tune to Service 2 and verify that audio PID 1207, nar 0x03 audio type is mixed with audio PID 1208 audio and is audible. Expected result: Supplementary audio can be enabled and disabled from the menu. IRD supports both broadcast mixed and receiver mixed supplementary audio. IRD is able to select a correct audio track according to user preference setting. |
| Test result(s) |  |  |  |
| Conformity | OK Fault Major Minor, define fail reason in comments |  |  |
| Comments | If possible describe if fault can be fixed with software update: YES NO Describe more specific faults and/or other information |  |  |
| Date |  | Sign |  |

| Test Case | IRD Internal Reference Level |  |  |
| --- | --- | --- | --- |
| Section | NorDig Unified 6.12 |  |  |
| Requirement | The level for reference tones for transmission will be 18 dB below clipping level, in accordance with EBU Recommendation R.68 ”Alignment level in digital audio production equipment and in digital recorders” as recommended by ETSI TS 101 154. |  |  |
| IRD variants and capability | all IRDs |  |  |
| Test procedure | The IRD manufacturer shall describe the used test procedure and the used test setup. |  |  |
| Test result(s) | The IRD manufacturer shall describe the used test procedure and the used test setup. |  |  |
| Conformity | OK Fault Major Minor, define fail reason in comments |  |  |
| Comments | If possible describe if fault can be fixed with software update: YES NO Describe more specific faults and/or other information |  |  |
| Date |  | Sign |  |

| Test Case | Loudness levels - Audio Output Levels |  |  |
| --- | --- | --- | --- |
| Section | NorDig Unified 6.13.3 |  |  |
| Requirement | In order to match the loudness of audio streams in a downstream AV receiver, PCM streams at a target reference level of -23 dBFS shall (1) be reduced in level by 8dB before being output on S/PDIF or HDMI to that device. PCM streams at a target reference level of -31 dBFS shall not be increased in level prior to output. If an HDMI (or HDMI eARC) receviving device indicates that it supports multichannel PCM audio, then stereo or multichannel PCM should output be at a target reference level of -31 dBFS. If an HDMI (or HDMI eARC) receviving device indicates that it only supports Basic Audio (i.e. twochannel L-PCM), then the output should be leveled to a target reference level of -23 dBFS. For analog (stereo) outputs, the target reference level should be leveled to -23 dBFS (as measured on a digital signal). |  |  |
| IRD variants and capability | all IRDs |  |  |
| Test procedure | Purpose of test: To verify IRD audio output levels are according to requirement. Test procedure: Use a test stream with audio encoded in MPEG-1 Layer II at on a beforhand measured integrated level that is approximately -23 LUFS (integrated value, measured according to EBU R 128 or other close standard such as ITU-R BS.1770). Available input format; Measured Output on S/PDIF (Integrated value in LUFS); OK/NOK. When Stereo is selected (default) – The Target Reference Level is -23 dBFS; When Multichannel is selected – The Target Reference Level is -31 dBFS. MPEG1 layer II i.e. measured to be -23 LUFS; -23 LUFS (same as input); -31 LUFS (8 dB attenuation). AC-3 i.e. measured to be -23 LUFS with dialnorm level set to = -23 dBFS; -23 LUFS; -31 LUFS. E-AC-3 i.e. measured to be -23 LUFS with dialnorm level set to = -23 dBFS; -23 LUFS; -31 LUFS. HE AAC i.e. measured to be -23 LUFS with prog_ref_level set to = -23 dBFS; -23 LUFS; -31 LUFS. Available input format; Measured Output on HDMI* (Integrated value in LUFS); OK/NOK. When Stereo is selected (default) – The Target Reference Level is -23 dBFS; When Multichannel is selected – The Target Reference Level is -31 dBFS. MPEG1 layer II i.e. measured to be -23 LUFS; -23 LUFS (same as input); -31 LUFS (8 dB attenuation). AC-3 i.e. measured to be -23 LUFS with dialnorm level set to = -23 dBFS; -23 LUFS; -31 LUFS. E-AC-3 i.e. measured to be -23 LUFS with dialnorm level set to = -23 dBFS; -23 LUFS; -31 LUFS. HE AAC i.e. measured to be -23 LUFS with prog_ref_level set to = -23 dBFS; -23 LUFS; -31 LUFS. * An audio analyser with HDMI-input is not common. A HDMI-to-S/PDIF-converter might be of use or other type of equipment. Available input format; Measured Output on S/PDIF (Integrated value in LUFS); OK/NOK. When Stereo is selected (default) – The Target Reference Level is -23 dBFS; When Multichannel is selected – The Target Reference Level is -31 dBFS. MPEG1 layer II i.e. measured to be -23 LUFS; -23 LUFS (same as input); -31 LUFS (8 dB attenuation). AC-3 i.e. measured to be -23 LUFS with dialnorm level set to = -23 dBFS; -23 LUFS; -31 LUFS. E-AC-3 i.e. measured to be -23 LUFS with dialnorm level set to = -23 dBFS; -23 LUFS; -31 LUFS. HE AAC i.e. measured to be -23 LUFS with prog_ref_level set to = -23 dBFS; -23 LUFS; -31 LUFS. For analog (stereo) outputs, the target reference level should be leveled to -23 dBFS (as measured on a digital signal).- Make the same tests as “Stereo” as above. Expected result: IRD supports correct loudness levels. Make sure that the audio level drops when changing from “Stereo” mode to “Multichannel” mode and vice versa. |
| Test result(s) |  |  |  |
| Conformity | OK Fault Major Minor, define fail reason in comments |  |  |
| Comments | If possible describe if fault can be fixed with software update: YES NO Describe more specific faults and/or other information |  |  |
| Date |  | Sign |  |

| Test Case | AC-4: Requirements should this change to “NGA Requirements” to match 6.12.1 in the UR or stay as AC-4 Requirements as in 6.2.4 of the UR? |  |  |
| --- | --- | --- | --- |
| Section | NorDig Unified 6.2.4.1 |  |  |
| Requirement | NorDig HEVC IRD supporting AC-4 shall • decode AC-4 streams at all bit rates and sample rates listed in ETSI TS 103 190-2. • be capable of transcoding AC-4 bitstreams to E-AC-3 or AC-3 bitstreams according to TS 102 366. |  |  |
| IRD variants and capability | HEVC IRD |  |  |
| Test procedure | Purpose of test: To verify that the IRD supports decoding of AC-4 level 3 bitstreams at various configurations. This test is to be performed at the main output of the IRD. Equipment: Audio decoder MUX Exciter IRD TS Source The TS shall contain three services each with one AC-4 audio stream at native frame rate and one HE-AAC stream, both streams shall be signaled as Danish. The AC-4 audio stream in the three services shall be according to: - AC-4 stereo - AC-4 multichannel - AC-4 immersive Test procedure: - If the main output of the IRD is HDMI, connect the IRD to an audio receiver, e.g. Home Theater System with HDMI. - Verify that there are three services and that each have an HE-AAC and an AC-4 audio stream. - On the receiver menu, select service 1. - Verify that AC-4 stereo is decoded or transcoded at the main output of the IRD. - On the receiver menu, select service 2. - Verify that AC-4 multichannel is decoded or transcoded. - On the receiver menu, select service 3. - Verify that AC-4 immersive is decoded or transcoded. Expected result: IRD support for AC-4 input audio signal and reproduction at its main output. |  |  |
| Test result(s) | OK or NOK. Stereo. Multichannel 5.1. Immersive. |
| Conformity | OK Fault Major Minor, define fail reason in comments |  |  |
| Comments | If possible describe if fault can be fixed with software update: YES NO Describe more specific faults and/or other information |  |  |
| Date |  | Sign |  |

| Test Case | AC-4: Analogue audio output |  |  |
| --- | --- | --- | --- |
| Section | NorDig Unified 6.2.4.1, 6.8.1. |  |  |
| Requirement | The NorDig IRD with analogue outputs shall always have an audio signal present on the analogue outputs (to built-in loudspeakers (see note 1)) if any of the supported formats is received. If the NorDig IRD has analogue stereo output(s), it shall be capable of decoding and downmixing the supported audio formats for the analogue outputs. NorDig IRD’s that support AC-4 shall • decode AC-4 streams at all bit rates and sample rates listed in ETSI TS 103 190-2. |  |  |
| IRD variants and capability | HEVC IRD |  |  |
| Test procedure | Purpose of test: To verify that receiver IRD decodes AC-4 bitstream and audio output level can be adjusted. This test in only mandatory for STB or iDTV with SCART or analogue audio output interface. Equipment: Source MUX Exciter DVB receiver Monitor A transport stream containing a service with AC-4 multichannel audio. Test procedure: - Verify that service has a multichannel AC-4 audio available. - Verify that AC-4 multichannel is downmixed to stereo at analog audio output. Expected result: IRD supports AC-4 audio decoding and audio is downmixed to stereo in analog audio output(s). The audio output level can be adjusted. |  |  |
| Test result(s) |  |  |  |
| Conformity | OK Fault Major Minor, define fail reason in comments |  |  |
| Comments | If possible describe if fault can be fixed with software update: YES NO Describe more specific faults and/or other information |  |  |
| Date |  | Sign |  |

| Test Case | AC-4: required output formats for HDMI, HDMI ARC and HDMI eARC |  |  |
| --- | --- | --- | --- |
| Section | NorDig Unified 6.2.4.3 |  |  |
| Requirement | Where HDMI, HDMI ARC (or eARC) is implemented, NorDig HEVC IRDs shall be capable of providing the following formats on the HDMI output connector from an AC-4 bitstream (see chapter 16 for factory default settings): • AC-4 bitstream decoded (up to 5.1 channels) and re-encoded to E-AC-3 at a minimum bit rate of 640 kbps. • AC-4 bitstream decoded (up to 5.1 channels) and re-encoded to AC-3 at a fixed bit rate of 640 kbps. • Decoded and downmixed (if > 2 channels) to PCM stereo bitstream |  |  |
| IRD variants and capability | HEVC IRD with option for HDMI, or HDMI ARC, or HDMI eARC |  |  |
| Test procedure | Purpose of test: To verify that STB IRD decodes/transcodes AC-4 bitstreams and outputs the appropriate format via HDMI. To verify that iDTV IRD decodes/transcodes AC-4 bitstreams and outputs the appropriate format via HDMI ARC and (if supported) HDMI eARC. To verify that for AC-4 bitstreams decoded and downmixed (if >2 channels) to PCM stereo, the audio output level can be adjusted. This test is only relevant for STB IRDs with HDMI output and iDTV IRDs with HDMI ARC and/or HDMI eARC output. Equipment: TS source MUX Exciter STB IRD HDMI receiver Monitor HDMI ARC receiver HDMI eARC receiver iDTV IRD A transport stream with AC-4 multichannel or immersive audio. Test procedure: - Connect the IRD to an HDMI (ARC, eARC) receiver e.g. Home Theatre System with HDMI (ARC, eARC) input, that supports E-AC-3 or AC-3 decoding. - Verify that service has a multichannel or immersive AC-4 audio available. - In receiver menu, select stereo audio. - Verify that AC-4 multichannel or immersive is decoded and downmixed to PCM stereo bitstream for HDMI (ARC, eARC) output. - Verify the audio output level can be adjusted. - In receiver menu, select multichannel audio. - Verify that AC-4 multichannel or immersive is transcoded to E-AC-3 or AC-3 multichannel at HDMI (ARC, eARC) output depending on the capabilities of the HDMI (ARC, eARC) receiver. Expected result: IRD supports AC-4 audio decoding and downmixing to PCM stereo when stereo audio is selected. The main audio level can be adjusted. IRD supports AC-4 multichannel audio transcoding to E-AC-3 and AC-3 at its HDMI (ARC, eARC) output when multichannel audio is selected. |  |  |
| Test result(s) | HDMI (ARC, eARC) receiver; OK or NOK. Stereo; any. Multichannel; E-AC-3. Multichannel; AC-3. |
| Conformity | OK Fault Major Minor, define fail reason in comments |  |  |
| Comments | If possible describe if fault can be fixed with software update: YES NO Describe more specific faults and/or other information |  |  |
| Date |  | Sign |  |

| Test Case | AC-4: recommended output formats for HDMI and HDMI eARC only (optional) |  |  |
| --- | --- | --- | --- |
| Section | NorDig Unified 6.2.4.3 |  |  |
| Requirement | Where HDMI (or HDMI eARC) is implemented, NorDig HEVC IRDs should be capable of providing the following formats from an AC-4 bitstream: - Decoded to PCM audio bitstreams with metadata (e.g., MAT, HDMI 2.1). |  |  |
| IRD variants and capability | HEVC IRD with option for immersive output over HDMI or HDMI eARC |  |  |
| Test procedure | Purpose of test: To verify that STB IRD decodes an immersive or multichannel AC-4 bitstream to PCM audio bitstreams with metadata for HDMI output. To verify that iDTV IRD decodes an immersive or multichannel AC-4 bitstream to PCM audio bitstreams with metadata for HDMI eARC output. This test is only relevant for STB IRDs with HDMI output and iDTV IRDs with HDMI eARC output. Equipment: TS source MUX Exciter STB IRD HDMI receiver Monitor HDMI eARC receiver iDTV IRD A transport stream with immersive or multichannel AC-4 audio. Test procedure: - Connect the IRD to an HDMI (eARC) receiver e.g. Home Theatre System with HDMI (eARC) input, that supports PCM audio bitstreams with metadata. - Verify that service has immersive or multichannel AC-4 audio available. - Verify that immersive and multichannel AC-4 audio is decoded to PCM with metadata for HDMI (eARC) output. Expected result: IRD supports decoding of multichannel or immersive AC-4 audio to PCM with metadata for HDMI (eARC) output. |  |  |
| Test result(s) |  |  |  |
| Conformity | OK Fault Major Minor, define fail reason in comments |  |  |
| Comments | If possible describe if fault can be fixed with software update: YES NO Describe more specific faults and/or other information |  |  |
| Date |  | Sign |  |

| Test Case | AC-4: recommended output formats for HDMI ARC (optional) |  |  |
| --- | --- | --- | --- |
| Section | NorDig Unified 6.2.4.3 |  |  |
| Requirement | Where HDMI, HDMI ARC (or eARC) is implemented, NorDig HEVC IRDs should be capable of providing the following formats from an AC-4 bitstream: - AC-4 bitstream decoded (e.g. 5.1.2 channels) and re-encoded using E-AC-3 according to ETSI TS 102 366 in combination with the JOC extension to E-AC-3 according to ETSI TS 103 420. |  |  |
| IRD variants and capability | HEVC iDTV with option for immersive output over HDMI ARC |  |  |
| Test procedure | Purpose of test: To verify that iDTV IRD transcodes an immersive AC-4 bitstream to an E-AC-3 JOC bitstream for HDMI ARC. This test is only relevant for iDTV IRDs with HDMI ARC. Equipment: TS source MUX Exciter STB IRD HDMI receiver Monitor HDMI ARC receiver HDMI eARC receiver iDTV IRD A transport stream with immersive AC-4 audio. Test procedure: - Connect the IRD to an HDMI ARC, receiver e.g. Home Theatre System with HDMI ARC input, that supports E-AC-3 JOC decoding. - Verify that service has immersive AC-4 audio available. - Verify that immersive AC-4 audio is transcoded to E-AC-3 JOC for HDMI ARC output. Expected result: IRD supports transcoding of AC-4 immersive audio to E-AC-3 JOC for HDMI ARC output. |  |  |
| Test result(s) |  |  |  |
| Conformity | OK Fault Major Minor, define fail reason in comments |  |  |
| Comments | If possible describe if fault can be fixed with software update: YES NO Describe more specific faults and/or other information |  |  |
| Date |  | Sign |  |

| Test Case | AC-4: required output formats for S/PDIF |  |  |
| --- | --- | --- | --- |
| Section | NorDig Unified 6.2.4.3 |  |  |
| Requirement | The NorDig IRD supporting AC-4 and including an S/PDIF output shall be capable of providing the following formats on the S/PDIF connector from an AC-4 bitstream: • AC-4 bitstream transcoded to AC-3 bitstream • Decoded and downmixed (if > 2 channels) to PCM stereo bitstream |  |  |
| IRD variants and capability | HEVC IRD |  |  |
| Test procedure | Purpose of test: To verify that IRD decodes/transcodes AC-4 bitstreams. This test is only relevant for IRD with S/PDIF output. Equipment: MPEG-2 source MUX Exciter DVB receiver S/PDIF receiver Monitor A transport stream with AC-4 multichannel or immersive audio. Test procedure: - Connect the IRD to a S/PDIF receiver e.g. Home Theater System with S/PDIF. - Verify that service has a multichannel or immersive AC-4 audio available. - In receiver menu, select stereo audio. - Verify that AC-4 multichannel or immersive is decoded and downmixed to PCM stereo bitstream for S/PDIF output. - In receiver menu, select multichannel audio. - Verify that AC-4 multichannel or immersive is transcoded to AC-3 multichannel at the S/PDIF output. Expected result: IRD supports AC-4 audio decoding to PCM stereo bitstream when stereo audio is selected. IRD supports AC-4 transcode to AC-3 multichannel when multichannel audio is selected. |  |  |
| Test result(s) |  |  |  |
| Conformity | OK Fault Major Minor, define fail reason in comments |  |  |
| Comments | If possible describe if fault can be fixed with software update: YES NO Describe more specific faults and/or other information |  |  |
| Date |  | Sign |  |

| Test Case | Dialogue Enhancement |  |  |
| --- | --- | --- | --- |
| Section | NorDig Unified 6.10 |  |  |
| Requirement | Next Generation Audio enable Dialogue Enhancement which gives the possibility to adjust the relative level of the dialogue, to aid speech intelligibiltiy or to suit the preference of the user. See section 16.2.3 for the user preference settings that are related to Dialogue Enhancement. In additions for NGA capable NorDig HEVC IRD user shall be able to select storable user preferences (stored as persistent settings) for following audio related functions (see chapter 16.4 for factory default values): • Dialogue Enhancement (on/off), see section 6.10 AC-4 enable Dialogue Enhancement accessibility services by offering the possibility to adjust the relative level of the dialogue. In additions for NGA capable NorDig HEVC IRD user shall be able to select storable user preferences (stored as persistent settings) for following audio related functions (see chapter 16.4 for factory default values): - Dialogue Enhancement (on/off), see section 6.10 |  |  |
| IRD variants and capability | HEVC IRD |  |  |
| Test procedure | Purpose of test: To verify that the IRD supports Dialogue Enhancement Equipment: Audio decoder MUX Exciter IRD TS Source The TS shall contain a service using AC-4 which include typically a noise signal with Dialogue Enhancement meta data which gain modules the noise when applied. Test procedure: - Verify that service has an AC-4 audio available including dialogue enhancement meta data. - In IRD menu, select Dialogue Enhancement off. - Verify that AC-4 is decoded and static noise is played. - In IRD menu, select Dialogue Enhancement on. - Verify that AC-4 is decoded and that gain modulated noise is played. |  |  |
| Test result(s) |  |  |  |
| Conformity | OK Fault Major Minor, define fail reason in comments |  |  |
| Comments | If possible describe if fault can be fixed with software update: YES NO Describe more specific faults and/or other information |  |  |
| Date |  | Sign |  |

| Test Case | Automatic Audio PID/Stream prioritisation for NGA capable NorDig HEVC IRDs |  |  |
| --- | --- | --- | --- |
| Section | NorDig Unified 6.5.2 |  |  |
| Requirement | NGA capable NorDig HEVC IRD shall prioritise the selection of NGA/AC-4 audio PID/stream over other audio PIDs/streams using other audio codecs when: - NGA stream signalised a language that matches IRD user preference settings for primary audio language. (See section 6.5.4.1 how IRD shall detect language for PMT signalling), or - NGA stream does not signalise any language (in PMT), or - No NGA stream matches the user preference settings for primary audio language and no non-NGA stream matches the IRD user preference settings for primary and secondary audio language. Otherwise, the non-NGA audio PIDs/streams shall be selected in accordance with section 6.5.3 below. If NGA capable NorDig HEVC IRD finds more than one NGA stream that matches the primary audio language, or finds none of the NGA streams match the primary audio language the NGA stream with lowest PID shall be selected. |  |  |
| IRD variants and capability | HEVC IRD |  |  |
| Test procedure | Purpose of test: To verify that the IRD selects the right PID/stream based on IRD primary language setting. Equipment: Audio decoder MUX Exciter IRD TS Source A transport stream containing a service with multiple audio PID in different languages and audio codecs, as specified in the table below. Note that the content is that of the reference streams and might be replaced by the tester. PID; Codec; Language signaling; Content. 1st audio PID; HE-AAC; Swedish; Sine tone. 2nd audio PID; HE-AAC; Swedish Audio Description; Speech*. 3rd audio PID; AC-4; 5 Presentations: Finnish Swedish Swedish (SpS) Swedish (AD+SpS) Swedish (AD); Finnish: music Swedish: : Immersive channel check and speech*. 4th audio PID; HE-AAC; English; White noise. * Speech content is a narration in English indicating the selected language and audio type Test procedure: - Setup the system as indicated in the diagram - In the IRD user preference settings, set the primary audio language to Swedish - Verify the IRD selects the 3rd audio PID and plays back the content corresponding to Swedish language and not the 2nd PID. - In the IRD user preference settings, set the primary audio language to English - Verify the IRD selects the 4rd audio PID and plays back the content corresponding to English language. - In the IRD user preference settings, set the primary and secondary audio language to any languages other than Swedish, Finnish or English. - Verify the IRD selects the 3rd audio PID and plays back the content corresponding to Finnish language. Expected result: There are three verification steps in this test procedure: In step 3, the IRD prioritises the NGA PID because it matches the user preference for primary audio language. In step 5, the IRD prioritises the non-NGA PID because the NGA PIDs don’t match the user preference for primary audio language and the non-NGA PID does. In step 7, the IRD prioritises the NGA PID because neither the NGA nor the non-NGA PIDs match the user preference for primary audio language. |
| Test result(s) | Primary/secondary language preference; Decoded audio stream; OK / NOK. Swedish; NGApresentation corresponding to Swedish language. English; HE-AAC PID corresponding to English language. Any languages other than Swedish, Finnish or English; NGA presentation corresponding to Finnish language. |
| Conformity | OK Fault Major Minor, define fail reason in comments |  |  |
| Comments | If possible describe if fault can be fixed with software update: YES NO Describe more specific faults and/or other information |  |  |
| Date |  | Sign |  |

| Test Case | Automatic Audio PID/Stream prioritisation for NGA capable NorDig HEVC IRDs |  |  |
| --- | --- | --- | --- |
| Section | NorDig Unified 6.5.2 |  |  |
| Requirement | NGA capable NorDig HEVC IRD shall prioritise the selection of NGA/AC-4 audio PID/stream over other audio PIDs/streams using other audio codecs when: - NGA stream signalised a language that matches IRD user preference settings for primary audio language. (See section 6.5.4.1 how IRD shall detect language for PMT signalling), or - NGA stream does not signalise any language (in PMT), or - No NGA stream matches the user preference settings for primary audio language and no non-NGA stream matches the IRD user preference settings for primary and secondary audio language. Otherwise, the non-NGA audio PIDs/streams shall be selected in accordance with section 6.5.3 below. If the NGA capable NorDig HEVC IRD finds more than one NGA stream that matches the primary audio language or finds that none of the NGA streams match the primary audio language, the NGA stream with lowest PID shall be selected. |  |  |
| IRD variants and capability | HEVC IRD |  |  |
| Test procedure | Purpose of test: To verify that the IRD selects the right PID/stream when the NGA stream does not signal any language in PMT Equipment: Audio decoder MUX Exciter IRD TS Source A transport stream containing a service with multiple audio PID in different languages and audio codecs as specified in the table below. Note that the content is that of the reference streams and might be replaced by the tester. PID; Codec; Language signaling; Content. 1st audio PID; HE-AAC; Swedish; Sine tone. 2nd audio PID; AC-4; Finnish and Swedish No signaling in the PMT; Finnish: music Swedish: Immersive channel check and speech*. * Speech content is a narration in English indicating the selected language Test procedure: - Setup the system as indicated in the diagram - In the IRD user preference settings, set the primary audio language to Swedish - Verify the IRD selects the 2nd audio PID and plays back the content corresponding to Swedish language and not the 2nd PID. Expected result: The IRD prioritises the NGA stream when it does not signal any language in the PMT. |
| Test result(s) | Primary/secondary language preference; Decoded audio stream; OK / NOK. Swedish; NGA presentation corresponding to Swedish language. |
| Conformity | OK Fault Major Minor, define fail reason in comments |  |  |
| Comments | If possible describe if fault can be fixed with software update: YES NO Describe more specific faults and/or other information |  |  |
| Date |  | Sign |  |

| Test Case | Automatic Audio Prioritisation inside the NGA Audio PID/stream |  |  |
| --- | --- | --- | --- |
| Section | NorDig Unified 6.5.6 |  |  |
| Requirement | This section applies only to the NGA capable NorDig HEVC IRD. The NGA capable NorDig IRD shall prioritise the preselection in the NGA stream according to Table 6.4. IRD settings →; Audio Description OFF; Audio Description ON. Spoken Subtitles OFF; Spoken Subtitles ON; Spoken Subtitles OFF; Spoken Subtitles ON. IRD behaviour depending on above IRD settings and preselection properties. Priority; Priority with respect to best matching preselection; Priority with respect to best matching preselection. 1; AD off and SpS off Preselection matching primary audio language; AD off and SpS on Preselection matching primary audio language; AD on and SpS off Preselection matching primary audio language (1); AD on and SpS on Preselection matching primary audio language. 2; AD off Preselection matching primary audio language; SpS on Preselection matching primary audio language; AD on and SpS on Preselection matching primary audio language; AD on and SpS off Preselection matching primary audio language. 3; Preselection matching primary audio language; Preselection matching primary audio language; SpS off Preselection matching primary audio language; SpS on Preselection matching primary audio language. 4; AD off and SpS off Preselection matching secondary audio language; AD off and SpS on Preselection matching secondary audio language; Preselection matching primary audio language; Preselection matching primary audio language. 5; AD off Preselection matching secondary audio language; SpS on Preselection matching secondary audio language; AD on and SpS off Preselection matching secondary audio language settings; AD on and SpS on Preselection matching secondary audio language. 6; Preselection matching secondary audio language; Preselection matching secondary audio language; AD on and SpS on Preselection matching secondary audio language; AD on and SpS off Preselection matching secondary audio language. 7; (if no match) default Preselection; (if no match) default Preselection; SpS off Preselection matching secondary audio language; SpS on Preselection matching secondary audio language. 8; Preselection matching secondary audio language; Preselection matching secondary audio language. 9; (if no match) default Preselection; (if no match) default Preselection. Table 2.4 Audio Priority between NGA Preselections for NGA “Accesibility Services” with Audio description on/off and Spoken Subtitles on/off. A lower number refers to higher priority. Note 1: This selection requires the IRD to evaluate the audio preselection descriptor. |
| IRD variants and capability | HEVC IRD |  |  |
| Test procedure | Purpose of test: To verify that the IRD selects the right preselection based on IRD primary language setting. Equipment: Audio decoder MUX Exciter IRD TS Source A transport stream containing a service with multiple audio PID in different languages and audio codecs as specified in the table below. Note that the content is that of the reference streams and might be replaced by the tester. PID; Codec; Language signaling; Content. 1st audio PID; HE-AAC; Swedish; Sine tone. 2nd audio PID; HE-AAC; Swedish Audio Description; Speech*. 3rd audio PID; AC-4; 5 Presentations: Finnish Swedish Swedish (SpS) Swedish (AD+SpS) Swedish (AD); Finnish: music Swedish: Immersive channel check and speech*. * Speech content is a narration in English indicating the selected language and audio type The NGA PID contains multiple preselections in different languages, including Accessibility services. - Finnish preselection with music as content - Swedish preselection with Immersive channel check as content - Swedish (Audio Description) preselection with Immersive channel check and speech* as content - Swedish (Spoken Subtitles) preselection with Immersive channel check and speech* as content - Swedish (Audio Description and Spoken Subtitles) preselection with Immersive channel check and speech* as content * Speech content is a narration in English indicating the selected language and audio type Test procedure: - Setup the system as indicated in the diagram - In the IRD user preference settings, set the primary audio language to Swedish - Verify the IRD selects the second preselection and plays back the content corresponding to Swedish language. - In the IRD user preference settings, set the primary audio language to Icelandic and the secondary audio language to Finnish - Verify the IRD selects the third preselection and plays back the content corresponding to Finnish language. - In the IRD user preference settings, set the primary audio language to Swedish and enable accessibility settings for Audio Description. - Verify the IRD selects the fourth preselection and plays back the mix corresponding to Swedish language with Audio Description - In the IRD user preference settings, set the primary audio language to Swedish and enable accessibility settings for Spoken Subtitles. - Verify the IRD selects the fourth preselection and plays back the mix corresponding to Swedish language with Spoken Subtitles - In the IRD user preference settings, set the primary audio language to Swedish and enable accessibility settings for both Audio Description and Spoken Subtitles. - Verify the IRD selects the fourth preselection and plays back the mix corresponding to Swedish language with Audio Description and Spoken Subtitles Expected result: There are five verification steps in this procedure to verify the IRD implements audio prioritisation inside the NGA PID including language and Accessibility preferences. |
| Test result(s) | Primary/secondary language preference; Accessibility settings; Decoded audio stream content; OK / NOK. Primary: Swedish; AD off SpS off; NGA presentation corresponding to Swedish language. Primary: Icelandic Secondary: Finnish; AD off SpS off; NGA presentation corresponding to Finnish language. Primary: Swedish; AD on SpS off; NGA presentation corresponding to Swedish language with Audio Description. Primary: Swedish; AD off SpS on; NGA presentation corresponding to Swedish language with Spoken Subtitles. Primary: Swedish; AD on SpS on; NGA presentation corresponding to Swedish language with Audio Description and Spoken Subtitles. |
| Conformity | OK Fault Major Minor, define fail reason in comments |  |  |
| Comments | If possible describe if fault can be fixed with software update: YES NO Describe more specific faults and/or other information |  |  |
| Date |  | Sign |  |

| Test Case | Manual Audio PID/Stream selection for NorDig IRDs |  |  |
| --- | --- | --- | --- |
| Section | NorDig Unified 6.5.1, 13.5.1 and 13.5.2 |  |  |
| Requirement | From Section 6.5.1 In addition, the user shall be able to manually select between all audio streams and/or NGA Preselections (1) that are associated with the active service, these settings should be stored in the IRD´s memory separately for each service. Note 1: NGA Preselections only applicable for NGA capable NorDig HEVC IRDs. From Section 13.5.1 and 13.5.2 For services with multiple audio streams (PIDs), or in case of multiple NGA Preselections (1), the NorDig IRD shall be able to display information about available incoming audio streams / NGA Preselections (1), for the user to temporarily select one audio stream / NGA Preselection (1). The NorDig IRD shall be able to display information about different incoming subtitling pages within the selected subtitle stream/PID, for the user to be able to temporary select subtitling page or disable displaying subtitling. This task also covers Task 14:2: User Information about service components – Audio and Subtitling Note 1: Only applicable/mandatory for Nordig HEVC IRD. |  |  |
| IRD variants and capability | HEVC IRD |  |  |
| Test procedure | Purpose of test: To verify that IRD support selecting between available audio alternatives Equipment: Audio decoder MUX Exciter IRD TS Source A transport stream containing a service with multiple audio PID in different languages and audio codecs as specified in the table below. Note that the content is that of the reference streams and might be replaced by the tester. PID; Codec; Language signaling; Content. 1st audio PID; HE-AAC; Swedish; Sine tone. 2nd audio PID; HE-AAC; Swedish Audio Description; Speech*. 3rd audio PID; AC-4; 5 Presentations: Finnish Swedish Swedish (SpS) Swedish (AD+SpS) Swedish (AD); Finnish: music Swedish: Immersive channel check and speech*. 4th audio PID; HE-AAC; English; English: White noise. * Speech content is a narration in English indicating the selected language and audio type Test procedure: - Setup the system as indicated in the diagram - Verify that the IRD supports selecting between available audio altneratives. - Verify that upon slecting a PID/Streams or NGA preselection that the expected audio comes out, - NGA Preselections only applicable for NGA capable NorDig HEVC IRDs. Expected result: Non NGA capable IRD’s can select and decode - Swedish, - Swedish Audio Description, - English NGA capable IRD’s can select and decode - Swedish, - Swedish Audio Accessibility, - Finnish - English |
| Test result(s) | PID stream Selected; Decoded audio stream; OK / NOK. Swedish; NGA presentation corresponding to Swedish language. English; HE-AAC PID corresponding to English language. Finish; NGA presentation corresponding to Finnish language. |
| Conformity | OK Fault Major Minor, define fail reason in comments |  |  |
| Comments | If possible describe if fault can be fixed with software update: YES NO Describe more specific faults and/or other information |  |  |
| Date |  | Sign |  |

| Test Case | Manual selection of audio preselection based on information from the Audio Preselection Descriptor |  |  |
| --- | --- | --- | --- |
| Section | NorDig Unified 6.5.1, 6.5.6 and 12.6.11 |  |  |
| Requirement | From Section 6.5.1 In addition, the user shall be able to manually select between all audio streams and/or NGA Preselections (1) that are associated with the active service, these settings should be stored in the IRD´s memory separately for each service. Note 1: NGA Preselections only applicable for NGA capable NorDig HEVC IRDs. From Section 6.5.6 For manual selection, the information from the Audio Preselection Descriptor (APD) shall be used (1). The NGA audio stream(s) may contain additional or fewer preselections than those listed in the Audio Preselection Descriptor. Note 1: Optional for NorDig HEVC IRDs launched before 1 July 2020 From Section 12.6.11 The NGA capable NorDig HEVC IRD supporting NGA services shall support the Audio Preselection Descriptor. The Audio Preselection Descriptor may contain the following information for a list of preselections: - The language of the preselection - Indication if the preselection contains audio description - Indication if the preselection enables dialogue enhancement - Indication if the preselection contains spoken subtitles Indication if the preselection contains enables user interactivity - Indication for the preselection regarding a preferred reproduction channel configuration (stereo, multichannel, immersive, headphones) |  |  |
| IRD variants and capability | HEVC IRD |  |  |
| Test procedure | Purpose of test: To verify that IRD correctly parses the information from the Audio Preselection Descriptor and message_descriptor. Equipment: Audio decoder MUX Exciter IRD TS Source A transport stream containing an TV service with NGA audio, and the NGA has the following structure (4 presentations in AC4 TOC but only 2 presentations in APD): #; Audio Preselection Descriptor; Ac4_toc; Content. 1; preselection_id: 10 audio_rendering_indication: 3 audio_description: 0 spoken_subtitles: 0 dialogue_enhancement: 1 interactivity_enabled: 0 language_code_present: 1 text_label_present: 1 multi_stream_info_present: 0 future_extension: 0 ISO_639_language_code: (eng); presentation_id: 10 content_classifier: 000b (Complete Main) language: en; Immersive channel check. 2; Not present; presentation_id: 11 content_classifier: 000b (Complete Main) language: sv; Noise. 3; preselection_id: 12 audio_rendering_indication: 2 audio_description: 0 spoken_subtitles: 0 dialogue_enhancement: 1 interactivity_enabled: 0 language_code_present: 1 text_label_present: 1 multi_stream_info_present: 0 future_extension: 0 ISO_639_language_code: (eng); presentation_id: 12 content_classifier: 000b (Complete Main) language: en; Cinematic content. 4; Not present; presentation_id: 13 content_classifier: 010b (Visually Impaired) language: en; Immersive channel check + speech (narration for Audio Description). The message_descriptor in the SDT contains also two entries: message_id: 0 text: Standard message_id: 1 text: Artist commentary Test Procedure: - Setup the system as indicated in the diagram - Verify that the IRD supports selecting between all NGA preselections described in the audio preselection descriptor as per table above - Verify that the IRD displays the preselection names defined in the message descriptor Expected result: The IRD uses the information in the Audio Preselection Descriptor (APD) to identify the available preselections for manual selection. Thus it presents 2 audio preselections with text labels from the message_descriptor. |
| Test result(s) |  |  |  |
| Conformity | OK Fault Major Minor, define fail reason in comments |  |  |
| Comments | If possible describe if fault can be fixed with software update: YES NO Describe more specific faults and/or other information |  |  |
| Date |  | Sign |  |

| Test Case | Audio Prioritisation inside the NGA Audio PID/stream based on information from elementary stream |  |  |
| --- | --- | --- | --- |
| Section | NorDig Unified 6.5.6 |  |  |
| Requirement | Audio Prioritisation inside the NGA Audio PID/stream can be done based on the Audio Preselection Descriptor or based on the ac4_toc, as specified in ETSI TS 103 190–2 [98] in the elementary stream. The information from the elementary stream (ac4_toc) shall be used for audio prioritisation inside the NGA audio PID/stream based on the user preference settings (1). For manual selection, the information from the Audio Preselection Descriptor (APD) shall be used (1). The NGA audio stream(s) may contain additional or fewer preselections than those listed in the Audio Preselection Descriptor. If the presentation corresponding to a manually selected preselection from the Audio Preselection Descriptor is not found in the bitstream, then the IRD shall fall back to preference-based presentation selection based on IRD settings and signalling in the elementary stream as specified above. In case the presentation currently playing does not have corresponding information in the Audio Preselection Descriptor (APD), the information should be extracted from the elementary stream (ac4_toc) or, if that is not feasible, a default called “currently playing” or similar could be displayed. Similarly, when a presentation present in APD but not present in the elementary stream (ac4_toc) is manually selected by the user, the NorDig HEVC IRD applies preference-based presentation selection based on the elementary stream (ac4_toc), resulting in a selection different to the user’s. This selection should be reflected in the UI by extracting the information from the elementary stream (ac4_toc) or, if that is not feasible, a default called “currently playing” or similar could be displayed. Additionally, when a presentation present in APD but not present in the elementary stream (ac4_toc) is manually selected by the user, the NorDig HEVC IRD may inform the user that the selected audio presentation is not available in the current broadcast. It is expected that quasi-static APD signaling could result in an overrepresentation of presentations in the APD e.g. during advertisement breaks, where during the advert break there could be only one audio track available to all languages. |  |  |
| IRD variants and capability | HEVC IRD |  |  |
| Test procedure | Purpose of test: To verify that the audio prioritization logic uses the information in the AC-4 TOC when the audio_preselection_descriptor contains inaccurate information. Equipment: Audio decoder MUX Exciter IRD TS Source Two transport streams containing the same AC-4 elementary stream but different signaling in the PMT. The list of preselections in the audio_preselection_descriptor does not match the AC-4 TOC accurately. - The audio_preselection_descriptor contains only a subset of the preselections in the AC-4 TOC that doesn’t include presentation 11 (language 2, e.g. Swedish) - The audio_preselection_descriptor contains all of the preselections in the AC-4 TOC and one more that is not found in the bitstream. That is preselection_id 14: lauage 2, e.g. Swedish with Audio Description AC-4 TOC: #; AC-4_TOC; Content. 1; presentation_id: 10 content_classifier: 000b (Complete Main) languag 1, e.g. en (English); Immersive channel check. 2; presentation_id: 11 content_classifier: 000b (Complete Main) language 2, e.g. sv (Swedish); Noise. 3; presentation_id: 12 content_classifier: 000b (Complete Main) language 1, e.g. en (English); Cinematic content. 4; presentation_id: 13 content_classifier: 010b (Visually Impaired) language 1, en (English); Immersive channel check + speech (narration for Audio Description). Note that the content is that of the reference streams and might be replaced by the tester. Test procedure: Execute the following procedure for each of the two transport streams. - Setup the system as indicated in the diagram - Set Swedish as the preferred audio language and enable Audio Description - Play the transport stream - Verify that the IRD selects the second preselection (language 2, e.g. Swedish) and plays the corresponding content. Additionally, for the second transport stream: - If the IRD offers a list of available preselections including language 2, e.g. Swedish with Audio Description, verify that manually selecting it also results in the playback of the second preselection (language 2, e.g. Swedish). Expected result: The list of preselections in the AC-4 TOC is always accurate, whereas the Audio Preselection Descriptor (APD) might occassionally miss some preselections or include others that are no longer available. In step #4, the IRD uses the information from the AC-4 TOC for audio prioritisation based on user preferences. Particularly: - In the first transport stream, no Swedish preselection is signaled in the APD. However the IRD selects it, being the closest to the user preferences of Swedish with Audio Description (AD). - In the second transport stream, even if a Swedish preselection with AD is signaled in the APD, it’s not available. Hence the IRD selects the Swedish preselection without AD and not the English one. In step #5, the IRD falls back to preference-based presentation selection due to a manual selection of a preselection that is not found in the elementary stream. |
| Test result(s) | Test; OK / NOK. Transport Stream 1; APD missing preselections from AC-4 stream. Transport Stream 2; APD with extra preselections not present in AC-4 stream. Transport Stream 2; Manual selection falls back to preference-based. |
| Conformity | OK Fault Major Minor, define fail reason in comments |  |  |
| Comments | If possible describe if fault can be fixed with software update: YES NO Describe more specific faults and/or other information |  |  |
| Date |  | Sign |  |