# Test Plan for NorDig  - Test Cases

## Task 11: The System Software Update

System Software Update

### Test equipment summary

To configure the minimum test setup described in these test procedures, the test setup seen in Figure 3. Generic test setup for SSU / CI+ CAM test procedure can be used.

Frequencies are dependent of the broadcast system

IRD

System software source

Monitor

DVB-x  modulator

MPEG-2 MUX

TS source

TS source

f1 ≠ f2

f2

MPEG-2 MUX

DVB-x  modulator

Combiner

Figure 3. Generic test setup for SSU / CI+ CAM test procedures

Note! The needed equipment  may vary depending on the used system and can be purchased from many vendors even in a compact all-in-one unit. However, most of the tests can done using one general test setup.

The SSU / CI+ CAM service is carried within a transport stream. The TS must contain correct type of signaling information in order for the receiver to detect correct SSU / CI+ CAM service. The following sections describe minimum signaling information.

The NIT must contain:

- Linkage_descriptor 0x4A to DVB SSU service using linkage_type 0x09.

Example for linkage_descriptor for IRD system download located at Tsid/Onid 0x0456/0x22F1.

| Descriptor_tag | 0x4A |
| --- | --- |
| Transport_stream id | 0x0456 |
| Original_network_id | 0x22F1 |
| Service_id | 0x1194 |
| Linkage_type | 0x09 |
| OUI *) |  |
| selector_bytes |  |
| private_data_byte **) |  |

*) DVB OUI or manufacture specific OUI

**) The private_data_byte shall be used as specified in the DVB Data Download Specification; Part 1: Simple Profile. (ETSI TS 102 006 v1.3.1 )

The PMT must contain:

(The descriptor shall be placed in the component loop of the PSI PMT table.)

- Data_broadcast_id_descriptor

| Descriptor_tag | 0x66 |
| --- | --- |
| Data_broadcast_id | 0x000A |
| Id_sector_bytes *) |  |
| OUI **) |  |
| update_type ***) |  |
| update_versioning_flag |  |
| update_version |  |
| selector_bytes |  |
| private_data_bytes |  |

*) The id_sector_bytes shall be used as specified in the DVB Data Download Specification (ETSI TS 102 006)

**) The OUI value in the PMT shall match the OUI value in the NIT linkage to SSU descriptor.

***) Update_type, use value below specified for simple or enhanced profiles

| proprietary update solution (not allowed) | 0x0 |
| --- | --- |
| standard update carousel (i.e. without notification table) via broadcast | 0x1 |
| system software update carousel with notification table (UNT) both available via broadcast | 0x2 |
| system software update signalled via broadcast UNT, update available from the return channel | 0x03 |
| system software update signalled via broadcast UNT, update available from the Internet | 0x04 |
| Reserved for future use | 0x5 – 0xFF |

The UNT must contain:

Signalling of UNT is relevant in case of  the data_broadcast_id_descriptor parameter update_type is set to 0x02 in PMT.

The parameters for different descriptors in UNT are specified below:

| Descriptor_tag | 0x4B |
| --- | --- |
| table_id | 0x01 |
| OUI 1) |  |
| processing_order 2) |  |
| common_descriptor_loop 3) |  |
| target_descriptor_loop 4) |  |
| operational_descriptor_loop 5) |  |

1) The OUI value in the PMT shall match the OUI value in the NIT linkage to SSU descriptor.

2) Depending of the SSU OTA mechanism

3) Common_descriptor_loop carries information which is intended for descriptors which apply to all platform/target devices listed in target_descriptor_loop and operational_descriptor_loop

4) Target_descriptor_loop can contain descriptor

- target_serial_number_descriptor

5) operational_descriptor_loop can contain  descriptors

- scheduling_descriptor

Example for scheduling_descriptor

| descriptor_tag | 0x01 |
| --- | --- |
| start_date_time |  |
| end_date_time |  |
| final_availability | 0 |
| periodicity_flag |  |
| period_unit |  |
| duration_unit |  |
| estimated_cycle_time_unit |  |
| period |  |
| duration |  |
| estimated_cycle_time |  |

- update_descriptor

- ssu_location_descriptor

- message_descriptor

- ssu_subgroup_association_descriptor

- private_data_specifier_descriptor

See more details regarding settings in DVB Data Download Specification (ETSI TS 102 006)

NorDig T2-IRD SSU OTA in DVB-T2 system

- The SSU stream is broadcasted with parameters settings PLP_ID = 1, T2_System_Id = 1 and Cell_Id = 1 in a single PLP mode.

### Test cases

| Test Case | IRD System software update using DVB SSU simple profile |  |  |
| --- | --- | --- | --- |
| Section | NorDig Unified 10.1, 10.3 and 10.6 |  |  |
| Requirement | The NorDig IRD shall provide a software download mechanism that enables download of software modules, to add a new software module or replace an existing software module. The actual upgrade of NorDig IRD software shall be initiated by the user (by update user preference setting and/or by user interaction). The user shall be able to disable any automatic update in the user preference settings for SSU. In cases where the user is prompted to confirm an update, the user shall be able to confirm or to abort/postpone the update. The user should be able to control when the installation will be performed, like proceed immediately or perform update in standby. The user shall be able to choose the update approach for the IRD (tested in Task 11:4) Any notification to the user (pop-up message) related to SSU shall be displayed in the same language as the language setting of the IRD. If the system software update does not allow the normal utilization of the NorDig IRD, the user shall be warned by some means. For example, by displaying information if the display is not available,(tested in Task 11.4). The progress of the download shall be displayed by the NorDig IRD. If the user chooses to abort the system software upgrade, the NorDig IRD shall remind the user during next restart or shutdown of the IRD to upgrade the receiver if the new software is available over the broadcast channel. The IRD shall inform the user whether there is an IRD update or CIP-CAM update. The IRD manufacturer shall ensure that download of non-certified system-software is prevented. If the NorDig IRD System software is corrupt, the IRD manufacturer shall provide a backup mechanism, either on local storage or via download, which can make the IRD operational again. The NorDig IRD shall be implemented with a protection mechanism for the existing system software. It shall ensure that the existing software will not be corrupted in case the System Software Update (SSU) is interrupted before the new system software is fully downloaded. The NorDig IRD shall be provided with a mechanism ensuring that only newer software versions than the existing System Software are accepted. The IRD manufacturer shall ensure that there are not any compatibility issues if different System Software versions are broadcast via different operators. The NorDig IRD shall support the SSU simple profile using the signaling in NIT, BAT and PMT, in accordance with the DVB-SSU specification. The Linkage descriptor in the NIT table, for linking to the SSU service is defined in section 12.2.6 (The UNT is not used for this profile, see chapter 5 of ref ETSI TS 102 006 ). |  |  |
| IRD Profile(s) | Basic, IRD, FE |  |  |
| Test procedure | Purpose of test: To verify the IRD system software update process in broadcast channel using DVB SSU simple profile. The SSU end user functionality is not tested in this case. It is tested in Task 11:4 SSU end user functionality. This test is convenient to do parallel with Task 11:4 SSU end user functionality. Equipment: Test setup described in 2.11.1 Test equipment summary Software to upgrade 2 pcs IRDs with older software than the version to upgrade or older software to downgrade the IRD under testing. Selection of two different ONID for different networks corresponding country1 and country2 in the test procedure. Test procedure: - Connect and start up the instruments. - Configure three outgoing download streams from the system software source. The system software PID, PMT PID and SID may not conflict with each other. - Configure the multiplexer to transmit three outgoing system software download streams within TS on the frequency f1. The system software PID, PMT PID and SID may not conflict with each other. (Simulates three system software streams.) - Configure linkage_descriptor for the two system software download streams not to be suitable for the receiver under test. The linkage_descriptor parameters for the third download stream shall be suitable for the receiver under test. The third system software download stream shall be the last in order. - Fill in the test results which parameters are unmatching with the receiver under test. - For the frequency f2 configure linkage_descriptor to refer to the frequency f1 with suitable parameters for the over-the-air download. (Simulates frequency change). - Do a channel search and check that all the services are possible to receive. - Tune the receiver to a service on the frequency f1. - Fill in the test results if the IRD has a setting for automatic search and it is set to “auto search”. - Initiate the download. - Verify that the software is being updated. - Fill in the test protocol. - Download older software to the receiver in use or use other receiver. - Tune the receiver to a service on the frequency f2. - Initiate the download again. - Verify that the software is being updated. - Fill in the test protocol. - Downgrade older software to the receiver in use or use another receiver with older software as broadcasted software version. - Initiate the download again. - Plug out the electric cable to corrupt the downloading of the system software. - Plug in the electric cable. - Verify that the receiver is still usable. - Fill in the test protocol. Fill in extended information in the comments section if the receiver ends up in an error state. Error messages etc. - Initiate the download again. - Plug out the antenna cable. (Simulates RF disturbances). - Plug in the antenna cable. - Fill in the test protocol. Fill in extended information in the comments section if the receiver ends up in an error state. Error messages etc. - Downgrade the IRD to the original software, or use another sample of the IRD with the original software if available. - Verify which ONID values correspond country1 and country2 settings in IRD. - Make sure that the IRDs country setting is equal to country2. - Configure the TS from f1 carrying the SSU service so that the ONID is equal to country1. - Configure the TS from f2 so that the ONID is equal to country2. - Fill in the test protocol which ONID values are broadcasted in TS in f1 (country1) and f2 (country2). - Fill in the test protocol which country settings are selected in the IRD. - Control that the IRD will not download and install the SSU. - Fill in the test protocol. Expected result: Receiver performes a software update from DVB SSU simple profile OTA broadcast available stream. |  |  |
| Test result(s) | Unmatched SSU stream number; Unmatched parameter name. #1. #2. ONID value. ONID value in TS in f1 (country1). ONID value in TS in f2 (country2). Test points; OK or NOK. 9; Does IRD have a setting for automatic SSU search set by defaut to “auto search” ?. 10- 12; The correct system software on the frequency f1 is upgraded.. 13 – 17; The correct system software, when initiated on frequency f2, is upgraded.. 18 – 23; Receiver starts after electric break using old (working) software.. 24 – 27; The receiver clarifies the RF signal reception disturbancies under OTA download. It has current software.. 28 – 36; The Receiver does not download or install the SSU from other ONIDs than the ONID that matches the country setting in the IRD.. |
| Conformity | OK Fault Major Minor, define fail reason in comments |  |  |
| Comments | If possible describe if fault can be fixed with software update: YES NO Describe more specific faults and/or other information |  |  |
| Date |  | Sign |  |

| Test Case | IRD System software update using DVB SSU enhanced profile - scheduling |  |  |
| --- | --- | --- | --- |
| Section | NorDig Unified 10.1, 10.2, 10.3, 10.5 and 10.6 |  |  |
| Requirement | The NorDig IRD shall support the SSU UNT Enhanced profile using the signaling in NIT, BAT, PMT, and UNT, in accordance with the DVB-SSU specification. The Linkage descriptor in the NIT table, for linking to the SSU service is defined in section 12.2.6 [1]. The descriptors of the UNT Enhanced profile shall be as specified in Section 12.7 [1]. |  |  |
| IRD Profile(s) | Basic, IRD, FE |  |  |
| Test procedure | Purpose of test: To verify the IRD system software update process in broadcast channel using DVB SSU enhanced profile. The SSU end user functionality is not tested in this case. It is tested in Task 11:4 SSU end user functionality. This test is convenient to do parallel with Task 11:4 SSU end user functionality. Equipment: Test setup described in 2.11.1 Test equipment summary Software to upgrade suitable for SSU enhanced profile and for the IRD under test, scheduled in the future, with more than one schedule_descriptor in UNT. 2 pcs IRDs with older software than the version to upgrade or older software to downgrade the IRD under testing. Test procedure: Refer to Task 11:1 but use DVB SSU enhanced download stream instead. - Connect and start up the instruments. - Configure three outgoing download streams from the system software source. The system software PID, PMT PID and SID may not conflict with each other. - Configure the multiplexer to transmit three outgoing system software download streams within TS on the frequency f1. (Simulates three system software streams.) - Configure linkage_descriptor for the two system software download streams not to be suitable for the receiver under test. The linkage_descriptor parameters for the third download stream shall be suitable for the receiver under test. The third system software download stream shall be the last in order. - Fill in the test results which parameters are unmatching with the receiver under test. - Configure UNT inclusive scheduling_descriptor by creating schedule with three scheduling descriptors, one in the past and two in the future for the system download streams. - For the frequency f2 configure linkage_descriptor to refer to the frequency f1 with suitable parameters for the over-the-air download. (Simulates frequency change). - Fill in the test protocol if the IRD support automatic search and it is set to “automatic search” by default. - Turn off the IRD and wait until IRD is standby. - Turn on the IRD from standby. - A message shall be displayed telling that a new software is available at the time specified in the scheduling_descriptor. The IRD shall find the nearest available scheduled time. - Verify that both “OK to download ” and “EXIT to abort” options works by verifying that download occurs at scheduled time and that no download occur when user selects EXIT. - Verify that IRD doesn’t upgrade software at specified time and date. - Fill in the test protocol. - Turn off the IRD and wait until IRD is standby. - Turn on the IRD from standby. - A message shall be displayed telling that a new software is available at the time specified in the scheduling_descriptor. The IRD shall find the nearest available scheduled time. - Verify that both “OK to download ” and “EXIT to abort” options works by verifying that download occurs at scheduled time and that download occurs when user chooses “OK to download”. - Verify that IRD is upgraded. - Reset the IRD to its original sw. - Enhanced profile with software for the IRD under test, scheduled active now, with more than one schedule_descriptor in UNT. - Initiate a manual download from the menu. - A message shall be displayed telling that a new software is available right now. - Verify that both OK and EXIT works. - Reset the IRD - Perform 21,23,24 but initiate the download via a background/standby search. - Fill in the test test results. Expected result: Receiver performs a system software update from DVB SSU enhanced profile OTA broadcast for future available stream. |  |  |
| Test result(s) | Unmatched SSU stream number; Unmatched parameter name. #1. #2. ONID value. ONID value in TS in f1 (country1). ONID value in TS in f2 (country2). Test points; OK or NOK. 8; Does the IRD have a setting for automatic SSU search and it is set to “automatic search” by default?. 9-14; Does the IRD display a pop-up message telling the closest time and date for the available OTA ?. 9-14; Does the pop-up message have selection “OK to download” and “EXIT to abort” ?. 9-14; Selecting EXIT doesn’t upgrade the receiver at specified time and date.. 15-20; Does the IRD upgrade the future available software at available time and date when turned off and turned on ?. 21-26; Does the IRD upgrade when the new sw is available right now when searching, From menu and from standby. |
| Conformity | OK Fault Major Minor, define fail reason in comments |  |  |
| Comments | If possible describe if fault can be fixed with software update: YES NO Describe more specific faults and/or other information |  |  |
| Date |  | Sign |  |

| Test Case | IRD System software update using DVB SSU Notifications update type 0x4 |  |  |
| --- | --- | --- | --- |
| Section | NorDig Unified 10.1, 10.2, 10.3, 10.5, 10.6 and 12.7 |  |  |
| Requirement | The NorDig IRD shall support the SSU notifications update type 0x4 using UNT in accordance with the DVB-SSU specificatiom. Linkage descriptor in the NIT table, for linking to the SSU service is defined in section 12.2.6 [1]. The descriptors of the SSU UNT shall be as specified in Section 12.7 [1]. |  |  |
| IRD Profile(s) | Basic, IRD, FE |  |  |
| Test procedure | Purpose of test: To verify the IRD system software update process in broadcast channel using DVB SSU Notifications. A typical use case for SSU Notification is to reach all non-connected connectable IRDs via broadcast channel with information that new SSU software is available but the new SSU software size is too large to be distributed via broadcast channel and/or requires a higher bandwidth over broadcast channel than is available. Manufacturers are recommended to: - Pre-store SSU messages in their IRDs and messages should be in all available languages that the IRD supports, - Mainly using the message_index (in the enhanced_message_descriptor, see 12.7.10) to reference which pre-stored message to be displayed for the user, - Minimise the broadcast text (in the enhanced_message_descriptor and/or message descriptor, see 12.7.10). Equipment: Test setup described in 2.11.1 Test equipment summary Software/configuration information to enable the SSU Notification process in the IRD. Test procedure: - Configure three outgoing download streams from the system software source. The system software PID, PMT PID and SID may not conflict with each other. - Configure the multiplexer to transmit three outgoing system software download streams within TS on the frequency f1. The system software PID, PMT PID and SID may not conflict with each other. (Simulates three system software streams.) - Configure linkage_descriptor for the two system software download streams not to be suitable for the receiver under test. The linkage_descriptor parameters for the third download stream shall be suitable for the receiver under test. The third system software download stream shall be the last in order. - For the frequency f2 configure linkage_descriptor to refer to the frequency f1 with suitable parameters for the over-the-air download. (Simulates frequency change). - Tune the receiver to a service on the frequency f1. - Perform a manual software update search. - Verify that a notification is presented - Reset the IRD to its factory default settings. - Repeat step 1-5. - Perform an automatic search from standby. - Turn on the IRD from standby. - Verify that a notification is presented - Reset the IRD to its factory default settings. - Tune the receiver to a service on the frequency f2. - Perform a manual software update search. - Verify that a notification is presented - Reset the IRD to its factory default settings. - Repeat step 1-5. - Perform an automatic search from standby. - Turn on the IRD from standby. - Verify that a notification is presented - Fill in the test results. Expected result: The IRD performs an SSU Notification update. . |  |  |
| Test result(s) | Unmatched SSU stream number; Unmatched parameter name. #1. #2. ONID value. ONID value in TS in f1 (country1). ONID value in TS in f2 (country2). OK or NOK Test points 6-7; OK or NOK Test points 10-12; OK or NOK Test points 15-16; OK or NOK Test points 19-21. Verify that any Notification to the user (pop-up message) related to SSU shall be displayed in the same language as the language setting of the IRD.. Verify the Notification informs the user about where the new software is available(internet adress) and how to proceed with the update process. Verify the user have a choice to postpone/abort/reject the system software update Notification.. If the user didn’t choose to abort/ reject the notification or if the SW in the IRD was upgraded to the latest version, verify that the user is reminded with the notification after restart of the IRD. |
| Conformity | OK Fault Major Minor, define fail reason in comments |  |  |
| Comments | If possible describe if fault can be fixed with software update: YES NO Describe more specific faults and/or other information |  |  |
| Date |  | Sign |  |

| Test Case | - IRD System software update using over-the-network download |  |  |
| --- | --- | --- | --- |
| Section | NorDig Unified 10.1 and 10.3 |  |  |
| Requirement | The NorDig IRD shall provide a software download mechanism that enables download of software modules, to add a new software module or replace an existing software module. The upgrade of NorDig IRD software shall be initiated by the user (by update user preference setting and/or by user interaction). The user shall be able to choose the update approach for the IRD (see 10.2.1) and the user shall be able to disable any automatic update. See section 16 for the NorDig IRD requirements of user preference settings for SSU. In cases where the user is prompted to confirm an update, the user shall be able to confirm or to abort/postpone the update (for example with a Yes and No option). If the user selects to abort/postpone an available update or by other ways cancel an available update, the NorDig IRD shall remind the user as stated in section 10.1.6 The NorDig IRD shall be provided with a mechanism ensuring that only newer software versions than the existing System Software are accepted. A connected IRD receiving updates in this way shall still arbitrate between software versions available via the broadcast network and the IP-based broadband network to ensure that only newer versions are downloaded and installed (according with section 10.1.4.2). If the NorDig IRD System software is corrupt (due to normal operation of the IRD or due to updating the system software), the IRD manufacturer shall provide a backup mechanism, either on local storage or via download, which can make the IRD operational again. The NorDig IRD shall be implemented with a protection mechanism for the existing system software. It shall ensure that the existing software will not be corrupted in case the System Software Update (SSU) is interrupted before the new system software is fully downloaded. |  |  |
| IRD Profile(s) | Basic, IRD, FE |  |  |
| Test procedure | Purpose of test: To verify the IRD system software update process in broadcast channel using DVB SSU simple profile. The SSU end user functionality is not tested in this case. It is tested in Task 11:4 SSU end user functionality. Equipment: Test setup described in 2.11.1 Test equipment summary For the testing three IRD SSU software is needed, “SW v1“,“SW v2“ and “SW v3“, (“SW v1“ as starting point, “SW v2“ for testing upgrading and “SW v3“ for testing ˮnon-downgradingˮ). IRD manufacturer shall ensure a software upgrade (“SW v2”) is available at the internet. Internet connection which can be enabled / disabled during the test (e.g. LAN connection through a local switch or Wi-Fi through a local access point) Two approaches for the SSU testing, either via using: - one single IRD with possibility to upgrade and downgrade the SW by other means plus SSU sw images of “SW v1“ and “SW v3“, - alternative via using multiple IRDs. Two IRDs with software “SW v1“ and one IRD with software “SW v3“. Test procedure: - Make sure the IRD has “SW v1” installed. - Connect the IRD to live TV network. - Connect the IRD to internet. - Perform factory reset to the IRD and new installation. During installation, enable network connection. - Check quickly that services are found and decoded properly. - Access the navigator. Look for a menu option for SSU over the Internet. - Enable SSU download over the Internet. - Verify that the software is being updated. - Fill in the test protocol. - Look for software version information in the IRD navigator. - Verify that the software in IRD is successfully updated to “SW v2”. - Perform factory reset to the IRD and new installation. During installation, enable network connection on the IRD. - Enable SSU download over the Internet. - Verify that no software update occurs. - Upgrade the IRD software to “SW v3”. - Perform factory reset to the IRD and new installation. During installation, enable network connection on the IRD. - Enable SSU download over the Internet. - Verify that no software update occurs. - Downgrade IRD software to “SW v1” - Initiate the download again. - During the download, plug out the electric cable to corrupt the downloading of the system software. - Plug in the electric cable. - Verify that the receiver is still usable. - Fill in the test protocol. Fill in extended information in the comments section if the receiver ends up in an error state. Error messages etc. - Initiate the download again. - Cut the network connection temporarily by e.g. disabling the network access point uplink. - Re-enable the network connection. - Fill in the test protocol. Fill in extended information in the comments section if the receiver ends up in an error state. Error messages etc. Expected result: Receiver performes a software update over-the-network. |  |  |
| Test result(s) | Test points; OK or NOK. 1-11; IRD is able to upgrade the System Software over-the-network.. 12-18; IRD has a mechansim that ensures only newer software version than the existing System Software is accepted. 19-24; Receiver starts after electric break using old (working) software.. 25-28; The receiver clarifies the network disturbancies under OTN download. Download is either aborted or resumed after a short network break.. |
| Conformity | OK Fault Major Minor, define fail reason in comments |  |  |
| Comments | If possible describe if fault can be fixed with software update: YES NO Describe more specific faults and/or other information |  |  |
| Date |  | Sign |  |

| Test Case | - Task 11:4 SSU end user functionality |  |  |
| --- | --- | --- | --- |
| Section | NorDig Unified 10.1, 10.2, 10.5, 16.4 |  |  |
| Requirement | Any (pop-up) messaging to the user related to SSU shall be displayed in the same language as the language setting of the IRD. If the system software update does not allow the normal utilization of the NorDig IRD, the user shall be warned by some means. For example, by displaying information if the display is not available according to NorDig unified 2.6 sec 10.1.3. If the user selects to abort/postpone an available update or by other ways cancel an available update, the NorDig IRD shall remind the user. The user shall be able to choose the update approach for the IRD and the user shall be able to disable any automatic update. If the IRD supports Fully Automatic mode the user must be able to disable Fully Automatic mode Depending on if the Nordig IRD is connectable or Non connectable the delivery alternatives and the update approaches can vary according to Nordig Unified 2.6 Table 10.1. |  |  |
| IRD Profile(s) | Basic, IRD, FE |  |  |
| Test procedure | Purpose of test: To verify that the IRD has settings required for SSU end user functionality. This test is convenient to do parallel with Task 11:1 IRD System software update using DVB SSU simple profile Equipment: Test setup described in 2.11.1 Test equipment summary Software to upgrade 2 pcs IRDs with older software than the version to upgrade or older software to downgrade the IRD under testing. Test procedure: - Verify that the IRD supports one of the described upgrade delivery altenatives and approach alternatives specified in [1]. - Fill in the test results. - Verify that any notification to the user (pop-up message) related to SSU shall be displayed in the same language as the language setting of the IRD. - Verify that the user is informed about the upgrade process if the SSU does not allow the normal utilization of the NorDig IRD. - Verify the user can cancel the upgrade process - Verify that if the user selects to abort/postpone an available update or by other ways cancel an available update, the NorDig IRD shall remind the user. - Fill in the test results. Expected result: All the test result are OK, and the available upgrade processes works as expected. |  |  |
| Test result(s) | Delivery alternatives; Non connectable IRD; OK or NOK or N/A; Internet connectable IRD; OK or NOK or N/A. #D1; Mandatory; Mandatory to implement at least one of the alternatives #D1, #D2 or #D3.. #D2; n/a. #D3; Optional. #D4; n/a; Optional. #D5; Optional; Optional. Approach alternatives; Non connectable IRD; OK or NOK or N/A; Internet connectable IRD; OK or NOK or N/A. #A1; Mandatory to implement at least one of the alternatives #A1, #A2 or #A3.; Mandatory to implement at least one of the alternatives #A1, #A2, #A3 or #A4.. #A2. #A3. #A4; Optional. #A5; Optional; Optional. OK or NOK. IRD displays pop-up message with language selected in the settings. IRD informs user about upgrade process when applicable. User can cancel the ongoing upgrade process. User can abort/postpone available update. User is reminded about available update. User can disable Fully automatic mode.. |
| Conformity | OK Fault Major Minor, define fail reason in comments |  |  |
| Comments | If possible describe if fault can be fixed with software update: YES NO Describe more specific faults and/or other information |  |  |
| Date |  | Sign |  |

| Test Case | - Task 11:5 Common interface plus (CI+) CAM module system software update |  |  |
| --- | --- | --- | --- |
| Section | NorDig Unified 10.6 and Content Security Extensions to the Common Interface. Version 1.4 (CI Plus specification) |  |  |
| Requirement | In the case of IRDs with CIP- CAM, the IRD shall also support to update the System Software on the CIP-CAM when such software is broadcast. The IRDshall inform the user whether there is an IRD update or CIP-CAM update. |  |  |
| IRD Profile(s) | Basic, IDTV, FE |  |  |
| Test procedure | Purpose of test: To verify the CI+ CAM module can be updated. Equipment: Test setup described in 2.11.1 Test equipment summary Software to upgrade Test procedure: 1 Setup equipment for broadcast CI+ CAM update 2 Tune the IRD to the multiplex which is carrying the CI+ CAM update 3 Verify that CI+ CAM can be updated with CI+ IRD Expected result: All test results are OK. |  |  |
| Test result(s) |  |  |  |
| Conformity | OK Fault Major Minor, define fail reason in comments |  |  |
| Comments | If possible describe if fault can be fixed with software update: YES NO Describe more specific faults and/or other information |  |  |
| Date |  | Sign |  |

## Task 17: User Preferences

User preferences

| Test Case | Stored preferences |  |  |
| --- | --- | --- | --- |
| Section | NorDig Unified 16.1 and 16.2 |  |  |
| Requirement | The user shall be able to store preference settings in persistent memory. All user preference settings listed below shall remain when changing service and when re-starting the IRD. The following user preferences shall be implemented in the NorDig IRD, unless it is stated below as optional (should) requirement. (See section 16.4 for factory default values). |  |  |
| IRD Profile(s) | Basic, IRD, FE |  |  |
| Test procedure | Purpose of test: To verify stored user preferences Equipment: Test procedure: - Verify following user preferences: * Service list * Coutry settings * Talking menus/Text-to-speech * SSU settings * HbbTV settings * HDCP preferences (Optional) - * Video display preferences * Audio preferences * Primary and secondary audio language preferences * Primary and secondary subtitling language preferences * Service list preferences * Country settingspreferences * Audio transcoding preferences * HDMI preferences * Supplementary Audio settings Expected result: User preferences are as defined. |  |  |
| Test result(s) |  |  |  |
| Conformity | OK Fault Major Minor, define fail reason in comments |  |  |
| Comments | If possible describe if fault can be fixed with software update: YES NO Describe more specific faults and/or other information |  |  |
| Date |  | Sign |  |

| Test Case | Reset to factory mode |  |  |
| --- | --- | --- | --- |
| Section | NorDig Unified 16.4 |  |  |
| Requirement | The IRD shall provide a function to reset all parameters to factory mode, thus removing all service lists, user preferences, etc. After reset, the IRD shall enter installation state. |  |  |
| IRD Profile(s) | Basic, IRD, FE |  |  |
| Test procedure | Purpose of test: To verify the reset function. Equipment: Test procedure: - Locate the reset to factory mode function in the IRD and initiate it. - Check that the service list and user preferences have been deleted. - Enter the navigator and check that the factory mode meets the requirements. - Fill in the test protocol. Expected result: The IRD has the user preference settings according the requirements. |  |  |
| Test result(s) | Test protocol Requirement; Value; NOK or OK. RF input DC power supply source for satellite front-end:; on. RF input DC power supply source for terrestrial front-end:; off (1). RF-output preset channel:; Channel 43 (PAL-G) (2). RF bypass gain; Disabled. Menu language:; equal to country settings. Audio (normal); On. Primary audio language:; equal to country settings. Audio format setting:; Stereo. Audio; hard of hearing/hearing impaired:; Off. Subtitling (normal):; on. Primary subtitling language:; as country settings. Subtitling; hard of hearing/hearing impaired:; off. HbbTV Interactivity; On. HDMI Audio output; Automatic using EDID information. HDMI Video output; Automatic using EDID information. HDCP; ON or as specified by the relevant network/ CA operator see section 9.11.4 (1). SSU – mode; Semi-automatic (if supported, otherwise a mode that enables automatic search). PVR recording priority, SD vs HD; HD. Talking menus (Text-to-Speech), optional; Off. |
| Conformity | OK Fault Major Minor, define fail reason in comments |  |  |
| Comments | If possible describe if fault can be fixed with software update: YES NO Describe more specific faults and/or other information |  |  |
| Date |  | Sign |  |