# Audio

<started re-numbering section 6 Audio sub-sections to match IRD spec>

< draft, not final>

Following text is only during drafting and will be removed before final NorDig RoO specification

DRAFTING GUIDELINES / Explanation from the editors related to DRAFT versions:

This NorDig RoO spec. for v3.2.2 draft document is based on the official NorDig RoO v3.2.1.

Yellow highlight marking marks changes in text compared to NorDig RoO v3.2.1.

  - New modified text: without strikethrough marks new additional text,

  - Removed text: with strikethrough marks old text proposed to be removed

- Green marked text: highlighting text that under extra scrutiny during this update (not yet agreed).

- Blue marked text: comments or other raw text that will be removed before final version.

- Grey marked text: refers to text that not are relevant to this review/update.

Guide: To improve version handling and readability, old text from NorDig RoO v3.2.1 that is proposed to be deleted in future v3.2.2 should not be removed from draft version. 
Use instead strikethrough and yellow highlighted marking. Microsoft Word function “Track Changes”, will be used in addition to highlight changes, BUT from one draft version to another draft sometimes all “Track Changes” are Accepted to easier read changes in updates of proposals during our work.

When drafting a proposal, cross-references should be manually set and same for proposing correction, i.e. yellow mark and manual reference value. NorDig editor will update cross-references when preparing final draft.

## General

This section includes several aspects regarding the set-up up of audio parameters within broadcast television and radio services, such as:

- The quality of audio at different encoding bit rates and indications for the selection of appropriate bitrate for different audio quality.

- A brief note regarding subjective test method for audio quality.

- A recommended set-up of audio parameters at the encoding headend.

- Recommendations regarding the handling of multichannel audio in the production, encoding and decoding domains.

- Commentary to assist the understanding of employing the correct audio metadata for loudness and down-mixing.

- Detail regarding the coding artefacts of audio which may occur when re-encoding audio for onward distribution.

<Text below proposed to be moved here from 6.2 (old 6.3) to match IRD spec>

NorDig has defined four audio encoding formats:

- MPEG-1 Layer II, which refers to MPEG-1 Layer II up to stereo (2.0) channel encoding, see section 6.2.1.

- E-AC-3, which refers to E-AC-3 streams (including AC-3) up to 5.1 multi-channel encoding, see section 6.2.2.

- HE-AAC, which refers to MPEG-4 HE AAC Level 4 (including AAC-LC) up to 5.1 multi-channel encoding, see section 6.2.3.

- AC-4, which refers to AC-4 as defined in ETSI TS 103 190-2 [98] for multi-channel, immersive and personalised audio, see section 6.2.4.

Note: Some NorDig broadcasters have aligned the delivery of their MPEG-4 based services to include HE-AAC, E-AC-3 or AC3 stream types only, this is optional within NorDig and MPEG 4 services may legally be transmitted with MPEG-1 Layer II encoding format.

HEVC services shall always use NGA, i.e. AC-4 audio. To serve legacy HEVC IRDs without support for NGA, legacy stream types (e.g. MPEG-1 Layer II, HE-AAC, AC-3, E-AC-3) may be broadcasted in addition to NGA.

Summery NorDig IRD “Profiles” (variants and capabilities) – Audio Decoding (informative)

Audio decoders comply with the DVB Implementation Guidelines for the use of MPEG-2 Systems, Video and Audio in satellite, cable and terrestrial Broadcasting Applications ETSI TS 101 154 [26].

Within the NorDig network, where there is no single operator responsible for the acceptance of the Integrated Receiver Decoder, the NorDig Basic IRD supports as a minimum:

- MPEG-1 Layer II

- E-AC-3 (AC-3)

- HE-AAC.

Within the NorDig network, where there is a single operator or regulator responsible for specifying and accepting the functionality of the IRD and for ensuring that the minimum requirements are met. The operator or regulator may specify one of following minimum audio decoding format alternatives for the NorDig Basic IRD to support (as a minimum) for that specific network:

- MPEG-1 Layer II, E-AC-3 and HE-AAC audio decoding, or

- MPEG-1 Layer II, E-AC-3 audio decoding, or

- MPEG-1 Layer II, HE-AAC audio decoding.

NorDig HEVC IRD (launched after 1st of July 2020) additionally supports AC-4 audio decoding, for all other NorDig IRDs, AC-4 decoding is optional.

NorDig HbbTV IRDs (launched after 1st of July 2020) support at least all mandatory features and requirements of HbbTV v2.0.2 (ETSI TS 102 796, v.1.5.1 [27]). Support for HbbTV is optional with NorDig Basic IRDs and for STBs according to the NorDig HEVC profile, but it’s mandatory for iDTVs according to the Nordig HEVC profile.

Note: As NorDig HEVC IRDs have to support E-AC-3 and AC-4, the conditional requirement from HbbTV v2.0.2 is fulfilled and all Nordig HEVC IRD also have to support these audio codecs from broadband delivery.

The following audio modes may be employed in the NorDig region.

### Audio User Preference Settings within NorDig IRD

No RoO specification.

### Audio terminology

NorDig has defined four properties for the audio, which here refer to the information of the audio that is signalled in the PMT for the audio stream and that the IRD uses to select the appropriate audio stream/PID to decode:

- Audio type: normal (main/undefined) or supplementary/accessibility (visual impaired/audio description, spoken subtitling etc) type of audio.

- Audio language: corresponds to the audio language signalled in the PMT for the audio stream.

- Audio format: mono, stereo, multichannel or (NGA) immersive audio format.

- Audio stream type: the audio codec (MPEG-1 L.II, AC-3, E-AC-3, HE-AAC or AC-4) for the audio stream.

Normal audio - the term “Normal” within this specification refers to audio streams that are:

- intended for the majority of users or users that are not interested in any supplementary audio.

Supplementary Audio for Accessibility Services

- Supplementary audio (SA) (1) refers here to audio for accessibility services (2) and refers to the following experiences:

  - Audio Description (AD): audio that includes narration describing the action of the scene and is targeted at users with visual or cognitive impairments.

  - Spoken subtitling (SS): audio that includes a spoken rendition of the broadcasted subtitle and is targeted at users with visual or cognitive impairments.

  - Dialogue Enhancement (DE): functionality that provides improved speech intelligibility. It is targeted at users with hearing impairments but can also serve as improvement for listening in noisy environments.

Audio Formats

- Mono audio is defined as an audio stream with a single channel of audio. (Note the old legacy case of dual mono mode is deprecated, for such use cases two separate (mono/stereo) audio streams shall instead be used).

- Stereo audio is defined as an audio stream with two channels of audio with loudspeakers positioned left and right of the center of the screen.

- Multichannel Audio, Immersive Audio is defined as an audio stream with more than two channels of audio with all loudspeaker positions in one plane.

- Immersive Audio is defined as an audio streams that contain more than 5.1 audio channels or exceed the horizontal plane are considered Immersive Audio is defined as audio containing additional height elements (i.e. more than 5.1 audio channels). The height elements may be carried either as additional audio channels, sound field representation, audio objects or a combination of these.

NGA Audio service (NGA)

- NGA refers to audio streams using NGA (e.g. AC-4) audio codec.

- The NGA Audio services support delivery of audio content from mono, stereo, and 5.1 channel-based audio sources, as well as Immersive Audio from Channel-based, Object-based or Scene-based audio sources. Additionally, NGA systems efficiently offer services such as multi-language support, accessibility services, personalization and interactivity.

Non-NGA Audio service

- Non-NGA refers here to audio streams using other audio codecs than NGA (e.g. MPEG-1 L.II, AC-3, E-AC-3 or HE-AAC).

NGA Preselections:

- NGA Preselections: alternative audio versions are made available to the user for 'User Personalization'. The NGA Preselections can share some of the Audio Elements/substreams contained within a single NGA stream, as these are combined in the audio decoder of the IRD receiver to create the final audio mix.

Note 1: The term supplementary audio may in other contexts also be used other purposes, like commentary audio etc, but NorDig section 6.11 focus upon the audio accessibility cases.
Note 2: In ETSI specifications, NGA refers to preselecions for supplementary audio.

### Methode of subjective audio quality assesssment

When encoding linear PCM audio into compressed audio, employing standard encoding schemes artefacts may be added to the original audio. These encoding artefacts may be audible to normal human hearing, to evaluate the effect of these artefacts there are several methods employed to subjectively assess the fidelity of the decoded audio when compared to the original baseband PCM audio. The resulting figures obtained by these methods indicate the subjective qualities of the audio encoding and decoding process.

ITU-R BS.1116 and BS.1284
One of the primary methods employed in subjective audio quality assessment is International Telecommunication Union Recommendation BS.1116-3. This method is primarily intended for the evaluation of audio content where the audio artefacts is judged to be minor or slightly impaired and are set out below in Table 6.1.

There is also the closely related International Telecommunication Union Recommendation BS.1284-1 which is intended for listening tests that are not as stringent as BS.1116-1, and the grading scales are accompanied with a perceived description.

| Impairment | Grade | Quality (only in ITU-R BS.1284-1) |
| --- | --- | --- |
| Imperceptible | 5.0 | 5 Excellent |
| Perceptible, but not annoying | 4.0 | 4 Good |
| Slightly annoying | 3.0 | 3 Fair |
| Annoying | 2.0 | 2 Poor |
| Very annoying | 1.0 | 1 Bad |

Table 6.1: Grading scale employed by ITU-R BS.1116-1 (originally ITU-R BS.1284-1).

The methodology employed in the subjective listening process is based on the ABX method, in this method, the listener is presented with three audio excerpt signals in sequence. Excerpt A is always the reference signal, excerpt B is a secondary reference and excerpt X is the excerpt under evaluation. The listener then has to decide if X sounds the same as either A or B and grade the audio quality of X according to the grading scale.

Typically, therefore signal A may be uncompressed baseband audio, signal B may be of poor or over compressed quality and signal X a proposed sampling and encoding bitrate to be assessed prior to use.

ITU-R BS.1534 (MUSHRA)

Another method employed in more recent subjective quality assessments is International Telecommunication Union Recommendation BS.1534-3. This method mirrors many aspects of Recommendation ITU-R BS.1116 and uses the same grading scale as is used for the evaluation of picture quality (i.e. Recommendation ITU-R BT.500). This method called “Multi Stimulus test with Hidden Reference and Anchor (MUSHRA)” is suitable for evaluation of intermediate audio quality and gives accurate and reliable results.

In the MUSHRA test method, the subject can switch at will between the reference signal and any of the systems under test, typically using a computer-controlled replay system. The subject is presented with a sequence of trials. In each trial the subject is presented with the reference version, the low and mid anchor, as well as all versions of the test signal processed by the systems under test. For example, if a test contains 8 audio systems, then the subject is allowed to switch near instantaneously between the 11 test signals and the open reference (1 reference + 8 test systems  1 hidden reference  1 hidden low anchor + 1 hidden mid anchor).

| 100 |  |  |  |
| --- | --- | --- | --- |
|  |  |  | Excellent |
| 80 |  |  |  |
|  |  |  | Good |
| 60 |  |  |  |
|  |  |  | Fair |
| 40 |  |  |  |
|  |  |  | Poor |
| 20 |  |  |  |
|  |  |  | Bad |
| 0 |  |  |  |

The assessors are required to score the stimuli according to the continuous quality scale (CQS). The CQS consists of identical graphical scales which are divided into five equal intervals with the adjectives as given in Fig. 6.1 from top to bottom.

Figure 6.1: Grading scale employed by ITU-R BS.1534.

In the MUSHRA test method, a high-quality reference signal is used and the systems under test are expected to introduce significant impairments. MUSHRA is to be used for assessment of intermediate quality audio systems. If MUSHRA is used with appropriate content, it is ideal that listener scores should range between 20 - 80 MUSHRA points. If scores for the majority of test conditions fall in the range of 80 - 100 it may be true that the results of the test are invalid.

Compared to Recommendation ITU-R BS.1116, the MUSHRA method has the advantage of displaying many stimuli at the same time so that the subject is able to carry out any comparison between them directly.

Recommended practices for evaluation design
Significant effort has to be made to ensure that the audio content employed for the evaluation of encoder and decoder chains is relevant and sensitive. Typical audio content that has historically been employed are certain specific music and speech excerpts along with that of audience applause, it is also important and necessary to include audio content that has been recorded with different microphone set-up techniques and/or the use of special audio imagery.

With the increasing use of modern low bit rate encoder’s audio content which intrinsically employs material that is based around high frequencies is of particular interest within the evaluation process. Encoder buffer clearance rates may also be revealed by the use of audio content with repeated transients which occur at higher frequency rates.

Within the evaluation process is the employment of expert listeners or so called “golden ears” rather than amateur or inexpert listeners is required, research has found that the results obtained from inexpert listeners may be skewed and show much larger differences among the answers than those obtained from expert listeners. Viz. inexpert listeners do not know what to listen for and therefore tend to allocate higher points than expert listeners do; inexpert listeners do not appear to hear as much difference between audio content excerpts as the expert listeners can and therefore cannot differentiate with the same ease and confidence.

## Audio Encoding

### MPEG-1 Layer II

#### MPEG-1 Layer II: Encoding

It shall be encoded according to ISO 11172-3 [49] and constrained according to ETSI TS 101 154 [26], section 6.1.

For MPEG-1 Layer II audio input, broadcasters shall ensure that MPEG-1 Layer II audio stream has an equivalent loudness level of -23 LUFS, as this is assumed by NorDig IRDs.

MPEG-1 Layer II: Recommendations / Requirements on Audio Handling
The following MPEG-1 Layer II modes may be employed:

Monaural, Joint Stereo, Stereo

Joint Stereo mode should be used with care only. At bitrate up to 160 kbps, it can increase encoding efficiency. At 192 kbps and above it usually better to use the stereo mode.

The following sampling frequencies may be employed:

48 kHz, 44.1 kHz, 32 kHz

The recommended sampling frequency is 48 kHz.

The following bit rates for coded audio may be employed:

64 kbit/s, 96 kbit/s, 128 kbit/s, 160 kbit/s, 192 kbit/s,
224 kbit/s, 256 kbit/s, 320 kbit/s, 384 kbit/s.

Recommended Audio Bitrates for good to excellent audio quality:

|  | Mono | Stereo |
| --- | --- | --- |
| MPEG-1 Layer II | 96 – 128 kbps | 192 - 256 kbps |

Table 6.2.

For MPEG-1 Layer II, it is recommended to use constant bit rate for the audio encoding for a format (mono or stereo), unless the network/operator is sure that all IRDs on the market support variable bit rate for these audio codecs. When changing audio format (between mono and stereo) it is often acceptable to change bit rate.

#### MPEG-1 Layer II:  NorDig IRD Audio Output

No RoO specification.

### E-AC-3 and AC-3

#### E-AC-3 and AC-3: Encoding

It shall be encoded according to ETSI TS 102 366 [33] and constrained according to ETSI TS 101 154 [26] section 6.2.

AC-3 and E-AC-3 encoders shall employ a sampling frequency of 48 kHz.

E-AC-3 and AC-3 is based on a constant bitrate (CBR) encoding scheme.

Recommended Audio Bitrates for good to excellent audio quality:

|  | Stereo |  | Multichannel 5.1 |  |
| --- | --- | --- | --- | --- |
|  | Good | Excellent | Good | Excellent |
| AC-3 |  | 256 kbps | 384 kbps | 448 kbps |
| E-AC-3 |  | 192 kbps | 256 kbps | 448 kbps |

Table 6.3.

For AC-3, it is recommended to use constant bit rate for the audio encoding for a format (mono, stereo or multi-channel), unless the network/operator is sure that all IRDs on the market support variable bit rate for this audio codec. For E-AC-3 NorDig IRDs should theoretically support variable bit rate but should be used with care. When changing audio format (between for example stereo and multi-channel) it is often acceptable to change bit rate.

#### E-AC-3 and AC-3: Metadata

With AC-3 and E-AC-3, the loudness reference level is transmitted in the bitstream’s parameter called dialnorm. The value of this data field shall represent the measured loudness in LUFS according to ITU-R BS.1770-4 [76] and EBU R 128 [72].

Dynamic Range Compression gain values are generated by the encoder based on the compression profile selected by the broadcaster or encoder operator. (1)

Downmix gains can be configured by the broadcaster or encoder operator.

Note 1: When AC-3 or E-AC-3 encoders are not fed with metadata, they will always add default metadata.

Note 2: In addition to the compression profile, the encoder takes care that DRC gains are suitable to prevent clipping in the audio decoder when audio has to be reproduced at a higher output level than encoded or when a multichannel audio representation has to be down-mixed.

Typical ”Dolby” Audio Metadata Parameters

The table below describes typical values for “Dolby” audio metadata that are often used and can be of help for a broadcaster to configure an audio encoder.

| Metadata Parameter | Professional | Consumer | Typical Value | Comment |
| --- | --- | --- | --- | --- |
| Program Configuration | X |  | --- | Mostly for Dolby E |
| Program Description Text | X |  | --- | Could be language, etc. |
| Dialogue Level |  | X | -23 | Recommended value for loudness alignment to EBU R 128 productions |
| Channel Mode |  | X | 3/2 or 2/0 | Alternates depending on content. “X/Y” where X is the number of front channels and Y is the number of surround channels |
| LFE Channel |  | X | Enabled | Typically enabled for 5.1 (needs three or more channels to be enabled) |
| Bitstream Mode |  | X | Complete Main |  |
| Line Mode Compression |  | X | Music Light | Suits most content types. |
| RF Mode Compression |  | X | Music Light | Suits most content types. |
| RF Overmodulation Protection |  | X | Disabled | Should normally be disabled |
| Center Downmix Level |  | X | 0.707 (-3 dB) | de facto commonly used for 5.1 content |
| Surround Downmix Level |  | X | 0.707 (-3 dB) | de facto commonly used for 5.1 content |
| Dolby Surround Mode |  | X | Not Indicated | Helps to not change user selected setting in consumers´ Dolby Surround decoders |
| Audio Production Information |  | X | No |  |
| Mix Level |  | X | --- |  |
| Room Type |  | X | Not Indicated |  |
| Copyright Bit |  | X | No |  |
| Original Bitstream |  | X | No |  |
| Preferred Stereo Downmix |  | X | Lo/Ro Preferred | Useful for music material. Most consumer decoders override to Lt/Rt downmix anyway. |
| Lt/Rt Center Downmix Level (Extended BSI) |  | X | 0.707 (-3 dB) | de facto commonly used for 5.1 content |
| Lt/Rt Surround Downmix Level (Extended BSI) |  | X | 0.707 (-3 dB) | de facto commonly used for 5.1 content |
| Lo/Ro Center Downmix Level (Extended BSI) |  | X | 0.707 (-3 dB) | de facto commonly used for 5.1 content |
| Lo/Ro Surround Downmix Level (Extended BSI) |  | X | 0.707 (-3 dB) | de facto commonly used for 5.1 content |
| Dolby Surround EX Mode |  | X | Not Indicated | Helps to not change user selected setting in consumers´ home cinema decoders |
| A/D Converter Type |  | X | Standard |  |
| DC Filter | X |  | Enabled | Could be disabled only if you are sure that there is no DC in the signal |
| Lowpass Filter | X |  | Enabled | Prevents aliasing |
| LFE Lowpass Filter | X |  | Enabled | Should only be disabled if the LFE signal has no signal above 120 Hz |
| Surround 3 dB Attenuation | X |  | Disabled | TV sound is normally produced in mixing rooms with all five channels at the same sound pressure level |
| Surround Phase Shift | X |  | Disabled | Enabled can disturb 5.1 listening to music productions. |

Table 6.4.

Professional: These parameters are either carried in a Dolby E bitstream or often statically set in the AC-3, E-AC-3, or AC-4 encoder.

Consumer: The consumer´s AC-3, E-AC-3 or AC-4 decoder uses these parameters to create the best possible audio program possible on each consumers playback system, ranging from a small TV set with built-in loudspeakers or a complete home cinema surround sound system.
The metadata parameters above are typically used by a broadcaster also for HE-AAC/LC-AAC encoding but are normally handled and converted into to similar parameters inside that encoder.
It is quite normal to use two or more sets of standardized metadata sets for different types of content such as “stereo”, “5.1 multichannel”, “film/drama”, “news”, etcetera, but can of course be set specifically for each program.

Further details on AC-3 and E-AC-3 metadata are described in ”Dolby Metadata Guide” [105].

#### E-AC-3 and AC-3:  NorDig IRD Audio Output

No RoO specification.

### MPEG 4 HE AAC

#### HE-AAC:  Encoding

It shall be encoded as described in ISO 14496-3 [xx] and constrained according to ETSI TS 101 154 [26] sections 6.4, 6.5 and Annex C.5.

The following channel mode may be employed with MPEG-4 HE AAC:

Mono, stereo, multi-channel up to 5.1

Encoders compliant to the MPEG-4 HE AAC family shall employ a sampling frequency of 48 kHz.

MPEG-4 audio bitstreams shall comply to either of the following profiles:

- AAC Profile

- High Efficiency AAC profile

The recommended Profile to use is the High Efficiency AAC Profile.

For each of these profiles, bitstreams shall not exceed Level 4 restrictions. In case of the High Efficiency AAC Profile, the downsampled SBR mode shall not be utilized, i.e. the sampling frequency of the AAC-LC Core shall be restricted to 24 kHz.

Informative note:

The AAC Profile is a subset of the High Efficiency AAC Profile. The AAC Profile utilizes the AAC-LC (AAC low complexity) coding tool only, while the HE AAC profile adds the SBR (spectral bandwidth replication) tool to enhance the coding efficiency by a parametric representation of the high frequency bands. For 2-channel stereo emissions the coding efficiency is further increased by utilizing the PS (parametric stereo) tool as available with the High Efficiency AAC v2 profile.

Within all profiles, level 2 restrictions allow for 2 channel stereo transmissions, while level 4 constraints additionally permit for multi-channel up to 5.1 audio.

Support for the High Efficiency AAC v2 profile is not mandated by the NorDig IRD specification. Therefore 2-channel stereophonic reproduction from streams utilizing the parametric stereo tool may not be supported by all IRDs. Unless it is guaranteed that all IRDs used in the network support it, the usage of the parametric stereo tool should be avoided.

MPEG-4 audio bitstreams shall be encapsulated in the LATM/LOAS bitstream format.

The MPEG-4 AAC and HE-AAC codecs does not require fixed bitrate levels (as MPEG-1 Layer II); bitrate figures below are approximations only (several MPEG TV Encoders on market often re-use bitrate levels from MPEG-1 Layer II into MPEG-4 AAC bitrate settings).

Recommended Audio Bitrates for good to excellent audio quality:

|  | Mono | Stereo |  | Multichannel 5.1 |  |
| --- | --- | --- | --- | --- | --- |
|  | Good | Good | Excellent | Good | Excellent |
| AAC Profile | 64 kbps | 128 kbps | 160 - 192 kbps | 256 kbps | 320 kbps |
| High Efficiency AAC Profile | 32 kbps | (64) – 96 kpbs | --- | (160) - 192 kbps | --- |

Table 6.5.

Note: NorDig has the intention to specify slightly different bitrates for Normal audio and supplementary audio streams in the future in the NorDig Rules of Operation.

Note: Operators may want to pay attention to possible high peak bitrates even in constant bitrate mode: The MPEG-4 specification requires a minimum buffer size of 6144 bits per audio channel per frame. Given the “AAC profile” (aka “AAC low complexity”) with its frame size of 1024 samples, this equals to 6 bits per audio sample.

#### HE-AAC:  Metadata

The MPEG-4 HE AAC audio stream should contain appropriate metadata to control the loudness, the dynamic range and (if applicable) the downmixing process in the decoder:

- program_reference_level: The value of this parameter, if present, shall match the value of the measured audio loudness in LUFS according to EBU R128 [72] (see section 6.1.4). If this parameter is not present, broadcasters shall ensure that the audio stream has an equivalent loudness level of -23 LUFS.

- dyn_rng_ctl and dyn_rng_sgn: These two data fields specify the “light compression” gain factors.

- compression_value: This data field specifies the “heavy compression” gain factor.

- matrix_mixdown_idx or preferably center_mix_level and surround_mix_level: gain factors to control the mixdown process

- drc_presentation_mode: This value specifies which DRC processing scheme (light or heavy compression) the decoder has to apply in the downmixing process to prevent any overloads.
For use in the Nordig region, this value shall be set to “mode 1”.

The encoded bitstreams shall have either sufficient headroom and/or dynamic range control values as specified by the drc_presentation_mode to prevent any overload when IRDs perform downmixing.

The requirements for drc_presentation_mode_1 imply that (considering the worst-case scenario):

- Both dynamic range control data (dyn_rng_ctl/dyn_rng_sgn and compression_value) shall be transmitted.

- DRC gains values in dyn_rng_ctl/dyn_rng_sgn, when applied in the decoder without scaling, it shall avoid any highly undesired overload for levelling towards a target level of -31 dB and downmixing to a stereo representation.

- DRC gains values in compression_value, when applied in the decoder, it shall avoid any highly undesired overload for levelling towards a target level of -23 dB and downmixing to a mono representation.

If no metadata are present in the transmitted audio stream, the NorDig IRD assumes the following default values:

- Program_reference_level: -23 dBFS

- No DRC gains for both light and heavy compression

- Downmix gains -3dB for both center and surround channels (if applicable by channel mode)

- Drc_presentation_mode: Mode 1 (use heavy compression for downmixes from 5.1 to stereo)

#### HE-AAC:  NorDig IRD Audio Output

No RoO specification.

### AC-4

#### AC-4: Encoding

General information

AC-4 as specified in ETSI TS 103 190-2 [98] is the only NGA codec selected for the use in NorDig and support therefore is mandatory for all NorDig HEVC IRDs launched after 1st of July 2020.

All NorDig HEVC IRDs launched after 1st of July 2020 are required to support AC-4 as specified in ETSI TS 103 190-2 [98].

In the context of AC-4, Audio Preselections are implemented as ‘AC-4 presentations’, and Audio Programme Components are implemented by ‘substream groups’.<AP Stephan S check wording>

AC-4 offers the possibility to carry many different audio programme components/substreams inside one stream/PID, and to combine these components/substreams into complete experiences in the IRD. Therefore, receiver mixing capabilities are fundamental to every AC-4 decoder. < AP Stephan S - align with similar text from IRD spec; “An NGA/AC-4 stream may consist of multiple substreams (such as Music & Effects, dialogue/multiple dialogues for different languages, supplementary audio etc), and mixing capabilities of different substreams into an AC-4 presentation are fundamental to every AC-4 decoder.”>

The intended typical use-case for AC-4 is that one service has only one AC-4 elementary stream/PID and that AC-4 stream includes multiple substreams for all languages, types and formats for that service. As a minimum, all experiences/preselections targeting one language (e.g. different types and/or output formats) shall be included in one single NGA stream as IRDs are not required to consider any other stream (neither coded in NGA nor legacy audio format) for presentation selection.

The AC-4 elementary streams/PIDs each carry a table of contents (TOC) which lists the different experiences (called presentations) that can be derived.

AC-4 presentations aggregate substream groups. In a presentation, substream groups take on a specific role (such as M&E, dialogue, accessibility, etc.). Substream groups, in turn, are aggregations of substreams.

It is important to note that there is a many-to-many relationship between these concepts. This provides flexibility to reuse substream groups and substreams many times over.

Further details on AC-4 audio are informatively described in Annex C.

Encoding constraints
AC-4 audio streams shall be encoded according to ETSI TS 103 190-2 [98] and shall be constrained according to TS 101 154 [26], clause 6.7. This implies that the bitstream_version field shall be set to the value 2 and the presentation_version field shall be set to the value 1. Additionally, AC-4 audio streams shall contain at least one “Normal Audio” presentation, which md_compat field is less than or equal to three. The number of presentations in an AC-4 audio stream shall be 64 or less.

AC-4 should be encoded using the average bitrate (ABR) encoding scheme. This encoding mode meets the requirements of the buffer models used by the constant bitrate (CBR) transmission channel of the MPEG-2 Transport Stream while it also preserves the audio quality under all characteristics of the encoded audio signal.

The raw AC-4 frames shall be encapsulated in the AC-4 Sync Frame format.

AC-4 encoder shall employ a sampling frequency of 48 kHz.

Recommended Audio Bitrates for good to excellent audio quality:

| Stereo | Multichannel 5.1 | NGA Immersive 5.1.4 | NGA Immersive 7.1.4 plus 4 objects | Dialogue or Accessibility tracks (mono) |
| --- | --- | --- | --- | --- |
| 48 - 64 kbit/s (Good- Excellent) | 96 - 144 kbit/s | 192 - 288 kbit/s | 320 - 512 kbit/s | 32 - 48 kbit/s |

Table 6.6.

AC-4 NorDig IRDs support variable bit rate. It is recommended to use the encoder in average bit rate mode which produces streams that adhere to the AC-4 buffer model (1).

AC-4 audio streams should be encoded at the same frame rate as the accompanying video stream. I-frames should be generated at the same interval and position corresponding to their decoding time stamp.

In case of HFR video, ideally the base frame rates are used that correspond to the video frame rates, i.e. video to audio frame rates result in an integer fraction. If higher audio frame rates are required for flexible segmenting of the audio stream, then the “efficient high frame rate” (EHFR) mode should be used.

Multiple Presentations
If the AC-4 elementary stream contains more than one presentation, the following rules shall apply:

- Each presentation shall contain a unique presentation_id

- The presentations shall be sorted in the AC-4 TOC in order of their preference

- The presentation_ids shall be in ascending order

Within one language, there shall be the default main only presentation first before all other presentations with the same language. If there are one or more presentations with audio description and spoken subtitles and one or more presentations with audio description only for the same language, then the default presentation with audio description and spoken subtitles shall precede the default presentation with audio description only.

Presentations in the elementary stream can be grouped either by language (i.e. first all presentations with language A, then all presentations with language B, a.s.o.) or by type (i.e. first all complete main only presentations, followed by all presentations with audio description, a.s.o.).

Presentations with the default language shall be listed before presentations with other languages of the same type.

If there are more than one presentation of the same language and audio type in the stream, then the default presentation shall be listed as the first presentation of all presentations with the same language and audio type. Additionally, SI signalling should include text labels to distinguish them (see section 6.4.4 “Signalling for NGA: Preselections” for further details).

All audio services targeting the same language shall be encoded as AC-4 presentations in one single AC-4 audio stream in order to be selected automatically in the receiver based on the user’s preferences. See section 6.4.4 (Signalling for NGA presentation) and section 6.5 (NGA prioritisation) for further information.

Multiple languages

If the AC-4 elementary stream contains presentations targeting multiple languages, then the ‘complete main’ or ‘dialogue’ substream group info shall include language information in BCP-47 format by providing at least the primary language subtag info in every frame of each presentation that has a language associated. If a presentation does not have an associated language, e.g. an ambience only presentation, then it shall either not use the language signalling or set the language in the substream_group info of the CM substream to ‘zxx’ (=”No linguistic content”).

The default presentation of every stream shall be tagged with the target broadcast region language tag, even if the audio remains in the original language but visual subtitles are available for that target language. This approach ensures that the IRD configured to default always selects the default presentation. E.g. if an English movie is transmitted to a Swedish audience with Swedish subtitles, the default presentation sets the language to Swedish (the target language). An additional optional presentation may also set the language to English while referring to the same main audio component.

Note that the AC-4 encoder might set dedicated language tags (in the range of reserved use of qaa to qtz) to substream groups used for accessibility services (Audio Description or Spoken Subtitles). Those tags are only used by the decoder and have no means to external components.

Note 1: In average bitrate mode (ABR), the maximum frame size is limited to 3,072,000bits/s dived by the frame rate in fps and does not exceed 16,384 bytes.

#### AC-4: Metadata

Loudness
With AC-4, the loudness reference level is transmitted as dialnorm in the bitstream’s parameter called dialnorm_bits separately for each presentation. The value of this data field shall represent the measured loudness in LUFS according to ITU-R BS.1770-4 [76] and EBU R 128 [72] for the corresponding presentation.

DRC
AC-4 defines multiple DRC decoder modes, each corresponding to a DRC profile and a reference output level. The AC-4 encoder applies and sends all predefined DRC decoder modes. In case neither a specific profile is assigned nor available from input metadata, the AC-4 encoder will apply a default profile value. The following table provides an overview on relevant standard DRC decoder modes, their targeted output level ranges, their mapping from E-AC-3 DRC modes and default profiles:

| DRC Decoder Mode | Output Level Range | E-AC-3 DRC Profile | Default Profile |
| --- | --- | --- | --- |
| Home Theater | -31 to -27 LUFS | Line Mode | Film Light |
| Flat Panel TV | -26 to -17 LUFS | RF Mode | Film Standard |

Table 6.7.

Note that AC-4 supports additional DRC Decoder Modes targeting portable devices.

Downmix metadata
For decoding channel-based content, when the number of output speakers are less than the number of encoded audio channels, downmixing is required to render the complete audio program.

AC-4 decoders downmix audio according to a standard set of downmix rules. Operators should specify various downmix metadata parameters to control the downmixing process in the decoder.

These parameters include:

- preferred downmix method

- stereo mixdown gains

- LFE downmix

- Downmix gains for channel-based immersive audio

Channel-based Immersive Presentations provide the option to carry custom downmix gains for lower order channel-based immersive output or 5.1 output. Rendering to stereo is always performed as a subsequent downmix from 5.1 using Stereo Downmix Gains

Additionally, the “Previous mix type” metadata is informational metadata used to indicate whether the signal was upmixed or downmixed prior to encoding and if so, what type of algorithm therefore was used.

Operators should consult their encoder provide for detailed guidance on possible downmix metadata.

Ducking metadata
Presentations that consist of a compositional mix include mixing metadata that define the gains for mixing the various components, including M&E and Dialogue mixing and Main-Associated mixing.

For mixing of associated audio, the Dolby AC-4 encoder can generate scaling factors to be applied on the normal audio component based on its built-in ducking algorithm.

Mixing a Dialogue component to a M&E component is equivalent to the use case of Dialogue Enhancement.

Dialogue Enhancement
Metadata to control the Dialogue Enhancement operation should be carefully configured by the broadcasters/operators.
A more detailed explanation of the operating modes of Dialogue Enhancement with AC-4 is described in Appendix C.

#### AC-4:  NorDig IRD Audio Output

No RoO specification.

## Stereo Audio

See section 6.1.2 for definitions of Stereo Audio.

## Multichannel Audio

See section 6.1.2 for definitions of Multichannel Audio.

## Immersive Audio

See section 6.1.2 for definitions of Immersive Audio.

## Audio Prioritisation within the NorDig IRD and signalling

### General audio stream selection (informative)

The NorDig IRD is required to always decode and output audio if the selected service has an audio stream (irrespective of type, language, format or codec used). For television services with more than one audio stream, the NorDig IRD is required to prioritise and select the preferred audio stream to decode and present an output according to IRD’s user preference settings.

NorDig IRDs is required to read the information that is signalled in the PMT (descriptors) for the audio stream to determind the property of the audio content (type, language, format and stream type) before selecting the appropriate audio stream/PID to decode as described in below sections. See 6.1.2 for definition of the audio properties (type, language, format and stream type).

### Audio Prioritisation for NGA capable NorDig HEVC IRD (informative)

The NGA capable NorDig HEVC IRD is required to prioritise the selection of NGA/AC-4 audio PID/stream over other audio PIDs/streams using other audio codecs when:

- NGA stream signalises a language that matches IRD user preference settings for primary audio language. (See section 6.4.4 on language signalling in PMT), or

- NGA stream does not signalise any language (in PMT), or

- No NGA stream matches the user preference settings for primary audio language and no non-NGA stream matches the IRD user preference settings for primary and secondary audio language.

Otherwise, the non-NGA audio PIDs/streams shall be selected in accordance with the information given below.

If the NGA capable NorDig HEVC IRD finds more than one NGA stream that matches the primary audio language or finds that none of the NGA streams match the primary audio language, the NGA stream with lowest PID is selected.

The selection among available presentation/preselections inside the AC-4 stream, is in accordance with section 6.5.3 6.6.6.

### Audio PID/Stream Prioritisation for Legacy / non-NGA streams within the NorDig IRD (informative)

The NorDig IRD is required to select the preferred audio stream to decode and present an output according to IRD’s user preference settings as given in the following table.

| IRD settings → |  | “Normal” audio mode |  | Supplementary audio mode |  |
| --- | --- | --- | --- | --- | --- |
|  |  | stereo mode (factory default) | multichannel mode | stereo mode | multichannel mode |
| IRD behavior depending on above IRD settings |  |  |  |  |  |
| Property of priority for audio | Priority | Order of priority |  |  |  |
| Audio type | 1 (highest) | 1.1 normal 1.2 supplementary audio | 1.1 normal 1.2 Supplementary audio | 1.1 supplementary audio 1.2 normal | 1.1 supplementary audio 1.2 normal |
| Language | 2 | 2.1 audio match primay audio language settings 2.2 audio match secondary audio language settings 2.3 (if no match) normal audio 2.4 (if no normal audio) any audio | 2.1 audio match primary audio language settings 2.2 audio match secondary audio language settings 2.3 (if no match) normal audio 2.4 (if no normal audio) any audio | 2.1 supplementary audio match primay audio language settings 2.2 supplementary audio match secondary audio language settings 2.3 (if no match) normal audio 2.4 (if no normal audio) any audio | 2.1 supplementary audio match primay audio language settings 2.2 supplementary audio match secondary audio language settings 2.3 (if no match) normal audio 2.4 (if no normal audio) any audio |
| Audio format | 3 | 3.1 stereo 3.2 Multichannel 3.3 mono | 3.1 Multichannel 3.2 stereo 3.3 mono | 3.1 stereo 3.2 Multichannel 3.3 mono | 3.1 Multichannel 3.2 stereo 3.3 mono |
| Stream type | 4 (lowest) | 4.1 MPEG-1 Layer II 4.2 HE-AAC 4.3 E-AC-3 4.4 AC-3 | 4.1 HE-AAC 4.2 E-AC-3 4.3 AC-3 4.4 MPEG-1 Layer II | 4.1 MPEG-1 Layer II 4.2 HE-AAC 4.3 E-AC-3 4.4 AC-3 | 4.1 HE-AAC 4.2 E-AC-3 4.3 AC-3 4.4 MPEG-1 Layer II |

Table 6.13: Audio Priority between incoming audio streams where a lower number refers to higher priority.

The IRD uses information from the PMT in order to select the appropriate audio stream or PID according to the user receiver preferences (Audio type, Audio Language, Audio format, Stream type) if several audio streams or PID’s are available. After selecting audio PID to be decoded, the IRD uses the audio metadata (PES/ES header or bitstream) within the audio stream for the decoding process. And this audio metadata (PES/ES header or bitstream) is normally more dynamic, (such as changing audio format between stereo and multichannel, downmix parameters, etc.). This means that the IRD does not use the EPG/EIT data for selecting the audio stream or PID, nor for the actual decoding. The information within the EIT/(EPG) regarding the audio is only intended for presentation to the viewer, e.g. EPG information. The values in the PMT should be more quasi static and describing the maximum use case (e.g. multichannel if the audio stream is dynamically changing between stereo and multichannel).

<Only IRD related, proposal is to move to IRD spec or remove it> It is deemed important importance that the NorDig IRDreceiver behaves in an elegant manner that it does not change audio stream type depending upon dependent up on how the content itself may have been produced.

<Only IRD related, proposal is to move to IRD spec or remove it>The intention is that the NorDig IRD adheres to an audio stream type in a semi-constant way, after the IRD has selected which audio stream type to decode and the NorDig IRD begins to decode the audio stream, only then does the audio decoder read and applies the Channel Mode audio metadata (e.g. for AC-3) to the appropriate outputs (can be stereo output to the TV-loudspeakers and multichannel bitstream to the digital audio output). It is therefore defined that the NorDig IRD must read the AAC_type field in the AAC_descriptor for AAC audio, and the number of channels flags in the AC-3 descriptor and Enhanced AC-3 descriptor for AC-3 and E-AC-3.

#### Audio PID/stream Prioritisation within the NorDig IRD, Type (Normal or supplementary) (informative)

No RoO specification.

#### Audio PID/stream Prioritisation within the NorDig IRD, Languages (informative)

No RoO specification.

#### Audio Prioritisation within the NorDig IRD, Format (multichannel or stereo) (informative)

The intention for the setting “Stereo” and “Multichannel” is to enable the user to set up the IRD to select the appropriate audio stream to suit their preference. Some broadcasters employ only stereo audio, others use stereo and multichannel and some use stereo audio and multichannel audio that rotates between the two depending upon the content (viz. from stereo up to 5.1). More combinations with Audio Description (AD) or Spoken Subtitle audio stream content may also be present.

### Audio related signalling considerations

NorDig IRDs is required to use the signalling information in the PMT to determine the audio property (language, audio type, audio format and stream type), before selecting audio PID/stream to decode.

The audio properties in the PMT can be signalled in multiple descriptors. There might be cases for some services that the broadcast signals different audio property value in the different descriptors for same audio stream/PID (for example in ISO639 as in supplementary audio descriptor might not signal the same audio language code). NorDig IRDs is therefor required to priorities between the information in the descriptors as shown in tables below (non-NGA IRDs and NGA IRDs), for more information see same section in NorDig Unified IRD specification.

| Audio Property | Supplementary audio descriptor | AAC descriptor, Enhanced AC-3 descriptor, AC-3 descriptor | ISO 639 descriptor | PMT stream type | if no signalling/descriptor |
| --- | --- | --- | --- | --- | --- |
| Audio Language | 1st | - | 2nd | - | - |
| Audio type | 1st | 2nd | 3rd |  | Normal |
| Audio format | - | 1st | - | - | stereo |
| Stream type | - | 2nd | - | 1st | - |

Table X.X NorDig IRD not supporting NGA/AC-4 expected priority inside each audio property between different signalling for audio streams and right column the IRD’s interpretation if no signalling available for an audio stream.

| Audio Property | Audio Preselection Descriptor (*if supported*) | Supplementary audio descriptor | AAC descriptor, Enhanced AC-3 descriptor, AC-3 descriptor, AC-4 descriptor | ISO 639 descriptor | PMT stream type | if no signalling/descriptor |
| --- | --- | --- | --- | --- | --- | --- |
| Audio Language | 1st (2) | 2nd | - | 3rd | - | - |
| Audio type | 1st | 2nd | 3rd | 4th | - | Normal |
| Audio format | 1st | - | 2nd | - | - | stereo |
| Stream type | - | - | 2nd | - | 1st | - |

Table X.X NGA capable NorDig HEVC IRD prioritization based on the available SI and descriptors.

See section 12.4.8.2 for more information related to audio specific signalling in the Event Information Table (EIT).

Note 2: Audio Language inside the Audio Preselection Descriptor is the only property used for PID/stream selection, while Audio type and Audio format inside the Audio Preselection Descriptor are used for presentation selection.

#### <new> Signalling for targeted language preference

For audio streams coded in either the MPEG-1 Layer II, AC-3, E-AC-3 or HE-AAC format, the preferred language of the targeted audience is signalled using the ISO_639_language_descriptor (section 12.6.4 and 12.1.8) or the supplementary audio descriptor (section 12.6.8 and 12.1.8).

If both descriptors are present on one audio stream, the ISO_639_language_code of the supplementary_audio_descriptor shall match that of the ISO_639_language_descriptor unless non-language tags are used for special purposes (see below for details).

NGA streams use the audio_preselection_descriptor (section 12.6.11) to signal the targeted languages of each signalled audio preselection. In addition, the ISO_639_language_descriptor can be used if IRDs supporting AC-4 but not the audio_preselection_descriptor is targeted (early NorDig HEVC IRDs released before July 2020).

Note 1: All NGA experiences targeting one language must be included in one single NGA stream as the IRDs are not required to will not consider any other stream (neither coded in NGA nor legacy audio format) for presentation selection, see 6.2.4.1.

Note 2: In absence of any descriptor (i.e. neither an ISO_639_language_descriptor nor an audio_preselection_descriptor is present), the NorDig HEVC IRD assumes that all languages are present in the NGA stream and in consequence will select that NGA stream.

Note 3: See section 12.1.8 on additional requirements and guidance on the use of language codes.

#### <new> Signalling for audio type (normal or supplementary Audio legacy audio only, i.e. non-NGA/non-AC-4)

For Non-NGA all Supplementary Audio streams (both Broadcast mixed and Receiver mixed) coded in MPEG-1 Layer II, AC-3, E-AC-3 or HE-AAC format shall be signalled by the broadcaster in the stream by means of the Supplementary Audio Descriptor, see section 12.6.8 (only exception if it is necessary to avoid mis-behaviours in legacy IRDs).

For NGA Supplementary Audio substreams (1) shall be (both Premixed mixed and Decoder mixed) coded in AC-4 format shall be signalled by the broadcaster in the stream by means of the the Audio Preselection Descriptor, see section 12.6.11.

Note 1: All experiences targeting one language must be included in one single NGA stream as IRDs are not required to consider any other stream (neither coded in NGA nor legacy audio format) for presentation selection, see 6.2.4.1.

Supplementary Audio Signalling using the Supplementary Audio descriptor

In the supplementary_audio_destriptor, mix_type is typically set to ‘1’. This applies to stream carrying “Normal Audio”, but also to streams providing “broadcast-mixed” accessibility services.
Only secondary audio streams intended to be mixed with a normal audio stream shall signal mix_type set to ‘0’ (“receiver mix”).

The editorial_classification shall indicate the type of the audio stream. Streams carrying “Normal audio” indicate 0x00, while accessibility services audio stream signal 0x01 for Audio Descriptor (AD) or 0x03 for Spoken Subtitles (SS) irrespectively of the mix_type employed.

For the ISO_639_language_code, the same rules as with the ISO_639_language_descriptor should apply.

Legacy IRD considerations on Supplementary Audio (informative)

Some (older) IRDs currently in the market (here referred to as legacy IRDs) handle supplementary audio in a variety of ways, and there are some legacy receivers which are unable to elegantly support the presence of Supplementary Audio.

To mitigate this and avoid unwanted behaviour, some Networks use special signalling for the Supplementary Audio. This means for example that in some networks a supplementary audio service carried as a separate broadcast-mixed audio track (PID) is signalled in the ISO_639_language_descriptor as ‘nar’ (=”Narrative”) in ISO_639_language_code field and 0x00 (=“normal/undefined”) in the audio_type field. The NorDig IRD recognizes the language code (‘nar’) and displays the word “Narrative” in the appropriate OSD and menu.

#### <new> Signalling to be used for audio format (stereo, multi…)

No RoO specification, for more information see section 12.6.

#### <new> Signalling to be used for audio stream type (codec)

No RoO specification, for more information see section 12.6.

<PerT 2025-02-18, text below section old 6.4.4/ new 6.5.4.5 subsection inside audio section does not excist in IRD spec. Almost the same text already exist in 12.6.11. Propose this to be move and merge into 12.6.11>

For each preselection contained in an NGA audio stream, the audio_preselection_descriptor (section 12.6.11) provides a language tag, some flags indicating the availability of accessibility features and an indication on the preferred reproduction layout in addition to an identifier for selection purposes towards the audio decoder. Optionally, preselections should be further differentiated by text labels carried in the message_descriptor (extension tag 0x08), if the beforementioned parameter doesn’t provide a comprehensive differentiation.

Not all presentations in the AC-4 elementary stream need to be reflected as preselections in an APD (audio_preselection_descriptor), but only those which are intended to be presented to and selectable by the user. On the other hand, the audio_preselection_descriptor may remain unchanged even if certain or all preselections are temporarily not available e.g. during commercial breaks and other interstitials. By this, the number of transitions in SI signalling is reduced compared to signalling within the NGA elementary stream where changes theoretically might occur every audio frame.

For AC-4 audio, Table 6.8 provides a mapping from elements of the APD (audio_preselection_descriptor) to corresponding elements in the AC-4 TOC.

| APD Element | AC-4 TOC Element |
| --- | --- |
| Audio Preselection | ac4_presentation_v1_info() |
| preselection_id | presentation_id |
| audio_rendering_indication | Preferred reproduction layout (see below.) |
| audio_description | Accessibility Service: Audio Description Associated audio ac4_substream_group present with content_classifier of 010b (Associated service: visually impaired). |
| spoken_subtitles | Accessibility Service: Spoken Subtitles Associated audio ac4_substream_group present with content_classifier of 111b (Associated service: voice over). |
| dialogue_enhancement | (typically set to ‘1’) |
| interactivity_enabled | (Reserved for future use.) |
| ISO_639_language_code | Primary language subtag of the language tag in the main audio or dialogue ac4_substream_group_info of the presentation, converted to ISO 639-2. |
| Text Label | Recommended but no expression in the elementary stream. |

Table 6.8.

Preferred reproduction channel layout

The audio_rendering_indication in the APD (audio_preselection_descriptor) should be set according to the pres_ch_mode and the b_pre_virtualized bit:

- If pres_ch_mode is 0 or 2, then audio_rendering_indication shall be set to 1.

- If pres_ch_mode is 1 and b_pre_virtualized is 0, then audio_rendering_indication shall be set to 1.

- If pres_ch_mode is 1 and b_pre_virtualized is 1, then audio_rendering_indication shall be set to 4.

- If pres_ch_mode is in the range from 3 to 8, then audio_rendering_indication shall be set to 2.

- If pres_ch_mode is 9 or higher, then audio_rendering_indication shall be set to 3.

- If pres_ch_mode is -1, then audio_rendering_indication shall be set to 3.

| pres_ch_mode | b_pre_virtualized | audio_rendering_indication |
| --- | --- | --- |
| 0 | any | 1 |
| 1 | 0 | 1 |
| 1 | 1 | 4 |
| 2 | any | 1 |
| 3 – 8 | any | 2 |
| 9 – 15 | any | 3 |
| -1 | any | 3 |

Table 6.9.

Multiple Languages with multiple streams
If audio streams targeting different languages are carried in multiple streams, then the audio_preselection_descriptor shall indicate all available languages in the NGA stream.

Additionally, the operator may send an ISO_639_language_descriptor. This may be necessary in order to serve HEVC IRDs not understanding the APD (audio_preselection_descriptor).

Note 1: If a selected preselection disappears, the AC-4 decoder will select an appropriate AC-4 presentation based on the user’s preferences until the selected preselection is available again.

Note 2: With NGA, accessibility services are an integral part of the NGA audio stream and therefore no supplementary audio streams will be used. Even if NGA receivers not understanding the APD (audio_preselection_descriptor) are targeted, accessibility services need not be signalled by legacy descriptors; those devices are still enabled to select an appropriate preselection from the NGA stream.

Note 3: As stated in section 6.3.5 all NGA experiences targeting one language must be included in one single NGA stream/PID.

Note 4: The language tags in the AC-4 elementary stream must strictly apply to BCP-47 while the language in the APD (audio_preselection_descriptor) is signalled using ISO 639-2 language tags. For example, the BCP-47 code of ‘en-US’ indicating “English as spoken in the United States” will read ‘eng’ in ISO 639-2.

Note 5: The audio_preselection_descriptor should only signal “real” languages. Special language tags like ‘und’, ‘mul’, ‘mis’, ‘qaa’ or ‘nar’ should be avoided as they will never match any user preference during the automated selection process. Instead, if an indication to the viewer of audio-related properties such as “original language” is desired, the audio_preselection_descriptor may reference a message_descriptor providing this information in a textual representation (With older audio codecs (such as MPEG1 L.II, MPEG4 AAC or Dolby AC-3) where accessibility audio (e.g. Audio Description) is carried within a service on their own PID and it is recommended to use language code ‘nar’ inside the ISO 639 language descriptor to facilitate user selection, while for NGA the accessibility audio is intended to be included in same PID as the normal audio PID  (see note 2) and therefore these special language tags are no longer needed).

Note 6: All NGA experiences targeting one language must be included in one single NGA stream as the IRD will not consider any other stream (neither coded in NGA nor legacy audio format) for presentation selection.

#### <old6.4.1> PID assignment for audio streams

In cases where multiple audio streams are intended to be simulcasted for one service, the IRD will prioritise streams with lower PID values in absence of comprehensive signalled information or if all signalling are equal (type (normal, supplementary), language, audio format (stereo/multichannel), stream type (codec)). In consequence, audio streams carrying experiences considered to be of primary interest by most users shall have assigned a lower PID number that those targeting a minor audience.

In particular, lower PID values shall be assigned to:

- Audio streams containing “Normal Audio” services compared to streams carrying “Supplementary Audio” services.

- Audio streams tagged with the primary preferred language of the majority of users in the reception area (=”default language”) compared to other languages.

- Legacy encoding formats compared to newer, more enhanced formats to increase compatibility with receivers based on older versions of the IRD specification

Note: Similar rules for prioritisation also apply to NGA preselections within one NGA stream, where receivers will prioritise, those presentations signalled first.

#### <old6.4.2> Codec-format specific signaling inside PID stream

MPEG-1 Layer II

The value of the stream_id field for MPEG-1 Layer II audio packetized elementary streams shall be 110x xxxx, where each x can be either 0, or 1.

The value of stream_type for MPEG-1 Layer II audio packetized elementary streams shall be 0x03 (indicating ISO/IEC 11172-3 Audio).

Note that MPEG-1 Layer II audio streams do not require an audio_stream_descriptor to be present (IRDs uses stream_type to identify that the stream is MPEG-1 Layer II audio), but an audio_stream_descriptor may be present in the ES_info_loop of the PMT for the service, see sections 12.6.2 for details.

E-AC-3 and AC-3

The value of the stream_id field for an AC-3 and Enhanced AC-3 elementary stream shall be 0xBD (indicating private_stream_1).

The value of stream_type for an AC-3 or Enhanced AC-3 elementary stream shall be 0x06 (indicating PES packets containing private data). Multiple AC-3 or Enhanced AC-3 streams may share the same value of stream_id since each stream is carried with a unique PID value.

Note that AC-3 and E-AC-3 audio streams require an ac-3_descriptor or an enhanced_AC-3_descriptor to be present in the ES_info_loop of the PMT for the service, respectively. See sections 12.6.5 and 12.6.6 for details.

MPEG 4 HE AAC

The value of the stream_id field for LATM/LOAS formatted MPEG-4 AAC, MPEG-4 HE AAC and MPEG-4 HE-AAC v2 packetized elementary streams shall be 110x xxxx, where each x can be either 0, or 1.

The value of stream_type for MPEG-4 AAC, MPEG-4 HE AAC and MPEG-4 HE AAC v2 packetized elementary streams shall be 0x11 (indicating ISO/IEC 14496-3 [53] Audio with the LATM/LOAS transport syntax).

Note that MPEG-4 HE-AAC (incl AAC-LC) audio streams require an AAC_descriptor in the ES_info_loop of the PMT for the service, even if some IRDs uses stream_type to identify that the stream is MPEG-4 AAC audio. See sections 12.6.7 for details.

AC-4

The value of stream_id in the packetized elementary stream (PES) header shall be 0xBD (indicating private_stream_1). Multiple AC-4 streams may share the same value of stream_id since each stream is carried using a unique PID value.

The AC-4 elementary stream shall be byte-aligned within the PES packet payload. This means that the first byte of an AC-4 frame shall reside in the first byte of the PES packet payload.

One or more AC-4 frames may be packaged into one PES packet, but each PES packet shall contain an integer number of AC-4 frames only. AC-4 frames shall not be split over two or more PES packets.

For an AC-4 elementary stream the value of stream_type in the associated elementary stream loop instance in the PMT shall be set to 0x06 (indicating PES packets containing private data).

Note that AC-4 audio streams require an ac-4_descriptor to be present in the ES_info_loop of the PMT for the service. See section 12.6.12 for details.

<PerT 2025-02-24, text below section old 6.4.5/ new 6.6.4.8 subsection inside audio section does not excist in IRD spec. Exactly the same text already exist in 12.4.8. Propose to remove this subsection and only keep 12.4.8 and to add a general reference in 6.6.4 above to audio info in EIT>

The type and the editorial characteristics of audio component should be signaled by dedicated component_types in the component_descriptor.
The presence of the component_descriptor in the EIT is optional and up to the broadcaster/operator.

The following table provides an overview on the available component_types used to characterize related audio components:

| Audio codec | configuration | stream_content | stream_content_ext | component_type |
| --- | --- | --- | --- | --- |
| MPEG-1 Layer II audio | Mono | 0x2 | n/a 0xF | 0x01 |
|  | stereo |  |  | 0x03 |
|  | receiver-mixed audio description |  |  | 0x47 |
|  | broadcast-mixed audio description |  |  | 0x48 |
| (E-)AC-3 |  | 0x4 | n/a 0xF | See below |
| MPEG-4 AAC / HE AAC (1) | Mono | 0x6 | n/a 0xF | 0x01 |
|  | Stereo |  |  | 0x03 |
|  | Multichannel |  |  | 0x05 |
|  | receiver-mixed audio description |  |  | 0x47 |
|  | broadcast-mixed audio description |  |  | 0x48 |
| AC-4 |  | 0x9 | 0x1 | 0x0E |
| NGA preselections |  | 0xB | 0xE | See below |
| Note 1: Applicable for MPEG-4 audio streams that conform to the AAC Profile (often referred to as AAC low complexity) and the High Efficiency AAC Profile; not applicable for streams that conform to the High Efficiency AAC v2 Profile. |  |  |  |  |

Table 6.10: Audio specific component_type value assignment.

component_type for (E-)AC-3 audio modes

For the usage with audio coding modes supported in the scope of the NorDig specification, the component_type assignments can be simplified as follows:

| component_type bits |  |  |
| --- | --- | --- |
| b7 (MSB) | b6 to b3 | b2 to b0 |
| E-AC-3 flag: 0: stream is AC-3 1: stream is E-AC-3 | Service type: 1000: Normal audio 1010: broadcast-mixed AD 0010: receiver-mixed AD (supplementary stream) | Channel Mode: 000: Mono 010: Stereo 100: Multichannel (5.1) |

Table 6.11.

For example, a normal audio, multichannel 5.1 stream encoded in E-AC-3 uses a component_type of 11000100 = 0xC4. A supplementary audio stream carrying the receiver-mixed audio description component uses a component_type of 10010000 = 0x90.

For all other assignments see the DVB SI specification in ETSI EN 300 468 [13].

component_type for NGA Preselections

For NGA, one instance of a component_descriptor should signal the used codec; in particular for AC-4 audio, this descriptor signals stream_content set to 0x9, stream_content_ext 0x1 and component_type set to 0x0E.

In addition, another instance of the compontent_descriptor can be put into the corresponding descriptor loop for each NGA preselection available for the whole service. In this case, with stream content set to 0xB, stream_content_ext set to 0xE and component type set according to the following table:

| component_type bits |  | Description |
| --- | --- | --- |
| b7 (MSB) |  | Reserved zero for future use |
| b6 |  | content is pre-rendered for consumption with headphones |
| b5 |  | content enables interactivity |
| b4 |  | content enables dialogue enhancement |
| b3 |  | content contains spoken subtitles |
| b2 |  | content contains audio description |
| b1 | b0 | Preferred reproduction channel layout: |
| 0 | 0 | no preference |
| 0 | 1 | stereo |
| 1 | 0 | two-dimensional |
| 1 | 1 | three-dimensional |

Table 6.12: Next-generation audio component_type value assignment.

### Examples of priority

See exampled in NorDig Unified IRD specification (section 6.5.5).

### Presentation selection from inside NGA streams

Once the IRD has selected an NGA audio stream for decoding, the desired presentation from that stream is selected by two mechanisms:

Per default, the IRD selects the most appropriate presentation based on the user’s preferences. This is based on the information contained in the ac4_toc in the elementary stream.
Additionally, the user may select a presentation from the list of available presentation signalled in the audio_preselection_descriptor (optionally enriched by textual descriptions from message descriptors) (1). Based on this signalling, the user interface can inform the NGA/AC-4 decoder about the user’s selection through a unique identifier.
If the signalled choice is temporarily unavailable - for instance during an advertisement break - the AC-4 decoder will instantaneously fall back on the user-preferences based presentation selection based on information from the ac4_toc to provide a seamless experience to the listener and will revert the personalised choice once it becomes available again.

Note 1: The audio_preselection_descriptor was optional for NorDig HEVC IRDs launched before 1. July 2020.

Table 6.14 uses the terminology from the Audio Preselection Descriptor, however the ac4_toc is used for audio prioritisation based on user preferences. The ac4_toc functionality is typically part of audio decoder components. Therefore, the corresponding properties for Audio Type and Language are used which are described in the section above.

| IRD settings | Audio Description (AD) OFF |  | Audio Description (AD) ON |  |
| --- | --- | --- | --- | --- |
|  | Spoken Subtitles OFF | Spoken Subtitles ON | Spoken Subtitles OFF | Spoken Subtitles ON |
| IRD behaviour depending on above IRD settings and preselection properties |  |  |  |  |
| Priority | Priority with respect to best matching preselection |  | Priority with respect to best matching preselection |  |
| 1 | AD off and SpS off Preselection matching primary audio language | AD off and SpS on Preselection matching primary audio language | AD on and SpS off Preselection matching primary audio language (1) | AD on and SpS on Preselection matching primary audio language |
| 2 | AD off Preselection matching primary audio language | SpS on Preselection matching primary audio language | AD on and SpS on Preselection matching primary audio language | AD on and SpS off Preselection matching primary audio language |
| 3 | Preselection matching primary audio language | Preselection matching primary audio language | SpS off Preselection matching primary audio language | SpS on Preselection matching primary audio language |
| 4 | AD off and SpS off Preselection matching secondary audio language | AD off and SpS on Preselection matching secondary audio language | Preselection matching primary audio language | Preselection matching primary audio language |
| 5 | AD off Preselection matching secondary audio language | SpS on Preselection matching secondary audio language | AD on and SpS off Preselection matching secondary audio language settings | AD on and SpS on Preselection matching secondary audio language |
| 6 | Preselection matching secondary audio language | Preselection matching secondary audio language | AD on and SpS on Preselection matching secondary audio language | AD on and SpS off Preselection matching secondary audio language |
| 7 | (if no match) default Preselection | (if no match) default Preselection | SpS off Preselection matching secondary audio language | SpS on Preselection matching secondary audio language |
| 8 |  |  | Preselection matching secondary audio language | Preselection matching secondary audio language |
| 9 |  |  | (if no match) default Preselection | (if no match) default Preselection |

Table 6.14: Audio Priority between NGA Preselections for NGA “Accesibility Services” with Audio description on/off and Spoken Subtitles on/off. A lower number refers to higher priority.

Differentiation of audio formats is not relevant for prioritisation of presentations since the NGA decoder provides renderers for kinds of playback scenarios.

## Audio Output Formats out from NorDig IRDs

No RoO specification.

## Audio Video Synchronisation

No RoO specification. <would be good to have some text here>

## Audio Output Signals out from NorDig IRDs

No RoO specification.

## Dynamic Changes in the audio streams

No RoO specification. <would be good to have some text here>

## Dialogue enhancement

No RoO specification. <would be good to have some text here>

## Supplementary Audio Services for Accessibility Services

This section describes guidelines for handling supplementary audio for accessibility services.

Note: The term supplementary audio may in other contexts also be used other purposes, like commentary audio etc, but this NorDig section focus upon the audio accessibility cases.

Note: In ETSI specifications, NGA refers to preselecions for supplementary audio.

### Informative about Supplementary Audio

A supplementary audio service (as defined in ETSI TS 101 154 [26]) is specified below for the “in-service delivery” and applies when “nNormal” audio PIDs/substreams and the supplementary audio streams/substreams are available within the same service (i.e. listed within same PMT). For non-NGA the different audio streams (normal and supplementary) refers to delivery to the IRD as different audio PIDs while for NGA different audio streams normally refers to substreams within a common NGA audio PID. (This means for NGA, no separate PIDs/streams are required for the supplementary audio, see 6.2.4.1).

A Supplementary Audio (SA) service may be broadcasted as either:

- For non-NGA audio streams:

  - “Broadcast mixed”:  pre-mixed audio by the broadcaster where the Supplementary Audio stream is a complete self-standing audio which contains both the programme audio mixed together with the supplementary audio content.

  - “Receiver mixed”: audio containing only the supplementary audio content which is not a complete self-standing audio and is not intended to be presented on its own. The receiver mixed supplementary audio and programme audio are typically mixed together in the IRD, under some control of the broadcaster via broadcast of supplementary data.

- For NGA audio streams:

  - “Premixed”: the Supplementary Audio content is pre-mixed by the broadcaster with the Normal audio content. Although the Premixed Supplementary audio is a self-standing audio element/substream, its multiplexed into the same audio elementary stream/PID as the Normal audio element. Inside the IRD, the NGA decoder selects the desired element for decoding.

  - “Decoder mixed”: the Supplementary Audio element/substream contains only the supplementary audio content which is not a complete self-standing audio and is not intended to be presented on its own. The supplementary audio element/substream is typically multiplexed into the same audio elementary stream/PID as the Normal audio element/substream. Inside the IRD, the NGA decoder mixes, after decoding both elements/substreams, the supplementary audio and Normal audio content, under some control of the broadcaster (mixing level).

### General requirements for Supplementary Audio for Accessibility Services

Within NorDig, the most common way of using Supplementary Audio is to use the facility to broadcast spoken subtitles, audio description as broadcast mix is also in use as a Supplementary Audio service. Since audio dubbing of content is very rarely employed within NorDig, the original spoken audio language content is broadcasted together with subtitling and an accompanying supplementary audio track which may either be i) a spoken version of the subtitle or ii) an original language version track with narrative description (in the same language). The viewer is then at liberty to select their preference accordingly dependent upon their personal preference or needs.

Within NorDig, there are several different methods employed to broadcast Supplementary Audio for the viewer. In general, the following methods are employed:

- Broadcast mixed, Supplementary Audio signalled within the main service (in use in Sweden).

- Broadcast mixed, a separate TV service (in use in Denmark and Norway).

- Receiver mixed without metadata; Supplementary Audio signalled.

- Receiver mixed with metadata; Supplementary Audio signalled.

- Supplementary Audio only, on a separate TV service (no longer used in NorDig networks).

- Supplementary Audio only with video, on separate TV service.

- Broadcast mix Audio Descriptive service with a narrative voice that describes the scene portrayed during natural gaps in dialogue, allowing viewers with visual impairment to follow on screen action. This functionality is presented to the viewer as an alternate language track and selected by the viewer via the language/audio function on the remote control.  (This format is in use in Ireland, Sweden, Norway & Denmark).

When Supplementary Audio is signalled via SI within the DVB stream, the user may configure the IRD to use “Audio Description ON/Yes” in the set-up menu to select the supplementary audio track.

The broadcaster sources the appropriate type of supplementary audio, (broadcast mixed or receiver mixed). The standard method of broadcasting receiver mixed audio is together with metadata instruction to “duck” or reduce the audio level of the main programme stream during periods of descriptive narrative (see 6.12.6.1 below), this metadata is typically carried within a supplementary audio stream and follows operational practice set out BBC WHP 198 [104].

The alternate method for the broadcaster in supplying supplementary audio is to employ “broadcast mixed” supplementary audio. 
In some NorDig countries a separate duplicate TV service is employed for the supplementary audio, with only the audio track differing in content from that of the “main” service.
This can simplify selection of the accessibility service for the viewer; however, it is an inefficient use of bandwidth and can complicate service selection.

Note In the context of NGA services Supplementary Audio (SA) streams are not used in the context of NGA services. Instead, NGA Accessibility Services (Audio Description, Spoken Subtitles and Dialogue Enhancement) are integrated features within NGA streams and can’t be handled externally as SA streams.

The Nordig IRD’s audio decoder is requested to be capable of supporting ‘visual impaired’ Supplementary audio (SA) services, as defined in ETSI TS 101 154 [26] (however, control of pan and fade is optional).

The NorDig IRD is requested to support both Broadcast mixed and Receiver mixed Supplementary Audio.

The NGA capable Nordig HEVC IRD audio decoder is requested to be capable of supporting the NGA supplementary audio for Accessibility services “Audio Description”, “Spoken Subtitles” and “Dialogue Enhancement” within NGA audio streams, as defined in section 6.1.

### IRD settings for Supplementary Audio for accessibility Services (informative)

NorDig IRDs is required to have a stored user preference setting(s) for enable and disable (priotisation of) supplementary audio for accessibility services. NorDig IRD is recommended to be able to display information about availability of supplementary audio for accessibility services in current selected service to the user.

For more information around IRD settings for supplementary audio for accessibility servies, see NorDig Unified IRD specifications [xx] sections 6.11.3, 6.14.1 and 16.2.

When Supplementary Audio is signalled via SI within the DVB stream, the user may configure the IRD to use “Audio Description ON/Yes” in the set-up menu to select the supplementary audio track.

### NorDig IRD selection of audio streams (informative)

No RoO specification.

### Signalling for Supplementary Audio

All non-NGA Supplementary Audio streams (both Broadcast mixed, and Receiver mixed) shall/should be signalled by the broadcaster by means of Supplementary Audio descriptors (see section 12.6.8) and for services with several audio streams (e.g one Normal/ plus one Supplementary Audio stream) also an ISO 639 language descriptor (see section 12.6.4).

All NGA Supplementary Audio shall/should be signalled in the broadcast stream by means of the Audio Preselection Descriptor (see section 12.6.12 12.6.11).

The signalling of the NGA Supplementary Audio in the Audio Preselection Descriptor shall be mapped to the NGA codec specific values according to ETSI EN 300 468 [13] Table M.1.

Non-NGA Broadcast mixed Supplementary Audio streams shall/should, in the ISO 639 descriptor use audio type 0x00 ‘undefined’ (to avoid issues in legacy IRDs).

Non-NGA Receiver mixed Supplementary Audio streams shall/should in the ISO 639 descriptor use audio type 0x03 ‘visual impaired’ or 0x00 ‘undefined’ (to avoid issues in legacy IRDs).

See section 6.5 for how NorDig IRDs priorities between different descriptors for audio streams and section 12.1.6 12.6 for more information about the descriptors for audio (for non-NGA see 12.6.8 and 12.6.4 and for NGA see 12.6.11).

<moved from section 6.11.9 – stephan 2025-02-12>

<Below a copy of the text from NorDig Unified IRD specification just during editing of RoO>

Legacy IRDs currently in the market are known to handle supplementary audio in a variety of ways, and there are some cases where legacy IRDs are even “disturbed” by the presence of Supplementary Audio. To mitigate this and avoid unwanted behaviour, some Networks use special signalling for the Supplementary Audio. This means for example that in some networks a broadcast pre-mixed supplementary audio may be signalled in the ISO639 descriptor as Normal (‘undefined’) audio type but with language code ‘nar’, or a receiver mixed supplementary audio may be signalled in the ISO639 descriptor as ‘visual impaired’ audio type but with a different language to that of the associated Normal audio. Language code ‘nar’ refers here to as ‘narrative’.

The IRDs may/should support to translate language code ‘nar’ in ISO639 descriptor as ‘narrative’ language for the user and may/should support ‘narrative’ as user selectable for audio language in user preference settings. The IRD may interpret audio signalled with language code ‘nar’ in ISO639 descriptor as Broadcast mixed Supplementary Audio (if no supplementary audio descriptor is available for the PID/stream).

### Receiver mixing

#### Receiver mixed, Pan and fade control

Spoken Subtitle receiver-mixed supplementary audio: ducking

The receiver mixing between supplementary audio and normal audio can be done in various ways. Generally, when the supplementary audio is active, the normal audio is attenuated or “ducked”. When supplementary audio is inactive, the normal audio returns to its original level.
The timing, attenuation speed, release speed, attenuation level, and delay of the supplementary audio in relation to when the subtitle is presented on screen, can be adjusted at the play-out chain. It is generally deemed preferable for the viewer if the normal audio that is not reduced too much but also maintains a good speech intelligibility of the spoken subtitle. The attenuation of the normal audio with the range of -6 to -12 dB is considered to adequate in order to achieve this. Too fast a change of normal audio level might give the impression of “pumping” audio and is undesirable.

It has been found through subjective observation that some slight delay of the spoken subtitles compared to when the subtitle is displayed on screen can be advantageous to the eye and assist comprehension.

Example:

Figure 6.2: Spoken Subtitle supplementary audio.

### Receiver mixed Supplementary audio on its own for headphones output of the NorDig IRD

No RoO specification.

### Broadcast mixed

The NorDig IRD is required to support non-NGA broadcast mixed supplementary audio services encoded per MPEG-1 Layer II, HE-AAC, AC-3 and E-AC-3 coding standards and NGA premixed supplementary audio services encoded per AC-4 coding standard, where those codecs are supported.

< AP Johan L draft text similar guidelines about ducking etc but made by the broadcasters instead of inside the IRD as above in 6.12.6, check if reference new EBU recommendation TR084. Check if we should add some more parameters for new/other types of playout systems. Review if change some in Receiver mixed section above, maybe move figure down to broadcast mixed and from receiver mix reference to figure>

## Internal Reference Level

Within NorDig the level for reference or lineup tone for transmission shall be -18 dB FS below clipping level, in accordance with EBU Recommendation R.68 [74] ”Alignment level in digital audio production equipment and in digital recorders” as recommended by ETSI TS 101 154 [26].

Note: The alignment level for sound-programme exchange does not need to be changed with the introduction of loudness normalization.

## Loudness levels

To achieve loudness and dynamic range consistency over stream types (= audio codecs), the operator shall follow the following guidelines on audio leveling and dynamic range control settings.

Measuring programme loudness shall be accomplished using a loudness meter compliant with ITU-R BS.1770-4 [76] and EBU R 128 [72]. Those meters provide a loudness reading in units of LUFS (=”Loudness Units relative to Full Scale”).

For immersive audio (including height elements, as well as object-based and scene-based audio), loudness meter needs to be compliant with ITU-R BS.1770-5. Although EBU R 128 [72], EBU Tech 3343 and EBU Tech 3344  [73] are not yet updated to this version of the ITU recommendation, the principles of all EBU documents still apply.

It is strongly recommended to use the loudness levels defined in EBU Tech 3343 and EBU Tech 3344  [73]. In general, it should be assumed that each content provider that supplies programs content to the platform head-end will follow the rules set in EBU Tech 3344. These guidelines state that program loudness for each program should be -23 LUFS.
A television service may consist of a sequence of programming content in successive order, including commercials and interstitials and all are interpreted as individual programming. The overall long-term integrated loudness for a complete service is therefore also meant to equate to -23 LUFS (service loudness).

Since accordng to section 5.1.3, the alignment level is specified to be at -18 dBFS (1 kHz tone), loudness meters will read it as −18 LUFS with the absolute scale (or +5 LU on the relative EBU-mode scale), provided that the 1 kHz tone is present (in phase) on both the left and right channel of a stereo or surround sound signal. If the 1 kHz tone of −18 dBFS is used only in a single front channel, the loudness meter will read −21 LUFS (or +2 LU on the relative scale).

According to EBU R 128 [72], the permitted Maximum True Peak Level measured according to ITU-R BS.1770 [76] shall not exceed -1 dBFS in linear production environments. For broadcast applications using stream types (=audio codecs) at reasonable bitrates, the recommended Maximum Permitted True Peak Level should be restricted to −2 dBTP. Relatively low bitrates might require even decreased limits for the Maximum True Peak Level.

Note: This recommendation obsoletes the “Maximum Permitted Level (PML)” of -9 dBFS in ITU-R BS.645.

In order for this to successfully work as intended all the way to the domestic receiver, the audio level itself must be under control, but also, it is of utmost importance that the audio metadata for loudness carries the correct values. To accommodate for the specific properties of the different stream types (=audio codecs), the rules given in the associated audio encoding sections (in section 6.3) shall apply.

Caveat: Only set static values when the content provider does not supply loudness metadata. Some content providers may supply dynamically changing loudness metadata.

Occasionally, the platform head-end may take their own loudness measurements in order to verify that each content provider has aligned the loudness levels in a correct way according to EBU Tech 3344 [73] and is legal. Additonal information on handling on audio loudness can be found in EBU tech 3344 [73].

### <new> HE-AAC, AC-3/E-AC-3 and AC-4 Audio Output

As stated above it is recommended that the program loudness should be -23 LUFS for all audio streams.

It is highly recommended to include audio metadata describing the current actual targeted loudness level for all HE-AAC, AC-3/E-AC-3 and AC-4 audio streams. If loudness metadata is not included in an audio stream, NorDig IRDs will assume loudness level -23 LUFS (see section 6.2).

The broadcaster shall ensure that sufficient headroom and/or dynamic range control values are included in the transmission to prevent any overload when downmixing.

#### <new> HE-AAC Encoding

When metadata for HE-AAC is included, it shall be set according to section 6.2.3.1.

#### <new> E-AC-3 and AC-3 Encoding

When metadata for E-AC-3 or AC-3 is included, it shall be set according to section 6.2.2.2.

#### <new>  AC-4 Encoding

When metadata for AC-4 is included, it shall be set according to section 6.2.4.2.

#### <new> Summary

<Below a copy of the text from NorDig Unified IRD specification just during editing of RoO>

A summary of which levelling data and dynamic range control parameters are used by the NorDig IRD in conjunction with which stream type target reference level and DRC presentation mode can be found in Table 6.6. below.

| Stream type | Target reference level | Levelling data | DRC Presentation Mode | Dynamic range control parameter |
| --- | --- | --- | --- | --- |
| AC-3 and E-AC-3 | -23 dBFS | dialnorm per ETSI TS 102 366 | n/a | compr per ETSI TS 102 366 |
| AC-3 and E-AC-3 | -31 dBFS | dialnorm per ETSI TS 102 366 | n/a | dynrng per ETSI TS 102 366 |
| HE-AAC | -23 dBFS | prog_ref_level per ISO/IEC 14496-3 | Mode 1 | compression_value per ETSI 101 154 Annex C |
| HE-AAC | -31 dBFS | prog_ref_level per ISO/IEC 14496-3 | Mode 1 | dyn_rng_sgn/dyn_rng_ctl per ISO/IEC 14496-3 |
| HE-AAC | -23 dBFS | prog_ref_level per ISO/IEC 14496-3 | Mode 2 | dyn_rng_sgn/dyn_rng_ctl per ISO/IEC 14496-3 |
| HE-AAC | -31 dBFS | prog_ref_level per ISO/IEC 14496-3 | Mode 2 | dyn_rng_sgn/dyn_rng_ctl per ISO/IEC 14496-3 |
| AC-4 | -23 dBFS | dialnorm per ETSI TS 103 190-1 | n/a | drc_decoder_mode_id = 1 “Flat panel TV” per ETSI TS 103 190-1 |
| AC-4 | -31 dBFS | dialnorm per ETSI TS 103 190-1 | n/a | drc_decoder_mode_id = 0 “Home Theatre” per ETSI TS 103 190-1 |

Table 6.6 Target reference levels.

### <new> MPEG-1 Layer II Audio Output

Since MPEG-1 Layer II audio coded streams do not support loudness audio metadata, IRDs do no level adjustments during the decoding. Therefore, broadcasters shall ensure that MPEG-1 Layer II audio stream has an equivalent loudness level of -23 LUFS.

### <new> Audio Output Levels from the NorDig IRD

No RoO specification.

# Service Information

<PerT include RoO section 12.6.1 in order to propose move and merge section new 6.5.4.5 (old 6.4.4) above to this 12.6.11 below and to review 12.4.8.2>

## Event Information Table

### Component Descriptor

#### General

A component_descriptor (0x50) identifies all the components associated with the service for the running event, this can indicate whether a current or future event has additional components which may be of interest to the viewer, such as subtitles or audio description.

The component_descriptor provides additional technical information of the program event (e.g. this program event has multichannel audio, this event has Hard of Hearing subtitling etc). The IRD typically uses the information inside the component_descriptor together with other event information (event name, event description, schedule time etc) when presenting for example EPG or info banner for the viewer (i.e. not used for the actual part of the decoding process of the selected service). As a guideline it could be to include component information that is of special interest among viewers and information that differ from the regular broadcast for this service or other services inside the network (e.g. a service that regularly broadcast stereo audio, but sometimes has multichannel, it could be of interest to at least include component information for the few events that has real multichannel audio). Other information which are more static and common for the service or all services inside the network, these could be excluded in order to save data and bandwidth for the EIT (like a service that is always broadcasted in 16:9 video or always carries an EBU Teletext service, these component information values could be omitted).

#### Audio specific signalling in the Event Information Table

The type and the editorial characteristics of audio component should be signalled by dedicated component_types in the component_descriptor.

The presence of the component_descriptor in the EIT is optional and up to the broadcaster/operator.

The following table provides an overview on the available component_types used to characterize related audio components:

| Audio codec | configuration | stream_content | stream_content_ext | component_type |
| --- | --- | --- | --- | --- |
| MPEG-1 Layer 2 audio | Mono | 0x2 | n/a 0xF | 0x01 |
|  | stereo |  |  | 0x03 |
|  | receiver-mix audio description |  |  | 0x47 |
|  | broadcast-mix audio description |  |  | 0x48 |
| (E-)AC-3 |  | 0x4 | n/a 0xF | See below |
| MPEG-4 AAC / HE AAC (1) | Mono | 0x6 | n/a 0xF | 0x01 |
|  | Stereo |  |  | 0x03 |
|  | Multichannel |  |  | 0x05 |
|  | receiver-mix audio description |  |  | 0x47 |
|  | broadcast-mix audio description |  |  | 0x48 |
| AC-4 |  | 0x9 | 0x1 | 0x0E |
| NGA preselections |  | 0xB | 0xE | See below |
| Note 1: Applicable for MPEG-4 audio streams that conform to the AAC Profile (often referred to as AAC low complexity) and the High Efficiency AAC Profile; not applicable for streams that conform to the High Efficiency AAC v2 Profile. |  |  |  |  |

Table 12.14: Audio specific component_type value assignment.

component_type for (E-)AC-3 audio modes
For the usage with audio coding modes supported in the scope of the NorDig specification, the component_type assignments can be simplified as follows:

| component_type bits |  |  |
| --- | --- | --- |
| b7 (MSB) | b6 to b3 | b2 to b0 |
| E-AC-3 flag: 0: stream is AC-3 1: stream is E-AC-3 | Service type: 1000: Normal audio 1010: broadcast-mixed AD 0010: receiver-mixed AD (supplementary stream) | Channel Mode: 000: Mono 010: Stereo 100: Multichannel (5.1) |

Table 12.15.

For example, a normal audio, multichannel 5.1 stream encoded in E-AC-3 uses a component_type of 11000100 = 0xC4. A supplementary audio stream carrying the receiver-mix audio description component uses a component_type of 10010000 = 0x90.
For all other assignments see the DVB SI specification in ETSI EN 300 468 [13].

component_type for NGA Preselections
For NGA, one instance of a component_descriptor should signal the used codec; in particular for AC-4 audio, this descriptor signals stream_content set to 0x9, stream_content_ext 0x1 and component_type set to 0x0E.
In addition, another instance of the compontent_descriptor can be put into the corresponding descriptor loop for each NGA preselection available for the whole service. In this case, with stream content set to 0xB, stream_content_ext set to 0xE and component type set according to the following table:

| component_type bits |  | Description |
| --- | --- | --- |
| b7 (MSB) |  | Reserved zero for future use |
| b6 |  | content is pre-rendered for consumption with headphones |
| b5 |  | content enables interactivity |
| b4 |  | content enables dialogue enhancement |
| b3 |  | content contains spoken subtitles |
| b2 |  | content contains audio description |
| b1 | b0 | Preferred reproduction channel layout: |
| 0 | 0 | no preference |
| 0 | 1 | stereo |
| 1 | 0 | two-dimensional |
| 1 | 1 | three-dimensional |

Table 12.16: Next-generation audio component_type value assignment.

## Conditional Access and Program Map Tables

### Audio Preselection Descriptor (NGA services only)

Audio preselections in NGA / AC-4 audio streams shall be signalled by the audio_preselection_descriptor (APD) as per ETSI EN 300 468, section 6.4.1. [13]. The descriptor shall be placed in the descriptor loop for the audio element of the PMT using the DVB extension_descriptor and an extension tag value of 0x19.

For each preselection, the audio_preselection_descriptor (APD) provides a language tag, some flags indicating the availability of accessibility features and an indication on the preferred reproduction layout in addition to an identifier for selection purposes towards the audio decoder. Optionally, preselections should be further differentiated by text labels carried in the message_descriptor (extension tag 0x08), if the beforementioned parameter does not provide a comprehensive differentiation.

Not all presentations in the AC-4 elementary stream need to be reflected as preselections in an APD (audio_preselection_descriptor), but only those which are intended to be presented to and selectable by the user. On the other hand, the audio_preselection_descriptor may remain unchanged even if certain or all preselections are temporarily not available e.g. during commercial breaks and other interstitials (1). By this, the number of transitions in SI signalling is reduced compared to signalling within the NGA elementary stream where changes theoretically might occur every audio frame.

For AC-4 audio, Table 12.24 provides a mapping from elements of the APD (audio_preselection_descriptor) to corresponding elements in the AC-4 TOC.

| APD Element | AC-4 TOC Element |
| --- | --- |
| Audio Preselection | ac4_presentation_v1_info() |
| preselection_id | presentation_id |
| audio_rendering_indication | Preferred reproduction layout (see below.) |
| audio_description | Accessibility Service: Audio Description Associated audio ac4_substream_group present with content_classifier of 010b (Associated service: visually impaired) (2). <fix table layout> |
| spoken_subtitles | Accessibility Service: Spoken Subtitles Associated audio ac4_substream_group present with content_classifier of 111b (Associated service: voice over) (2). <fix table layout> |
| dialogue_enhancement | (typically set to ‘1’) |
| interactivity_enabled | (Reserved for future use.) |
| ISO_639_language_code | Primary language subtag (3) of the language tag in the main audio or dialogue ac4_substream_group_info of the presentation, converted to ISO 639-2. |
| Text Label | Recommended but no expression in the elementary stream. |

Table 12.24.

The audio_preselection_descriptor should only signal “real” languages. Special language tags like “und”, “mul”, “mis”, “qaa” or “nar” should be avoided. If audio streams targeting different languages are carried in multiple streams, then the audio_preselection_descriptor shall indicate all available languages in the NGA stream (4).

Preferred reproduction channel layout

The audio_rendering_indication in the APD (audio_preselection_descriptor) should be set according to the pres_ch_mode and the b_pre_virtualized bit:

- If pres_ch_mode is 0 or 2, then audio_rendering_indication shall be set to 1.

- If pres_ch_mode is 1 and b_pre_virtualized is 0, then audio_rendering_indication shall be set to 1.

- If pres_ch_mode is 1 and b_pre_virtualized is 1, then audio_rendering_indication shall be set to 4.

- If pres_ch_mode is in the range from 3 to 8, then audio_rendering_indication shall be set to 2.

- If pres_ch_mode is 9 or higher, then audio_rendering_indication shall be set to 3.

- If pres_ch_mode is -1, then audio_rendering_indication shall be set to 3.

| pres_ch_mode | b_pre_virtualized | audio_rendering_indication |
| --- | --- | --- |
| 0 | any | 1 |
| 1 | 0 | 1 |
| 1 | 1 | 4 |
| 2 | any | 1 |
| 3 – 8 | any | 2 |
| 9 – 15 | any | 3 |
| -1 | any | 3 |

Table 12.25.

Multiple Languages with multiple streams
If audio streams targeting different languages are carried in multiple streams, then the audio_preselection_descriptor should indicate all available languages in the NGA stream unless all languages are available from this one and only NGA stream (5).

Additionally, the operator may send an ISO_639_language_descriptor. This may be necessary in order to serve HEVC IRDs not understanding the APD (audio_preselection_descriptor).

Note 1: If a selected preselection disappears, the AC-4 decoder will select an appropriate AC-4 presentation based on the user’s preferences until the selected preselection is available again.

Note 2: With NGA, accessibility services are an integral part of the NGA audio stream/PID and therefore separate supplementary audio streams/PIDs should not be used. Even if NGA receivers that do not understand the audio_preselection_descriptor are targeted, NGA streams containing accessibility services do not need to be signalled by legacy descriptors (e.g. supplementary_audio_descriptor); those NGA receivers are still able to select an appropriate preselection from the NGA stream.

Note 3: The language tags in the AC-4 elementary stream must strictly apply to BCP-47 while the language in the audio_preselection_descriptor is signalled using ISO 639-2 language tags. For example, the BCP-47 code of ‘en-US’ indicating “English as spoken in the United States” will read ‘eng’ in ISO 639-2.

Note 4: The audio_preselection_descriptor should only signal “real” languages. Special language tags like ‘und’, ‘mul’, ‘mis’, ‘qaa’ or ‘nar’ should be avoided as they will never match any user preference during the automated selection process. Instead, If an indication to the viewer of audio-related properties such as “original language” is desired, the audio_preselection_descriptor may reference a message_descriptor providing this information in a textual representation (With older audio codecs (such as MPEG-1 Layer II, MPEG-4 AAC or Dolby AC-3) where accessibility audio (e.g. Audio Description) is carried within a service on their own PID and it is recommended to use language code ‘nar’ inside the ISO 639 language descriptor to facilitate user selection, while for NGA the accessibility audio is intended to be included in same PID as the normal audio PID  (see note 2) and therefore these special language tags are no longer needed).

Note 5: As stated in section 6.2.4.1 all experiences/preselections targeting one language must be included in one single NGA stream asIRDs are not required to consider any other stream (neither coded in NGA nor legacy audio format) for presentation selection.