# Audio

## General

NorDig has selected the following audio formats:

- MPEG 1 Layer II, which refers to MPEG-1 Layer II up to stereo (2.0) channel decoding [53].

- E-AC-3, which refers to E-AC-3 streams (including AC-3) up to 5.1 multichannel decoding [33].

- HE-AAC, which refers to MPEG-4 HE-AAC Level 4 (including AAC-LC) up to 5.1 multichannel decoding [53]. (Might be used for MPEG-4 based services).

- AC-4, which refers to AC-4 as defined in ETSI TS 103 190-2 [98] where the md_compat field as defined by clause 6.3.2.2.3 of ETSI TS 103 190-2 [98] shall be less than or equal to three and constrained by ETSI TS 101 154 [26] section 6.7.

Regarding the audio decoder (“codec”), the NorDig IRD shall meet all the audio codec requirements specified by the network(s) for where the NorDig IRD is intended to be used, see below (i.e. the audio codec requirements might slightly differ from one NorDig network to another).

Informative: Some NorDig networks have aligned all (MPEG-4) services into always using HE-AAC and some other networks have aligned all (MPEG-4) services into always using E-AC-3 or AC-3, this among other things to limit the number of audio decoders in the IRDs and in some cases also to save broadcast capacity. However, some NorDig networks are using HE-AAC for some (MPEG-4) services and E-AC-3/AC-3 for some other of their (MPEG-4) services.

HEVC Services: It is expected that all HEVC services will always use NGA.

In NorDig networks/regions where there is no single operator for acceptance of the IRDs, the NorDig IRD shall support:

- MPEG-1 Layer II and E-AC-3 and HE-AAC audio decoding.

In NorDig networks/regions where there is a single operator/regulator in charge for specifying the functionality of the IRD and ensuring that the minimum requirements are met, the operator/regulator specifies one of following minimum audio decoding format alternatives for the NorDig IRD to minimum support for that relevant network (i.e. always minimum support for two audio format):

- MPEG-1 Layer II and E-AC-3 and HE-AAC audio decoding,

- MPEG-1 Layer II and E-AC-3 audio decoding,

- MPEG-1 Layer II and HE-AAC audio decoding.

Additionally, the NorDig HEVC IRD shall support AC-4 audio decoding.

For other NorDig IRD´s, AC-4 audio decoding is optional, but if supported, then the AC-4 audio decoding shall follow the AC-4 NGA requirements speficied for the NorDig HEVC IRD.

The Audio decoders shall fully comply with the DVB Implementation Guidelines for the use of MPEG-2 Systems, Video and Audio in satellite, cable and terrestrial Broadcasting Applications ETSI TS 101 154  [26].

### Audio User Preference Settings

The NorDig IRD shall have User Preference Settings for audio functions as stated in section 16.2 (for example primary and secondary audio language, audio format, audio type, audio delay, etc).

### Audio terminology

NorDig has defined four properties for the audio, which here refer to the information of the audio that is signalled in the PMT for the audio stream and that the IRD uses to select the appropriate audio stream/PID to decode:

- Audio type: normal (main/undefined) or supplementary/accessibility (visual impaired/audio description, spoken subtitling etc) type of audio.

- Audio language: corresponds to the audio language signalled in the PMT for the audio stream.

- Audio format: mono, stereo, multichannel or (NGA) immersive audio format.

- Audio stream type: the audio codec (MPEG-1 Layer II, AC-3, E-AC-3, HE-AAC or AC-4) for the audio stream.

Normal audio – the term “Normal” refers in this specification to audio streams that are:

- intended for the majority of users and users that are not interested in any supplementary audio and

- signalled in the supplementary audio descriptor by mix_type ‘1’ and editorial description ‘0’ and in the ISO 639 language descriptor with audio_type ‘undefined’ and language not set to ‘nar” (1).

Supplementary Audio for Accessibility Services

- A Supplementary Audio (SA) (1) refers here to audio for accessibility services (2) and refers to the following experiences service may be either:

- Audio Description (AD): audio that includes a narration describing the action of the scene and is targeted at users with visual or cognitive impairments (for non-NGA streams, see section 6.11 for more information).

- Spoken Subtitles (SpS): audio that includes a spoken rendition of the subtitles and is targeted at users with visual or cognitive impairments (for non-NGA streams, see section 6.11 for more information).

- Dialogue Enhancement (DE): functionality that provides improved speech intelligibility. It is targeted at users with hearing impairments but can as well serve as improvement for listening in noisy environments (see section 6.10 for more information).

Audio Formats

- Mono audio is defined as an audio stream with a single channel of audio. (Note the old legacy case of dual mono mode is deprecated, for such use cases two separate (mono/stereo) audio streams shall instead be used).

- Stereo audio is defined as an audio stream with two channels of audio with loudspeakers positioned left and right of the center of the screen.

- Multichannel Audio is defined as an audio stream with more than two channels of audio with all loudspeaker positions in one plane.

- Immersive Audio is defined as audio containing additional height elements (i.e. more than 5.1 audio channels). The height elements may be carried either as additional audio channels, sound field representation, audio objects or a combination of these.

NGA Audio service (NGA)

- NGA refers to audio streams using Next Generation Audio (e.g. AC-4) audio codec.

- The NGA Audio services support delivery of audio content from mono, stereo, 5.1 channel and 7.1 channel-based audio sources, as well as Immersive Audio from Channel-based, Object-based or Scene-based audio sources. Additionally, NGA systems efficiently offer services such as multi-language support, accessibility services, personalization and interactivity.

Non-NGA audio service

- Non-NGA refers here to audio streams using other audio codecs than NGA (e.g. MPEG-1 Layer II, AC-3, E-AC-3 or HE-AAC).

NGA Accessibility Services:

- Accessibility services in the context of an NGA Audio service are Audio Description, Spoken Subtitles and Dialogue Enhancement as described above, see section 6.14 for more information.

NGA Preselections

- NGA Preselections offer alternative audio versions to the user for 'User Personalization' by sharing some of the Audio Programme Components contained within a single NGA stream and combining them in the audio decoder of the IRD receiver to create the final audio mix.

- An NGA stream may consist of multiple audio programme components (3) such as Music & Effects, dialogue/multiple dialogues for different languages, supplementary audio, etc., and mixing capabilities of different audio programme components is fundamental to every NGA decoder.

Stereo, multichannel and Immersive Audio:

- See sections 6.3 and 6.4 for definitions of Stereo, multichannel and Immersive Audio

Note 1: only applicable to non-NGA services. The term supplementary audio may in other contexts also be used other purposes, like commentary audio etc, but NorDig section 6.11 focus upon the audio accessibility cases.
Note 2: The Supplementary Audio (SA) term is not used in the context of NGA services. NGA Accessibility Services (Audio Description, Spoken Subtitles and Dialogue Enhancement) are integrated features of NGA and don’t need to be handled externally as SA. In ETSI specifications, NGA refers to preselections for supplementary audio.

Note 3: In the context of AC-4, the generic NGA term “audio programme component” referes to “AC-4 substream groups”. Likewise, “NGA Preselections” are implemented as “AC-4 presentations”.

## Audio Decoding

The majority of customer installed base of home theatre systems are not able to decode E-AC-3, HE-AAC or AC-4 formats. Furthermore, older home theatre systems can only decode DTS and AC-3 through S/PDIF.  Therefore, the NorDig IRD shall be able to re-encode or transcode audio streams as stated in sections 6.2.2.1, 6.2.3.1 and 6.2.4.1.

### MPEG-1 Layer II:  Requirements on Audio Handling

#### MPEG-1 Layer II: Decoding

The NorDig IRD shall support:

- decode MPEG-1 Layer II streams at all bit rates and sample rates listed in ETSI TS 101 154 [26].

The NorDig IRD should support:

- decode MPEG-1 Layer II streams at half-sampling rates (22.05 and 24 kHz).

#### MPEG-1 Layer II:  Audio Output

The NorDig IRD shall be capable of providing the following formats on the HDMI output connector from an MPEG-1 Layer II bitstream (see section 16 for factory default settings):

- Decoded to PCM stereo bitstream.

The NorDig IRD including an S/PDIF output shall be capable of providing the following formats on the S/PDIF connector from an MPEG-1 Layer II bitstream:

- Decoded to PCM stereo bitstream

### E-AC-3 and AC-3:  Requirements on Audio Handling

#### E-AC-3 and AC-3: All Pass-through, Decoding and Transcoding

NorDig IRD supporting E-AC-3 and AC-3 according to ETSI TS 102 366 [33] shall

- decode AC-3 streams at all bit rates and sample rates (not including Annex E ).

- (additionally) decode E-AC-3 streams with data rates from 32 kbps to 3 024 kbps and support all sample rates.

- be capable of transcoding E-AC-3 bitstreams to AC-3 bitstreams. Transcoding to AC-3 audio streams shall be at a fixed bit rate of 640 kbps.

#### E-AC-3 and AC-3: Metadata

The NorDig IRD supporting E-AC-3 and AC-3 shall support the use of a complete set of Dolby metadata [33] embedded in the audio stream when:

- decoding AC-3 or E-AC-3 bitstreams,

- transcoding E-AC-3 bitstreams to AC-3,

- or creating a PCM stereo downmix from a decoded E-AC-3 or AC-3 bitstream. (1) (2)

Note 1:	The E-AC-3 and AC-3 encoders always adds default metadata if they are not fed with metadata.
Note 2:	NorDig has the intension to include more information in the future in the NorDig Rules of Operation about the inclusion of metadata for E-AC-3 and AC-3.

#### E-AC-3 and AC-3:  Audio Output

Where HDMI, HDMI ARC (or eARC) is implemented, NorDig IRDs supporting E-AC-3 and AC-3 shall be capable of providing the following formats from an E-AC-3 or AC-3 bitstream (see chapter 16 for factory default settings):

- Pass-through of native bitstream (AC-3 and E-AC-3).

- E-AC-3 bitstream transcoded to AC-3 bitstream.

- Decoded and downmixed (if > 2 channels) to PCM stereo bitstream.

Where HDMI (or HDMI eARC) is implemented, NorDig IRDs supporting E-AC-3 and AC-3 should be capable of providing the following formats from an E-AC-3 or AC-3 bitstream:

- Decoded to PCM multichannel bitstream.

The NorDig IRD supporting E-AC-3 and AC-3 and including an S/PDIF output shall be capable of providing the following formats on the S/PDIF connector from an E-AC-3 or AC-3 bitstream:

- E-AC-3 bitstream transcoded to AC-3 bitstream.

- Decoded and downmixed (if > 2 channels) to PCM stereo bitstream.

- Pass-through of AC-3 bitstream.

### HE-AAC:  Requirements on Audio Handling

#### HE-AAC:  All Pass-through, Decoding and Re-encoding

NorDig IRDs supporting HE-AAC (and thereby also AAC-LC) shall be capable of:

- decoding HE-AAC Version 1 at Level 2 at sampling rates of 48 kHz according to ETSI TS 101 154 [26].

  - decoding, including downmixing HE-AAC Version 1 at Level 4 (multichannel, up to 5.1) at sampling rates of 48 kHz according to ETSI TS 101 154 [26], Annex C (downmix).

  - Re-encoding HE-AAC Version 1 at Level 4 (multichannel, up to 5.1) at sampling rates of 48 kHz according to TS 101 154 [26], Annex H to AC-3 or DTS.

NorDig IRDs shall be able to skip bitstream elements that are not recognized, i.e. unknown Fill elements and Data Stream elements.

If the NorDig IRD is supporting HE-AAC audio stream re-encoding to AC-3 audio stream, it shall be done according to TS 102 366 [33]. Re-encoding to AC-3 multichannel audio streams shall be at a fixed bit rate of 640 kbps.

If the NorDig IRD is supporting HE-AAC audio stream re-encoding to DTS audio stream, it shall be done according to ETSI TS 102 114 [30] at a fixed bit rate of 1.536 Mbps.

#### HE-AAC: Metadata

The NorDig IRD supporting HE-AAC shall support decoding of HE-AAC bitstreams both with and without audio metadata. It is highly recommended that the broadcast includes metadata for all HE-AAC bitstream, (NorDig has the intension to include more information in the future in the NorDig Rules of Operation about the inclusion of metadata for HE-AAC).

The NorDig IRD supporting HE-AAC shall support the use of the following MPEG-4 AAC metadata embedded in the audio stream when decoding HE-AAC and transcoding HE-AAC multichannel to AC-3 or DTS:

- Program Reference Level according to ISO/IEC 14496-3 [53] (prog_ref_level)

- Downmix Parameters according to "Transmission of MPEG4 Ancillary Data" part of DVB specification ETSI TS 101 154  [26] (center_mix_level, surround_mix_level)

- DRC Presentation Mode according to ETSI TS 101 154 [26] Annex C.5.2.2.3 (drc_presentation_mode) (see note 1)

- Dynamic Range Control (DRC) according to ISO/IEC 14496-3 [53] (dyn_rng_sgn, dyn_rng_ctl)

- Heavy Compression according to ETSI TS 101 154 [26] Annex C.5.2.5 (compression_on, compression_value)

The NorDig IRD capable of transcoding metadata to their output format shall not alter the level of the audio contained within the bitstream, shall pass all audio channels and shall transcode all metadata to the output format.
The NorDig IRDs that are re-encoding the incoming audio with metadata to an output format without metadata, shall apply the incoming metadata before the re-encoding. 

For HE-AAC bitstreams without metadata the NorDig IRD shall interpret that the bitstream uses default metadata values (for decoding and any re-encoding) according to ETSI TS 101 154 [26] chapter C.5.2.8 Table C.31 ‘Persistance of MPEG4 ancillary data’. In the absence of Program Reference Level metadata, it shall be assumed to be -23 dBFS (for mono, stereo and multichannel audio).

It is up to the broadcaster to ensure that sufficient headroom and/or dynamic range control values are included in the transmission to prevent any overload when downmixing.

Note: According to MPEG-4 HE-AAC specification (ISO/IEC 14496-3 [53] section 4.5.2.7.3 “Persistence of DRC information”) audio metadata is applied immediately during decoding whenever it is changed.

#### HE-AAC:  Audio Output

Where HDMI, HDMI ARC (or eARC) is implemented, NorDig IRDs supporting HE-AAC shall be capable of providing the following formats from a HE-AAC bitstream (see chapter 16 for factory default settings):

- Re-encoding to AC-3 or DTS bitstream.

- Decoded and downmixed (if > 2 channels) to PCM stereo bitstream.

Where HDMI, HDMI ARC (or eARC) is implemented, NorDig IRDs supporting HE-AAC should be capable of providing the following formats from a HE-AAC bitstream (see chapter 16 for factory default settings):

- Pass-through of native bitstream (HE-AAC) (1).

- Decoded to PCM multichannel bitstream.

Note 1:	HE-AAC over HDMI is defined in HDMI v 2.0 specification.

The NorDig IRD supporting HE-AAC and including an S/PDIF output shall be capable of providing the following formats on the S/PDIF connector from a HE-AAC bitstream:

- Decoded and downmixed (if > 2 channels) to PCM stereo bitstream.

- Re-encoding to AC-3 or DTS bitstream.

### AC-4:  Requirements on Audio Handling

#### AC-4: All Pass-through, Decoding and Re-encoding

NorDig IRD´s that support AC-4 shall

- Decode AC-4 streams at all bit rates and sample rates listed in ETSI TS 103 190-2 [98]

- Be capable of transcoding AC-4 bitstreams to E-AC-3 or AC-3 bitstreams according to ETSI TS 102 366 [33].

#### AC-4: Metadata

The NorDig HEVC IRD shall support the use of AC-4 metadata according to ETSI TS 103 190  [97] and ETSI TS 103 190 -2 [98] when;

- Decoding AC-4 bitstreams.

- Re-encoding AC-4 bitstreams to AC-3 or E-AC-3.

- Creating a PCM output from an AC-4 bitstream.

- Creating a MAT [99] output where supported.

#### AC-4: Audio Output

Where HDMI, HDMI ARC (or eARC) is implemented, NorDig HEVC IRDs shall be capable of providing the following formats from an AC-4 bitstream (see chapter 16 for factory default settings):

- AC-4 bitstream decoded (rendered up to 5.1 channels) and re-encoded to E-AC-3 at a minimum bit rate of 640 kbps.

- AC-4 bitstream decoded (rendered up to 5.1 channels) and re-encoded to AC-3 at a fixed bit rate of 640 kbps.

- Decoded and downmixed (if > 2 channels) to PCM stereo bitstream

Where HDMI, HDMI ARC (or eARC) is implemented, NorDig HEVC IRDs should be capable of providing the following formats from an AC-4 bitstream:

- AC-4 bitstream decoded (e.g. 5.1.2 channels) and re-encoded using E-AC-3 according to ETSI TS 102 366 [33] in combination with the JOC extension to E-AC-3 according to ETSI TS 103 420 [100].

Where HDMI (or HDMI eARC) is implemented, NorDig HEVC IRDs should be capable of providing the following formats from an AC-4 bitstream:

- Decoded to PCM multichannel bitstream (up to 5.1 channels).

- Decoded to PCM audio bitstreams with metadata (e.g., MAT [99], HDMI 2.1 [93]).

The NorDig HEVC IRD supporting AC-4 and including an S/PDIF output shall be capable of

providing the following formats on the S/PDIF connector from an AC-4 bitstream:

- AC-4 bitstream decoded (up to 5.1 channels) and re-encoded to AC-3 bitstream.

- Decoded and downmixed (if > 2 channels) to PCM stereo bitstream.

## Stereo Audio

The NorDig IRD shall be able to output an audio stream as stereo audio if there is a stereo or mono audio stream present for the chosen service in the incoming transport stream.

See more details in section 6.2 Audio decoding, 6.5 Audio Prioritisation and 6.8 Audio Output Signals.

## Multichannel Audio

Multichannel audio is defined as an audio stream with more than two channels of audio with all loudspeaker positions in one plane. Audio streams that contain more than 5.1 audio channels are considered Immersive Audio, see section below.

The NorDig IRD should be able to output an audio stream as non-PCM encoded audio if there is a multichannel audio stream present for the chosen service in the incoming transport stream.

The multichannel audio shall be decoded to a stereo downmix under the control of the format indicated in the preferred stereo downmix parameter in the audio metadata, i.e. a matrix encoded compatible stereo downmix (Lt/Rt signal) or to a normal stereo downmix (Lo/Ro signal), and shall be fed to the (stereo) analogue output connectors (RCA, headphones and/or built-in loudspeakers if present).

If audio does not include any preferred stereo downmix parameter in the audio metadata, the factory default method should be according to ETSI TS 101 154 [26], Annex C.5.2.4 (Lo/Ro) for AAC. For AC-3, and E-AC-3 and AC-4, there is always audio metadata included together with the audio bitstream.

The choice and priority between different stereo audio streams, and downmixed multichannel audio streams for the analogue outputs shall then be as specified in section 6.5 (Audio prioritisation). When multichannel audio (AC-3, E-AC-3, HE-AAC or AC-4) is present, it is recommended to show a visual indication.

See more details in section 6.5 Audio prioritisation and section 6.8 Audio Output Signals.

## Immersive Audio

NGA services also support Immersive Audio. Immersive Audio is defined as audio containing additional height elements. The height elements may be carried either as additional audio channels, sound field representation, audio objects or a combination of these.

Immersive Audio can be played back using either direct radiating speakers or simulating real speakers through various techniques such as soundbars directing sound in various directions and using sound reflections from walls and ceiling. Using appropriate virtualization techniques, immersive audio can also be experienced through headphones.

An example of immersive audio loudspeaker configuration is an eleven-speaker setup withplus one LFE subwoofer, that has seven loudspeakers positioned on one plane as in multichannel audio and four height loudspeakers at elevated positions, as specified in ETSI/ITU-R BS.2051-2 [103] as system J. This setup is also informally known as 7.1+4H or 7.1.4.

For stereo playback, the immersive audio shall be decoded to a stereo downmix under the control of the format indicated in the preferred stereo downmix parameter in the audio metadata, i.e. a matrix encoded compatible stereo downmix (Lt/Rt signal) or to a normal stereo downmix (Lo/Ro signal), and shall be fed to the (stereo) analogue output connectors (RCA, headphones and/or built-in loudspeakers if present).

The choice and priority between different stereo audio streams, downmixed multichannel audio streams and downmixing of immersive audio streams for the analogue outputs shall then be as specified in section 6.6 (Audio prioritisation). When AC-4 immersive audio is present, it is recommended to show a visual indication.

See more details in section 6.6 Audio Prioritisation and section 6.7 Audio Output Signals.

6.6 Stereo Audio

The NorDig IRD shall be able to output an audio stream as stereo audio if there is a stereo or mono audio stream present for the chosen service in the incoming transport stream.

See more details in section 6.2 Audio decoding, 6.5 Audio Prioritisation and 6.8 Audio Output Signals.

## Audio Prioritisation

### General

The NorDig IRD shall be able to automatically select audio PID/stream according to user preference settings. The NGA capable NorDig HEVC IRD shall, after PID/stream selection be able to automatically select NGA Preselection within an NGA stream according to user preferences.

In addition, the user shall be able to manually select between all audio streams and/or NGA Preselections (1) that are associated with the active service, these settings should be stored in the IRD´s memory separately for each service.

If manually selected audio is not able to be stored in the IRD´s memory per service, a global setting should be made possible to set manual prioritisation of stream type for all services.

For Audio language, Audio type (e.g. Normal or Supplementary Audio mode) and Audio format (stereo or multichannel, or immersive), the priority for the selected audio source shall be based on the user preference settings and the audio PID/stream shall be selected according to the prioritisation schema selection in 6.7.2 for NGA capable NorDig HEVC IRDs and in 6.5.3 for non-NGA capable NorDig IRDs below. (See section 16.2.3 for requirements of audio user preference settings for the NorDig IRD).

If the user preference settings are not matching any of the incoming audio streams, the NorDig IRD shall always select one of the audio streams which most closesly matches the user preference settings and will hereby provide audio to end-user.

The NorDig IRD should behave in an elegant manner (e.g. without audio dropouts etc) and should not change audio stream type depending upon how the content itself may have been produced (2).

Note 1: NGA Preselections only applicable for NGA capable NorDig HEVC IRDs.

Note 2:	The intention is that the NorDig IRD adheres to an audio stream in a semi-constant way, after the IRD has selected which audio stream to decode (according to prioritisation below). Only then does the audio decoder read and apply the Channel Mode audio metadata to the appropriate outputs.

### Audio Prioritisation for NGA capable NorDig HEVC IRD

This section describes how NGA capable NorDig HEVC IRDs shall prioritise and select the audio PID/stream to be decoded for services with several audio PIDs/streams.

NGA capable NorDig HEVC IRD shall prioritise the selection of NGA/AC-4 audio PID/stream over other audio PIDs/streams using other audio codecs when:

- NGA stream signalised a language that matches IRD user preference settings for primary audio language. (See section 6.5.4.1 how IRD shall detect language for PMT signalling), or

- NGA stream does not signalise any language (in PMT), or

- No NGA stream matches the user preference settings for primary audio language and no non-NGA stream matches the IRD user preference settings for primary and secondary audio language.

Otherwise, the non-NGA audio PIDs/streams shall be selected in accordance with section 6.5.3 below.

If NGA capable NorDig HEVC IRD finds more than one NGA stream that matches the primary audio language or finds that none of the NGA streams match the primary audio language, the NGA stream with lowest PID shall be selected.

The selection among available presentation/preselections inside the AC-4 stream, shall be in accordance with section 6.5.6.

### Audio PID/Stream Prioritisation for non-NGA streams

This section describes how NorDig IRDs shall prioritise and select the non-NGA audio PID/stream to be decoded for services with several non-NGA audio PIDs/streams.

For prioritisation between non-NGA audio PIDs/streams (for non-NGA capable NorDig IRDs or NGA capable NorDig IRDs that has not prioritised NGA stream(s) according to section 6.5.2), the audio source priority shall be based on the property of the supported audio streams and for different user preference settings of audio format and audio type as selected by the user according with table 6.1 below. See section 16.4 for factory default user preference settings related to audio.

| IRD settings → |  | Normal audio mode |  | Supplementary audio mode |  |
| --- | --- | --- | --- | --- | --- |
|  |  | stereo mode (factory default) | multichannel mode | stereo mode | multichannel mode |
| IRD behaviour depending on above IRD settings |  |  |  |  |  |
| Property of priority for audio | Priority | Order of priority |  |  |  |
| Audio type | 1 (highest) | 1.1 Normal 1.2 supplementary audio | 1.1 Normal 1.2 Supplementary audio | 1.1 supplementary audio 1.2 Normal | 1.1 supplementary audio 1.2 Normal |
| Language | 2 | 2.1 audio match primay audio language settings 2.2 audio match secondary audio language settings 2.3 (if no match) Normal audio 2.4 (if no Normal audio) any audio | 2.1 audio match primay audio language settings 2.2 audio match secondary audio language settings 2.3 (if no match) Normal audio 2.4 (if no Normal audio) any audio | 2.1 supplementary audio match primay audio language settings 2.2 supplementary audio match secondary audio language settings 2.3 (if no match) Normal audio 2.4 (if no Normal audio) any audio | 2.1 supplementary audio match primay audio language settings 2.2 supplementary audio match secondary audio language settings 2.3 (if no match) Normal audio 2.4 (if no Normal audio) any audio |
| Audio format | 3 | 3.1 stereo 3.2 multichannel 3.3 mono | 3.1 multichannel 3.2 stereo 3.3 mono | 3.1 stereo 3.2 multichannel 3.3 mono | 3.1 multichannel 3.2 stereo 3.3 mono |
| Stream type | 4 (lowest) | 4.1 MPEG-1 Layer II 4.2 HE-AAC 4.3 E-AC-3 4.4 AC-3 | 4.1 HE-AAC 4.2 E-AC-3 4.3 AC-3 4.4 MPEG-1 Layer II | 4.1 MPEG-1 Layer II 4.2 HE-AAC 4.3 E-AC-3 4.4 AC-3 | 4.1 HE-AAC 4.2 E-AC-3 4.3 AC-3 4.4 MPEG-1 Layer II |

Table 6.1 Audio Priority between incoming audio streams where a lower number refers to higher priority.

In the event that the NorDig IRD finds more than one stream with same audio property parameters (type, language, format and stream type) that fully or partially matches the IRD user preference settings, of the remaining streams (which have the same audio properties) the stream with the lowest PID shall be selected.

#### Audio PID/stream Prioritisation, Type (Normal or supplementary)

This section is for selection among non-NGA audio PIDs/streams (for the case that non-NGA capable NorDig IRDs or NGA capable NorDig HEVC IRDs do not prioritise the NGA stream(s) as above in section 6.5.2).

If the IRD’s user preference setting for audio type is set to prioritise Normal audio, for non-NGA audio PID/streams, the NorDig IRD shall only use audio PID/stream of types other than Normal if no Normal audio is available at input for the selected service (see 6.1.2 for definition of Normal audio).

If the IRD’s user preference setting for audio type is set to prioritise Supplementary audio, for non-NGA audio PID/streams, the NorDig IRD shall prioritise Supplementary Audio streams for the selected service that has language matching user preference settings for primary and secondary audio language. If the selected service has no Supplementary Audio with a language matching IRD’s user preference settings for either primary or secondary audio language, the NorDig IRD shall instead prioritise Normal audio (before any Supplementary audio with a language not matching audio language preferences).

#### Audio PID/stream Prioritisation, Languages

If an audio stream according to the primary audio language preference is not associated with the chosen service, the NorDig IRD shall automatically choose the audio stream according to the secondary audio language preference, if present.

#### Audio PID/Stream Prioritisation, Format (multichannel or stereo)

This section is for selection among non-NGA audio PIDs/streams (for the case that non-NGA capable NorDig IRDs or NGA capable NorDig HEVC IRDs do not prioritise the NGA stream(s) as above in section 6.5.2).

If the IRD’s user preference setting for audio type is set to multichannel mode and if both multichannel and stereo (non-NGA) streams are available for the selected language and audio type, the NorDig IRD shall use the multichannel audio stream to provide downmixed audio in analogue audio output(s), if applicable, and suitable digital bitstream format in digital audio output(s).

If stereo mode is selected and if both multichannel and stereo (non-NGA) streams are available for the selected language and audio type, the NorDig IRD shall use the stereo audio source to provide audio in analogue audio output(s), if applicable, and PCM stereo in digital audio output(s) as e.g. in examples in Annex G (Guidelines for NorDig IRD audio selection: “Example table when more than one audio stream is received”).

If the NorDig IRD has an optional external audio system mode, then when that external audio system mode is selected as the primary audio output, the NorDig IRD shall follow user preferences and the priorities in Table 6.1.

### Signalling to be used for audio property

The NorDig IRD shall use the signalling information in the PMT to determine the audio property (language, audio type, audio format and stream type) (1).

| Audio Property | Supplementary audio descriptor | AAC descriptor, Enhanced AC-3 descriptor, AC-3 descriptor | ISO 639 descriptor | PMT stream type | if no signalling/descriptor |
| --- | --- | --- | --- | --- | --- |
| Audio Language | 1st | - | 2nd | - | - |
| Audio type | 1st | 2nd | 3rd |  | Normal |
| Audio format | - | 1st | - | - | stereo |
| Stream type | - | 2nd | - | 1st | - |

Table 6.2 NorDig IRD priority inside each audio property between different signalling for audio streams and right column the IRD’s interpretation if no signalling available for an audio stream.

See Annex I for some example cases of priority between different incoming signalling for the audio property.

The NGA capable NorDig HEVC IRD shall use the signalling information in the PMT to determine the audio property (language, audio type, audio format and stream type).

| Audio Property | Audio Preselection Descriptor (*if supported*) | Supplementary audio descriptor | AAC descriptor, Enhanced AC-3 descriptor, AC-3 descriptor, AC-4 descriptor | ISO 639 descriptor | PMT stream type | if no signalling/descriptor |
| --- | --- | --- | --- | --- | --- | --- |
| Audio Language | 1st (2) | 2nd | - | 3rd | - | - |
| Audio type | 1st | 2nd | 3rd | 4th | - | Normal |
| Audio format | 1st | - | 2nd | - | - | stereo |
| Stream type | - | - | 2nd | - | 1st | - |

Table 6.3 NGA capable NorDig HEVC IRD prioritization based on the available SI and descriptors.

Note 1: The behaviour of the NorDig IRD depends upon this signalling in the PMT to make the audio priority decision, so it is expected that all broadcasts will include this signalling.
Note 2: Audio Language inside the Audio Preselection Descriptor is the only property used for PID/stream selection, while Audio type and Audio format inside the Audio Preselection Descriptor are used for presentation selection.

#### Signalling to be used for audio language

For the selection of audio language (see Table 6.1 and Table 6.2), the NorDig IRD shall use the ISO 639 language code from the supplementary audio descriptor and/or the ISO 639 language descriptor.

The NGA capable NorDig HEVC IRD shall use the ISO_639_language_code from all preselections in the Audio Preselection Descriptor.

If several of these descriptors are available for the same audio stream describing language, then the language inside the Audio Preselection Descriptor shall have highest priority, the supplementary audio descriptor has second priority and lowest priority is ISO 639 descriptor.

#### Signalling to be used for audio type

For the selection of audio type (see Table Table 6.1 and Table 6.2), the NorDig IRD shall use the signalling from supplementary audio descriptor (mix_type and editorial_classification), AAC descriptor (AAC_type), Enhanced _AC-3_descriptor (service type flags), AC-3 descriptor (service type flags) and/or the ISO 639 langauge descriptor (audio_type).

If several of these descriptors are available for the same audio stream, then the supplementary audio descriptor shall have highest priority, second priority is the AAC/Enhanced_AC-3/AC-3 descriptor and lowest priority is the ISO 639 descriptor.

Audio type shall be ignored for NGA PID/stream selection. The audio type that is used for audio prioritisation inside the NGA PID/stream is taken either from the Audio Preselection Descriptor or from the ac4_toc, as specified in ESTI TS 103 190–2 [98] in the elementary stream.

It is assumed that an NGA stream carries all audio types for each language for the service, for example Normal audio, audio descripton and spoken subtitling are carried within the same NGA PID/stream, therefore the audio type in the audio descriptors is normally set to ‘undefined’ even though the NGA stream carries Supplementary audio.

#### Signalling to be used for audio format

For the selection of audio format (mono, stereo or multichannel, see see Table 6.1 and Table 6.2), the NorDig IRD shall use:

- the AAC_type field in the AAC_descriptor for AAC audio,

- the number of channels flags in the AC-3 descriptor and Enhanced AC-3 descriptor for AC-3 and E-AC-3.

In any case where, for some reason, this information is not carried in the PMT for a particular audio stream, then the IRD shall prioritise based on the assumption that that audio stream contains Normal stereo content.

Audio format shall be ignored for NGA stream/PID selection. The audio format is also not relevant for audio prioritisaion inside the NGA PID/stream. The audio format that is used for audio prioritisation inside the NGA PID/stream is taken either from the Audio Preselection Descriptor or from the ac4_toc, as specified in ESTI TS 103 190–2 [98] in the elementary stream.

#### Signalling to be used for audio stream type

For the selection of audio stream type (see see Table 6.1 and Table 6.2), the NorDig IRD shall use stream_type in the PMT and any complementing descriptor (AAC, Enhanced AC-3, or AC-3 descriptor or AC-4 descriptor) to decide which audio codec the stream has.

For example, the presence of an AC-4 descriptor in the ES_info_loop of the PMT indicates it is an AC-4 audio stream.

### Examples of priority

Below are two examples aimed at explaining how the priority table shall be used. In general, the IRD’s priority decision is completed once a single stream is identified when moving down the priority table. Hence it can occur that e.g. 4th priority rules do not have to be applied because the streams to decode have already been identified.

Example 1: IRD is in stereo mode, two audio streams are available to the IRD, both have same language, normal hearing, and are stereo, stream A is coded using HE-AAC and stream B is encoded using MPEG-1 Layer II. IRD shall decode and playback MPEG-1 Layer II since, priority 1, 2 or 3 does not provide a preference between the two streams, but MPEG-1 Layer-II has higher priority than HE-AAC.

Example 2: IRD is in multichannel mode, two audio streams are available to the IRD, both have same language, normal hearing, stream A is multichannel coded using E-AC-3 and stream B is stereo encoded using HE-AAC. IRD shall decode and playback the E-AC-3 stream since, priority 1, 2 does not provide a preference between the two streams, but the IRD is in multichannel mode and there is only one multichannel stream; the E-AC-3 stream. In this example, the stream type priority is not used since already in the audio format priority a single stream could be identified.

In case of multiple audio streams being broadcasted, a clarifying example table can be found in Annex G “Example table when more than one audio codec is received”, for how to select which audio stream to decode in multichannel or stereo user selected situation. That table is given by the property rules of Table 6.1.

### Audio Prioritisation inside the NGA Audio PID/stream

This section applies only to the NGA capable NorDig HEVC IRD.

Audio Prioritisation inside the NGA Audio PID/stream can be done based on the Audio Preselection Descriptor or based on the ac4_toc, as specified in ETSI TS 103 190–2 [98] in the elementary stream.

The information from the elementary stream (ac4_toc) shall be used for audio prioritisation inside the NGA audio PID/stream based on the user preference settings.

For manual selection, the information from the Audio Preselection Descriptor (APD) shall be used.

The NGA audio stream(s) may contain additional or fewer preselections than those listed in the Audio Preselection Descriptor.

If the presentation corresponding to a manually selected preselection from the Audio Preselection Descriptor is not found in the bitstream, then the IRD shall fall back to preference-based presentation selection based on IRD settings and signalling in the elementary stream as specified above.

In case the presentation currently playing does not have corresponding information in the Audio Preselection Descriptor (APD), the information should be extracted from the elementary stream (ac4_toc) or, if that is not feasible, a default called “currently playing” or similar could be displayed.

Similarly, when a presentation present in APD but not present in the elementary stream (ac4_toc) is manually selected by the user, the NorDig HEVC IRD applies preference-based presentation selection based on the elementary stream (ac4_toc), resulting in a selection different to the user’s. This selection should be reflected in the UI by extracting the information from the elementary stream (ac4_toc) or, if that is not feasible, a default called “currently playing” or similar could be displayed. Additionally, when a presentation present in APD but not present in the elementary stream (ac4_toc) is manually selected by the user, the NorDig HEVC IRD may inform the user that the selected audio presentation is not available in the current broadcast. It is expected that quasi-static APD signaling could result in an overrepresentation of presentations in the APD e.g., during advertisement breaks, where during the advert break there could be only one audio track available to all languages.

Language:
For audio prioritisation inside the NGA PID/stream based on the Audio Preselection Descriptor the NGA capable NorDig HEVC IRD shall use the ISO 639 language code per preselection. For audio prioritisation inside the NGA PID/stream based on the ac4_toc, as specified in ETSI TS 103 190–2 [98] in the elementary stream the NGA capable NorDig HEVC IRD shall use the primary language subtag of the IETF BCP 47 [101]language code in the content_type of the main audio or dialogue substream group.

Audio Type:
For audio prioritisation inside the NGA PID/stream based on the Audio Preselection Descriptor the NGA capable NorDig HEVC IRD shall use the audio_description and spoken_subtitles flags per preselection. For audio prioritisation inside the NGA PID/stream based on the ac4_toc, as specified in ETSI TS 103 190 – 2 [98] in the elementary stream, the NGA capable NorDig HEVC IRD shall use the presentation_config and, if present, the content_classifier of the associated audio substream group. 

Audio Format:
NGA preselections can carry rendering information for immersive, multichannel, and stereo playback scenarios. Therefore, the audio format is not relevant for audio prioritisation inside the NGA PID/stream.

Audio Prioritisation:
The NGA capable NorDig IRD shall prioritise the preselection in the NGA stream according to Table 6.4. below.

Table 6.4 uses the terminology from the Audio Preselection Descriptor, however the ac4_toc is used for audio prioritisation based on user preferences. The ac4_toc functionality is typically part of audio decoder components. Therefore, the corresponding properties for Audio Type and Language are used which are described in the section above.

| IRD settings → | Audio Description (AD) OFF |  | Audio Description ON |  |
| --- | --- | --- | --- | --- |
|  | Spoken Subtitles (SpS) OFF | Spoken Subtitles ON | Spoken Subtitles OFF | Spoken Subtitles ON |
| IRD behaviour depending on above IRD settings and preselection properties |  |  |  |  |
| Priority | Priority with respect to best matching preselection |  | Priority with respect to best matching preselection |  |
| 1 | AD off and SpS off Preselection matching primary audio language | AD off and SpS on Preselection matching primary audio language | AD on and SpS off Preselection matching primary audio language (1) | AD on and SpS on Preselection matching primary audio language |
| 2 | AD off Preselection matching primary audio language | SpS on Preselection matching primary audio language | AD on and SpS on Preselection matching primary audio language | AD on and SpS off Preselection matching primary audio language |
| 3 | Preselection matching primary audio language | Preselection matching primary audio language | SpS off Preselection matching primary audio language | SpS on Preselection matching primary audio language |
| 4 | AD off and SpS off Preselection matching secondary audio language | AD off and SpS on Preselection matching secondary audio language | Preselection matching primary audio language | Preselection matching primary audio language |
| 5 | AD off Preselection matching secondary audio language | SpS on Preselection matching secondary audio language | AD on and SpS off Preselection matching secondary audio language settings | AD on and SpS on Preselection matching secondary audio language |
| 6 | Preselection matching secondary audio language | Preselection matching secondary audio language | AD on and SpS on Preselection matching secondary audio language | AD on and SpS off Preselection matching secondary audio language |
| 7 | (if no match) default Preselection | (if no match) default Preselection | SpS off Preselection matching secondary audio language | SpS on Preselection matching secondary audio language |
| 8 |  |  | Preselection matching secondary audio language | Preselection matching secondary audio language |
| 9 |  |  | (if no match) default Preselection | (if no match) default Preselection |

Table 6.4 Audio Priority between NGA Preselections for NGA “Accesibility Services” with Audio description on/off and Spoken Subtitles on/off. A lower number refers to higher priority.

## Audio Output Formats

The following Table 6.5 shows the expected output format based on the currently decoded audio and per digital audio out setting (multichannel mode or stereo mode) as selected by the user. It should be noted that this section defines minimum requirements; no restriction to use any new and innovative method to carry audio via digital audio interfaces is implied.

Please note that Table 6.5 defines audio output formats after audio priority has been made between incoming audio streams (see section 6.5 and Table 6.1 for audio priority between incoming streams).

| Currently Decoded Audio | Output on S/PDIF and HDMI (incl. HDMI ARC and HDMI eARC, see note 10) (see note 5) |  | Output on 2 ch analogue output(s) and/or built in loudspeaker(s) (see note 6) |
| --- | --- | --- | --- |
|  | When Stereo mode is selected | When multichannel mode is selected |  |
| MPEG-1 layer II | PCM stereo | PCM stereo (see note 8) | Decoded MPEG-1 Layer II |
| HE-AAC stereo | PCM stereo | PCM stereo or optionally HE-AAC stereo on HDMI (if IRD supports HE-AAC on HDMI, see note 4) HE-AAC transcoded to AC-3 or DTS on S/PDIF | Decoded HE-AAC |
| E-AC-3 stereo | PCM stereo | E-AC-3 on HDMI (see note 2) E-AC-3 transcoded to AC-3 on S/PDIF (see note 1) | Decoded E-AC-3 |
| AC-3 stereo | PCM stereo | AC-3 (see note 1) | Decoded AC-3 |
| AC-3 multichannel | Downmixed PCM stereo | AC-3 (see note 1) or PCM multichannel (see note 7) | Decoded downmixed AC-3 |
| E-AC-3 multichannel | Downmixed PCM stereo | E-AC-3 on HDMI (see note 2) E-AC-3 transcoded to AC-3 on S/PDIF (see note 1) PCM multichannel (see note 7) | Decoded downmixed E-AC-3 |
| HE-AAC multichannel | Downmixed PCM stereo | HE-AAC multichannel on HDMI (if IRD supports HE-AAC on HDMI, see note 4) HE-AAC transcoded to AC-3 or DTS on S/PDIF PCM multichannel (see note 7) | Decoded downmixed HE-AAC |
| AC-4 | Rendered to PCM stereo | AC-4 on HDMI (if IRD supports AC-4 on HDMI, see note 9) E-AC-3 including JOC on HDMI (see notes 2, 3) AC-3 on S/PDIF (see note 1) PCM multichannel (see note 7) MAT (PCM and metadata, see note 10) | AC-4 decoded and rendered to stereo signal |

Table 6.5 Audio Output formats.

Note 1:	The S/PDIF output shall in any case comply with the content of the table above. For HDMI however, the following feature should be implemented:
If an HDMI, HDMI ARC (or HDMI eARC) receiving device indicates that it only supports Basic Audio (i.e. two-channel L-PCM from the original stereo signal or from a stereo down-mix from the multichannel signal), then the IRD output will be Basic Audio. This feature would then take precedence over the requirement of AC-3, E-AC-3, HE-AAC multichannel and DTS in the table above whenever the receiving device indicates that only Basic Audio is supported. Observe however that the HDMI output could be different from S/PDIF output, since S/PDIF still has to comply with multichannel format requirements as in the table above.

Note 2:	If an HDMI, HDMI ARC or HDMI eARC receiving device indicates that AC-3 decoding is supported, but E-AC-3 decoding is not supported, the IRD shall transcode E-AC-3 or AC-4 streams to AC-3 prior to HDMI transmission.

Note 3: If an HDMI, HDMI ARC or HDMI eARC receiving device indicates that E-AC-3 decoding is supported, but AC-4 or MAT decoding is not supported, AC-4 Immersive audio shall be encoded into E-AC-3 including the JOC extension containing the additional immersive height elements using relevant metadata

Note 4:	For IRDs that support the optional output HE-AAC on HDMI: If an HDMI, HDMI ARC or HDMI eARC receiving device indicates that AC-3 or DTS is supported, but HE-AAC decoding is not supported, the IRD shall transcode HE-AAC streams to AC-3 or DTS prior to HDMI transmission.

Note 5:	For IDTVs, this shall apply when External Audio System (if supported) is selected as audio output. If built-in loudspeakers are selected for audio output the minimum requirement is to have PCM output signal in the digital output, where supported.

Note 6:	When external audio system (if supported) is selected as main audio output, IDTVs may
optionally mute the TV speakers.

Note 7:	If the IRD supports PCM multichannel over HDMI or HDMI eARC (not applicable for HDMI ARC).

Note 8: If content is stereo, output shall be PCM stereo also in case of multichannel mode.

Note 9: For IRDs that support the optional output AC-4 on HDMI: If an HDMI, HDMI ARC or HDMI eARC receiving device indicates that E-AC-3 decoding is supported, but AC-4 decoding is not supported, the IRD shall re-encode AC-4 streams to E-AC-3 prior to HDMI transmission. If an HDMI, HDMI ARC or HDMI eARC receiving device indicates that AC-3 decoding is supported, but neither of E-AC-3 or AC-4 decoding is supported, the IRD shall re-encode AC-4 streams to AC-3 prior to HDMI transmission.

Note 10: NorDig recognises that HDMI eARC will enable delivery of multichannel audio between suitably equipped iDTVs and A/V decoders. Revisions of this document will consider including recommendations/requirements regarding this output method. For example, MAT [99].

## Audio Video Synchronisation

The NorDig IRD shall not introduce more than ±5 ms of relative delay between the audio and video components on the primary output (1) and not more than ±25 ms between the primary video output and a secondary audio output (2).

The requirement for the relative delay shall be met from 5 seconds after the start of decoding a service. The relative delay between the audio and video components shall be synchronised continuously using the service’s PCR and PTS values in the MPEG transport stream, resulting in maintaining correct synchronisation until a change of service.

If the NorDig IRD, as a part of an integrated digital TV set (IDTV) has an audio output, the audio and/or decoded PCM digital audio output shall be in sync with the video display.

Where audio leaves the IRD in an encoded form (such as in IEC 61937 [41] bitstream from S/PDIF, HDMI, HDMI ARC or HDMI eARC outputs, the IRD shall compensate for the decoding latency of the selected audio format, as specified for the relevant reference decoder for the selected format (e.g AC-3), such that the output of the reference decoder would be ±5 ms with respect to the decoded video. This applies for all audio systems that the IRD supports.

Note 1:	In this case, primary output for STB is primary HDMI and for IDTV is the video display and internal loudspekars (see sections 8.5 and 8.6).
Note 2:	In this case, primary video output is the same as note 1. Secondary audio outputs for STB are i.e. secondary HDMI, analogue RCA, and S/PDIF (see section 8.5). Secondary audio outputs for IDTV are i.e. analogue RCA, S/PDIF, HDMI ARC, HDMI eARC, headphone output, and “external audio system”.

### Adjustment of Audio/Video delay

The NorDig IRD (1) shall support the possibility to adjust the audio-delay on the HDMI and S/PDIF output (if available) at least up to 250 ms and it should be adjustable in 5 ms steps or smaller steps, as the IRD may have several different user set-ups, resulting in different a/v delays, e.g. the IRD may be connected to several types of external audio amplifiers and the IRD may be connected to several types of external screens.

The NorDig IRD should also be able to automatically adjust the audio/video synchronization via HDMI (minimum) version 1.3 interface to compensate for delay times of video displays.

Note 1: Optional (recommended) for NorDig IDTV.

## Audio Output Signals

### Analogue Audio Output (RCA)

If the IDTV has an analogue stereo audio output on a pair of RCA connectors and/or headphone connector, the audio shall be in sync with the video display, see section 6.7.

If the NorDig IRD has analogue stereo output(s), it shall be capable of decoding and downmixing the supported audio formats for the analogue outputs.

The NorDig IRD with analogue outputs shall always have an audio signal present on the analogue outputs (stereo out, see section 8.5 and to built-in loudspeakers (see note 1) if any of the supported formats is received.

For the NorDig IRD with analogue audio outputs (e.g. built-in loudspeakers), it shall be possible to control the audio level on the analogue outputs primarily used for TV viewing (e.g. built-in loudspeakers) with the remote-control unit and buttons on the IRD (if present). It should be possible to control the audio level on other analogue audio outputs (like RCA, headphone etc) using the same controls.

Note 1: 	For IDTVs that provide a setting to choose the audio output between the built-in TV speakers and the external audio system, it is optional to have signal in the built-in TV speakers when the external audio system setting is selected. The reasons for this are that when the user selects output to an external audio system as the priority, then the audio levels and compression settings are optimized for an external audio system and may not be suitable for output to TV speakers (e.g., lower dialogue level, high dynamic range).

### Digital Audio Output (HDMI, HDMI ARC, HDMI eARC and S/PDIF)

For NorDig iDTV that cannot reproduce multichannel audio using built-in loudspeakers and/or connected loudspeakers, it should have an S/PDIF output and/or an HDMI Audio Return Channel (HDMI ARC) and/or HDMI eARC output, see sections 8.5 and 8.6.

The NorDig STB shall have an HDMI output and should have an S/PDIF output, see sections 8.6 (HDMI audio) and 8.5.3 (S/PDIF).

For NorDig STB, where only PCM audio will be carried on the HDMI output primarily used for TV viewing (e.g. stereo only HDMI sink (2), selection of “Stereo mode” by the user), it shall be possible to control the audio level on this HDMI with the remote control unit and buttons on the IRD (if present). Where only PCM audio will be carried on other digital audio outputs (S/PDIF, secondary HDMI port etc), it should be possible to control the audio level on other digital audio outputs using the same controls.

The digital audio output as defined in section 8.5.3 (S/PDIF) and 8.6 (HDMI) shall always give either a valid PCM-output according to IEC 60958 [40] or a non-PCM audio bit-stream according to IEC 61937 [41]. The user shall be able to choose between the following storable output modes on the digital audio output interface (1):

- Forced PCM output according to IEC 60958 [40] (Part1 General, Part 3 Consumer).

- Non-PCM audio bit-stream output according to IEC 61937 [41] when present and if not present output PCM according to IEC 60958 [40]. Non-PCM audio bitstream formats like AC-3, and DTS shall be possible to order and enable/disable according to priority set by the user.

- This chapter is also valid for HDMI, HDMI ARC and HDMI eARC, as HDMI transmits digital audio according to IEC 60958 [40] and IEC 61937 [41]. Observe that HDMI supports more stream types than S/PDIF.

See sections 6.3 and 6.5 for more details.

Note 1: 	The IRD’s different digital audio outputs may use use different audio output formats, for example
 HDMI using PCM stereo and S/PDIF using non-PCM audio bitstream (like AC-3 or DTS).
Note 2:	A “stereo only HDMI sink” refers here to a downstream device to which the STB is connected to  and which can only render stereo audio, for example a TV set without a multichannel decoder.

## Dynamic Changes in the audio streams

The NorDig IRD shall be able to handle dynamic changes of audio component(s) (PID/PIDs/preselections) in a service. The IRD shall automatically identify if an audio component is added or removed. The NorDig IRD should gracefully handle change of service or audio format at the audio outputs without significant disturbances to the end user.

The NorDig IRD shall be able to handle the following dynamic changes without user interaction and start decoding within one second after reception of a change (like PMT update, elementary stream header signalling, audio metadata):

- change of number of audio channels (still same audio codec), (for example from mono to stereo to multichannel, from multichannel to stereo to mono, from immersive to multichannel to stereo to mono, from mono to stereo to multichannel to immersive).

- changes in audio metadata such as loudness, downmix coefficients, DRC, etc. (this does not apply to MPEG-1 Layer II.).

- change of bitrate for an audio component (for example from 192 kbps to 160 kbps).

- change of audio PID value (typical examples are during regional news insertions) (case SVT regional news insertion), the NorDig IRD shall use next preferred audio component/PID.

- removal of one or more audio component(s), the NorDig IRD shall use next preferred audio component (for example; a service broadcasts two or more audio components and then removes one of them, like removal of AC-3 during regional news broadcast) (case SVT regional news insertion).

- addition of one or more audio component(s) with higher preferred user settings (for example; a service adds an audio component/PID, like add AC-3 audio component, after a regional news broadcast) (case SVT regional news insertion).

The NorDig IRD shall handle the dynamic changes after change of selected service ("zapping") or dynamic PMT update (i.e., shall not require to re-install services) and shall be able to handle the following dynamic changes without user interaction and start decoding within one second after reception of change:

- change of the audio codec, (for example change from MPEG-1 Layer II into AC-3).

- change of ISO 639-2 [68] language for an audio component.

- AC-4 internal configuration changes: Change of accessibility service(s), Audio Description, language, see 6.7.4 (Multiple audio programme components) of ETSI TS 101 154 [26].

The NorDig IRD shall be able to read the audio information contained in the DVB_SI stream_content and component_type of the component descriptor as defined in ETSI EN 300 468 [13], see also chapter 12 and section 13.3.2. The NorDig IRD should be able to present the audio information, including the descriptors for audio description for the visually impaired and audio for the hard of hearing, contained in the component descriptor to the user for information and selection purposes.

## Dialogue enhancement

Next Generation Audio enables Dialogue Enhancement which gives the possibility to adjust the relative level of the dialogue, to aid speech intelligibiltiy or to suit the preference of the user. See section 16.2.3 for the user preference settings that are related to Dialogue Enhancement.

## Supplementary Audio for Accessibility Services

This section describes how NorDig IRDs shall handle supplementary audio used for non-NGA PIDs/streams Audio accessibility services.

Note: The term supplementary audio may in other contexts also be used other purposes, like commentary audio etc, but this NorDig section focus upon the audio accessibility cases.

Note: In ETSI specifications, NGA refers to preselections for supplementary audio.

### Informative about for Supplementary Audio

A supplementary audio service (as defined in ETSI TS 101 154 [26]) is specified below for the “in-service delivery” and applies when (non-NGA) Normal audio PIDs/streams audio programme components and the supplementary audio PIDs/streams/audio programme components are available within the same service (i.e. listed within same PMT). For non-NGA the different audio streams (normal and supplementary) refer to delivery to the IRD as different audio PIDs while for NGA different audio streams normally refers to audio programme components within a common NGA audio PID. (This means for NGA, no separate PIDs/streams are required for the supplementary audio). A supplementary audio service as specified in this section 6.11, including all subsections, does not apply to the NGA PIDs/streams. Accessibility services for NGA PIDs/streams are specified in section 6.14.

A Supplementary Audio (SA) service may be broadcast as either:

- For non-NGA audio streams:

  - “Broadcast mixed”:  pre-mixed audio by the broadcaster where the Supplementary Audio stream is a complete self-standing audio which contains both the Normal audio mixed together with the supplementary audio content.

  - “Receiver mixed”: audio containing only the supplementary audio content which is not a complete self-standing audio and is not intended to be presented on its own. The receiver mixed supplementary audio and Normal audio is typically mixed together inside the IRD, under some control of the broadcaster (mixing level).

- For NGA audio streams:

  - “Premixed”: the Supplementary Audio content is pre-mixed by the broadcaster with the Normal audio content. Although the Premixed Supplementary audio is a self-standing audio programme component, its multiplexed into the same audio elementary stream/PID as the Normal audio programme component. Inside the IRD, the NGA decoder selects the desired element audio programme component for decoding.

  - “Decoder mixed”: the Supplementary Audio programme component contains only the supplementary audio content which is not a complete self-standing audio and is not intended to be presented on its own. The supplementary audio programme component is typically multiplexed into the same audio elementary stream/PID as the Normal audio programme component. Inside the IRD, the NGA decoder mixes, after decoding both audio programme components, the supplementary and Normal audio programme components, under some control of the broadcaster (mixing level).

### General requirements for supplementary audio for Accessibility Services

The Nordig IRD’s audio decoder shall be capable of supporting ‘visual impaired’ Supplementary audio (SA) services, as defined in ETSI TS 101 154 [26] (however, control of pan and fade is optional).

The NorDig IRD shall (1) support both Broadcast mixed and Receiver mixed Supplementary Audio.

The NGA capable Nordig HEVC IRD audio decoder shall be capable of supporting the NGA supplementary audio for Accessibility services “Audio Description”, “Spoken Subtitles” and “Dialogue Enhancement” within NGA audio streams, as defined in section 6.1.

### IRD settings for Supplementary Audio for accessibility services

#### IRD user preference settings for Supplementary Audio

The user shall be able to enable and disable Supplementary audio and to change the default user preference setting(s), (see section 16.416.2 for IRD factory default settings).

The NorDig IRD shall enable a persistent setting user selection of audio preferences for ‘Normal’ and ‘Supplementary’ Audio (see section 6.1.1) , as defined in section 16.2.  shall be a fixed setting (i.e. remain when changing service and when re-starting the IRD). (The wording for supplementary audio may typically be ‘Audio Description’, ‘AD’, referring to all versions of supplementary audio types).

#### Temporary selection for Supplementary Audio

The NorDig IRD should enable an easy, but temporary, method to have an alternative metod to temporarily select supplementary audio, e.g. via an audio key or an AD key on the remote control (see also section 13.6), (i.e. a temporary selection that does not change the IRD’s stored user preference settings). This temporary selection of ‘Audio Type’ shall not remain when changing service, language, stream type or when re-starting the IRD.

#### Audio priority for Supplementary Audio

Enabling a Supplementary Audio mode shall change the ‘Audio Type’ priority (see section 6.5). If no Supplementary audio streams areis received, then the NorDig IRD shall use Normal audio from the same selected language.

If several supplementary audio streams/audio programme components are available for the same language and the same IRD user supplementary audio preference settings (for example both Audio Description and Spoken Subtitling), then the IRD shall by default prioritise the audio that is first listed in the PMT.

If no NGA Supplementary audio for Accessibility service preselection is available, then the NorDig HEVC IRD shall use Normal audio preselection from the same selected language.

#### Display of available Supplementary audio streams

The IRD should present information to the user if the service has a supplementary audio available, for example in “info banner” after selecting such a service.

If the IRD offers the option to select a supplementary audio track as a temporary selection, then the IRD should indicate that this is of a supplementary audio type (preferably also which audio type it is; Audio Description, spoken subtitles and/or dialogue enhancement) together with its language.

#### Audio mixing level

The NorDig IRD shall be able for non-NGA to adjust the relative audio mixing level between the incoming receiver mixed supplementary audio and the Normal audio and for NGA to adjust the relative audio mixing level between the incoming decoder mixed type supplementary audio programme components, and the Normal audio programme components.

This relative adjustment of audio mixing level should be able to both enhance and reduce the supplementary audio level compared to the Normal audio before mixing depending on user settings.

### Selection of audio streams

The NorDig IRD shall be able to select the correct audio type stream according to user preference settings, see section 6.5.

When the user has selected Normal audio as preferred audio, the NorDig IRD shall by default select the Normal audio stream for services which have several audio streams with different audio type for selected language.

When the user has selected Supplementary Audio as preferred audio, the NorDig IRD shall by default select the supplementary audio stream for services which have several audio streams with different audio type for selected language.

The NorDig IRD with separate user preference settings for Audio Type selection of Audio Description and Spoken Subtitles (see section 16.2.3) shall select the appropriate Supplementary audio stream based for non-NGA audio on supplementary_audio_descriptor signalling and for NGA audio on audio_preselection_descriptor. Furthermore, if the non-NGA Supplementary Audio is signaled via ISO 639(1) language descriptor but no supplementary_audio_descriptor (i.e. supplementary_audio_descriptor is missing, meaning that the IRD can not distinguish if non-NGA Supplementary Audio is Audio Description or Spoken Subtitles), the non-NGA Supplementary Audio shall be selected if any of the user preferences settings Audio Description or Spoken Subtitles are turned on.

Observe that the Supplementary Audio descriptor overrides the language and audio type stated in the ISO639 descriptor, see section 6.5.4.

Note 1: ISO_639_language_descriptor audio_type 0x03 (visually impaired commentary) is used in combination with one particular language, while Normal Audio is signalled as audio_type 0x00.

### Signalling for Supplementary Audio

All non-NGA Supplementary Audio streams (both Broadcast mixed, and Receiver mixed) will be signalled by the broadcaster by means of Supplementary Audio descriptors and for services with several audio streams (e.g one Normal/ plus one Supplementary Audio stream) also an by ISO 639 language descriptors.

All NGA Supplementary Audio will be signalled in the broadcast stream by means of the Audio Preselection Descriptor (see section 12.6.12).

The signalling of the NGA Supplementary Audio in the Audio Preselection Descriptor shall be mapped to the NGA codec specific values according to ETSI EN 300 468 [13] Table M.1.

Non-NGA Broadcast mixed Supplementary Audio streams will, in the ISO 639 descriptor use audio type 0x00 ‘undefined’ (to avoid issues in legacy IRDs).

Non-NGA Receiver mixed Supplementary Audio streams will in the ISO 639 descriptor use audio type 0x03 ‘visual impaired’ or 0x00 ‘undefined’ (to avoid issues in legacy IRDs).

See section 6.5.1 for priority between different descriptors for audio streams and section 12.1.6 for more information about the descriptors for audio.

Legacy IRDs currently in the market are known to handle non-NGA supplementary audio in a variety of ways, and there are some cases where legacy IRDs are even “disturbed” by the presence of Supplementary Audio. To mitigate this and avoid unwanted behaviour, some Networks use special signalling for the Supplementary Audio. This means for example that in some networks a broadcast pre-mixed supplementary audio may be signalled in the ISO639 descriptor as Normal (‘undefined’) audio type but with language code ‘nar’, or a receiver mixed supplementary audio may be signalled in the ISO639 descriptor as ‘visual impaired’ audio type but with a different language to that of the associated Normal audio. Language code ‘nar’ refers here to as ‘narrative’.

The IRDs may/should support to translate language code ‘nar’ in ISO639 descriptor as ‘narrative’ language for the user and may/should support ‘narrative’ as user selectable for audio language in user preference settings. The IRD may interpret audio signalled with language code ‘nar’ in ISO639 descriptor as Broadcast mixed Supplementary Audio (if no supplementary audio descriptor is available for the PID/stream).

### Receiver mixing

The NorDig IRD shall support (non-NGA) receiver mixing/(NGA) decoder mixing of Supplementary Audio services encoded as MPEG-1 Layer II, HE-AAC, and E-AC-3 and AC-4 coding standards, where those codecs are supported.

The NorDig IRD shall for non-NGA audio support Receiver mixed Supplementary Audio when the two audio streams are of the same codec family, sampling rate and on two different PIDs.

The NorDig IRD is not required to support receiver mixing when Normal and Supplementary Audio are broadcast within the same E-AC-3 stream/PID.

The NorDig HEVC IRD shall support (NGA) decoder mixing when Normal and Supplementary audio programme components are broadcast within the same NGA stream/PID.

The NorDig IRD shall be able to mix the Normal audio stream/audio programme component together with the supplementary audio stream/audio programme component (‘visual impaired’).

The NorDig IRD shall not decode a (non-NGA) ‘receiver mixed’ audio stream /(NGA) ’decoder mixed’ supplementary audio programme component without also decoding and presenting it with the associated Normal audio stream either mixed to the same audio output, or optionally presenting Supplementary Audio on its own on the headphones output while presenting the Normal audio on the other audio outputs, see section  6.11.7.

#### Receiver mixed, Pan and fade control

For MPEG-1 Layer II and HE-AAC, the NorDig IRD should support pan and fade control of the receiver mixing via inband control metadata in the audio stream as described in ETSI TS 101 154 Annex E [26] and for E-AC-3 as in TS 102 366 [33].

For MPEG-1 Layer II and HE-AAC, the supplementary audio stream may carry the AD descriptor in its PES_private_data, as defined in ETSI TS 101 154 [26] Annex E.2. This AD descriptor provides the audio decoder with the information needed to control the mixing of the supplementary stream with the main stream (pan and fade). For MPEG supplementary audio streams without an AD descriptor in the PES header, the NorDig IRD shall use mixing as central forward presentation (pan = 0x00) and no fade (fade = 0x00).

For the E-AC-3 codec, the mixing data is carried in the supplementary E-AC-3 stream, as defined in ETSI TS 102 366 [33], and, for this codec, any AD descriptor shall be ignored.

### Supplementary audioReceiver mixed on its own for headphones output

The IRD with headphones output should also support presenting the non-NGA receiver mixed Supplementary Audio on its own on the headphones interface and at the same time presenting the Normal audio on its own on the other main audio outputs (HDMI, S/PDIF etc).

If NGA decoder-mixed Preselection (1) is available, the Nordig HEVC IRD with headphones output should also support presenting the NGA supplementary audio programme component on its own on the headphones interface and at the same time presenting the NGA Normal programme component on its own on the other main audio outputs (HDMI, S/PDIF etc).

The reason behind this alternative is that:

- other family members may not want to be disturbed by the Supplementary Audio, but still be able to watch a programme together with family members who are helped by the Supplementary Audio and/or

- use of a separate loudspeaker for the Supplementary Audio compared to the Normal audio, which can help to localise the different audio types and thereby give better intelligibility.

Note 1: NGA decoder-mixed preselections can’t be distinguished from pre-mixed preselections solely from the APD. Instead, inspecting the NGA audio stream is required. (For AC-4, see ETSI 103 190-1 [x], clause 4.3.3.8.8)

### Broadcast mixed

The NorDig IRD shall support (non-NGA) broadcast mixed/(NGA) premixed supplementary audio services encoded as MPEG-1 Layer II, HE-AAC, AC-3, and E-AC-3 and AC-4 coding standards, where those codecs are supported.

## IRD Internal Reference Level

The level for reference tones for transmission will be 18 dB below clipping level, in accordance with EBU Recommendation R.68 Alignment level in digital audio production equipment and in digital recorders as recommended by ETSI TS 101 154 [26].

## Loudness Levels – Dynamic Range Control and Downmixing

To achieve loudness and dynamic range consistency over codec formats, the Nordig IRD shall follow the following guidelines on audio levelling and dynamic range control. This section does not cover NGA AC-4. For NGA streams, see section 6.15.

A typical implementation for loudness levels on an IDTV-implementation is found in Annex H, “Loudness levels – Typical IDTV Audio Block diagram”.

### HE-AACAudio Input and , AC-3/E-AC-3 and AC-4 Audio Input

The NorDig IRD shall, by default, be able to present the decoded HE-AAC /AC-3/
E-AC-3 and AC-4 bitstream at an average loudness level of -23 LUFS. Where prog_ref_level as of ISO/IEC 14496-3 [53] or dialnorm as of ETSI TS 102 366 [33] or ETSI TS 103 190-1 [97] is included in the stream, it shall indicate the loudness of the audio essence contained within the bitstream using the numerically equivalent dBFS value (e.g. a measured loudness of -23 LUFS shall be indicated by a prog_ref_level / dialnorm value of -23 dBFS).

#### HE-AAC Decoding

For decoding to a Target Reference Level of -23 dBFS, the following applies:

If program level data of the form prog_ref_level as of ISO/IEC 14496-3 [53] is present in the bitstream, it shall be used; otherwise a prog_ref_level of -23 dBFS shall be used.

If drc_presentation_mode as of ETSI TS 101 154 Annex C.5.2.2.3 [26] is present in the bitstream and indicates DRC Presentation Mode 2, then any dynamic range control data of the form dyn_rng_sgn/dyn_rng_ctlas of ISO/IEC 14496-3 [53] also present in the bitstream shall be used and may not be scaled if they indicate negative gains. Otherwise if dynamic range data of the form compression_value as of ETSI TS 101 154 Annex C 5.2 [26] or of the form compression_value as of ISO/IEC 14496-3 [53] is present in the bitstream, it shall be used and it may be scaled.

It is expected that the broadcaster shall ensure that sufficient headroom and/or dynamic range control values are included in the transmission to prevent any overload when downmixing.

For NorDig IRDs that supports a multichannel audio output, the Nordig IRD should be able to decode utilizing a Target Reference Level of -31 dBFS. See section 6.13.3 for audio output levels.

For decoding to a Target Reference Level of -31 dBFS, the following applies:

If program level data of the form prog_ref_level as of ISO/IEC 14496-3 [53] is present in the bitstream, it shall be used; otherwise a prog_ref_level of -23 dBFS shall be used.

If dynamic range control data of the form dyn_rng_sgn/dyn_rng_ctl as of ISO/IEC 14496-3 [53] is present in the bitstream, it shall be used, and it may be scaled.

If downmixing is performed and dynamic range data of the form dyn_rng_sgn/dyn_rng_ctl as of ISO/IEC 14496-3 [53] is present in the bitstream, those shall be applied without scaling if they indicate negative gains.

#### E-AC-3 and AC-3 Decoding

For decoding to a Target Reference Level of -23 dBFS, the following applies:

If program level data of the form dialnorm as of ETSI 102 366 [33] is present in the bitstream, it shall be used, if dynamic range control data of the form compr as of  ETSI 102 366 [33]is present in the bitstream, it shall be used and it may not be scaled.

It is expected that the broadcaster shall ensure that sufficient headroom and/or dynamic range control values are included in the transmission to prevent any overload when downmixing. For NorDig IRDs that supports a multichannel audio output, the NorDig IRD should be able to decode utilizing a Target Reference Level of -31 dBFS.  See section 6.13.3 for audio output levels.

For decoding to a Target Reference Level of -31 dBFS, the following applies:

Program level data of the form dialnorm as of ETSI 102 366 [33] shall be used.

If dynamic range control data of the form dynrng as of of ETSI 102 366 [33] is present in the bitstream, it shall be used, and it may be scaled.

If downmixing is performed and dynamic range data of the form dyn_rng_sgn/dyn_rng_ctl as of ISO/IEC 14496-3 [53] or of the form dynrng as of of ETSI 102 366 [33] is present in the bitstream, those shall be applied without scaling if they indicate negative gains.

#### Loudness and Dynamic Range Control for AC-4 Decoding

For decoding to a Target Reference Level of -23 dBFS, the following applies:

Program level data of the form dialnorm as of ETSI 103 190-1 [97] shall be used.

If dynamic range control data according to section 4.3.13 of ETSI 103 190-1 [97] is present in the bitstream, it shall be used, and it may not be scaled.

The broadcaster must ensure that sufficient headroom and/or dynamic range control values are included in the transmission to prevent any overload when downmixing.

For NorDig IRDs that supports a multichannel audio output, the NorDig IRD should be able to decode utilizing a Target Reference Level of -31 dBFS.

For decoding to a Target Reference Level of -31 dBFS, the following applies:

Program level data of the form dialnorm as of ETSI 103 190-1 [97] shall be used.

If dynamic range control data according to section 4.3.13 of ETSI 103 190-1 [97] is present in the bitstream, it shall be used and it may be scaled.

#### Summary

A summary of which levelling data and dynamic range control parameters shall be used in conjunction with which stream type target reference level and DRC presentation mode can be found in Table 6.6. below.

| Stream type | Target reference level | Levelling data | DRC Presentation Mode | Dynamic range control parameter |
| --- | --- | --- | --- | --- |
| AC-3 and E-AC-3 | -23 dBFS | dialnorm per ETSI TS 102 366 | n/a | compr per ETSI TS 102 366 |
| AC-3 and E-AC-3 | -31 dBFS | dialnorm per ETSI TS 102 366 | n/a | dynrng per ETSI TS 102 366 |
| HE-AAC | -23 dBFS | prog_ref_level per ISO/IEC 14496-3 | Mode 1 | compression_value per ETSI 101 154 Annex C |
| HE-AAC | -31 dBFS | prog_ref_level per ISO/IEC 14496-3 | Mode 1 | dyn_rng_sgn/dyn_rng_ctl per ISO/IEC 14496-3 |
| HE-AAC | -23 dBFS | prog_ref_level per ISO/IEC 14496-3 | Mode 2 | dyn_rng_sgn/dyn_rng_ctl per ISO/IEC 14496-3 |
| HE-AAC | -31 dBFS | prog_ref_level per ISO/IEC 14496-3 | Mode 2 | dyn_rng_sgn/dyn_rng_ctl per ISO/IEC 14496-3 |
| AC-4 | -23 dBFS | dialnorm per ETSI TS 103 190-1 | n/a | drc_decoder_mode_id = 1 “Flat panel TV” per ETSI TS 103 190-1 |
| AC-4 | -31 dBFS | dialnorm per ETSI TS 103 190-1 | n/a | drc_decoder_mode_id = 0 “Home Theatre” per ETSI TS 103 190-1 |

Table 6.6 Target reference levels.

### MPEG-1 Layer II Audio Input

For MPEG-1 Layer II audio input, the default loudness is assumed to be -23 LUFS and therefore no level adjustments are expected to be made for decoding to a target reference level of -23 dBFS. It is expected that the broadcaster ensures that MPEG-1 Layer II audio stream has an equivalent loudness level of -23 LUFS.

For decoding to a Target Reference Level of -31 dBFS (i.e. digital audio output), the following applies:

An attenuation of 8 dB shall be made to the audio stream.

### Audio Output Levels

In order to match the loudness of audio streams in a downstream AV receiver, PCM streams at a target reference level of -23 dBFS shall be reduced in level by 8dB before being output on S/PDIF or HDMI to that device. PCM streams at a target reference level of -31 dBFS shall not be increased in level prior to output. If an HDMI (or HDMI eARC) receiving device indicates that it supports multichannel PCM audio, then stereo or multichannel PCM should be output at a target reference level of -31 dBFS.

If an HDMI, HDMI ARC (or HDMI eARC) receiving device indicates that it only supports Basic Audio (i.e. two-channel L-PCM), then the output should be levelled to a target reference level of -23 dBFS.

For analogue (stereo) outputs, the target reference level should be levelled to -23 dBFS (as measured on a digital signal).

## NGA Accessibility Services

The NGA capable Nordig HEVC IRD audio decoder shall be capable of supporting the NGA Accessibility services “Audio Description”, “Spoken Subtitles” and “Dialogue Enhancement”, as defined in section 6.1.

### IRD Settings for NGA Accessibility Services

The user shall be able to enable and disable Accessibility services and to change the default setting (see section 16.4 for IRD factory default settings).

The NGA capable NorDig HEVC IRD shall enable a persistent setting of audio preferences for Normal audio and Accessibility services (see section 6.1.1), as defined in section 16.2. (i.e. remain when changing service and when re-starting the IRD).

The NGA capable NorDig HEVC IRD should enable an easy, but temporary, method to select NGA Accessibility Services, e.g., via an audio key or an AD key on the remote control (see also section 13.6), i.e., a temporary selection that does not change the IRD’s stored user preference settings. This temporary selection of Accessibility services shall not remain when changing service, language, stream type or when re-starting the IRD.

If no NGA Accessibility service preselection is available, then the NGA capable NorDig HEVC IRD shall use Normal audio preselection from the same selected language.

### Display of available NGA Accessibility Services

The NGA capable NorDig HEVC IRD should present information to the user if the service has a NGA accessibilty service available, for example in “info banner” after selecting such a service.

If the NGA capable NorDig HEVC IRD offers the option to select an NGA accessibility service preselection as a temporary selection, then the NGA capable NorDig HEVC IRD should indicate which accessibility service type it is (Audio Description, spoken subtitles and/or dialogue enhancement) together with its language.

### NGA Accessibility signalling

All NGA Accessibility Services will be signalled in the broadcast stream by means of the Audio Preselection Descriptor (see section 12.6.12).

The signalling of the NGA Accessibility Services in the Audio Preselection Descriptor shall be mapped to the NGA codec specific values according to ETSI EN 300 468 [13] Table M.1.

### NGA Preselection with Audio Description over headphones output

The NGA capable Nordig HEVC IRD with headphones output should support presenting, if available, the Audio Description NGA Preselection on its own on the headphones interface and at the same time presenting the Normal NGA Preselection on its own on the other main audio outputs (HDMI, S/PDIF etc).

The Audio Description NGA Preselection may be identified by the audio_description element in the Audio Preselection Descriptor, or similarly by the corresponding signalling in the NGA audio stream according to ETSI EN 300 468 [13] Table M.1.

## Loudness Levels and Dynamic Range Control for NGA

### Loudness and Dynamic Range Control for AC-4

For decoding to a Target Reference Level of -23 dBFS, the following applies:

Program level data of the form dialnorm as of ETSI 103 190-1 [97] shall be used.

If dynamic range control data according to section 4.3.13 of ETSI 103 190-1 [97] is present in the bitstream, it shall be used, and it may not be scaled.

The broadcaster must ensure that sufficient headroom and/or dynamic range control values are included in the transmission to prevent any overload when downmixing.

For NorDig IRDs that supports a multichannel audio output, the NorDig IRD should be able to decode utilizing a Target Reference Level of -31 dBFS.

For decoding to a Target Reference Level of -31 dBFS, the following applies:

Program level data of the form dialnorm as of ETSI 103 190-1 [97] shall be used.

If dynamic range control data according to section 4.3.13 of ETSI 103 190-1 [97] is present in the bitstream, it shall be used and it may be scaled.

: Guidelines for NorDig IRD audio selection

Example table when more than one audio stream is received.

This table is given by the property rules of table 6.1. The table covers many possible cases, although not every possible one.

| Supported available input stream types and audio formats as signaled by note 1 in table 60 (examples) | Output on S/PDIF and HDMI (incl. HDMI Audio Return Channel) (Note 4) |  | Output on 2 ch analogue output(s) and/or built in loudspeaker(s) (see note 5) |  |
| --- | --- | --- | --- | --- |
|  | When Stereo mode is selected (default) | When multichannel mode is selected | When Stereo mode is selected (default) | When multichannel mode is selected |
| MPEG-1 lLayer II & AC-3 multichannel | PCM (from MPEG-1 lLayer II) | AC-3 (from AC-3 multichannel) (see note 1) | Decoded MPEG-1 Layer II | Decoded downmixed AC-3 |
| MPEG-1 lLayer II & E-AC-3 multichannel | PCM (from MPEG-1 lLayer II) | E-AC-3 on HDMI (from E-AC-3 multichannel) (see note 2) E-AC-3 transcoded to AC-3 on S/PDIF (from E-AC-3 multichannel) (see note 1) | Decoded MPEG-1 Layer II | Decoded downmixed E-AC-3 |
| MPEG-1 lLayer II & HE-AAC multichannel | PCM (from MPEG-1 lLayer II) | HE-AAC multichannel on HDMI (from HE-AAC multichannel) (see note 3) HE-AAC transcoded to AC-3 or DTS on S/PDIF (from HE-AAC multichannel) | Decoded MPEG-1 Layer II | Decoded downmixed HE-AAC |
| HE-AAC stereo & AC-3 multichannel | PCM (from HE-AAC stereo) | AC-3 (from AC-3 multichannel) (see note 1) | Decoded HE-AAC | Decoded downmixed AC-3 |
| HE-AAC stereo & E-AC-3 multichannel | PCM (from HE-AAC stereo) | E-AC-3 on HDMI (from E-AC-3 multichannel) (see note 2) E-AC-3 transcoded to AC-3 on S/PDIF (from E-AC-3 multichannel) (see note 1) | Decoded HE-AAC | Decoded downmixed E-AC-3 |
| HE-AAC stereo & HE-AAC multichannel | PCM (from HE-AAC stereo) | HE-AAC multichannel on HDMI (from HE-AAC multichannel) (see note 1 + 3) HE-AAC transcoded to AC-3 or DTS on S/PDIF (from HE-AAC multichannel) | Decoded HE-AAC stereo | Decoded downmixed HE-AAC multichannel |
| AC-3 stereo & AC-3 multichannel | PCM (from AC-3 stereo) | AC-3 (from AC-3 multichannel) (see note 1) | Decoded AC-3 stereo | Decoded downmixed AC-3 multichannel |
| HE-AAC multichannel & AC-3 multichannel | PCM (from downmixed HE-AAC) | HE-AAC multichannel on HDMI (from HE-AAC multichannel) (see note 1 + 3) HE-AAC transcoded to AC-3 or DTS on S/PDIF (from HE-AAC multichannel) | Decoded downmixed HE-AAC | Decoded downmixed HE-AAC |
| MPEG-1 lLayer II & AC-3 stereo | PCM (from MPEG-1 lLayer II) | AC-3 (from AC-3 stereo) (see note 1) | Decoded MPEG-1 Layer II | Decoded AC-3 |
| MPEG-1 lLayer II & HE-AAC stereo | PCM (from MPEG-1 lLayer II) | HE-AAC on HDMI (from HE-AAC) (see note 3) HE-AAC transcoded to AC-3 or DTS on S/PDIF (from HE-AAC multichannel) | Decoded MPEG-1 Layer II | Decoded HE-AAC |
| HE-AAC stereo & AC-3 stereo | PCM (from HE-AAC stereo) | HE-AAC on HDMI (from HE-AAC) (see note 3) HE-AAC transcoded to AC-3 or DTS on S/PDIF (from HE-AAC) | Decoded HE-AAC | Decoded HE-AAC |

Table Annex G 1: Example table, examples of possible simulcast situations and signals on analogue and digital outputs dependent on user settings with equal languages and normal audio types. For IDTVs, this shall apply when External Audio System (if supported) is selected as audio output.

Note 1:	The S/PDIF output shall in any case comply with the content of the table above. For HDMI however, the following feature should be implemented:

When an HDMI Sink device indicates in its E-EDID structure that it only supports Basic Audio (i.e. two-channel L-PCM from the original stereo signal or from a stereo down-mix from the multichannel signal), then the HDMI output will provide Basic Audio. This feature would then take precedence over the requirement of AC-3, E-AC-3, HE-AAC multichannel and DTS in the table above whenever the Sink device indicates that only Basic Audio is supported. Observe however that the HDMI output could be different from S/PDIF output, since S/PDIF still has to comply with multichannel format requirements as in the table above.

Note 2:	If an HDMI, HDMI ARC (or HDMI eARC) receiving device indicates that AC-3 decoding is supported, but E-AC-3 decoding is not supported, the IRD shall transcode E-AC-3 streams to AC-3 prior to HDMI transmission.

Note 3:	If an HDMI, HDMI ARC (or HDMI eARC) receiving device indicates that AC-3 or DTS is supported, but HE-AAC decoding is not supported, the IRD shall transcode HE-AAC streams to AC-3 or DTS prior to HDMI transmission.

Note 4: For IDTVs, this shall apply when optional External Audio System is selected as audio output. If built-in loudspeakers are selected for audio output, the minimum requirement is to have PCM output signal in the digital output.

Note 5: When External Audio System is selected as main audio output, IDTVs may optionally mute the TV speakers.

Example table when legacy and NGA audio streams is received by an NGA capable NorDig HEVC IRD

This table is given by the rules defined in section 6.5 and its sub sections. In the table below, HE-AAC is used as an example of legacy codecs. The same logic will apply if other legacy codecs are used in combination with AC-4.

| Supported available input stream types and audio formats as signaled by note 1 in table 61 (examples) | Output on S/PDIF and HDMI (incl. HDMI Audio Return Channel) (Note 4) |  | Output on 2 ch analogue output(s) and/or built in loudspeaker(s) (see note 5) |  |
| --- | --- | --- | --- | --- |
|  | When Stereo mode is selected (default) | When multichannel mode is selected | When Stereo mode is selected (default) | When multichannel mode is selected |
| HE-AAC stereo & AC-4 immersive audio Both streams have the same language | PCM (downmix from AC-4) | AC-4 rendered to 5.1 transcoded to AC-3 on S/PDIF (see note 1) AC-4 decoded to 5.1 or 5.1.2 re-encoded to E-AC-3 on HDMI (see note 2 and 8) AC-4 decoded to PCM with metadad and transported on HDMI (see note 6) | Decode downmix AC-4 | Decode downmix AC-4 |
| HE-AAC stereo & AC-4 immersive audio The two streams have different languages HE-AAC stream match the language preference set in the IRD, the AC-4 stream does not match. | PCM (from HE-AAC stereo) | HE-AAC on HDMI (from HE-AAC) (see note 3) HE-AAC transcoded to AC-3 or DTS on S/PDIF (from HE-AAC) | Decoded HE-AAC | Decoded HE-AAC |

Table Annex G 2: Example table when legacy and NGA audio streams is received by an NGA capable NorDig HEVC IRD.

Note 1:	The S/PDIF output shall in any case comply with the content of the table above. For HDMI however, the following feature should be implemented:

When an HDMI Sink device indicates in its E-EDID structure that it only supports Basic Audio (i.e. two-channel L-PCM from the original stereo signal or from a stereo down-mix from the multichannel signal), then the HDMI output will provide Basic Audio. This feature would then take precedence over the requirement of AC-3, E-AC-3, HE-AAC multichannel and DTS in the table above whenever the Sink device indicates that only Basic Audio is supported. Observe however that the HDMI output could be different from S/PDIF output, since S/PDIF still has to comply with multichannel format requirements as in the table above.

Note 2:	If an HDMI, HDMI ARC (or HDMI eARC) receiving device indicates that AC-3 decoding is supported, but E-AC-3 decoding is not supported, the IRD shall transcode E-AC-3 streams to AC-3 prior to HDMI transmission.

Note 3:	If an HDMI, HDMI ARC (or HDMI eARC) receiving device indicates that AC-3 or DTS is supported, but HE-AAC decoding is not supported, the IRD shall transcode HE-AAC streams to AC-3 or DTS prior to HDMI transmission.

Note 4:	For IDTVs, this shall apply when External Audio System (if supported) is selected as audio output. If built-in loudspeakers are selected for audio output the minimum requirement is to have PCM output signal in the digital output.

Note 5:	When external audio system (if supported) is selected as main audio output, IDTVs may
optionally mute the TV speakers.

Note 6:	If the IRD supports PCM multichannel over HDMI or HDMI eARC (not applicable for HDMI ARC).

Note 7: If content is stereo, output shall be PCM stereo also in case of multichannel mode.

Note 8: If an HDMI, HDMI ARC (or HDMI eARC) receiving device indicates that E-AC-3 decoding is supported, but AC-4 decoding is not supported, the IRD shall transcode AC-4 streams to E-AC-3 prior to HDMI transmission. If an HDMI, HDMI ARC (or HDMI eARC) receiving device indicates that AC-3 decoding is supported, but neither of E-AC-3 or AC-4 decoding is supported, the IRD shall transcode AC-4 streams to AC-3 prior to HDMI transmission.

Note 9: NorDig recognises that HDMI eARC will enable delivery of multichannel audio between suitably equipped iDTVs and A/V decoders. Revisions of this document will consider including recommendations/requirements regarding this output method. For example, MAT [99].

: Loudness levels – Typical IDTV Audio Block diagram

The following diagram represents a typical IDTV IRD audio implementation. The implementation though will vary from manufacturer to manufacturer and chassis to chassis depending on feature level and cost. For technical reasons, such as processing power, memory and general architecture, it is common for the functionality offered by some of the operational blocks shown to be mutually exclusive. e.g. only one decode process at a time may be supported.

The output levels shown are those suggested by recent publications by EBU (R128 [72] and Tech 3344 [73]). It is expected that the industry will move towards following the levels shown here, though it must be expected that changes to architectures required to support these levels depend on IC and chassis design lead times so must be still considered as guidance only.

Figure Annex H 1: Loudness levels – Typical IDTV Audio Block diagram.

Note: Informative. Diagram is based on diagram in UK D-Book 7A [65], and shared by kind permission of the DTG. This shall be interpreted as informative only.

The outputs shown are labeled ‘TV Output Mode’ and ‘AV Amplifier Output Mode’; these terms are intended to denote output to TV speakers and external AV Amplifier/System respectively.

Due to system architecture limitations, it is common for PCM outputs to external AV systems to follow the same levels as outputs to internal TV speakers. Architectures often mute the TV speakers in order to process audio in formats specifically suitable for external AV systems (e.g. -31 LUFS dialogue level with increased dymanic range).

The general concept of loudness levels at different outputs are:

– Programme reference level at analogue outputs should be -23 dBFS (as measured on a digital signal).

– Programme reference level at digital outputs (S/PDIF and HDMI) should be -31 dBFS.

In this way, users with lower quality of speaker systems will experience a dynamic range which is appropriate for these systems. Users with (usually external) higher quality systems will be able to experience larger (original) dynamic range.

As guidance, these standards and documents are to be considered:

- EBU Recommendation R 128, Loudness normalisation and permitted maximum level of audio levels [73]

- EBU Technical Recommendation R 68-2000, Alignment level in digital audio production equipment and in digital audio recorders [74].

- EBU Tech 3341, Loudness Metering: “EBU mode” metering to supplement loudness normalisation in accordance with EBU R 128 [72].

- EBU Tech 3344, Loudness normalisation in distribution

- ITU Recommendation ITU-R BS.1770-2 (March 2011), Algorithms to measure audio programme loudness and true-peak audio level [76]

- ITU Recommendation ITU-R BS.1771, Requirements for loudness and true-peak indicating meters [77].

: Examples of Signalling to be used for audio property

Example table of priority between audio signalling alternatives related to audio property.

This table is given by the property rules of section 6.5.4 and shows which signalling (descriptor or stream type) shall be used for each audio property of the audio stream during the selection of audio stream. The table covers a number of relevant cases, although not every possible one is listed. For services with multiple audio streams, the NorDig IRD has to parse the signalling inside the PMT for the audio streams to select the appropriate audio stream(s) to decode in accordance with the IRD user preference settings. After the selection of audio stream and during the actual audio decoding, the IRD shall primary rely on signalling inside the PES stream for the audio format. (Example for an audio that dynamically changes between stereo and multichannel, the audio format in the PMT is signalized as multichannel while the signalling inside the PES stream signalize stereo).

| codec |  |  |  | Signalling in incoming stream |  |  |  |  | IRD, Signalling to be used during the selection of audio stream for each audio property. |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MPEG -1 LayerII | HE-AAC | AC-3 | E-AC-3 | PMT stream type | Supplementary audio descriptor (SAD) |  | ##### [1] [1] AAC/ AC-3/ E-AC-3 descriptor | ##### [2] [2] ISO639 descriptor (lang, type) | Lang | Type | Format | stream type |
|  |  |  |  |  | mix type, edit class | ISO639 lang |  |  |  |  |  |  |
| X | X |  |  | X |  |  |  |  |  | (“normal”) | (stereo) | stream type |
| X | X |  |  | X |  |  |  | X | [2] | [2] | (stereo) | stream type |
|  | X | X | X | X |  |  | X |  |  | [1] | [1] | [1] |
|  | X | X | X | X |  |  | X | X | [2] | [1] | [1] | [1] |
|  | X | X | X | X | X |  | X | X | [2] | SAD | [1] | [1] |
| X | X |  |  | X | X |  |  | X | [2] | SAD | (stereo) | stream type |
|  | X | X | X | X | X | X | X | X | SAD | SAD | [1] | [1] |
|  | X | X | X | X | X | X | X |  | SAD | SAD | [1] | [1] |
| X (codec): applicable case for these audio codecs, X (signalling); signalling/descriptor included in incoming MPEG-2 TS for the audio stream SAD: Supplementary Audio Descriptor (“normal”): Audio Type is not includedin signalling, IRD shall then assume that the audio stream is of a Normal type. (Stereo): Audio Format is not includedin signalling, IRD shall then assume that the audio stream has stereo format. |  |  |  |  |  |  |  |  |  |  |  |  |

Table Annex I 1: Example table, examples of signalling alternatives for the audio property and priority the IRD shall have between the incoming signalling.

The table below provides examples of the descriptors that need to be evaluated for prioritisation schema selection according to section 6.5.2 by NGA capable NorDig HEVC IRDs if the AC-4 NGA codec is used together with a legacy codec.

| codec |  |  |  |  | Signalling in incoming stream |  |  |  |  |  | IRD, Signalling to be used during the selection of NGA audio stream based on language and codec. |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MPEG -1 LayerII | HE-AAC | AC-3 | E-AC-3 | AC-4 | PMT stream type | Supplementary audio descriptor (SAD) |  | ##### [1] [1] AAC/ AC-3/ E-AC-3/ AC-4 descriptor | ##### [2] [2] ISO639 descriptor (lang, type) | ##### [2] [3] Audio Pre-selection descriptor | Language (IRDs that do not support the audio_ preselection _descriptor) | Language (IRDs that do support the audio_ preselection _descriptor) |
|  |  |  |  |  |  | mix type, edit class | ISO639 lang |  |  |  |  |  |
| X | X |  |  | X | X |  |  | X |  |  | AC-4 | AC-4 |
| X | X |  |  | X | X |  |  | X | X |  | [2] | [2] |
|  | X | X | X | X | X |  |  | X |  |  | AC-4 | AC-4 |
|  | X | X | X | X | X |  |  | X | X |  | [2] | [2] |
|  | X | X | X | X | X | X |  | X | X |  | [2] | [2] |
| X | X |  |  | X | X | X |  | X | X |  | [2] | [2] |
|  | X | X | X | X | X | X | X | X | X |  | SAD (lang) | SAD (lang) |
|  | X | X | X | X | X | X | X | X |  |  | SAD (lang) | SAD (lang) |
| X | X |  |  | X | X |  |  | X |  | X | AC-4 | [3] |
| X | X |  |  | X | X |  |  | X | X | X | [2] | [3] |
|  | X | X | X | X | X |  |  | X |  | X | AC-4 | [3] |
|  | X | X | X | X | X |  |  | X | X | X | [2] | [3] |
|  | X | X | X | X | X | X |  | X | X | X | [2] | [3] |
| X | X |  |  | X | X | X |  | X | X | X | [2] | [3] |
|  | X | X | X | X | X | X | X | X | X | X | SAD | [3] |
|  | X | X | X | X | X | X | X | X |  | X | SAD | [3] |
| X (codec): applicable case for these audio codecs, X (signalling); signalling/descriptor included in incoming MPEG-2 TS for the audio stream SAD (lang): Supplementary Audio Descriptor, use language property only AC-4: If no language information is available, the AC-4 codec shall be prioritized over legacy codecs. |  |  |  |  |  |  |  |  |  |  |  |  |

Table Annex I 2: Example table, examples of signalling alternatives the IRD needs to consider for prioritisation schema selection of NGA capable NorDig HEVC IRDs.

: Flowchart describing Audio Prioritisation for NGA capable NorDig HEVC IRD

- Introduction

Find below a flowchart that describes the PID selection as detailed in section 6.5.2 Audio Prioritisation for NGA capable NorDig HEVC IRD.

PID Selection for NGA capable NorDig HEVC IRD:

Figure Annex F 1: PID Selection for NGA capable NorDig HEVC IRD.